This program was developed as an alternative to IBM'S "ADVANCED DIAGNOSTICS"

for the hard disk. Although you get more than a hard disk diagnostic with

IBM'S diagnostics, you pay $295 for the package. All I ask in return for

the FREE use of this diagnostic is that you do not change the copyright

nor charge for distribution of this program.

Any problems or suggestions should be forwarded to:

JIM BRACKING

967 PINEWOOD DRIVE

SAN JOSE, CA 95129

(408) 725-0628

PROGRAM FEATURES

----------------

This program is menu driven and in most cases should be self-explanatory. It

run on a IBM AT, PC or compatible computer.

The diagnostics are divided into two categories, destructive and non destructive

tests. These tests can be useful in installing a new disk, finding and

flagging bad data areas or testing the general working condition of your hard

disk and controller.

NON-DESTRUCTIVE TESTS

---------------------

In an IBM compatible hard disk bios the last cylinder is reserved for running

diagnostic tests on. This program uses that cylinder for diagnostic writes.

F1.......WRITE/READ/VERIFY TEST     This test verify's that read and write work

by writing one sector then reading it and

comparing it to the value it wrote.

F2.......SEEK TEST		    This test first verify's every cylinder

then exercises the read/write heads by

performing a crossing seek pattern.

F3.......HEAD SELECT		    This test selects one head on every

cylinder.

F4.......ECC TEST		    Most controllers have some form of onboard

error correction that writes four bytes of

ECC data with every data record. When the

data is read these four bytes of ECC are

used to determine the validity of the data

and make corrections for any incorrect

bits depending on the controller. This test

does a read long, modify's four bits and

write's the record back with a write long.

The next read should return the corrected

data.

F5.......RUN ALL TESTS		    This runs tests 1 through 4 and allows the

the user to loop up the tests.

F6.......PREPARE FOR RELOCATION     Moves the read/write heads to the landing

zone. The computer should be shut off after

executing this or the heads will move back

onto the data surface.

F7.......DEFECT SCAN		    Reads every sector the specified number of

times and reports the errors.

DESTRUCTIVE TESTS

-----------------

F8.......SURFACE ANALYSIS	    This test writes the worst case data

pattern (6DB6) on every sector and reads

it back three times. If any errors are

found they are reported and the track if

flagged.

F9.......FORMAT DISK		    This performs a physical format of the hard

disk and should not be confused with DOS

format. If you are formatting a disk for

the first time then you should specify "y"

to clear the defects, otherwise a quick

scan will be made to identify any flagged

tracks. After the format is done the tracks

that were flagged as bad will be flagged

again.

F10......FLAG DEFECTS		    Its a sad but true fact that most hard

come from the factory with some known

defects and some will be developed over

time. This routine is used to flag those

defects to prevent any data from being

written there.

A-F10....FORMAT TRACK		    The same as the format drive except it

only formats one track.

MISCELLANEOUS FUNCTIONS

-----------------------

A-F9.....TOGGLE PRINTER ON/OFF	    The printer can be used to log any errors.

A-F10....EXIT			    Return to DOS

SOURCE DOCUMENTATION

The hard disk diagnostic is made up of many smaller programs that perform a

specific function. Below is a brief description of each program. All of the

BAT files and INCLUDE statements in the programs assume that you have the

the following sub directories setup.

SCRMGR   -	SCREEN MAMAGER SOURCE CODE

HD	     -	HARD DISK DIAGNOSTIC SOURCE CODE

LST      -	ASSEMBLY LISTINGS

OBJ      -	OBJECT CODE

SCREEN MANAGER DATA

SCRMGR.ASM	    Does all the screen management and keyboard functions.

SCRNWORK.ASM	    Equates and structures

HD SUPPORTING FILES

HD.DOC		    Documentation

HDLINK.BAT	    HD link bat file

HDLINK.DAT	    HD link data file (input to hdlink)

ASM.BAT 	    Assembler bat file

HD SOURCE CODE

HD.MAC		    HD macros

HDSUB.ASM	    HD subroutines external declarations

HDDATA.ASM	    External declares

HDEQU.ASM	    Common equates

HDMAIN.ASM	    Main module

HDALL.ASM	    Run all tests

HDECC.ASM	    Ecc test

HDFMT.ASM	    Format program

HDFSEC.ASM	    Flag bad sectors

HDHS.ASM	    Head select test

HDMSG.ASM	    Common messages

HDRW.ASM	    Raed write test

HDSCAN.ASM	    Scan test

HDSEEK.ASM	    Seek test

HDSHIP.ASM	    Prtepare for relocation

HDSURF.ASM	    Surface analysis

HDTFMT.ASM	    Format a specific track