325D/333D BIOS Patch for QEMM VIDRAM utility

This patch, installed as a device driver in CONFIG.SYS, fixes an anomaly

in the system BIOS that effectively disables Quarterdeck's VIDRAM utility

(which runs under QEMM and extends conventional memory by 96K by using

graphics memory).

Installation is very easy.  A "DEVICE=QEMMFIX.SYS" line should be added

to CONFIG.SYS _before_ the line which installs QEMM.  For example,

CONFIG.SYS may look like the following:

DEVICE=\utils\qemm\QEMM386.SYS RAM AdapterRAM=CC00-CE00

SHELL=\dos\COMMAND.COM C:\DOS /e:1000 /p

BUFFERS=5

FILES=30

The patch should be installed so CONFIG.SYS now looks like this:

DEVICE=QEMMFIX.SYS

DEVICE=\utils\qemm\QEMM386.SYS RAM AdapterRAM=CC00-CE00

SHELL=\dos\COMMAND.COM C:\DOS /e:1000 /p

BUFFERS=5

FILES=30

Please report any problems encountered using this patch to Dell Computer

Corp. Technical Support.