Chapter 15 - Subroutines                                      157

________________________

What if both strings are in memory but we don't know which

segments they are in? In that case, the calling subroutine needs

to pass both the segment and the offset for both the from_string

and the to_string. Let's do this in C.

move_string ( from_string, to_string ) ;

In C, this will pass the addresses of the arrays, not the arrays

themselves. C, once again, pushes these variables in right to

left order. If the compiler is set up to move both the segment

and the offset, then the generated C code will be:

mov  ax, seg to_string

push ax

mov  ax, offset to_string

push ax

mov  ax, seg from_string

push ax

mov  ax, offset to_string

push ax

call move_string

add  sp, 8                    ; 4 pushes = 8 bytes

On the 8086, the low two bytes are ALWAYS the offset and the high

two bytes are ALWAYS the segment. Remember, SEG gets the segment

starting address of the named variable.

We will do the subroutine as a near routine. After setting up BP,

we will have:

to_string segment        bp + 10

to_string offset         bp + 8

from_string segment      bp + 6

from_string offset       bp + 4

old IP                   bp + 2

bp   ->   old bp                   bp + 0

In the subroutine we will have to move the segment and the offset

for each pointer. Luckily for us, there are two 8086 instructions

for doing this:

LDS (load DS) loads the first two bytes into the named

register and the next two bytes into DS.

LES (load ES) loads the first two bytes into the named

register and the next two bytes into ES.

If we write:

LES  di, [bp + 8]

then the 8086 will load the first two bytes (bp+8 and bp+9) into

DI and the next two bytes (bp+10 and bp+11) into ES. This loads

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                        158

______________________

the offset and the segment in the same instruction. If we write:

LDS  si, [bp + 4]

the 8086 will load the first two bytes (bp+4 and bp+5) into SI

and the next two bytes (bp+6 and bp+7) into ES, loading both the

offset and segment in one instruction.

LDS and LES allow you to load the offset into any full arithmetic

register (AX, BX, CX, DX, SI, DI, BP or SP) but you can't use AX,

CX or DX as addressing registers, so it only makes sense to load

BX, SI, DI and BP for use as pointers. The two strings will now

be addressed by DS:SI and ES:DI. DS is SI's normal segment, so we

don't need to do anything, but we need a segment override for

ES:DI. Here is the code for a C subroutine:

A C string ends with a 0 byte, that is with a byte having the

numeric value 0. It can be any length, but we need to test each

byte to find out if it is 0.

Notice that we are using (and changing) both DS and ES this time,

so we have to PUSH and POP them, just like other registers.

; - - - - -

move_string   proc near

FROM_POINTER  EQU  [bp+4]

TO_POINTER    EQU  [bp+8]

push bp

mov  bp, sp

PUSHREGS  ds, es, ax, si, di

lds  si, FROM_POINTER

les  di, TO_POINTER

move_loop

mov  al, [si]            ; source to al

mov  es:[di], al         ; al to destination

inc  si                  ; pointers to next byte

inc  di

and  al, al              ; is al 0?

jnz  move_loop

POPREGS  ds, es, ax, si, di

pop  bp

ret                      ; calling routine pops variables

move_string  endp

; - - - - -

Basically, the only difference between this and the Pascal move

is that (1) here we check for 0 and there we had an actual count,

and (2) in Pascal we used "ret (4)" and here the calling routine

does the adjustment.

Chapter 15 - Subroutines                                      159

________________________

DRAWING THE STACK

Each time that we have used the stack we have drawn a picture of

where everything is on the stack. In case you think that this is

some trivial little learning technique, I'm telling you now that

at the assembler level, if you are passing variables on the stack

and you don't make a diagram on a piece of paper of where

everything is, you are guaranteed to consistently reference

things by the wrong address. ALWAYS make a paper diagram that

includes BP, ALWAYS use EQU statements, and you'll avoid a lot of

mistakes.

It is now time to get more complex. Everything that follows is

more advanced, so it requires some programming experience.

Everything that follows is about C modules and recursion. If this

gets too complicated or obscure, just skip to the summary at the

end of the chapter.

I am going to give you a sample subroutine in C and then show you

where all the variables go. The following is a complete C file:

/* a complete C file  - - - - - - - - - - - */

int            A, B ;

static int     C, D ;

extern int     E, F ;

sample_routine ( G, H )

int G, *H ;

{

int            I, J ;

static int     K, L ;

A = I ;   /* transfer the words around */

B = G ;

J = E ;

F = C ;

D = K ;

*H = I ;

return ;

}

/* end of C file   - - - - - - - - - - - - - */

The only thing this routine does is tranfer the words around.

This is so you can see where things are stored and how they are

accessed. If you don't know C, the only thing you really need to

know here is that '*H' means that 'H' is the ADDRESS of an

integer, not the integer itself, while '*H' is the actual integer

that is being addressed.

For those of you who DO know C, you need to know exactly what

extern, static and static mean. extern means that it is in an

The PC Assembler Tutor                                        160

______________________

external file. The first 'static' (which is outside of any

subroutine) means that the data is INTERNAL to the file; it won't

be shared with other files. Variables A and B, which don't have

the word 'static' are GLOBAL and will be shared with other files.

The other 'static' (inside the subroutine) means that its address

is fixed in memory while I and J, which are not 'static' have

their addresses generated every time you call the program. Say

what? Yes, that's correct, every time you call the program they

can be in a different place. That's what allows recursion, and

you'll see how that is implemented in a second.

Here is the equivalent program in assembler:

; - - - - - START DATA BELOW THIS LINE

PUBLIC A, B

EXTRN  E:WORD, F:WORD

A   dw   ?

B   dw   ?

C   dw   ?

D   dw   ?

K   dw   ?

L   dw   ?

; - - - - - END DATA ABOVE THIS LINE

; - - - - - START SUBROUTINE BELOW THIS LINE

sample_routine proc near

ADDRESS_OF_H EQU [bp + 6]

G EQU  [bp + 4]

I EQU  [bp - 2]

J EQU  [bp - 4]

push bp

mov  bp, sp         ; set up bp

sub  sp, 4          ; save space for I and J

PUSHREGS ax, si

mov  ax, I          ; A = I

mov  A, ax

mov  ax, G          ; B = G

mov  B, ax

mov  ax, E          ; J = E

mov  J, ax

mov  ax, C          ; F = C

mov  F, ax

mov  ax, K          ; D = K

mov  D, ax

mov  ax, I          ; *H = I

mov  si, ADDRESS_OF_H

mov  [si], ax

Chapter 15 - Subroutines                                      161

________________________

POPREGS  ax, si

mov  sp, bp         ; readjust sp

pop  bp

ret            ; a C routine so calling routine pops

sample_routine endp

; - - - - - END SUBROUTINE ABOVE THIS LINE

This time the setup is a little longer. It is:

push bp

mov  bp, sp

sub  sp, 4

PUSHREGS ax, si

I and J are not in the data segment, so we need to make space for

them somewhere, and we do it on the stack. We subtract 4 from sp

to provide ourselves with 4 bytes, two for I and two for J. There

are two EQU statements that say exactly where I and J will be. We

also push AX and SI because we will use them. After this setup,

we have:

address of H        bp + 6

G                   bp + 4

old IP              bp + 2

bp ->     old bp              bp + 0

I                   bp - 2

J                   bp - 4

old ax              bp - 6

sp ->     old si              bp - 8

We have created space for some variables below bp. This is

temporary and will disappear when we leave the subroutine. We do

our dummy calculations,{1} and then do the end adjustment. There

are two ways to readjust sp before returning:

add  sp, 4

just adds the amount that we subtracted, so it winds up in the

same place. But the place it winds up is ALWAYS the address of

the old BP, and bp is now pointing to that, so

mov  sp, bp

does exactly the same thing.

ARE YOU REALLY DEMENTED?

Yes, for those of you who are truly masochistic, we have -

ta-dah! - the Towers of Hanoi in assembler language.

____________________

1. And that's 'dummy' in more ways than one.

The PC Assembler Tutor                                        162

______________________

The Towers of Hanoi is a game with three posts and a number of

disks which have incrementally smaller diameters. At the

beginning of the game, all the disks are on post one, ordered by

size with the smallest on top and the largest on the bottom. For

five disks it looks like this:

(1)              (2)              (3)

X                X                X

X                X                X

XXXXX              X                X

XXXXXXX             X                X

XXXXXXXXX            X                X

XXXXXXXXXXX           X                X

XXXXXXXXXXXXX          X                X

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

The object is to wind up with all the disks on post three in the

same order. The game has only one rule: You may never put a

larger disk on top of a smaller disk.

The general solution to this problem is that if you have posts A,

B and C with N disks on post A and you want to move them to post

C, first move (N-1) disks to post B, move the bottom disk from

post A to post C, then move (N-1) disks from post B to post C.

This works out to be the optimal recursive solution. Try it out

with N = 1, N = 2 and N = 3. N = 4 is already complicated.

I won't discuss the game further or how to get the solution. If

you don't know about it, you can look it up in either Doug

Cooper's "Oh! Pascal" or Robert Kruse's "Data Structures and

Program Design".

The fact that I need to resort to such an unususl example to

illustrate recursion underlines the fundamental rule of

recursion, which is:

YOU SELDOM NEED TO USE RECURSION, BUT WHEN YOU NEED IT YOU

REALLY REALLY NEED IT.

First, here is the solution in C:

/* the C solution - - - - - - - - - - - - - - - */

#define MAXIMUM 9

main ()

{

int  count ;

while (1)

{

printf ("Enter a number less than 10.\n" ) ;

scanf ( "%d", &count ) ;

if ( count > MAXIMUM )

continue ;

Chapter 15 - Subroutines                                      163

________________________

towers_of_hanoi ( count, 1, 3, 2 ) ;

}

}

/* - - - - - - - - - - - - - - - - - - - - - - - - - */

towers_of_hanoi ( count, from, to, via )

int  count, from, to, via ;

{

if (count <= 0)

return ;

count-- ;

towers_of_hanoi ( count, from, via, to ) ;

printf ("Move a disk from %1d to %1d.\n" , from, to ) ;

towers_of_hanoi ( count, via, to, from ) ;

return ;

}

Notice that by letting a routine call itself, we have reduced it

to just a few lines. And this is a problem that looks very

complex. Here comes the assembler equivalent of the C code:

; + + + + + + + + + + + + + + + START DATA BELOW THIS LINE

MAX_COUNT  EQU   9

enter_message        db  "Enter a number less than 10", 0

make_a_move_message  db  "Move a disk from "

from_byte            db  "X to "

to_byte              db  "X.", 0

; + + + + + + + + + + + + + + + END DATA ABOVE THIS LINE

; + + + + + + + + + + + + + + + START CODE BELOW THIS LINE

outer_loop:

lea   ax, enter_message        ; count message

call  print_string

call  get_unsigned_byte        ; count in al

sub   ah, ah                   ; zero ah for 'push ax'

cmp   al, MAX_COUNT            ; too big?

ja    outer_loop

; move from post 1 to post 3 via post 2

mov   bx, 2                    ; post 2 = 'via'

push  bx

mov   bx, 3                    ; post 3 = 'to'

push  bx

mov   bx, 1                    ; post 1 = 'from'

push  bx

push  ax                       ; al = count, ah = 0

call  towers_of_hanoi

add   sp, 8                    ; adjust the stack

jmp   outer_loop

; + + + + + + + + + + + + + + + END CODE ABOVE THIS LINE

; + + + + + + + + + + + + START SUBROUTINES BELOW THIS LINE

The PC Assembler Tutor                                        164

______________________

towers_of_hanoi proc near

VIA   EQU  [bp + 10]

TO    EQU  [bp + 8]

FROM  EQU  [bp + 6]

COUNT EQU  [bp + 4]

push  bp                  ; set up bp

mov   bp, sp

push  ax

cmp   BYTE PTR COUNT, 0   ; if no disks, we are done

jbe   exit

dec   BYTE PTR COUNT      ; 1 less disk to move

; first half

push  TO

push  VIA

push  FROM

push  COUNT

call  towers_of_hanoi

add   sp, 8               ; adjust the stack

; print the message

mov   al, FROM            ; get 'from' number

add   al, '0'             ; convert to ascii

mov   from_byte, al       ; put into message

mov   al, TO              ; get 'to' number

add   al, '0'             ; convert to ascii

mov   to_byte, al         ; put into message

lea   ax, make_a_move_message

call  print_string

; second half

push  FROM

push  TO

push  VIA

push  COUNT

call  towers_of_hanoi

add   sp, 8               ; adjust the stack

exit:

pop   ax

pop   bp

ret

towers_of_hanoi endp

; + + + + + + + + + + + + END SUBROUTINES ABOVE THIS LINE

The main routine checks that the number you enter is not too big

and then calls 'towers_of_hanoi'. After setting up BP, the stack

looks like this:

VIA post       bp + 10

TO post        bp + 8

Chapter 15 - Subroutines                                      165

________________________

FROM post      bp + 6

count          bp + 4

old IP         bp + 2

bp   ->   old bp         bp + 0

We make some EQU statements to define where each variable is and

set up BP. We are using only one register, so we have a single

PUSH instead of using PUSHREGS. We then check to see if the count

is 0. If it is we are done. If not, we decrement the count by 1

which gives us 'count - 1' and divide the problem into three

parts. Part 1 calls 'towers_of_hanoi' and moves 'count - 1' disks

from 'from' to 'via'. Part 2 prints a message of where to move

the bottom disk. It converts the post numbers into ascii and

inserts them in the string where the 'X's are.{2}  Part 3 calls

towers_of_hanoi again, this time moving the 'count - 1' disks

from 'via' to 'to'. Assemble and link it with ASMHELP. When you

run it, start with just 1 disk, then 2 then 3 etc. If you use the

maximum number (9), it will print 511 lines. For N disks, you

need (2 ** N) - 1 moves, so this gets very big very fast.

If you still feel no need to draw a picture of the stack or to

use EQU statements, why don't you try writing this subroutine

using the actual pointer values, i.e:

push [bp + 6]

and so on. See how long it takes and see how easy it is to read

once it's done. Can you get it to work correctly?

By the way, how large does the stack get? Well, if you raised the

limit to 30 disks and entered 30, It would take your computer

about a year, running 24 hours a day, to complete the solution

(about 1 billion moves). The maximum stack size would be

((disks+1) * stack_use_per_disk). The extra 1 is for the calling

routine. That is 31 * 14 bytes (including pushing IP, BP, and

AX), or a mere 434 bytes.

____________________

2 It uses the fact that for a data declaration, a variable

name has the address of the first piece of data on that line.

The PC Assembler Tutor                                        166

______________________

SUMMARY

To call a procedure you use CALL with the procedure name:

call subroutine1

Procedures may be either FAR or NEAR. If there is a mismatch

between which type the assembler thinks it is and which type it

really is, there will be an error, either at the assembler level

for internal subroutines or at the linker level for external

subroutines. You may override the default type for procedures

with PTR:

call NEAR PTR subroutine5

call FAR PTR subroutine6

This is normally needed if the procedure comes after the call in

the file and is not the default type.

To allow other files to use a procedure you declare it PUBLIC

within its own segment.

PUBLIC  subroutine1

To use a PUBLIC procedure from another file you declare it EXTRN,

stating which type it is:

EXTRN  subroutine1:NEAR, subroutine2:FAR

You should make this declaration in the code segment and you

should make it before the procedure is referenced.

A procedure is defined by giving a name followed by the word

'proc' (procedure) followed by either FAR or NEAR

subroutine1 proc near

subroutine2 proc far

and a procedure is ended by giving the procedure name followed by

'endp' (end of procedure):

subroutine2 endp

One procedure must be ended before another is begun.

Data is normally passed from one procedure to another on the

stack:

push variable1

push variable2

push variable3

call subroutine1

If this is done, then the called procedure references this data

by using BP, the base pointer. The standard setup code is:

Chapter 15 - Subroutines                                      167

________________________

push bp             ; save old bp

mov  bp, sp         ; set bp to current top of stack

What the stack looks like at this point depends on whether it is

a near or a far procedure. For a near procedure, we have:

variable1           bp + 8

variable2           bp + 6

variable3           bp + 4

old IP              bp + 2

bp   ->   old BP              bp + 0

For a far procedure we have:

variable1           bp + 10

variable2           bp + 8

variable3           bp + 6

old CS              bp + 4

old IP              bp + 2

bp   ->   old BP              bp + 0

Although it is theoretically possible to access these variables

by their pointer definition:

mov  ax, [bp + 10]

It is much less error prone and much clearer to use EQU

statements:

VAR1 EQU [bp + 10]

mov  ax, VAR1

If you are writing a recursive procedure and you need temporary

variables, you can allot space on the stack for these variables:

sub  sp, 6     ; room for 6 bytes of temp. variables

This should be done before any other pushes are done:

push bp

mov  bp, sp

sub sp, 6

PUSHREGS ax, bx, cx, dx

These variables should also be named with EQU statements, and as

always, you should draw a picture of what is on the stack:

variable1           bp + 10

variable2           bp + 8

variable3           bp + 6

old CS              bp + 4

old IP              bp + 2

bp   ->   old BP              bp + 0

VAR4                bp - 2

The PC Assembler Tutor                                        168

______________________

VAR5                bp - 4

VAR6                bp - 6

VAR4 EQU  [bp - 2]

VAR5 EQU  [bp - 4]

VAR6 EQU  [bp - 6]

Data which is passed to the procedure is at a positive offset to

BP while data that is created in the procedure is at a negative

offset to BP.

If you have created a data area for yourself on the stack, then

you must eliminate it before leaving the procedure. There are two

ways of doing this. One way is to add back what you have

subtracted:

POPREGS ax, bx, cx, dx

add  sp, 6

pop  bp

ret

The other way is to give SP the value in BP because this is the

place where SP will wind up anyway:

POPREGS ax, bx, cx, dx

mov  sp, bp

pop  bp

ret

Use whichever one is clearer to you.

When you return from a procedure that has had data passed to it,

the data must be taken off the stack. There are two ways of doing

this. The C standard is that it is the calling program's

responsibility to do this:

push variable1

push variable2

push variable3

call subroutine1

add  sp, 6          ; 3 pushes = 6 bytes

The Pascal standard is that you take them off the stack on the

return. There is a special return instruction for that:

ret (6)             ; 3 pushes = 6 bytes

DATA

Data can be made available to other files and data can be

accessed from other files. To make data available, declare it

PUBLIC:

PUBLIC variable1, variable2, variable3

Chapter 15 - Subroutines                                      169

________________________

To access PUBLIC data from other files, use an EXTRN statement

which includes the data type:

EXTRN  variable7:BYTE, variable8:WORD, variable9:DWORD

EXTRN  variable10:QWORD, variable11:TBYTE

This EXTRN statement must be in a segment which has the same

ASSUME segment register as will be used when accessing the data.

Normally this is DS, but it can be something else. For instance,

if the above EXTRN statements were in MORESTUFF and you have:

ASSUME es:MORESTUFF

then every time you access variable8:

mov  dx, variable8

the assembler will code an ES segment override.

PUSHREGS and POPREGS

When writing a subroutine, you should always save any registers

that you use by pushing them.

push ax

push bx

push cx

They are then popped before returning

pop  cx

pop  bx

pop  ax

In order to save a lot of lines of code, there are two macros,

PUSHREGS and POPREGS. They are designed so you may use a word

processor to copy them. PUSHREGS pushes in left to right order

and POPREGS pops in right to left order:

PUSHREGS ax, bx, cx, dx

POPREGS ax, bx, cx, dx

is a matched pair.

LES and LDS

LDS (load DS) loads the first two bytes into the named register

and the next two bytes into DS. LES (load ES) loads the first two

bytes into the named register and the next two bytes into ES.

les  si, [bp+6]

lds  di, [bp+10]