===================================================

| PibTerm:  Communications and Terminal Emulation |

===================================================

For the IBM PC and compatibles

Version 3.0

Philip R. Burns

October 7, 1985

Documentation for PibTerm version 3.0               07 Oct 85   Page 1

Table of Contents

1  Table of Contents

4  Introduction

4  Getting a copy of the source

5  PibTerm features

6  Starting PibTerm

6     Setting the environment variable PIBTERM

7  Using PibTerm

8     Reading the configuration files

8     The phone directory

8     Initializing the modem

8     Terminal emulation mode

9     Using the PibTerm menus

9     Three important PibTerm commands

9  PibTerm Commands

9  Alt-A:  Edit line on screen

10     Keys used in editing

10  Alt-B:  Send Break

11  Alt-C:  Clear screen

11  Alt-D:  Dial a number

11     Carrier detect line and dialing

11     Dialing directory format

11     Using the dialing directory

12        Dialing a specific directory entry

13        Moving around the dialing directory

13        Revising a dialing entry

13        Dialing prefixes

14        Other dialing commands

14     Automatic baud rate adjustment

14  Alt-E:  Toggling local echo

15  Alt-F:  File manipulation

15     Changing the active directory

15     Copying a file

15     Displaying the current directory

15     Erasing a file

16     Find free space on drive

16     Changing the logged drive

16     Viewing a file

16     Quit file manipulation

16  Alt-G:  Enter Gossip mode

16  Alt-H:  Hang up the phone

17  Alt-I:  Produce command list

17  Alt-J:  Jump to DOS

17  Alt-K:  Set input keys

18     Reading key definition from a file

18     Entering key definitions at keyboard

18     Writing key definitions to a file

18     Quiting key definition mode

18     Contents of key definition strings

21  Alt-L:  Log session to printer

21  Alt-M:  Toggle Mute Mode

21  Alt-N:  New communications parameters

22  Alt-O:  Output session to disk

22  Alt-P:  Set parameters

23     Format of configuration file

Documentation for PibTerm version 3.0               07 Oct 85   Page 2

Table of Contents

23     PibTerm parameter names

27     Sample configuration file

28  Alt-Q:  Redial last number dialed

29  Alt-R:  Receive file from remote

29     Ascii protocol

30     Xmodem protocol

30     Modem7 protocol

30     Ymodem protocol

30     Telink protocol

31     Kermit protocol

32     Mahoney BBS autodownload protocol

33     CompuServe B protocol

33     Downloads overwrite existing files

33     Cancelling a download

33     Download buffer size

33     Processing of CANs in Xmodem

34  Send file to remote system

34     Cancelling an upload

34     Ascii file uploads

34     Uploads with CIS B protocol

35  Alt-T:  Get translate table

36  Alt-U:  Screen dump to file

36  Alt-V:  View Back Scroll Buffer

36  Alt-W:  Wait for call

36  Alt-X:  Exit PibTerm

37  Alt-Y:  Display elapsed time

37  Alt-Z:  Get area code

37  Shift-Tab:  Toggle adding linefeeds

37  Notes on terminal emulation

37  VT52 mode

38  VT100 mode

39  ANSI/BBS mode

39  Using Script File

39     Executing a script file

40  Script file commands

42  General syntax of script commands

42  Detailed description of script commands

42     Addlf -- add line feeds

42     Break -- send a break

43     Capture -- capture session to disk

43     Clear -- clear screen

43     Delay -- delay execution for specified interval

43     Dial --- dial a number

44     Dos -- execute DOS command

44     Echo --- toggle local echo

44     Else -- part of IF statement

44     Endif --- ends an IF block

44     EndWhile -- terminate WHILE block

44     Exit -- terminate script execution

45     Hangup -- hang up the phone

45     Host -- enter Host mode

45     If -- test for condition

45        Conditions allowed in IFs

47     Input -- get input from keyboard

Documentation for PibTerm version 3.0               07 Oct 85   Page 3

Table of Contents

47     Key -- get function key values

47     KeySend -- Send function key value to remote system

48     Log -- toggle session logging to printer

48     Message -- display message on screen

48     Mute -- toggle mute mode

48     Param -- set program parameter

49     Receive -- receive file from remote

49     Redial -- redial last number dialed

50     Repeat -- repeat block of statements

50     Reset -- reset script execution to start of script

50     RInput -- receive input from remote system

50     Send -- send file to remote system

52     SText -- send text to remote system

52     Suspend -- suspend execution of script

52     Text -- send text to remote system

52     Translate -- read in translate table

53     Until -- ends repeat block

53     Wait -- wait for time of day

53     WaitString -- wait for specified string from remote

54     When -- wait for specified string from remote and respond

54  While -- execute statements WHILE condition true

55  Example scripts

55     For CompuServe

56     For CDC/NOS

56     For VAX/VMS

57     For an RBBS system

57     For an IBBS system

58     For FIDO

58  Notes on error handling

58     Critical DOS error handler

59     Invalid keyboard entries and mute mode

59  Introduction to Host Mode

59  Files in host mode

59     Pibterm.usf --- user file

59     Pibterm.msg --- message file

60     Pibterm.xfr --- file transfer list

60     Pibterm.cmt --- comments

60     Pibterm.log --- host session log file

61  Using host mode

61     Autobaud detection

61     Welcome message

62     Entering user name and password

62     Main menu

63     File transfer menu

64  Acknowledgements

Documentation for PibTerm version 3.0               07 Oct 85   Page 4

PibTerm:  Communications and Terminal Emulation for the IBM PC

INTRODUCTION

============

PibTerm  is  a  communications  and terminal emulation program for IBM

personal computers and compatible units.  I,  the  author,  Philip  R.

Burns,   reserve  the  copyright  to  this  program  and  all  related

materials.  You are granted a license to use the program.

You may also make unlimited copies of the program and distribute these

copies  as  you desire, except that the author reserves the sole right

to distribute the program for profit.  You may request  a  duplication

charge, not to exceed $10 per copy.  You should distribute the program

without any changes you have made, and then a  SEPARATE  version  with

any  changes you've made.  You should clearly mark the changes in some

sort of documentation file, as well as in the source files you change.

You  may  use  the  source  code  and  modify  it  as  you  please for

NON-COMMERCIAL APPLICATIONS  ONLY.   You  may  NOT  use  the  code  in

developing commercial applications without my permission.

I  encourage  you  to  extend  this  program  and  add interesting new

features.  I also encourage you to upload these changes to your  local

BBS  in  order  to share your work with others.  What I do NOT want is

for you to rip off this code as if it were yours and  sell  it  for  a

profit.  That's not nice.

HOW TO GET A COPY OF THE SOURCE

===============================

In  many cases the source to PibTerm will be available on the same BBS

from which you downloaded the executables, perhaps in several  archive

(.ARC)  or  library  (.LBR)  files.   If  not,  then  you can probably

download it from some one of the BBSs mentioned in the sample  dialing

directory  included  with  the  executable version of PibTerm.  If you

can't find it there, or the phone expense is too great, then  you  can

send $12 for two disks with a complete .ARC copy of the source to:

Philip R. Burns

Vogelback Computing Center

Northwestern University

2129 Sheridan Road

Evanston, IL.  60201

The $12 is a media, mailing, and handling cost, not a program charge.

The  source  to PibTerm contains all sorts of interesting Turbo Pascal

routines:  not  just   the   terminal   emulation   and   asynchronous

communications  parts,  but  PibMenus, a menuing and windowing system,

Documentation for PibTerm version 3.0               07 Oct 85   Page 5

routines for performing directory and file manipulation, an  interface

to  MS  DOS's  file  handle  input/output system, a routine to emulate

Basic's PLAY statement, a routine for providing full-screen editing, a

routine  to  invoke a secondary command processor from Turbo, and much

much more.  Should keep you Turbo hackers busy for a while, and should

also  keep  some  of  you  busy figuring out how to improve or add new

features to the code!

PIBTERM FEATURES

================

1. Several different terminal emulation methods:

-- DEC VT52

-- DEC VT100 (large subset)

-- BBS ANSI (ANSI.SYS color graphics and music)

-- Dumb glass TTY

-- Dumb glass TTY in split-screen mode

2. The ability to act as a host program (mini BBS).

3. Ring buffered, interrupt driven, serial I/O.

4. A number of popular file transfer methods:

-- Xmodem Checksum and CRC

-- Modem7 batch with either Checksum or CRC

-- Ymodem (YAM)

-- Telink

-- Ascii with XON/XOFF

-- Autodownload for Bob Mahoney's EXEC PC BBS

-- CompuServe B Protocol

-- Kermit

5. Telephone dialing directory.

6. Session logging/capture to printer and/or disk.

7. On-screen editing (block mode) for use with mainframes.

8. Menu-driven commands.

9. Macro-key  definition   for   constructing   terminal   keypad

emulations or holding other often-used text sequences.

10. Escapes to the operating system (and return from DOS) within a

PibTerm session.

11. Built-in file and directory manipulation, including  directory

display, file display, file copying, file erasure.

12. Screen dumps to a file.

Documentation for PibTerm version 3.0               07 Oct 85   Page 6

13. Customizable modem support.

14. Automatic  change  of  baud  rate to the proper one with smart

modems like the Hayes 1200 and USR Courier.

15. Built-in phone area code directory.

16. Dialing session and total session time elapsed counters.

17. Internal line capture buffer which may be viewed at any time.

18. Script files  for  constructing  sequences  of  commonly  used

commands, especially useful for generating auto-logon scripts.

STARTING PIBTERM

================

PibTerm  is  distributed  in  both  source  form  (Turbo  Pascal)  and

executable  form (.COM).  PibTerm is a large program (over 34000 lines

of Turbo Pascal code) for a micro and operates  in  an  overlay  mode.

You  do  not  need  to  compile  PibTerm  to  use  it; you can use the

executable modules directly.  If you do wish to compile  PibTerm,  you

must  have  version 3.0 of the IBM version of Turbo Pascal (or later).

You will also need about 2 megabytes of free disk space  to  effect  a

complete compilation.

The executable modules are:

PIBTERM.COM

PIBTERM.000

PIBTERM.001

PIBTERM.002

PIBTERM.003

PIBTERM.004

PIBTERM.005

PIBTERM.006

PIBTERM.007

You  must place these nine files in the same directory.  You can begin

execution of PibTerm by entering:

PIBTERM

in response to the DOS command prompt.  PibTerm looks for its overlays

in  the  current directory.  You may want to invoke PibTerm from other

directories as well.  In that case you can  set  the  DOS  environment

variable  PIBTERM  to the name of the directory containing the PibTerm

overlays, and use the DOS PATH statement to ensure that  the  selected

directory  is  searched when you invoke PibTerm by name.  For example,

if you've named your PibTerm directory "MYPIB", located  on  drive  C,

then  you  could  use  the  following  DOS control statements to allow

access to PibTerm from any other directory:

Documentation for PibTerm version 3.0               07 Oct 85   Page 7

PATH C:\MYPIB

SET PIBTERM=C:\MYPIB

PibTerm also uses a number of configuration files.  For  the  terminal

emulation mode these are:

PIBTERM.CNF --- primary configuration file

PIBTERM.PRE --- dialing prefix file

PIBTERM.FON --- dialing directory

PIBTERM.FNC --- function key definitions

PIBTERM.TRA --- translate table

PIBTERM.SCR --- main script file

In  addition,  for  the host mode, these extra configuration files are

needed:

PIBTERM.USF --- user name file

PIBTERM.CMT --- comments file

PIBTERM.MSG --- message file

PIBTERM.XFR --- file transfer list file

PIBTERM.LOG --- session log file

These files must reside in  the  same  directory  as  PIBTERM.COM,  as

indicated by the PIBTERM environment variable.

I  have supplied samples of the emulation mode files.  These include a

dialing directory with  numbers  for  some  good  BBS  systems  and  a

configuration  file suitable for use with the Hayes 1200 SmartModem or

compatibles.  The other  emulation  mode  files  will  be  created  as

necessary by PibTerm.

The  exact format of PIBTERM.CNF is documented under the ALT-P command

below.  The formats for PIBTERM.FON  and  PIBTERM.PRE  are  documented

under  ALT-D  below.   The  format  of PIBTERM.FNC appears under ALT-K

below.  PIBTERM.TRA is  described  under  Alt-T,  and  PIBTERM.SCR  is

described in the section on scripts.

I  have  also  supplied sample input key definition files for use with

the VT100 emulator in PibTerm under the VAX/VMS system and the CDC/NOS

system.

See  the section below on host mode for a description of the host mode

files and how to create them.  You do not need  to  create  host  mode

files if you only want to use the terminal emulation mode.

USING PIBTERM

=============

There  are  basically  two  general  classes  of remote communications

programs.  One class contains programs  like  PibTerm,  PC  Talk,  and

Qmodem  in which you are always in terminal emulation mode.  You enter

commands to  the  communications  program  by  means  of  special  key

sequences,  but  you never really leave terminal mode.  Other programs

Documentation for PibTerm version 3.0               07 Oct 85   Page 8

like Kermit or Modem are designed with an  explicit  separate  command

mode  and  a  terminal  emulation mode.  You move from command mode to

emulation mode with a command, and from emulation mode to command mode

by a special key sequence.

I  personally prefer programs which are always in terminal mode, which

is why PibTerm is written that way.

If you have used programs like QMODEM, PC  TALK,  ETERM,  PROCOMM,  or

TMODEM  then  the operation of PibTerm in emulation mode will be quite

familiar to you, and you can probably just fire up  PibTerm  and  work

through the menus to see how to use the program.  The major difference

is that ALL PibTerm commands  are  entered  using  ALT+key  sequences;

keypad keys are not used so that they are free for use by the terminal

emulators.

If you have not used one of these programs, then you should  read  the

following instructions carefully.

After  you enter the PIBTERM command to DOS, PibTerm begins execution.

You will see a program notice including the current version  and  date

of that version:

PibTerm Version 3.0 (October, 1985) Ready.

Hit Alt-I for command list.

PibTerm reads the configuration files, if present, or else prompts you

with a series of menus to obtain the necessary information  to  create

the configuration files.

If  you  don't  use the sample supplied phone directory, then an empty

directory will be created,  unless  QMODEM.FON  --  the  QMODEM  phone

directory  --  or  PCTALK.FON  --  the  PC  Talk phone directory -- is

present in the same DOS directory from which you execute PibTerm.   If

PibTerm finds QMODEM.FON, it will ask you if QMODEM.FON should be read

to get the dialing directory entries.  If you respond yes,  QMODEM.FON

is  used to create PIBTERM.FON.  If PibTerm can't find QMODEM.FON, but

finds PCTALK.FON, then PCTALK.FON is read to get the dialing directory

entries for PIBTERM.FON.

After  the configuration files are read, PibTerm initializes the modem

if a modem initialization string appears in  the  configuration  file.

PibTerm  can  also  be  used  on a hard-wired line without a modem, in

which case the modem initialization string if  present  will  probably

have no effect.  If you do have a modem attached, you should set it so

that the carrier detect line  accurately  reflects  the  true  carrier

status  --  i.e.,  the carrier detect is NOT permanently set on -- and

also so that the data terminal ready (DTR) line is set  to  accurately

reflect  the status of DTR.  Some modems unfortunately have a tendency

to hang up the phone connection  when  communication  parameters  like

baud rate are changed if DTR is allowed to reflect reality, so you may

need to set your modem to always have DTR turned on (set high).

After initialization is complete, PibTerm  enters  terminal  emulation

Documentation for PibTerm version 3.0               07 Oct 85   Page 9

mode,  emulating  whichever  terminal  you  choose as a default in the

configuration file.  At this  time  you  may  begin  entering  PibTerm

commands.   All  PibTerm  commands are input by depressing and holding

the ALT key along with the indicated ALPHA key.  Most commands cause a

"pop-up"  menu  box  to  be displayed over the current contents of the

display.  Upon completing the command, PibTerm restores  the  previous

screen contents.

You choose an item from a menu by EITHER

1. Typing the first character of the menu item, OR

2. Using  the  up and down arrow keys to move to the desired menu

selection, and then hitting the enter key.

The currently selected item is always highlighted in inverse video  in

the menu.

Here are three PibTerm commands you should know about right away:

1. ALT-I provides a complete list of available commands.

2. ALT-X  terminates execution of PibTerm and returns you to DOS.

NOTE THAT THE PHONE LINE IS NOT NECESSARILY HUNG UP  WHEN  YOU

LEAVE  PIBTERM.  If you set DTR normal on your modem, then the

phone WILL BE hung up by exiting PibTerm.  If you have DTR set

permanently  high,  then  the  phone IS NOT hung up by exiting

PibTerm.  This is a useful  feature  to  allow  you  to  leave

PibTerm  and  re-enter  your  session  later.  However, if you

forget, you may run up a big phone bill, so it's best to  hang

up  the  modem/phone  manually before you leave PibTerm unless

you plan to re-enter an on-going session.

3. ALT-H hangs up the phone.

PIBTERM COMMANDS

================

Following is a complete description of each PibTerm command.

Alt-A: Edit line on screen

===========================

When Alt-A is pressed, a highlighted bar appears  at  line  1  if  the

cursor is currently on lines 2 through 25, or on line 25 if the cursor

is on line 1.  An XOFF is sent to the host to prevent it from  sending

more  data.  You may then use the arrow keys to move around the screen

to any displayed line and edit that line using the keypad keys  listed

below.   Pressing the INS key toggles insertion mode (the default) and

overwrite mode, just like in the  Turbo  editor.   A  carriage  return

causes  an  XON  to  be  sent to the host, followed by the text of the

newly-edited line.  Only one line at a time may  be  edited  and  sent

using ALT-A.

Documentation for PibTerm version 3.0               07 Oct 85   Page 10

Alt-A  is intended to provide a simple block-mode feature for use with

remote hosts that do not provide full-screen features.   For  example,

on  IBM  systems, if you are unfortunate enough to be stuck with using

line editors like QED or EDIT under TSO, you can use the ALT-A feature

to  good  effect  since  the entry of a line starting with a number is

interpreted as an insertion.  For example, supposed you are in  a  TSO

QED  session  and  you  enter  a list command for line 20.  The system

responds by listing the current text of line 20:

L 20

00020 HERE IS THE TEXT OF LINE 20

You can now enter Alt-A, move the cursor using the arrow keys  to  the

displayed line, and change the text, perhaps to read:

00020 HERE IS THE REVISED TEXT OF LINE 20

Now, hitting the return (or enter) key sends the newly revised line to

QED.  QED inserts the revised line 20 in your TSO file in place of the

old line 20.

Another  good  use  for  this  local  editing feature is to correct an

improperly  entered  system  command  for  those  systems  without  an

edit-last-command feature.

Here is a list of available editing key sequences for use with Alt-A:

Key Sequence            Resulting edit action

===================================================================

INS                 Toggles insert/delete mode

DEL                 Deletes current character under cursor

<-                  Moves one character to left in line

->                  Moves one character to right in line

HOME                Moves to start of line

END                 Moves to end of line

Back Space          Erases character to left of cursor, line moves

to left to fill hole.

Other characters    Insert mode:    character inserted at current

character position

Overwrite mode: character replaces character

at current cursor position

===================================================================

Alt-B: Send Break

==================

Pressing Alt-B sends a sustained break signal.  This is the equivalent

of hitting the attention key on  many  termianls,  and  is  frequently

necessary to gain the attention of a host system when using PibTerm on

a hard-wired line.

Documentation for PibTerm version 3.0               07 Oct 85   Page 11

Alt-C: Clear Screen

====================

Alt-C clears the entire screen, except in Gossip mode, when it  clears

your half of the screen only.

Alt-D: Dial a number

=====================

Alt-D invokes the dialing facilities.

VERY IMPORTANT NOTE:

===================

PibTerm monitors the carrier detect line to see if a remote session is

in progress.  If PibTerm detects a carrier, it WILL NOT allow  you  to

dial  another  number and initiate another session.  If your system or

modem MUST keep the carrier detect line high (on) all the  time,  then

you  must go to the Alt-P menu and choose "n) miscellaneous", and tell

PibTerm to IGNORE the carrier detect line for dialing purposes.

Dialing directory format

======================== The dialing directory file PIBTERM.FON  is  a

random-access  text  file in which each record corresponds to one host

system.  PibTerm as distributed provides for  200  directory  entries.

The format of each entry is:

Field Name       Length in characters

=====================================

Name of system        25

Phone Number          15

Baud rate              5

Parity                 1

Data Bits              1

Stop Bits              1

All  fields  are  plain  ascii  text.   PibTerm  uses  the  associated

communications parameters when dialing the selected phone number.

After pressing ALT-D you will see a the  first  page  of  the  dialing

directory:

Documentation for PibTerm version 3.0               07 Oct 85   Page 12

-[ Dialing Directory ]---------------------------------------------------

|

|No.  --------- Name ----------  --- Number ---    -Baud- -Bits- -Par- St

|

|  1  BOSS BBS                   1 201 568 7293     2400    8      N    1

|  2  Bob Blacher BBS            1 202 547 2008     2400    8      N    1

|  3  Met Chem BBS               1 203 281 7287     1200    8      N    1

|  4  Craig Derouen's FIDO       1 206 745 2497     1200    8      N    1

|  5  Invention factory BBS      1 212 431 1194     1200    8      N    1

|  6  Dan Doman's BBS            1 212 924 6598     1200    8      N    1

|  7  Programmer's Toolbox       1 301 540 7230     1200    8      N    1

|  8  Rich Schinnel BBS          1 301 949 8848     2400    8      N    1

|  9  Pete Olympia Darwin BBS    1 301 251 9206     2400    8      N    1

| 10  Plain Vanilla PC HOST      1 301 986 9408     2400    8      N    1

| 11  Bob Klahn's Fido           1 302 764 7522     2400    8      N    1

| 12  Sid Bratkovich's Fido      1 312 338 8827     1200    8      N    1

| 13  Gene Plantz BBS            1 312 885 7144     2400    8      N    1

| 14  Ron Fox BBS                1 312 940 6496     2400    8      N    1

| 15  John Friel's BBS           1 319 277 0044     2400    8      N    1

|

| -->    R Revise entry    P Revise prefix    C Clear entry    Q Redial

|        PgUp/PgDn Page    Esc Exit           ^/v Scroll       H Hang up

|        Entry to dial     Home First page    End Last page

|        M Manual dial     ENTER dials highlighted entry

|

-------------------------------------------------------------------------

The  top  part  of  the display shows the first fifteen numbers in the

dialing directory.  This part of  the  display  is  scrollable.   What

cannot  be  shown on this printed copy is that the first number's line

is displayed on the PC's screen in reverse video.  Whichever number is

currently  at  the  top  of the display is the default number.  If you

just hit the enter key (return key), PibTerm will  automatically  dial

this number.

The  bottom  part  of  the  display  indicates  the  available dialing

commands.

Dialing a specific directory entry

==================================

--> prompts you to enter a directory entry number to dial.

Type the entry number of the system  you  wish  to  connect  to.   The

corresponding phone number in the directory is dialed.  You may prefix

the number with one of the dialing prefix characters described  below,

in which case the corresponding prefix string is appended to the front

of the phone number.  The communications parameters are  also  set  to

those  present in the dialing directory.  You may hit the ESC (escape)

key to cancel the dialing at any time during the progress of  a  call.

Once  a connection is established, however, hitting the escape key has

no effect.

For example, if you want to dial Gene Plantz's BBS, which is number 13

Documentation for PibTerm version 3.0               07 Oct 85   Page 13

in  this  directory,  then  you  enter 13 when asked for the number to

dial.

Moving around the dialing directory

===================================

PgDn --- takes you forward a page in the dialing directory.

PgUp --- takes you back a page in the dialing directory.

Up arrow --- takes you down one line in the dialing directory.

Down arrow --- takes you up one line in the dialing directory.

Home --- takes you to first page of directory.

End --- takes you to last page of directory.

Revising a dialing entry

========================

R --- allows you to revise a dialing directory  entry.   You  will  be

prompted  for  the  number  of  the  entry to revise, and then the new

values for that entry.

C --- allows you to clear out a dialing entry.

Dialing prefixes

================

P --- allows you to define or revise the five dialing prefixes, or set

a default prefix character.

PibTerm provides for dialing prefixes to be attached to the front of a

phone number.  You may define up to five such prefixes which are  then

stored  in  the  text  file  PIBTERM.PRE.  Associated with each prefix

number is a prefix character.  The five  available  prefix  characters

are  the  same  as  those in QMODEM: '+', '-', '!', '@', '#'.  You use

these prefix characters to cause PibTerm to add the extra access codes

you need for alternate long-distance services like MCI or SPRINT.  For

example, if you use SPRINT then you need  to  dial  the  local  SPRINT

access  number,  followed  by your SPRINT access code, followed by the

actual number.  For clarity let's assume that the local SPRINT  access

number  is,  say,  888  8888,  and  our access code is 1234567.  For a

Hayes-like modem we could define the prefix string for '+' to be

8888888,,,,1234567

where each comma results in a  one  second  pause  to  provide  SPRINT

enough time to answer the call and request the access code.  To define

a prefix, use the P command.

Let's assume that you have used the P command to  define  that  prefix

for  the  '+'  key.   If you enter then enter +13 PibTerm prefixes the

Documentation for PibTerm version 3.0               07 Oct 85   Page 14

number for Gene Plantz's BBS  from  the  dialing  directory  with  the

string associated with '+' and dials:

8888888,,,,1234567 1 312 885 7144

You  can  also  set  a default dialing prefix character.  For example,

assume that you hit the P key get to the prefix revision  menu.   Then

hit  the  S  key  to define (or remove) a default dialing prefix.  The

default prefix character MUST be one of the prefix characters above or

a blank.  The corresponding prefix string is automatically appended to

the front of a dialing directory number IF that number  contains  more

than  seven digits AND you request the number be dialed by hitting the

Enter key -- that is, the number to be dialed must  be  the  currently

highlighted  directory  entry.  The default prefix is NOT added to the

front of a dialing directory number  when  you  explicitly  type  that

number.

Other dialing commands

======================

M --- allows you to enter a complete phone number to be dialed.

H --- hangs up the phone line.

Q --- redials the last number dialed.  This feature works the same way

as the Alt-Q command, so see the section below on  the  Alt-Q  command

for further details.

ESC --- takes you out of the dialing mode.

When  entering a command, you need only hit that key to get PibTerm to

respond.  You do NOT need to hit the Enter key as well.  However, when

you  enter  a  directory  number  to be dialed, you DO need to hit the

enter key after the last digit of the number.

If a successful connection to the remote system occurs, PibTerm checks

the  modem  connect  message  returned  by  the modem (if any) for the

string designated to be the modem connect message (MC= parameter).  If

it  is  found,  then  PibTerm  assumes  that  any  digits  immediately

following the modem message comprise the baud rate at which the  modem

connected.   If  that  baud  rate  differs from the current baud rate,

PibTerm adjusts the baud rate  automatically  to  match  that  of  the

remote host as indicated by the modem.

If  the  connection  to  the remote system is not successful, then the

dialing directory is displayed again so that you  can  choose  another

number to dial if you wish.

Alt-E: Toggle local echo

=========================

Some  systems  do  not  echo characters you type back to you.  In this

Documentation for PibTerm version 3.0               07 Oct 85   Page 15

case you can enter ALT-E to cause PibTerm to echo characters you type.

In  other words, if you can't see what you're typing when connected to

a remote host, enter Alt-E to toggle local echo  ON.   You  can  later

enter ALT-E to toggle local echo OFF again.

Alt-F: File manipulation

=========================

Depressing ALT-F causes PibTerm to display a menu of file manipulation

facilities.  The menu displayed is:

-[ Choose File Function:  ]--

| A)ctive directory change  |

| C)opy file                |

| D)irectory display        |

| E)rase file               |

| F)ree space on drive      |

| L)ogged drive change      |

| V)iew a file              |

| Q)uit                     |

-----------------------------

Changing the active directory

=============================

A)ctive directory change prompts you for the name of a  new  directory

path to be the logged directory.

Copying a file

==============

C)opy  file prompts you for the name of a file to be copied and a name

for the copy.  Both names may include drive names and  full  directory

paths.  An exact byte-for-byte copy of the selected file is made.  The

length of the copied file will be  identical  to  the  length  of  the

original and NOT rounded up to a multiple of 128.

Displaying the current directory

================================

D)irectory  display  provides  a  listing  of all files in the current

logged directory, their lengths, creation time  and  dates,  and  time

required  for  Xmodem  transmission  using  the current set baud rate.

(You can see what the currently  defined  baud  rate  is  by  entering

ALT-N.)

Erasing a file

==============

E)rase file allows you to specify a file to be erased.  BE CAREFUL!

Documentation for PibTerm version 3.0               07 Oct 85   Page 16

Find free space on drive

========================

F)ree  space  on  drive  prompts  you  for the letter of a drive.  The

amount of free space in bytes for that drive is displayed.

Changing the logged drive

=========================

L)ogged drive change lets you enter the letter of a new  drive  to  be

used as the logged (current) drive.

Viewing a file

==============

V)iew  a  file  lets  you list the contents of an Ascii file.  You can

request that a non-Ascii file be displayed, but  the  result  will  be

pretty much useless.

Quit file manipulation

======================

Q)uit ends file manipulation mode and returns to terminal mode.

Alt-G: Enter gossip mode

=========================

Gossip  mode  is  a  special split-screen mode that allows you to chat

with another remote user using PibTerm or Eterm.  The current  display

is  split  into  two  pieces: the top for characters received from the

remote system, and the bottom for characters you type.  This allows  a

conversation  to  take  place electronically in a full-duplex fashion,

i.e., both you and the remote system user can be entering text at  the

same time.

You  can also use gossip mode to segregate what you type from what the

system types.

Gossip mode is a dumb  terminal  mode  --  escape  sequences  are  not

processed.

Alt-H: Hang up the phone

=========================

ALT-H  causes  PibTerm  to  hang  up the phone using the modem hang-up

command defined in the configuration file, if any.  If no  command  is

defined, or the command fails to hang up the phone, then PibTerm drops

DTR.  This should hang up the phone if  DTR  is  not  permanently  set

high.   PibTerm  reports if the phone was successfully hung up or not.

If not, you may have to manually hang up the phone line.

Documentation for PibTerm version 3.0               07 Oct 85   Page 17

Alt-I: Produce command list

============================

ALT-I displays a brief list of available PibTerm commands.

Alt-J: Jump to DOS

===================

ALT-J causes Pibterm to invoke a secondary copy  of  the  DOS  command

processor  using  DOS function 4B.  This function requires that 64K of

free space be available and will  fail  if  that  much  space  is  not

available.   If you are using the distributed version of PibTerm, then

the Alt-J functions requires a system with at least 360K.

If the invocation succeeds, then you may enter DOS or  other  commands

to  your  heart's content.  Entering EXIT as a DOS command returns you

to PibTerm.   Note,  however,  that  invoking  another  program  which

commandeers  the  same  communications  port that PibTerm is using may

cause trouble, as may executing a program which in turn calls  up  yet

another  copy of the DOS command processor.  You may need to reboot if

your system locks up in either of these cases.

Before  invoking  DOS,  PibTerm  changes  the  cursor  shape  from  an

underline  to  a  square  box.   This  reminds  you  that you are in a

secondary command processor level.  Upon return to PibTerm the  cursor

is set back to an underline.

Alt-K: Set input keys

======================

ALT-K  allows  you  to  define  strings  to be sent to the host system

whenever any one of the forty function keys (F1 through F10, Shift  F1

through  Shift  F10,  Alt F1 through Alt F10, and Ctrl F1 through Ctrl

F10), or anyone of the keypad keys (Home, Ins, Del, PgUp,  PgDn,  End,

arrow  keys,  and  Ctrl  and  Alt version of those keys) is depressed.

Strings up of to 65 characters can be assigned to the  function  keys,

and  strings  up  to  12 characters long can be assigned to the keypad

keys.  This facility provides  great  flexibility  in  creating  logon

scripts,  special  command sequences for host systems, terminal keypad

values, and so on.

When you hit ALT-K, you will see a menu with the following entries:

-[ Choose Key Definition Method:  ]--

| R)ead definitions from file       |

| E)nter definitions from keyboard  |

| W)rite definitions to file        |

| Q)uit key definition              |

-------------------------------------

Documentation for PibTerm version 3.0               07 Oct 85   Page 18

Reading key definitions from a file

===================================

R)ead definitions from file causes PibTerm to ask you for the name  of

a file containing key definitions.  You need only enter the first part

of the file name; the suffix .FNC is assumed.   For  example,  if  you

enter  "MYKEYS"  as  the  file  name,  that  is  the  same as entering

"MYKEYS.FNC."  However, entering "MYKEYS."  causes PibTerm to look for

the  file  "MYKEYS" WITHOUT the .FNC suffix.  Of course, you can enter

the full file name if you  wish:  MYKEYS.FNC,  or  even  a  name  like

MYKEYS.DAT.

When PibTerm is initially invoked, the file PIBTERM.FNC, if it exists,

is read in order to set the default  function  key  values.   You  may

enter key definitions at the keyboard as described in the next section

and then write the resulting definitions to PIBTERM.FNC so that  those

definitions will automatically exist when PibTerm is executed.

Entering key definitions at keyboard

====================================

E)nter definitions from keyboard is used to set the key definitions by

hand.  You can use E)nter to look at keys you've already  defined,  or

to create a file to be used in a later session.

Writing key definitions to a file

=================================

W)rite  definitions  to  file causes the current key definitions to be

written out.  These can be read later using R)ead.   Again,  you  need

only  enter  the  first  part  of  the  file  name; the suffix .FNC is

automatically added if none appears in the file name you specify.

Quiting key definition mode

===========================

Q)uit leaves key definition mode and  returns  to  terminal  emulation

mode.

Contents of key definition strings

==================================

The  actual  key  definitions  are  comprised  of ordinary Ascii text.

There are four special characters defined for use with the  input  key

facility:

1. A  character  to represent a carriage return -- the default is

'|' (ascii 174).  This character is  defined  by  FC=  in  the

PIBTERM.CNF configuration file.

2. A  character to cause a one-second delay -- the default is '~'

(ascii 126).  The configuration parameter is FD=.

3. A character to cause  PibTerm  to  wait  for  the  immediately

Documentation for PibTerm version 3.0               07 Oct 85   Page 19

following  character  to  be  sent by the remote system before

sending any more of the function key string.  The  default  is

'`' (ascii 96).  The configuration parameter is FW=.

4. A  character  to mark the immediately following character as a

control character.   The  default  is  '^'  (ascii  94).   For

example,  entering  '^G' in a string (the ascii characters 94,

71) results in a single character being entered in the string,

ascii  7  -- the bell character, control-G.  When a key string

containing control characters is written to a  file  using  W,

all  the  embedded  control  characters  are converted to this

marker character format.  Upon being  read  in  using  R,  the

characters are converted back to control characters.

These  characters may also be used when defining modem control strings

using the modem definition submenu of ALT-P.

As an example, let's create a one-key auto-logon script for a  typical

RBBS-style  bulletin  board  system.   The usual logon sequence for an

RBBS system is:

1. Several carriage returns to select the baud rate;

2. The first name, last name, and password.

An appropriate script is:

|~|~|~`?PHILIP;BURNS;PASSWORD|

We send three carriage returns to help RBBS determine the  baud  rate.

Then  we wait for the prompting question mark which asks for the first

name.  On RBBS systems you can enter  the  entire  name  and  password

separated  by  semicolons on one line.  We'll define the key ALT-F1 to

contain the logon script.  We begin by hitting ALT-K to get  into  key

definition  mode, and then hit E to get the keyboard entry menu, which

looks like this:

-[ Select keys to define: ]-

| 1) Function keys 1 to 10 |

| 2) Shifted function keys |

| 3) Ctrl + function keys  |

| 4) Alt + function keys   |

| 5) Keypad keys           |

| 6) Alt + keypad keys     |

| 7) Ctrl + keypad keys    |

| 8) Quit                  |

----------------------------

We then hit 4 to choose the Alt + function  keys.   The  result  is  a

display   of   the  current  values  of  the  Alt-F1  through  Alt-F10

definitions, which will be blank since we have  not  yet  entered  any

definition.   Hit  1  to indicate that it is Alt-F1 we wish to define.

We then input the logon script  we  developed  above.   The  resulting

Documentation for PibTerm version 3.0               07 Oct 85   Page 20

screen  looks like this (this is slightly squeezed so we can fit it on

the document page):

-[ Input key definition ]--...--------------------------------

|  # -Key Name- --------   ... Definition ... ------------   |

|                                                            |

|                                                            |

|  1 Alt F1                                                  |

|  2 Alt F2                                                  |

|  3 Alt F3                                                  |

|  4 Alt F4                                                  |

|  5 Alt F5                                                  |

|  6 Alt F6                                                  |

|  7 Alt F7                                                  |

|  8 Alt F8                                                  |

|  9 Alt F9                                                  |

| 10 Alt F10                                                 |

|                                                            |

|                                                            |

|  Enter key number to redefine or <CR> to quit: 1           |

|  Enter new key definition ...                              |

|  -->|~|~|~`?PHILIP;BURNS;PASSWORD|                         |

|                                                            |

|                                                            |

--------------------------------------------------------------

Hitting the enter key completes the key definition.  Hitting 8he enter

key  again  takes  us  back to the previous menu.  Entering 8 for quit

takes us back to the very first Alt-K menu.  Hitting Q there lands  us

back in emulation mode.

We could now dial an RBBS system in our dialing directory and wait for

the connection to be made.  Then, hitting ALT-F1  would  automatically

cycle through the logon sequence.

Using  a  single  function key to define an auto-login script is quite

limited.  PibTerm provides a much more  powerful  script  facility  as

well.  See the section later in this document on "Using Scripts."

As  another  example  we can map the VT100's keypad for use with VT100

emulation mode in PibTerm.  The sample file VT100.FNC,  included  with

PibTerm,  defines  F1  through  F10  to  be the left half of the VT100

keypad, and Shift F1 through Shift F10 to be the  right  half  of  the

VT100 keypad.

To use PibTerm with VAX/VMS, enter the VMS command

SET TERMINAL/VT100

which  tells  VMS that you're using a VT100 terminal, and select VT100

emulation by hitting ALT-P and selecting "h) terminal definition."

The file CDC.FNC contains another mapping of the VT100  keys  for  use

with  Control Data's NOS operating system.  Here the function keys are

Documentation for PibTerm version 3.0               07 Oct 85   Page 21

set to automatically send a carriage return after the actual  function

key,  as  the NOS operating system requires this.  To use PibTerm with

NOS, enter the command

SCREEN,VT100.

when you have achieved a successful login  to  NOS.   As  before,  use

ALT-P to select the VT100 emulation mode.

When  running  on  VM/CMS  on  an  IBM  mainframe, with the Yale Ascii

front-end processor, enter VT100 as the terminal  type  when  you  are

prompted  for the terminal name by the system.  Again, ensure that you

are actually in PibTerm VT100 emulation mode by using ALT-P to  select

terminal emulation mode.

At  this  time, not all the possible keypad key combinations will work

properly.  That is, not all the keypad characters  displayed  actually

allow  assignment  of  strings.   Try and see if a particular sequence

works on your machine or not.  Later versions of PibTerm will be  able

to handle this problem.

Alt-L: Log session to printer

==============================

ALT-L toggles the logging of your session to the printer.  The printer

is written using the Turbo device LST, which is generally  DOS  device

LPT1.  Initially printer logging is turned off.

Alt-M: Toggle Mute Mode

==========================

ALT-M  toggles  noise production by PibTerm.  If mute mode is ON, then

any bells, beeps, or music received from the  host  is  ignored.   The

default is to have those sounds come through.

Normally when PibTerm wants to alert you that you've made a mistake in

typing some kind of entry (especially in the menus), PibTerm rings the

terminal  bell.   In  mute  mode,  PibTerm  flashes the word "<ALERT>"

instead on line 25 of the PC's display.

Alt-N: New communication parameters

====================================

ALT-N allows you to quickly set new  communications  parameters.   Two

menus  are displayed: the first prompts you for a new parity/data word

length/stop bit combination, and the second prompts you for a new baud

rate.   In  both  cases,  the  current settings are highlighted in the

menus, so you can also use ALT-N to see what the current settings  are

without  changing them -- just hit the enter key to select the current

settings.

Documentation for PibTerm version 3.0               07 Oct 85   Page 22

Alt-O: Output session to disk

==============================

ALT-O toggles the capture of the current session to disk.  This  works

like ALT-L, except that instead of sending the session to the printer,

the session is sent to a disk file.  PibTerm prompts you for the  name

of  the  disk file.  If it exists, the captured session is appended to

the end of the file.

Any open capture file is closed when you escape to DOS.   This  allows

you  to  look at the captured session with a file-listing program like

TYPE, MORE, Buerg's LIST, or PibList.  When you re-enter PibTerm,  the

capture  file  is  re-opened  and  the captured text for the continued

session is appended to the end of the already captured text.   If  you

have changed the directory, then the capture file cannot be re-opened.

You will need to re-issue the Alt-O command in this case.

Alt-P: Set parameters

======================

ALT-P allows you to set  program  parameters.   These  parameters  are

initially   read   from   the   configuration  file  PIBTERM.CNF.   If

PIBTERM.CNF does NOT exist  when  PibTerm  is  invoked,  then  PibTerm

cycles through the menus in the ALT-P series.

The menu displayed when you press ALT-P is:

--[ Set parameters ] ---------

| a) Communications Port     |

| b) Speed (Baud Rate)       |

| c) Parity                  |

| d) Data Bits               |

| e) Stop Bits               |

| f) Backspace               |

| g) Linefeed toggle         |

| h) Terminal Type           |

| i) Modem setup             |

| j) File transfer protocol  |

| k) Kermit parameters       |

| l) Screen dump file name   |

| m) Video mode and colors   |

| n) Miscellaneous           |

| s) Execute Script file     |

| w) Write new config file   |

| q) Quit setup              |

|                            |

------------------------------

Each  of the menu entries (except w and q ) in turn calls up a submenu

which allows you to set the corresponding program parameters.  A  list

of  these  parameters appear below.  As an example, assume that we hit

b) for speed.  The following menu box appears:

Documentation for PibTerm version 3.0               07 Oct 85   Page 23

-[ Choose Baud Rate ]-

| a) 110             |

| b) 150             |

| c) 300             |

| d) 600             |

| e) 1200            |

| f) 2400            |

| g) 4800            |

| h) 9600            |

----------------------

The default baud rate will be highlighted in inverse video.   You  can

either  enter one of the letters to select a new baud rate, or use the

arrow keys to move to a new baud rate and then hit the enter key.

The format of the file PIBTERM.CNF is as follows: in  columns  1-2  of

each record is a code to identify a parameter.  After the parameter is

an '=' sign.  Following the "=" the value of the parameter appears.

The parameter names are:

DA   --- number of data bits per character (7 or 8)

PA   --- parity

= N: none

= E: even

= O: odd

ST   --- number of stop bits (nearly always 1)

BA   --- default baud rate (300 through 9600)

PO   --- number of serial port (1 or 2)

BS   --- decimal ascii value of backspace character

(8 if backspace sends backspace, 127 if backspace

sends delete)

DE   --- decimal ascii value of delete character

(127 if delete sends delete, 8 if delete sends backspace)

TE   --- terminal type to emulate

= 0:  Dumb terminal

= 1:  Dec VT52

= 2:  BBS/ANSI  (private DEC sequences not recognized)

= 3:  Dec VT100 (private DEC sequences ARE recognized)

= 4:  Gossip (split screen)

= 5:  Host mode

LF   --- if line feeds to be added automatically when carriage return

received from remote system.

= 0: no

= 1: yes

Documentation for PibTerm version 3.0               07 Oct 85   Page 24

MI   --- modem initialization string

MD   --- modem dialing command (e.g., ATDT for Hayes)

MC   --- modem connect message (e.g., CONNECT for Hayes)

MN   --- modem no carrier message (e.g., NO CARRIER for Hayes)

MB   --- modem line busy message (e.g., BUSY for Hayes)

ME   --- modem escape characters (e.g., +++ for Hayes)

MT   --- milleseconds to surround modem escape sequence

(1500 for Hayes)

MH   --- modem hangup command (e.g., ATH0 for Hayes)

MO   --- modem timeout value in seconds, i.e., how long

to wait for a connection to be made.

MR   --- delay in seconds between redials

MA   --- modem command to answer phone (e.g., ATA for Hayes)

MS   --- modem host mode setup string

SM   --- silent mode

= 0: off

= 1: on

LE   --- local echo

= 0: off

= 1: on

FC   --- function key carriage return character

FD   --- function key delay character

FW   --- function key wait for following character

FM   --- function key escape character marker

TT   --- default file transfer protocol

= XK --- Xmodem checksum

= XC --- Xmodem CRC

= TC --- Telink

= M7 --- Modem7/CRC

= MK --- Modem7/Checksum

= AS --- Ascii

= YM --- Ymodem

= KE --- Kermit

SD   --- name of screen dump file

Documentation for PibTerm version 3.0               07 Oct 85   Page 25

TB   --- number of bells to ring after file transfer complete

TM   --- text mode for display

= M --- monochrome display

= C --- color display

CF   --- Foreground color for text display.

= 0: Black         = 8:  DarkGray

= 1: Blue          = 9:  LightBlue

= 2: Green         = 10: LightGreen

= 3: Cyan          = 11: LightCyan

= 4: Red           = 12: LightRed

= 5: Magenta       = 13: LightMagenta

= 6: Brown         = 14: Yellow

= 7: LightGray     = 15: White

CB   --- BackGround color for text display.

Same values as for CF.

CT   --- Color for menu text.

Same values as for CT.

CM   --- Color for menu frames.

Same values as for CM.

BP   --- if CompuServe B protocol recognized in VT52 or ANSI mode.

= 0: no

= 1: yes

You should keep this option turned off unless you

are actually using CompuServe to prevent line noise

from accidentally throwing you into CompuServe mode.

CompuServe B protocol requests are signalled by the

reception of an Ascii 05 character (ENQ).

EB   --- if EXEC PC BBS requests recognized in VT52 mode

0 = no, 1 = yes

This parameter if set to 1 allows you to perform

autodownloads from Bob Mahoney's EXEC PC BBS in Milwaukee.

DP   --- default dialing prefix character

= blank    :  no default prefix character

= +,-,!,@,#:  that is the default character, and the

corresponding PIBTERM.PRE entry if the

default dialing prefix string.

If DP is not blank, then hitting the return (enter) key at

the dialing directory menu causes the highlighted number to

be dialed as usual, but the prefix string corresponding to

BP=  is prefixed to the phone number ONLY IF the phone number

is longer than seven digits (i.e., is apparently a

long-distance number).

Documentation for PibTerm version 3.0               07 Oct 85   Page 26

BB   --- number of lines in review (backscroll) buffer

= 0:  no backscroll buffer

> 0:  number of line, maximum of 800.

Number will be adjusted to available memory.

XM   --- if "exploding" menus to be used.

= 0:  no

= 1:  yes

XB   --- the number of bytes to be used as a download

buffer in file transfers.  The default is 2048 bytes,

which is 16 Xmodem 128 byte sectors.

AB   --- if Host mode autobaud detection is to be based upon

information extracted from the modem connect message.

= 0:  no, wait for caller to enter carriage returns

and detect speed from that.

= 1:  yes, read modem connect message and get

speed from that (i.e., CONNECT 2400).

VB   --- Background color for VT100 emulation mode.

= colors 0 through 15 as defined for CF= above.

VF   --- Foreground color for VT100 emulation mode.

= colors 0 through 15 as defined for CF= above.

VE   --- Foreground color for high-intensity VT100 mode.

= colors 0 through 15 as defined above.

VU   --- Underline color for VT100 emulation mode.

= colors 0 through 15 as defined above.

KC   --- Block check type for Kermit.

= 1: 6-bit checksum (default)

= 2: 12-bit checksum

= 3: 16-bit CRC

KE   --- Marks end of Kermit packet

= Ascii ^M = CR (should not have to be changed)

KH   --- Character marking start of Kermit packet.

= ^A = SOH (should not have to be changed)

KN   --- Number of pad characters to put in Kermit packet.

Default is zero (no padding).

KP   --- Default pad character.  Default is ^@ (Ascii 0).

KQ   --- Control-quote character for Kermit (default is #).

KR   --- Repeat data character for Kermit (default is ~).

KS   --- Size of Kermit data packets (default is 80 characters).

Documentation for PibTerm version 3.0               07 Oct 85   Page 27

KT   --- Seconds to wait for character to arrive before timing out

in Kermit.

KW   --- Seconds to wait before sending first file of batch

in Kermit upload mode.

K8   --- Eighth-bit quoting character for Kermit

= & as default.

There are some other parameters but they are  for  debugging  purposes

and so can be ignored.

There  are  two  sample  configuration  files  provided on the PibTerm

distribution disk.  You should rename PIBBW.CNF to PIBTERM.CNF and use

it   if  you  have  a  monochrome  monitor.   Rename  PIBCOLOR.CNF  to

PIBTERM.CNF and use it if you have a color monitor.

Here are the values provided by the sample PIBCOLOR.CNF file  included

with  the  distribution copy of PibTerm.  The values for PIBBW.CNF are

identical EXCEPT for the TM, CF, CB,  CT,  CM,  VB,  VF,  VE,  and  VU

parameters.

FC=|

FD=~

FW=`

FM=^

DA=8

PA=N

ST=1

BA=1200

PO=1

BS=127

DE=8

TE=3

AG=0

LF=0

MI=ATZ|~ATX1|~ATS10=30|

MD=ATDT

MC=CONNECT

MN=NO CARRIER

MB=BUSY

ME=+++

MT=1500

MH=ATH0|

MO=  60

MR=  45

MA=ATA|

MS=ATZ|~ATX1|~ATS0=1|

MW=50

SM=1

PM=0

LE=0

Documentation for PibTerm version 3.0               07 Oct 85   Page 28

TT=XC

SD=scrndump.dat

GD=6

TB=8

TM=C

CF=6

CB=0

CT=2

CM=4

BP=0

EB=1

DP=+

BB=100

XM=0

CH=0

XB=2048

AB=1

VB=0

VF=6

VE=14

VU=1

KC=1

KD=1

KE=^M

KH=^A

KN=0

KP=^@

KQ=#

KR=~

KS=80

KT=5

KW=15

K8=&

Since  PIBTERM.CNF  is  an ordinary Ascii text file, you can create or

edit it with  an  text  editor  that  produces  such  files.   Illegal

parameters are ignored by PibTerm.

Alt-Q: Redial last number dialed

=================================

ALT-Q  starts  a  re-dial  of  the  last number dialed, if any.  If no

number has been previously dialed in the current PibTerm session, then

you  are  prompted  to  enter  a dialing entry, just like in the ALT-D

command above.  Redialing  continues  until  you  terminate  it  or  a

connection is made.  Only one system can be re-dialed at a time.

During  the  period  between  redials  PibTerm  displays a continually

decremented timer telling you how long it  will  be  before  the  next

redial attempt.  You also receive the prompt

R = redial now ESC = stop C = change delay

Documentation for PibTerm version 3.0               07 Oct 85   Page 29

If  you  hit R then an immediate redial occurs.  If you hit the escape

key (ESC) then the redial is cancelled.  If you hit  C  then  you  are

prompted  for  a  change  in  the  redial delay.  Enter the new redial

delay, which is the number of seconds between  redial  attempts.   The

redial   delay  is  initially  set  from  the  parameter  MR=  in  the

PIBTERM.CNF file.

Alt-R: Receive file from remote

================================

ALT-R allows you to receive a file from the remote  system.   You  are

prompted  for  the  name(s)  of  the  files  to be transferred and the

transfer protocol.  For all protocols you will see  a  menu  detailing

the  progress  of the transmission -- blocks received, lines received,

errors encountered, and so on.

The following transfer protocols are available in PibTerm:

-- Xmodem Checksum and CRC

-- Modem7 batch with either Checksum or CRC

-- Ymodem (YAM)

-- Telink

-- Ascii with XON/XOFF

-- Autodownload for Bob Mahoney's EXEC PC BBS.

-- Kermit

-- CompuServe B protocol

The menu you see when you hit Alt-R looks like this:

-[ Choose file transfer protocol for download: ]------

| a) Ascii                                           |

| b) Xmodem (Checksum)                               |

| c) Xmodem (CRC)                                    |

| d) Kermit                                          |

| e) Telink                                          |

| f) Modem7 (Checksum)                               |

| g) Modem7 (CRC)                                    |

| h) Ymodem                                          |

| i) Ymodem (Batch)                                  |

|                                                    |

|                                                    |

|                                                    |

|                                                    |

|                                                    |

|                                                    |

------------------------------------------------------

Ascii

=====

Ascii file  transfer  receives  a  file  without  any  kind  of  error

checking.   This is the fastest transmission method for text files and

generally works well on hard-wired lines.  It does NOT work for binary

Documentation for PibTerm version 3.0               07 Oct 85   Page 30

files,  and  it is NOT well-suited for use over phone lines or digital

transmission units.  This is because of the high probability  of  line

noise  introducing  spurious characters -- especially on long-distance

calls.

Xmodem

======

Xmodem Checksum and Xmodem CRC are the two commonest transfer  methods

for  use  with  remote  bulletin  board systems and many mini-computer

systems.  These  are  single-file-at-a-time  transfer  methods.   Ward

Christensen  originated the Xmodem protocol.  Files are transmitted in

128 character blocks along with  the  block  number  and  checksum  or

cyclic redundancy check information.  This allows the receiving system

to ensure that the transmission has occurred correctly to a very  high

degree  of  probability.  The CRC error-checking method is superior to

the Checksum method, so you should always use the CRC  method  if  the

remote  system to which you are connected accepts it.  RCPM, Fido, and

PC Host bulletin board systems  support  Xmodem  CRC;  recently,  RBBS

systems  have  started  to  support it.  Few mainframe systems support

Xmodem; CDC's NOS is one that does.

Modem7

======

Modem7 batch protocol is a variant of Xmodem which sends the file name

before  the  file  itself.  This provides for sending a batch of files

all at one time.  Each file is preceded  by  the  file  name.   Either

checksum or CRC error-checking can be used with Modem7.

Ymodem

======

Ymodem  is  another  Xmodem  variant  which  provides  for  long  1024

character blocks -- more efficient for higher baud  rates  like  2400,

4800,  and  9600  baud -- and for sending the exact file size.  Ymodem

also has facilities for sending the file creation  time.   Ymodem  can

act  either  as  a  single-file  or a batch-file protocol.  Ymodem was

developed by Chuck Forsberg and Ward  Christensen.   PibTerm  supports

both  the  single-file  and batch version of Ymodem, and also supports

the file length and file creation time information.

Telink

======

Telink is a variant of Modem7  proposed  by  Tom  Jennings  that  adds

information  on  file  size  and  file creation time to the file name.

PibTerm will use the file size  and  creation  time  information  from

Telink  if it appears.  Telink is most useful with FIDO bulletin board

systems.  PibTerm always requests the CRC  version  of  Telink,  since

there is no reason to use the Checksum version.

Incidentally,  if  you  enter  the wrong Xmodem variant protocol name,

PibTerm will try and adjust as it  receives  blocks  from  the  remote

Documentation for PibTerm version 3.0               07 Oct 85   Page 31

system.  In other words, if the remote system decides to send the file

using Telink, but you just say Modem7, PibTerm will adjust  to  Telink

anyway.   Likewise,  a  Ymodem  1024-character  block is automatically

handled, as is the Ymodem header block if any, even if  you  say  that

you  want an Xmodem transfer.  Of course, PibTerm cannot always adjust

properly, so it's best to tell it the proper transfer  type  to  avoid

problems.

Kermit

======

Kermit  is  a  protocol  developed  at  Columbia  University.   It  is

available on a large  number  of  computer  systems,  from  micros  to

mainframes.   It  is  probably the commonest and most widely available

protocol on mainframes.  It is also the  only  protocol  available  in

PibTerm  that  provides  for  transferring  binary files even when the

transmission parameters are set for 7 bits.  Kermit does  this  though

the  use  of  a  "quoting"  mechanism  so  that  an 8-bit character is

transmitted as a special marker character  indicating  the  high-order

bit  is  turned  on  and a second character containing the other seven

bits.  Kermit can transmit a single file or a  group  of  files.   The

version  implemented  in PibTerm can be used with a remote server, but

PibTerm itself does not act as a Kermit server.

Here is the menu which appears when you choose Kermit protocol:

-[ Choose Kermit function: ]-----------

|                                     |

| a) GET text file                    |

| b) GET binary file                  |

| c) RECEIVE text file                |

| d) RECEIVE binary file              |

| L) Logout remote server             |

| Q) Quit kermit                      |

|                                     |

|                                     |

---------------------------------------

With Kermit you need to distinguish between receiving a text file  and

receiving a binary file.  Many mainframe Kermits will only accept text

files, while some  microcomputer  Kermits  only  accept  binary  files

(including  text  files  send as binary files).  When receiving a file

you  must  also  distinguish  between  the  Kermit  GET  and   RECEIVE

functions.   Use  RECEIVE  to  transfer  a file when you have manually

requested the remote Kermit to SEND a file.  Use  GET  to  transfer  a

file  when  the  remote system is in server mode.  You can also logout

the remote Kermit server from PibTerm using the L) Logout entry in the

Kermit transfer menu.  PibTerm leaves you at the Kermit menu until you

type Q) quit; this allows you to GET  many  files  without  having  to

first type an Alt-R for each.

If  you choose a) GET text file or b) GET binary file then you will be

prompted for the name of the file to receive.  This should be the name

Documentation for PibTerm version 3.0               07 Oct 85   Page 32

of  the file on the remote system.  If the name is not compatible with

MS DOS file naming conventions, PibTerm will try to alter it  to  make

it  compatible.   If  the  file name already exists, then PibTerm will

rename the incoming file.  In either case you will be  told  what  the

revised file name will be.

If  you choose c) RECEIVE text file or d) RECEIVE binary file then the

(possibly altered) file name from the remote system is used.

You  may  enter  a  wildcard  specification  if  the   remote   system

understands it.

There  are  a  number of Kermit parameters which can be defined at the

Alt-P K) Kermit parameters menu.  In general, you will never  need  to

alter  those  values.   If  you  do, consult the Kermit manual for the

remote system in order to see what settings you need to use.

Because of the text/binary complication, you may find  that  a  Kermit

transfer  fails if you specify the wrong type.  Most often, you should

use the TEXT transfers to mainframes,  and  the  BINARY  transfers  to

micros.   You  may  also  need to set parameters in the remote Kermit,

such as character set type or  timeout  values.   Again,  consult  the

reference manual for the remote system's Kermit for details.

There  are  three  different  error-checking  methods  used in various

Kermit implementations.  The first method, which all Kermits allow, is

the   6-bit  checksum.   Some  Kermits  optionally  provide  a  12-bit

Checksum, which is the second method.  The third method is  to  use  a

16-bit  CRC,  similar  to  the  one in Xmodem-CRC but differing in the

definition of the bit string order.  PibTerm's Kermit will  work  with

any  of  those  three block checks.  If the remote Kermit provides the

16-bit CRC, you should use that on phone lines.   The  6-bit  Checksum

seems  reliable  enough  for  hard-wired  connections.   On downloads,

PibTerm automatically switches to  whatever  block  check  the  remote

Kermit  uses.   You  generally  need  to issue a command to the remote

Kermit to get it to use the optional block checks.  See the  reference

manual for the remote Kermit for details.

The Kermit implementation in PibTerm is based in part upon the TKERMIT

program written by Jeff Duncan.  Thanks Jeff!

Mahoney BBS autodownload

========================

Bob Mahoney implemented a special variant of Modem7 protocol  for  use

on  his  EXEC PC BBS system in Milwaukee, Wisconsin.  This protocol is

ONLY available when you are in VT52 emulation mode;  this  corresponds

to  the  IBM  3101  mode  used  by Bob's BBS.  To use the autodownload

feature on EXEC PC BBS, you must be in the file transfer menu of  EXEC

PC.  You then enter

A;file1;file2;file3;....

where  the  A  indicates an autodownload is desired, and file1, file2,

Documentation for PibTerm version 3.0               07 Oct 85   Page 33

etc.  are the file names of files to download.   You  need  NOT  enter

ALT-R  at  all,  since PibTerm will receive instructions directly from

EXEC PC BBS to begin and execute the download.  Greg Ryan provided the

original  code in Turbo Pascal to implement this autodownload facility

(thanks, Greg!).

CIS B

=====

CompuServe B Protocol, like the special Exec PC BBS protocol, does NOT

require  an  explicit  Alt-R  for a download.  Instead, you must be in

VT52 or Ansi or VT100 mode, and you must have  set  the  Compuserve  B

recognized  flag  (using  Alt-P, then requesting the "n) miscellaneous

parameters" menu).  CompuServe will issue an Ascii 05 character (ENQ),

which  automatically  starts the transfer.  You can halt a transfer by

hitting the ESC (escape) key.  The actual cancellation time can be  as

much  as  thirty  or  forty  seconds.   You  should avoid setting this

protocol ON by default unless you  are  about  to  do  a  transfer  on

CompuServe.   The  reason  is  that  line  noise  often appears as ENQ

characters,  resulting  in   spurious   CompuServe   transfers   being

initiated.

In  the  single-file  tranmission  methods,  you  are  informed  if  a

downloaded file would overwrite an existing  file.   No  such  warning

occurs  in the batch transfer methods, to avoid timeout problems.  The

best approach is to change the logged directory when downloading files

to be an empty directory.

You may cancel a transfer by hitting ALT-R again while the transfer is

in progress.  Do not expect an immediate response from  PibTerm  since

the  cancellation  requires  some  handshaking between PibTerm and the

remote system.

At this time, PibTerm does NOT check if  the  download  disk  contains

enough  room  to hold the received file.  You should ensure that there

is enough room before starting a transfer.

The default download buffer for Xmodem is large enough to hold sixteen

128-byte  or  two  1024-byte sectors.  This means that a disk write is

usually done only after collecting sixteen  128-byte  sectors  or  two

1024-byte  sectors.   You  can adjust this buffer size by altering the

value of the 'XB=' parameter (see the Alt-P section above) to indicate

the  number  of 128-byte sectors to hold in memory before a disk write

is performed.  The expanded buffer size is also used by Kermit.

PibTerm requires AT LEAST TWO CAN characters (Ctrl-X or Ascii  24)  to

cancel  a download.  This eliminates unnecessary cancellations because

of spurious CANs attributable to line noise.

Documentation for PibTerm version 3.0               07 Oct 85   Page 34

Alt-S: Send file to remote

===========================

ALT-S allows you to send files to a remote system.  The same protocols

are  available as for sending files, with the exception of the EXEC PC

method, for which there is no corresponding auto-upload.  The menu for

uploads resembles that for downloads.

For  single file transmission methods you are prompted for the name of

the file to be transmitted.  For the batch methods  you  may  enter  a

wildcard   specification,   and   all   files  matching  the  wildcard

specification will be transmitted to the remote system.  For  example,

if the current logged directory has three files

A.PAS

B.PAS

C.PAS

then  entering  the  file name as *.PAS to, say, Modem7 results in all

three files being transmitted to the remote system.

You may cancel a transfer (except in CompuServe  B  mode)  by  hitting

ALT-S  again  while  the  transfer  is  in progress.  Do not expect an

immediate response from PibTerm since the cancellation  requires  some

handshaking between PibTerm and the remote system.

When  you  upload  an Ascii file, you will be prompted for the name of

the file.  Then, you will be asked the following questions:

1. What delay to use between characters;

2. What delay to use at the end of a line;

3. What character to wait for from the remote system in order  to

send the next line (the pacing character);

4. Whether  to  send  a carriage return only or a carriage return

and line feed combination at the end of each line.

The inter-character delay and inter-line delays  provide  a  means  of

slowing  down  the transfer to accomodate slow mainframe systems which

have difficulty accepting large batches of  characters  all  at  once.

The  pacing  character  instructs  PibTerm  to wait until the selected

character appears from the remote before sending each  line  of  text.

Some  remote  systems  prefer  to have an ascii file entered only with

carriage returns, not carriage returns and line feeds together.

CompuServe B Protocol does  NOT  require  an  explicit  Alt-S  for  an

upload.   Instead,  you must be in VT52 or Ansi or VT100 mode, and you

must have set the Compuserve B recognized flag (using Alt-P  and  then

requesting  the  "n) miscellaneous parameters" menu).  CompuServe will

handle the transfer automatically.  You can halt a transfer by hitting

the  ESC (escape) key.  The actual cancellation time can be as much as

thirty or forty seconds.  You should avoid setting this protocol ON by

Documentation for PibTerm version 3.0               07 Oct 85   Page 35

default  unless  you  are  about  to do a transfer on CompuServe.  The

reason is that line noise often appears as ENQ  characters,  resulting

in spurious CompuServe transfers being initiated.

Alt-T: Get translate table

===========================

ALT-T  allows  you  to  define  a  translation  map  for  all incoming

characters.  You may request that any one of the 256 Ascii  characters

be  changed  to  any other character.  This facility is most useful in

stripping unwanted  characters  --  just  request  that  the  incoming

unwanted  characters be translated to nulls (binary 0 -- also known as

control-@).

ALT-T causes PibTerm to prompt you for the name of a  file  containing

translation  information.   You  can  enter  the name of a file if one

exists, or just hit the enter key if you want to enter the translation

mapping from the PC's keyboard.  You need only enter the first part of

the name; the suffix .TRA is assumed.  The translate file  PIBTERM.TRA

(if  it  exists)  is automatically read when PibTerm is first invoked.

You can create that file here using keyboard definition mode.

If you hit enter rather than a  file  name  PibTerm  displays  a  menu

containing  the current translation definitions for ascii characters 0

through 127.  You can hit S to select  a  display  of  characters  128

through  255.   Use  arrows  to  move  up/down/left/right  through the

displayed table.  To change a character  definition,  just  enter  the

decimal  value  of  the  ascii  character to which to map the selected

character.  You may define as many characters as you please.

To exit the translate definition mode, hit the ESC (escape) key.   You

will  be  prompted  for  the  name  of  a  file  to  which to save the

definitions.  You can just hit  the  enter  key,  in  which  case  the

definitions are not saved to a file.  If you do enter a file name, you

need only enter the first  part  of  the  name;  the  suffix  .TRA  is

assumed.   You  can  then  use  that  file  later by hitting ALT-K and

providing that file name.

You may have as many different files of translation  mappings  as  you

want, but of course only one translation mapping can be in effect at a

time.  You can change the mapping by hitting ALT-K to read a new  file

or edit the definitions by hand.

The file STRIPHI.TRA provided as part of the PibTerm release materials

contains a translation table that causes the Ascii characters from 128

through  255  to be mapped into the characters 0 through 127 in order.

This mapping is useful if you are calling  a  remote  system  using  8

bits,  and  No Parity as the communications parameters -- a good first

choice -- and you see mostly garbage on the screen  after  connecting.

The  garbage  is often a result of the remote system using even or odd

parity.  When using STRIPHI.TRA, PibTerm will strip the parity bit off

each incoming character so that you can read it and figure out what to

do next.

Documentation for PibTerm version 3.0               07 Oct 85   Page 36

Alt-U: Screen dump to file

===========================

ALT-U prompts you for a file name to receive the text of the currently

displayed screen image.  Only the text on the screen is written to the

file, not the attributes (like color, or blink).  You can only perform

a  screen dump when you are in terminal mode, NOT when you are already

at another menu or in host mode.

ALT-V: View Back Scroll Buffer

===============================

If the BB=n parameter is greater than zero,  then  PibTerm  saves  the

last n lines displayed on the screen in a back scroll buffer.  You can

display the saved lines by  hitting  Alt-V.   The  result  will  be  a

split-screen  display in which the first 20 lines of the screen act as

a window over the back scroll buffer, and the available  commands  and

the  number  of  the current line being viewed appear at the bottom of

the screen.

The PC keypad keys control viewing.  Use the up  and  down  arrows  to

scroll  up  and down one line at a time through the saved lines.  PgUp

and PgDn take you up and down a page of 20 lines, respectively.   Home

takes  you to the first page, and End takes you to the last page.  ESC

exists the view mode.

The view buffer does NOT take account  of  escape  sequences,  so  the

resulting  saved  display  for  a  full-screen session may not be very

readable.

Alt-W: Wait for call

=====================

ALT-W causes PibTerm to enter host mode.  In this  mode  PibTerm  acts

like  a mini-BBS and answers the phone, providing a simple message and

file-transfer facility.  More details on using host  mode  in  PibTerm

can be found in a special section on PibTerm host mode below.

Alt-X: Exit PibTerm

====================

Alt-X serves three purposes:

1. If a "WHEN" string is active, Alt-X is used to stop the "WHEN"

processing from happening.

2. If  a  script  is  executing,  Alt-X  stops  the  script  from

executing.

3. If no script is running, Alt-X is used to exit PibTerm.

The  phone  line is NOT necessarily hung up when you exit PibTerm.  It

generally will be if you have the DTR line set  normal.   This  allows

Documentation for PibTerm version 3.0               07 Oct 85   Page 37

you  to re-enter the session at a later time if you so desire.  If you

set DTR permanently high, then the phone line is probably NOT hung  up

when you exit PibTerm.  Hence, you should be sure to hang-up the phone

using ALT-H before leaving PibTerm if you do not plan  to  re-enter  a

session.

Alt-Y: Display elapsed time

============================

Alt-Y  displays  two elapsed time values: the total time since PibTerm

started executing, and the elapsed  time  since  the  last  successful

remote dialing session began.  You may clear the latter timer value by

entering a C when prompted.

Alt-Z: Get area code

=====================

ALT-Z invokes PibTerm's built-in  dialing  area  directory.   You  can

enter  a  dialing  code,  a  state,  or  a  country  and  receive  the

corresponding state, country, or dialing  codes.   For  domestic  area

codes,  you  also receive a list of representative cities and towns in

that area.

Shift-Tab: Toggle adding linefeeds

===================================

Some systems may not send a carriage return/line  feed  sequence,  but

only  a  carriage return, assuming that a line feed will automatically

be added by the  remote  terminal.   Hitting  Shift-Tab  toggles  this

automatic  addition of a line-feed character to each incoming carriage

return.  If you log into a system and all the lines  from  the  remote

system are being displayed on a single line, then you probably need to

toggle linefeed mode by hitting Shift-Tab.

NOTES ON TERMINAL EMULATION

===========================

PibTerm  provides  dumb  terminal  emulation  (and  gossip   mode,   a

split-screen  dumb  terminal mode), an ANSI mode for connecting to BBS

systems which use ANSI graphics and music, Dec VT52  mode  (which  can

also be used to display some IBM 3101 codes), and Dec VT100 mode.  The

VT52, VT100, and ANSI/BBS modes need some further explanation.

VT52 MODE

=========

The VT52 mode is quite complete, and features a couple  of  additional

escape  sequences  for  use  with  Bob  Mahoney's EXEC PC BBS (for box

drawing  and  auto-downloading).   Many  mainframe  and  mini  systems

recognize the VT52 terminal.

The  keypad  is mapped onto the function keys F1 through F10 and Shift

Documentation for PibTerm version 3.0               07 Oct 85   Page 38

F1 through Shift F10.  F1 through F10 are the LEFT half of the keypad;

Shift  F1 through Shift F10 are the RIGHT half of the keypad.  You may

also use the PC's arrow keys as the VT52  cursor  keys.   Double-width

keys in the real VT52 keypad are mapped on to two adjacent keys on the

PC's function keys.  Pressing either of the PC function keys which map

to one VT52 key will activate that key.

Note  that  the  VT52  function  key  definitions  OVERRIDE  ANY OTHER

DEFINITIONS YOU MAKE for the function keys.

VT100 MODE

==========

The PibTerm VT100 emulator processes all the codes that  the  standard

MS  DOS  ANSI.SYS  processes,  plus  a  number  of  private DEC escape

sequences.  VT102 codes for character insert  and  delete  or  printer

control  are NOT implemented (yet!).  Mode change requests ( ESC [ ...

l or ESC [ ...  h ) that change character  sizes  are  ignored.   This

means no double height or double width characters, no 132 column mode,

and so on.  Also, only fixed position tabs are allowed -- tab  setting

is  not  implemented.  In general, any escape sequence not implemented

in PibTerm is simply swallowed up and ignored.

Nevertheless, the implemented sequences  are  sufficient  to  use  the

full-screen  editors EDT under Dec's VAX/VMS, XEdit under IBM's VM/CMS

(through some Ascii front end protocol converter, e.g.,  Yale's),  and

FSE under CDC's NOS operating system.

Unlike  the  VT52  mode,  the  VT100 keypad has NOT been mapped onto a

fixed set of keys.  Instead, you can define  whatever  keys  are  most

convenient  for  your  particular  application.   The  file  VT100.FNC

provided with this copy of PibTerm contains definitions which map  the

standard  VT100  keypad  applications  mode keys onto function keys F1

through F10 and Shift F1 through Shift F10 -- again, the left half  of

the  VT100  keypad  is mapped to F1 through F10, and the right half is

mapped to Shift F1 through Shift F10.  The PC's  arrow  keys  function

like the keypad application mode cursor keys.

The  file  CDCNOS.FNC  contains  definitions  useful  under  CDC's NOS

operating system.  Unlike the VAX, the  CDC  system  requires  that  a

carriage   return   follow  each  keypad  character  in  order  to  be

recognized.  CDCNOS.FNC defines the proper codes for use with the  FSE

editor,  and  adds  the  required carriage return.  This makes it very

convenient to use the PC as a terminal to NOS since you do not have to

constantly  hit  the  carriage return key after each VT100 keypad key.

Note that the order of the keys is DIFFERENT  in  CDCNOS.FNC  than  in

VT100.FNC.

PibTerm's  VT100  emulation DOES understand requests for the alternate

VT100 graphics characters.  PibTerm maps the VT100 graphics characters

onto similar PC graphics characters.

Documentation for PibTerm version 3.0               07 Oct 85   Page 39

ANSI/BBS MODE

=============

The  ANSI/BBS mode interprets a small set of ANSI commands, sufficient

to handle the codes usually sent by BBSs.  In  addition,  the  special

escape  codes  for  music  are interpreted and played through the PC's

speaker (unless mute mode -- selected by ALT-M -- is in effect).

USING SCRIPT FILES

==================

PibTerm provides an extended script file facility which allows you  to

create  a file containing a set of instruction for PibTerm to execute.

The instructions can include dialing up a  remote  system,  performing

file  transfers,  and waiting for a specific time before initiating an

event.  The script facility allows you to set up PibTerm to run in  an

unattended mode and take advantage of off-hours rates on host computer

systems, as well as to automate such chores as logging  in  to  remote

systems.

Executing a script file

=======================

There  are  two  ways to execute a script file.  The first is to enter

the file name of a script file on the PibTerm command line  itself  at

the DOS prompt:

PIBTERM script.SCR

where  'script.SCR'  is  the  name  of  a  script  file.  You need not

explicitly give the .SCR, it is understood if no  other  extension  is

provided.

When  PibTerm begins execution, it will take its input from the script

file.  You may still make keyboard entries if you  desire,  and  there

are  some  script  commands  which  explicitly request keyboard entry.

This provides quite a bit of flexibility in case something goes  wrong

during  the  execution  of  a script and you are present to repair the

damage.

If no script file name appears  on  the  PibTerm  invocation,  PibTerm

looks for the file PIBTERM.SCR and executes it as a script if found.

To  exit a script, use the ALT-X command.  When a script is executing,

you will be prompted as to whether or not you want to  stop  executing

the script.  If you then want to exit PibTerm, type Alt-X again.

The second way to execute a script file is to go to the Alt-P menu and

select S)cript file execution.  You will be prompted for the  name  of

the  script  file  to execute.  Again, you need only provide the first

part of the file name; .SCR is understood.

Before executing a script file, PibTerm scans it and converts it to an

Documentation for PibTerm version 3.0               07 Oct 85   Page 40

in-memory  list of commands.  During the scanning process, you will be

informed if any script command line is  syntactically  bad.   (PibTerm

only looks for the first bad line at this time.)  Any erroneous script

statement prevents PibTerm from executing the script.

Script files are limited to 32767 characters in length.   This  should

be  sufficient  for nearly any purpose.  Also, the size of a script is

limited by the available memory to hold the  "compiled"  script  code.

If there is not enough memory for the script, it will not be executed.

SCRIPT FILE COMMANDS

==================

Script file commands consist  of  an  initial  command  word  possibly

followed  by  one  or  more arguments.  Blank lines or lines beginning

with an "*" may be used as comments.  Many  of  the  commands  perform

similar functions to the regular Alt-letter keyboard entries described

previously.

Here is a complete list  of  PibTerm  script  commands  with  a  brief

description  of  the  purpose of each.  A detailed description of each

command follows.

AddLF       --- adds line feeds to carriage returns (like Ctrl-Tab).

Break       --- issue a break sequence (like Alt-B).

Capture     --- toggles session capture to specified file (like Alt-O).

Clear       --- clears the screen (like Alt-C).

Delay       --- Delays PibTerm execution for specified length of time.

Dial        --- dial a number in the dialing directory (like Alt-D).

Dos         --- executes a specified DOS command (somewhat like Alt-J).

Echo        --- toggles local echo (like Alt-E).

Else        --- FALSE branch of an IF statement.

Endif       --- ends an IF statement.

EndWhile    --- ends a WHILE statement.

Exit        --- stop executing the script.

Hangup      --- hangs up the phone (like Alt-H).

Host        --- enters Host mode (like Alt-W).

If          --- test for several different types of conditions;

Else is also available and Endif terminates an IF block.

Documentation for PibTerm version 3.0               07 Oct 85   Page 41

Input       --- prompt for input from local user.

Key         --- sets function keys from specified file (like Alt-K

"Read definitions from a file").

KeySend     --- sends specified function key's definition to remote

system.

Log         --- toggles session logging on printer (like Alt-L).

Message     --- display message on PC's screen; message is not sent

to remote system.

Mute        --- toggles mute mode (like Alt-M).

Param       --- sets program parameters (syntax like that used in the

PIBTERM.CNF file -- see Alt-P description above).

Receive     --- receives a file from a remote system (like Alt-R).

Redial      --- redials the last phone number entered (like Alt-Q).

Repeat      --- begins block of statements to be repeatedly executed unti

condition on matching UNTIL statement is true.

Reset       --- starts executing script from the beginning again.

RInput      --- prompt for input from remote system.

Send        --- sends a file to a remote system, like Alt-S.

SText       --- send text to remote system, and interpret embedded

characters like function key characters.

Suspend     --- suspends script execution for specified length of time

(PibTerm continues executing).

Text        --- send text to remote system without interpretation.

Translate   --- reads in a translation table (like Alt-T).

Until       --- terminates REPEAT block and provides termination conditio

Wait        --- stops PibTerm execution until a specified time

(in HH:MM:SS form) is reached, at which time execution

proceeds.

WaitString  --- waits for a given string to appear from the

remote system (only done once).

When        --- waits for a given string to appear from the

remote system, and then sends a specified response

string (done as often as required string appears).

Documentation for PibTerm version 3.0               07 Oct 85   Page 42

While       --- Begins block of statements to be repeatedly executed as

long as specified condition is true.  A While block is

terminated by an EndWhile statement.

Syntax of script commands

=========================

A couple of general comments about script command syntax are in  order

here.

1. Command  names may be entered in any case; "CLEAR" = "Clear" =

"ClEaR".

2. Each command must fit on a single input  line.   There  is  no

provision for multi-line commands.

3. When  quoted  strings  are called for, either single or double

quotes may be used.  To insert a quote into a string in  which

that  same  quote is being used as the string delimiter, write

two adjacent quotes.  For example, the string

Here's a string

can be written as:

'Here''s a string'.

4. Quoted strings cannot span a line.  If the closing quote for a

string  is  not encountered before the end of the line, then a

quote is automatically assumed at the end of the line.

SCRIPT COMMANDS IN DETAIL

=========================

This section describes each Script command  in  detail.   Necessarily,

the  examples  for  some commands use other commands which will not be

described until later.  You may need to make two passes through  these

descriptions in order to fully understand the examples.

AddLF

=====

'AddLF'  toggles  the  addition of a line feed to each carriage return

received from the remote system.  'AddLF' acts exactly the same as the

Ctrl-Tab PibTerm keyboard entry.

Break

=====

'Break'  issues a sustained break signal over the communications port.

This is exactly the same as hitting the Alt-B.

Documentation for PibTerm version 3.0               07 Oct 85   Page 43

Capture

=======

'Capture' toggles session capture  to  a  specified  file.   The  file

should  be specified in quote marks.  For example, to initiate session

capture to file "GETIT.DAT", use the script command:

Capture "GETIT.DAT"

To turn off capture later in the same script, just enter Capture  with

no argument:

Capture

Be careful -- if you enter Capture with no arguments, and you have not

previously requested session capturing,  then  PibTerm  will  issue  a

prompt  for  the  capture  file  name.   The intent here is to provide

flexibility when running a script in attended mode; but if you're  not

careful,  you  may  find an unattended script gets hung up waiting for

the capture file name entry.

Clear

=====

'Clear' clears the screen, just like the Alt-C keyboard command.

Delay

=====

'Delay' delays PibTerm execution for the number of tenths of a  second

specified by the argument.  For example,

Delay 10

delays  PibTerm execution for one second (ten tenths = one).  To delay

execution for half a second, enter:

Delay 5

'Delay' differs from  the  'Suspend'  command  described  below.   The

Suspend  command only halts SCRIPT processing for the specified length

of time.  Delay halts ALL PibTerm processing (except the  asynchronous

reception of incoming characters).

Dial

====

'Dial'  dials  a  number  in  the PibTerm dialing directory, just like

Alt-D.  Specify the number to be dialed in quotes.   For  example,  to

dial directory entry 3, use the command:

Dial "3"

You can add the dialing prefix characters if you wish:

Documentation for PibTerm version 3.0               07 Oct 85   Page 44

Dial "-3"

You can also request a manual dial by prefixing the entire number with

an M:

Dial "M1234567"

Dos

===

'Dos' executes a specified DOS command --  somewhat  like  Alt-J,  but

only  for  a  single  command.   Specify the command to be executed in

quotes.

For example, to execute a DIR command, enter:

Dos "DIR"

Unlike the Alt-J command, 'Dos' automatically returns to PibTerm after

executing  the  single  specified  DOS  command,  unless the specified

command is null.  In that case, 'Dos' WILL drop to a new level of  the

command  processor,  and  you  will  have  to  enter EXIT to return to

PibTerm.

Echo

====

Echo toggles local echo mode, just like the keyboard entry Alt-E.   In

local  echo  mode,  PibTerm  displays characters as you type them, and

doesn't wait for the remote system to echo them back.

Else ====

'Else' is described as part of the 'If' command below.

Endif

=====

'Endif' is described as part of the 'If' command below.

EndWhile

========

'EndWhile' is described as part of the 'While' statement below.

Exit

====

'Exit' terminates execution  of  the  script.   PibTerm  automatically

inserts  an  exit  at  the end of a script, so you need not provide it

there.  Exit is most useful when you want  to  stop  script  execution

somewhere in the middle of a script.

Documentation for PibTerm version 3.0               07 Oct 85   Page 45

Note  that  terminating  a script does NOT terminate processing of any

defined 'When' string.  See the section on the 'When'  script  command

below for further details.

Hangup

======

'Hangup' hangs up the phone, just like the keyboard entry Alt-H.

Host

====

'Host'  causes  PibTerm  to  enter  Host  mode, just like the keyboard

command Alt-W.  In this mode PibTerm acts like  a  mini-BBS  (bulletin

board  system).   Remote  callers  can  dial your system and transfers

files and leave messages.  See the section on PibTerm Host mode  below

for more details.

If

==

'IF'  provides  a  mechanism  for  testing  several different types of

conditions.  The general form of the 'If' statement in PibTerm is:

IF condition

< script statements executed if condition is TRUE >

ELSE

< script statements executed of condition is FALSE >

ENDIF

The 'Else' part is optional and  need  not  be  specified.   You  MUST

specify the 'Endif'.

The following conditions can be specified on the If statement:

Waitstring --- If the string specified in the last previous

WaitString command appeared (see the WaitString

command for more details).

Connected --- If PibTerm is connected to a remote system.

This check is performed by examining whether the

carrier detect line is active or not.

Local --- If the reply string provided to the last previous

'Input' command (which see) contains the string

specified immediately following the 'Local' on

the IF statement.  The reply string from the

'Input'

statement must match in case with the text on the

'If' command.  Note that the test is a CONTAINMENT

test, not and EQUALITY test.

Remote --- If the reply string provided to the last previous

Documentation for PibTerm version 3.0               07 Oct 85   Page 46

'RInput' command (which see) contains the string

specified immediately following the 'Remote' on

the IF statement.  The reply string from the

'INPUT'

statement must match in case with the text on the

'If' command.  Note that the test is a CONTAINMENT

test, not and EQUALITY test.

You  need  only  provide  the first three letters of these conditions.

You may also prefix each of these conditions with a "NOT" to  indicate

that you wish to test the reverse of the specified condition.

For example, to see if PibTerm is online to a remote system, you could

write:

If Connected Then

Message "PibTerm is connected to a remote system."

Else

Message "PibTerm is NOT connected to a remote system."

Endif

The 'Then' on the If statement is optional, and is ignored if present.

It's  just there so that inveterate Pascal programmers who can't STAND

to write an If without a Then will be happy.

To ask a yes/no answer to a question about hanging up the  phone,  and

test for a yes/no answer, you could write:

Input "Do you want to hang up the phone?  "

If local "Y,y" Then

Message " "

Message "OK, hanging up the phone."

Else

If local 'N,n' Then

Message " "

Message "OK, phone not hung up."

Else

Message "Bozo, you didn't enter Y or N."

Endif

Endif

This example indicates that If statements may be nested.  You may nest

If statements up to 10 deep.  This example also illustrates  that  the

If  statement  checks  are  case-sensitive, but you can easily include

both cases of text to check for in the If statement.

The next example demonstrates how  to  check  that  the  text  from  a

WaitString command has appeared:

WaitString "Username:" 20

If NOT WaitString Then

Message "Username prompt did not appear within twenty seconds."

Hangup

Exit

Documentation for PibTerm version 3.0               07 Oct 85   Page 47

Endif

Here  the  WaitString  command  (which  see) is looking for the string

"Username:", and a time limit of 20 seconds has  been  placed  on  its

appearance.  If the WaitString doesn't appear in those twenty seconds,

then the next statement in the script file -- here the If statement --

is  executed.   The  If  statement checks if the "Username:" text ever

arrived, and if not, hangs up the phone and quits script execution.

The conditions tested by the If statement can also be  tested  in  the

'Repeat/Until'  and  'While/EndWhile' statements, providing a facility

for conditional looping in scripts.   See  the  'Repeat'  and  'While'

statements for more details.

Input

=====

'Input'  prompts  for  input  from the keyboard.  The prompt should be

enclosed in quotes.  The keyboard input is stored in a string and  can

be tested by the 'If Local' statement.

For  example,  to  ask  a  yes/no question about hanging up the phone,

write:

Input "Do you want to hang up the phone?  "

To see how the resulting input can be used,  see  the  'If'  statement

description above.

Key

===

'Key'  sets  function  keys  from  a  specified  file,  like the Alt-K

keyboard entry "Read definitions from a file".  Put the file name from

which the definitions are to be read in quotes.  For example,

Key "mydefs"

reads  key  definitions  from  file  "mydefs.fnc"  --  the  ".fnc"  is

understood.

KeySend

=======

'KeySend' sends the specified function key's definition to the  remote

system.   The  function  key  name  should  NOT appear in quotes.  The

allowed names are:

F1 through F10;

A1 through A10 (for Alt-F1 though Alt-F10);

C1 through C10 (for Ctrl-F1 through Ctrl-F10);

S1 through S10 (for Shift-F1 through Shift-F10).

For example, to send the text associated with function key F1  to  the

Documentation for PibTerm version 3.0               07 Oct 85   Page 48

remote system, enter:

KeySend F1

Log

===

'Log'  toggles  session  logging  on  printer, like the keyboard entry

Alt-L.  You should ensure that your printer is turned on before  using

Alt-L.   When logging is active, all the output from the remote system

is echoed on your printer.

Message

=======

'Message' displays a message on your screen; the message is  NOT  sent

to  any remote system to which you may be connected.  Specify the text

of the message to be sent in quotes.  For example:

Message "Here is a sample message."

The message text cannot span more than a single line.  You  may  enter

as many 'Message' statements in a row as you want, if you wish to have

more than one line of text appear.

Mute

====

'Mute' toggles mute mode, like the  keyboard  entry  Alt-M.   In  mute

mode,  no  sounds  like  bells or music are issued.  This is ideal for

those late-night sessions when you want to log in to those  BBSs  that

play  the entire top 40 rock hits before they allow you to do anything

useful, and for those mainframe systems which beep  at  you  everytime

you do ANYTHING at all.

Param

=====

'Param' sets Pibterm program parameters.  The syntax follows that used

in the PIBTERM.CNF file (see Alt-P description  above).   You  specify

the  two-letter  name  of the parameter to change, then an equal sign,

and then the value to be assigned to that  parameter.   The  parameter

names  and  values  are  described  in  the  description  of the Alt-P

keyboard command above.

For example, to change the terminal emulation to VT100 mode, write:

Param te=3

To change the baud rate to 1200 baud, write:

Param ba=1200

Documentation for PibTerm version 3.0               07 Oct 85   Page 49

Receive

=======

'Receive' causes Pibterm to receive a file from a remote system,  like

the  keyboard  command  Alt-R.   Specify  the  name  of the file to be

received in quotes, and the transfer protocol  after  the  file  name.

The protocol names are:

A  --  Ascii

X  --  Xmodem CheckSum

XC --  Xmodem CRC

Y  --  Ymodem

YB --  Ymodem Batch

T  --  Telink

M  --  Modem7 Batch Checksum

MC --  Modem7 Batch CRC

K  --  Kermit (Text file)

KB --  Kermit (Binary file)

For example, to receive the file YUMYUM.PAS using Xmodem-CRC, write:

Receive "yumyum.pas" xc

To receive a batch of files using Telink, write:

Receive "" t

The  file  names are automatically provided by the sender with Telink,

so you don't have to specify the name -- just provide a null  filename

as shown above.

Note  that "Receive" only sets up PibTerm to receive a file -- it does

NOT initiate a transfer from the remote  system.   You  will  need  to

issue  a command to the remote system to get it to start a transfer so

that Pibterm can  receive  the  file(s).   For  example,  if  you  are

connected  to  the file section of an RBBS system, you might enter the

following sequence of statements to download a file you know you want:

*

* Request that file "goodstuf.lbr" be sent to me.

*

Stext "s;goodstuf.lbr;x|"

Receive "goodstuf.lbr" x

Redial

======

'Redial'  redials  the  last  phone  number entered, like the keyboard

command Alt-Q.  However, you can specify a phone number in  quotes  on

the 'Redial' command and PibTerm will redial that phone number until a

connection is established.  For  example,  to  repeatedly  dial  phone

directory entry 3 until a connection is established, write:

Documentation for PibTerm version 3.0               07 Oct 85   Page 50

Redial "3"

Repeat

======

'Repeat'  begins a block of statements to be repeatedly executed until

the condition on a matching UNTIL statement is true.

Repeat

stext '|'

waitstring "Username:" 1

Until waitstring found

Testable conditions are described under the "If" statement above.

Reset

=====

'Reset' starts executing the current script from the beginning of  the

script again.  Reset has no arguments.

RInput

======

'RInput' prompts for input from the remote system.  Specify the prompt

to be issued in quotes.  The prompt string  may  contain  the  special

function  key  characters for carriage return, control-key definition,

and 1-second delay described under the Alt-K keyboard command.

The  response  can  be  tested  for  content  using  the  "If  Remote"

statement.

For example, to prompt a remote caller for a yes/no response you could

write:

RInput "Do you want to quit (enter Y or N ) ?  "

Send

====

'Send' sends a file to a remote  system,  like  the  keyboard  command

Alt-S.   Specify the file(s) to be sent in a quoted string, and follow

that by the protocol to be used for the transfer.  Protocol names  are

the same as for the 'Receive' command documented above.

For  example,  to  send the file "yumyum.lbr" to a remote system using

Xmodem-CRC, write:

Send "yumyum.lbr" xc

To send all files ending in .PAS using  Telink  to  a  remote  system,

write:

Send "*.pas" tc

Documentation for PibTerm version 3.0               07 Oct 85   Page 51

'Send'  only  causes  PibTerm  to  begin  sending  files to the remote

system.  You must have previously instructed the remote system  to  be

ready  to receive the files.  For example, if you are connected to the

file section of an RBBS system, you could send the  file  "yumyum.lbr"

using Xmodem with the commands:

stext "r;yumyum.lbr;x|"

suspend 400

Send "yumyum.lbr" x

The waitstring is included to ensure we don't start the send procedure

before RBBS is ready.

Kermit transfers are particularly easy if the remote  system  can  run

Kermit  in  server mode.  Just run Kermit on the remote system, put it

into server mode,  and  issue  Sends  to  your  hearts  content.   For

example, to send a bunch of files to a Vax system running Kermit under

VMS, you could write:

*

* Wait for VMS command prompt

*

waitstring "$"

*

* Start up Kermit

*

stext "Kermit|"

*

* Wait for Kermit's prompt, then send command

* to put Kermit in server mode.

*

Waitstring ">"

stext "Server|"

*

* Wait for blurb about server mode to go by, then

* send a bunch of files, then receive a file.

*

suspend 500

send "*.pas" k

send "*.doc" k

*

receive "newstuf.doc" k

*

* Take remote Kermit out of server mode.

*

stext "finish|"

*

* Tell remote Kermit to stop executing.

*

stext "stop|"

Documentation for PibTerm version 3.0               07 Oct 85   Page 52

SText

=====

'SText' sends text to the remote system.  Specify the text to be  sent

in  quote  marks.   The  special  characters used in function keys for

carriage returns, marking control characters, and delays may be  used.

(See  the  section  on the Alt-K keyboard command for details on these

special characters.)

You've already seen a number of examples of Stext  in  action  in  the

description of previous commands.

Suspend

=======

'Suspend'  suspends  script  execution  for  a  given  length  of time

(PibTerm  continues  executing).   Specify  the  length  of  time   in

hundredths of a second.

For example, to stop script processing for one second, enter:

Suspend 100

To stop script processing for half a second, enter:

Suspend 50

'Suspend' differs from 'Delay' in that 'Delay' stops EVERYTHING except

the reception of remote characters, while  'Suspend'  only  stops  the

execution of script commands.  Note, however, that WaitString and When

searches will continue  during  the  suspension  period.   That  makes

'Suspend' useful in writing conditional loops with recalcitrant remote

systems that may require a variable number of, say,  carriage  returns

before the remote system wakes up.

Text

====

Text   sends   text   to   the  remote  system,  WITHOUT  any  special

interpretation (e.g., unlike SText).  Specify the text to be  sent  in

quotes.

Translate

=========

'Translate'  reads  in  a translation table, just like the "read file"

option of Alt-T.  Specify  the  file  name  from  which  to  read  the

translation  table  in  quotes.  For example, to read a table from the

file "Striphi.tra", write:

Translate "striphi"

Documentation for PibTerm version 3.0               07 Oct 85   Page 53

The ".tra" is assumed.

Translate tables are useful in stripping parity bits (as in the sample

file  STRIPHI.TRA  provided with the PibTerm release materials), or in

removing unwanted characters.

Until

=====

See the 'Repeat' statement above.

Wait

====

'Wait' stops PibTerm execution until a given time (in  HH:MM:SS  form)

is  reached,  at  which  time execution proceeds.  Specify the time in

HH:MM:SS form in quotes.

For example, to stop PibTerm execution until 1 AM, write:

Wait "01:00:00"

The wait command only works within a given 24 hour period.

WaitString

==========

'WaitString' causes Pibterm to wait for a given string to appear  from

the remote system.  Specify the string to be waited for in quotes.  By

default, PibTerm waits for the string for 30 seconds.  You can specify

the  time  in  seconds  following  the string.  If the string does not

appear within the specified time period, then execution proceeds  with

the  next  script  command.   WaitString  can  be  combined  with  the

Repeat/Until, While/EndWhile, and If/Else/Endif script  statements  to

good effect, since the 'WaitString' condition is TRUE if the specified

string appeared, and FALSE if it did not appear.

To wait until the string 'Username:' appears, write:

WaitString 'Username:'

PibTerm will wait 30 seconds for 'Username:' to appear.  If  you  want

it to wait for, say 60 seconds, write:

WaitString 'Username:' 60

You can wait for single character:

WaitString ">"

'WaitString'  slows  down  PibTerm's execution somewhat, so you should

not be surprised if the output from the  remote  system  seems  to  be

slower than usual.

Documentation for PibTerm version 3.0               07 Oct 85   Page 54

When

====

'When'  waits  for  a given string to appear from the remote system --

like WaitString -- and then sends a specified  response  string.   You

specify  both  the  subject  and  response  strings in quotes.  'When'

searching stays active until you  de-activate  it  using  Alt-X,  even

after  the  script  itself  has  finished executing.  Hence, to stop a

script, you can enter Alt-X, but the WHEN keeps  on  going.   You  can

enter  another Alt-X to stop the WHEN.  A third Alt-X takes you out of

PibTerm entirely.

'When' slows down PibTerm's execution somewhat, so you should  not  be

surprised if the output from the remote system seems to be slower than

usual.

While

=====

'While' begins a block of statements to be repeatedly executed as long

as  the  specified  condition  on  the  'While'  statement is true.  A

'While' block is terminated by an 'EndWhile' statement.  The  form  of

the conditions for the 'While' statement are identical to those of the

'Until' and 'If' statements.

For example, to execute a  block  of  script  statements  as  long  as

PibTerm is connected to a remote system, write:

While Connected Do

.

.

.

EndWhile

The "Do" is optional and is ignored if present.

Documentation for PibTerm version 3.0               07 Oct 85   Page 55

EXAMPLE SCRIPTS

===============

The  script  language  in  PibTerm  is  complicated  enough  that some

extended examples are useful in showing how scripts can be constructed

for common login sequences.

Here  is  a sample script to log onto CompuServe and go to the Borland

user's group section.  The comment lines describe what each command is

supposed to do.

*

*     Set terminal to VT52

*

Param te=1

*

*     Dial CompuServe access number in directory

*

Dial "12"

*

*     Send Ctrl-C to get CompuServe's attention

*

SText '~^C'

*

*     Wait for User ID prompt, then send User ID

*

Waitstring "User ID:"

Stext "72000,1234|"

*

*     Wait for password prompt, then send password

*

Waitstring "Password:"

Stext "good-stuff|"

*

*     Wait for main menu prompt, then request entry to

*     Borland conference.

*

WaitString "!"

Stext "GO BOR100|"

*

*     Prompt for automatic capture to file COMPUS.DAT.

*     If automatic capture, also define a WHEN string to

*     automatically go to the next message after one has

*     been read.

*

Message " "

Input ">>> Automatic capture and non-stop messages? "

If localreplyin "Y,y"

Capture "COMPUS.DAT"

When "<CR> for menu :" "N"

Message " "

Message "Capturing to file COMPUS.DAT"

Message "Non-stop message mode."

Else

Documentation for PibTerm version 3.0               07 Oct 85   Page 56

Message " "

Message "Manual message reading, no capture."

Endif

Here  is  a  script for automatically logging into a CDC NOS mainframe

system:

*

*     Set VT100 emulation

*

param te=3

*

*     Dial up NOS system

*

dial "2"

*

*     Wake up NAM and tell it we've got a VT100.

*

stext "~|~|~|~| TC=7|"

*

*     Wait for the family prompt and reply with our username.

*

waitstring ":"

stext "myfam,myusrid,mypass,iaf|"

*

*     Wait for the control statement prompt, then tell NOS

*     our terminal type and user job name.

*

waitstring "/"

stext "SETJOB,UJN=MYUJN|"

stext "SCREEN,VT100.|"

Here is a script to log into a Vax system under VMS:

*

*   Set terminal to VT100 emulation

*

param te=3

*

*   Dial Vax system

*

Message 'Dialing Vax ... '

Dial "1"

*

*   Send CRs to wake up Vax's autobaud.

*   Keep doing it until "Username:" prompt appears.

*

Repeat

stext '|'

waitstring "Username:" 1

Until waitstring found

*

*   Send my username.

Documentation for PibTerm version 3.0               07 Oct 85   Page 57

*

Stext "MYUSRID|"

*

*   Wait for password prompt.

*

Waitstring 'Password:'

*

*   Send password.

*

Stext "MYPASS|"

Here is a script to login into an RBBS system:

*

*    Set BBS/ANSI

*

param te=2

*

*    Dial number

*

Message "Dialing RBBS"

Dial "1"

*

*    Send a few carriage returns to wake up RBBS

*

REPEAT

Stext "|"

WaitString "ame:" 1

UNTIL Waitstring

*

*    Send first name, last name, and password

*

Stext   "first;last;mypass|"

Here is a script to log into an IBBS system:

*

*    Set BBS/ANSI

*

param te=2

*

*    Dial number

*

Message "Dialing IBBS"

Dial "1"

*

*    Wait for ID request

*

Waitstring "ID: "

*

*    Send first name, last name, and password

Documentation for PibTerm version 3.0               07 Oct 85   Page 58

*    Also request message reading from last previous message

*

Stext   "first;last;mypass;m;r;*"

Here is a script to log into a FIDO system:

*

*    Set BBS/ANSI

*

param te=2

*

*    Dial number

*

Message "Dialing FIDO"

Dial "1"

*

*    Send some spaces until system wakes up

*

Repeat

SText " "

Waitstring "Your FIRST name:" 1

Until Waitstring

*

*    Send first name, last name.

*

Stext   "first last|"

*

*    Reply "y" to validation prompt

*

Waitstring "(y,n)"

Stext "y|"

*

*    Wait for password prompt and then send password.

*

Waitstring "assword:"

Stext "mypass|"

NOTES ON ERROR HANDLING

=======================

PibTerm incorporates an interrupt handler for DOS  interrupt  24,  the

critical  error  interrupt.   This  interrupt  is  signalled for error

conditions such as "disk not ready."

If PibTerm should abort, then  PibTerm's  error  handler  will  be  in

force.   PibTerm  DOES  try  to  handle all errors, and die reasonably

gracefully if possible.  Sometimes a system lockup cannot be  avoided,

so  it is best to re-boot (using CTRL-ALT-DEL) to avoid problems.  You

may also want to do this even if PibTerm appears  to  have  died  with

dignity,  since  if  any  files  were open and not closed by the error

handler, then the number of available MS  DOS  file  handles  will  be

considerably reduced.

Documentation for PibTerm version 3.0               07 Oct 85   Page 59

Normally,  when  you  make  an invalid keyboard entry in response to a

prompt, PibTerm sounds the terminal bell.  In mute mode  (Alt-M),  the

bell is not sounded, and instead the message "<ALERT>" appears on line

25 of the display for a second or so.  This <ALERT> also  flashes  any

other  time  a  bell (Control-G) character is received from the remote

system and mute mode is in effect.

INTRODUCTION TO HOST MODE

=========================

PibTerm provides simple host mode facilities for message  leaving  and

file  transfer.   PibTerm can be used as a very simple remote bulletin

board.  However, it lacks the security provisions needed for a genuine

BBS,  and  is  really  intended  to cover the need for a simple remote

facility for an individual or a small private group of users.

PibTerm host mode assumes a Hayes-compatible modem.  You may  need  to

modify  the  code if your modem doesn't return verbal codes sufficient

to determine the baud rate of the caller.  The modem is assumed to  be

set to answer the phone automatically.

To  invoke host mode after entering PibTerm, enter Alt-W.  If you want

the remote session echoed to the printer or captured to disk, then use

the  Alt-L and Alt-O commands as described above before using Alt-W to

invoke host mode.

FILES IN HOST MODE

==================

The following files are  required  for  use  with  the  host  mode  of

PibTerm:

PIBTERM.USF

===========

PIBTERM.USF  is  a simple text file containing the first name,

last name, and password for each authorized user.   This  file

can  be  created  using  any  text  editor that produces ascii

files.  The format is:

firstname;lastname;password

i.e., semicolons separating the first  name,  last  name,  and

password.

This  file  MUST  be  created outside of PibTerm; there are no

provisions for a remote caller to get added to the user file.

PIBTERM.MSG

===========

Documentation for PibTerm version 3.0               07 Oct 85   Page 60

PIBTERM.MSG is the message file,  also  a  simple  ascii  text

file.   Message  header  information  is  flagged  by  '==' in

columns one and two.  The end of a message is  marked  by  '==

End'  in  column one.  This file will be created by PibTerm if

it doesn't exist when a host session requires its presence.

To remove messages, use a text  editor  and  just  delete  the

header  lines and text for a message.  There are no provisions

for deleting messages remotely.

PIBTERM.XFR

===========

PIBTERM.XFR is the file transfer list.  This file  contains  a

list of files which may be downloaded by a remote user.  Files

NOT on the transfer list cannot be downloaded.

A file with the same name as a file on  this  list  cannot  be

uploaded  by  a  remote user.  Further, any file with the same

name as one of the PibTerm host mode files cannot be uploaded,

to  prevent  a  remote  user  from overwriting those host mode

files.

A simple way to create the transfer list is to execute the DOS

command:

DIR >PIBTERM.XFR

and then edit the resulting file using a text editor to remove

unneeded lines and get the file names into 'name.ext' form  as

required  by  PibTerm.   Each file name must appear in columns

1-14 of a separate line  in  PIBTERM.XFR.   You  can  use  the

remainder  of the line for file size, dates, a description, or

whatever you like.

Note that all files must be  in  a  single  directory  --  the

current logged directory when host mode is entered.

Here are sample lines from a PIBTERM.XFR file (the numbers are

column numbers and don't appear in the file):

1             15

VT100.FNC         428   5-28-85  12:26a

CDCNOS.FNC        211   6-17-85   4:13a

PIBTERM.CMT

===========

PIBTERM.CMT is the private comments file -- only  readable  by

you.  The format is the same as the message file.

PIBTERM.LOG

===========

Documentation for PibTerm version 3.0               07 Oct 85   Page 61

PIBTERM.LOG  is  the  log  file telling who logged on and when

they logged off.

Note that all these host mode files are simple sequential ascii files.

This implies that they should be kept small for reasonable performance

-- which is fine for a small group of users.  This implementation does

not  provide  good performance for a large group of users; if you need

that, you should obtain a real BBS program designed  to  handle  large

numbers of users.

USING HOST MODE

===============

After Alt-W is entered, PibTerm enters host mode and waits for a call.

The mode is assumed to be able to answer  the  phone.   The  modem  is

initialized using the host-mode initialization string described above.

PibTerm then scans the  user  file  and  stores  the  user  names  and

passwords  in  memory  for  quick  access.  The message bases are also

scanned to find the number of current messages.

Once in host mode, NO PIBTERM COMMANDS are recognized.   You  can  hit

the  ESC (escape) key to return to terminal emulation mode, or you can

hit the enter (return) key to start a host-mode session right at  your

PC.   When  a remote session is in progress, you can enter data at the

keyboard, which will appear to PibTerm as if the remote user had typed

the entry.

When  PibTerm  receives  a  call, it waits for the modem to answer the

phone.  PibTerm must now determine the calling characteristics  (speed

and parity) of the caller.  This is done in one of two ways, depending

upon  the  setting  of  the  configuration  parameter   AB   in   file

PIBTERM.CNF.

AB  =  1: PibTerm scans the modem connect message for the baud rate of

the caller, and determines the parity later from the initial  keyboard

entries.   This  is  the preferred method if your modem can return the

baud rate on the  connect  message.   For  example,  the  USR  Courier

returns  CONNECT  for  a  300-baud connection, CONNECT 1200 for a 1200

baud connection, and CONNECT 2400 for a 2400 baud connection.

AB = 0: PibTerm starts at 1200 baud, no parity.  PibTerm waits for the

remote caller to enter a series of carriage returns or spaces, SLOWLY.

PibTerm  determines  the  caller's  baud   rate   by   adjusting   the

communications  parameters  until  the  spaces or carriage returns are

recognized.  The parity can be determined at the same time.

After determining the proper baud rate, PibTerm issues a short welcome

message which looks like:

PibTerm v3.0, October, 1985

Beginning Remote Communications

Documentation for PibTerm version 3.0               07 Oct 85   Page 62

Test if line feeds required ...

Are these lines O V E R P R I N T I N G ?

The  caller  may  see  these  lines  as  above,  or  the  lines may be

overprinting  on  the  remote  terminal.   If  the   lines   are   NOT

overprinting,  then  N  should  be  entered  as a response to the "...

overprinting ?'  question.  If the  lines  ARE  overprinting,  then  Y

should  be entered.  PibTerm will then send line feeds as necessary to

the remote user.

Following this,  PibTerm  prompts  for  first  name,  last  name,  and

password:

Enter first name: <user enters first name>

Enter last name: <user enters last name>

Enter password: <user enters password>

The entries MUST be made on separate lines; you CANNOT enter the first

and last name on one line.  If either the user  name  or  password  is

incorrectly  entered,  then PibTerm issues the prompt again, a maximum

of three times.  If the information is not provided correctly  on  the

last try, then PibTerm hangs up the phone.

The case of the first and last names is irrelevant; any combination of

upper and lower case is acceptable (i.e., Philip = PHILIP  =  PhIlIp).

However,  PASSWORDS  ARE  CASE  SENSITIVE!!   This  means that Mypass,

MYPASS, and MyPass are ALL DIFFERENT PASSWORDS.

REPEATING: Passwords ARE case-sensitive!

If there are any messages addressed to the caller,  then  the  message

headers of those message are displayed.

Host  mode  provides two main sections: a main menu for message entry,

and a secondary menu for file transfers.  The  main  menu  appears  as

follows:

======================================================

=             PibTerm Host Mode Main Menu            =

======================================================

E=Enter message

R=Read message

S=Scan messages

P=Personal message scan

Q=Quit and logoff

F=File transfers

G=Gossip mode

X=Expert mode

C=Send comments

======================================================

Documentation for PibTerm version 3.0               07 Oct 85   Page 63

When  prompted  by  an 'Enter command ?  ' the caller can enter one of

the command letters.  No carriage return is needed.

E is used to enter a message in the message base.

R is used to read a message.

S is used to display the message numbers and headers.

P scans the message base for messages addressed to the caller.

Q ends the host session and logs off the caller.

F enters the file transfer section (see below).

G enters a "gossip" mode which allows the caller to "chat" with

the host mode operator.

X turns on expert mode and suppresses the long menu displays.

C is used to send a private message to the host system operator.

The file transfer menu appears when F is entered:

======================================================

=        PibTerm Host Mode File Transfer Menu        =

======================================================

U=Upload file

D=Download file

L=List files for transfer

M=Return to main menu

Q=Quit and logoff

X=Expert mode

======================================================

U is used to send a file to PibTerm.

D is used to receive a file from PibTerm.

L lists the files available for transfer.

M returns the caller to the main menu.

Q ends the host session and logs off the caller.

X turns on expert mode and short prompts.

In host mode, PibTerm looks for input  from  either  the  host  system

keyboard  or  the remote system.  Thus, you can run a "remote" session

from the host machine's keyboard itself.  This feature also allows you

to  enter  commands to complete what a remote user has input.  You may

also hit function keys F1 and F2: F1 enters gossip mode, and F2  hangs

up the remote user.

Documentation for PibTerm version 3.0               07 Oct 85   Page 64

If  you  have  used bulletin board systems like RBBS or IBBS, then you

should have no trouble understanding how to use PibTerm host mode.

The distributed executable version of PibTerm provides for  a  maximum

of  fifteen  users.   You  can  change the constant MaxUsers in module

PIBHOSTA.PAS of the source and  recompile  to  get  a  larger  allowed

number of users.

ACKNOWLEDGEMENTS

================

I would like to thank those who have made suggestions or provided code

for additional PibTerm features.  I would also  like  to  thank  those

Sysops  who  have tolerated my hogging their systems in order to shake

out the bugs in file transfers in ANSI emulation.  And, last  but  not

least,  I'd  like  to  thank those other software authors who provided

excellent programs to model PibTerm after, and  especially  those  who

contributed  source to the public domain.  The following list of names

is not complete, but I feel it is important to acknowledge those  whom

I   can  remember.   Programs  like  PibTerm  also  benefit  from  the

suggestions, modifications, and additions of others.   Some  of  these

individuals I have never actually communicated with, but PibTerm would

be much poorer without their software contributions.

Sysops:

======

Bob Blacher, Rod Bowman, Sid Bratkovich, Chuck Childers,

Craig Derouen, Howard Ekman, Ron Fox, Joel Garrett,

Mike Johnson, Bob Klahn, Bob Mahoney, Pete Olympia,

Gene Plantz, Rich Schinnell, Steve Stern, Vern Keenan

In particular, Bob Blacher and Joel Garrett have gone out of their way

to test PibTerm on various machines and configurations to which I have

no access.  Bob Klahn, Sid Bratkovich, and Craig Derouen --  all  Fido

sysops  --  showed  exceptional  patience  while I tried to get Telink

protocol to work properly with Fido.

Software authors, code contributors:

===================================

Alan Bishop, Don Conover, Jeff Duncan, C.  J.  Dunford,

Jim Everingham, John Friel, Guy Gallo, Tom Hanlin,

Mike Harrington, Tim Krauskopf, Bela Lubkin, Paul Meiners,

Jim Nutt, Michael Quinlan, Bob Ray, Greg Ryan, Ben Slivka

Others:

======

Charlie Price and Lon Mu Liu for many valuable suggestions.