EMS0030 PCjr Keyboard scan codes different

11/01/83

IBM PCjr System Unit/keyboard                      (4860067)

Single character scan codes are identical.

The extended scan codes  released  by  the PCjr keyboard are

slightly different than the PC's. If the Fn key is not

released before pressing the f7-10 keys the following is tru

**Shift  F9  gives no scan code on the PCjr.

**Ctrl F8, Ctrl F9, and Ctrl F10 give no scan code.

**Alt F7 also results in no scan code being released.

The shift print screen works just like the PC.  The  Control

Print Screen is replaced by Fn/Echo.

*** END OF DOCUMENT  ***

Note :  This modification or recommendation has not been subjected to

exhaustive testing and may not, in all circumstances, provide

a complete solution.

#sting and may not, in all circumstances, provide

a complete solution.