ERASABLE OPTICAL

DISK AND DRIVE TECHNOLOGY ADVANTAGES

ERASABLE

* UNLIMITED WRITE/READ/ERASE CYCLES

* OPTIMUM USE OF DISK SPACE

HIGH PERFORMANCE

* LARGE CAPACITY 3.5" DISK

* RAPID RANDOM ACCESS

* SCSI CONTROLLER

MICRO SIZE

* 3.5" MICRODISK FOOTPRINT

* HALFHEIGHT

REMOVABLE

* INTERCHANGEABLE AND PORTABLE DATA

* MORE CAPACITY PER DRIVE

RELIABLE

* NO HEAD CRASHES

* RESISTS ACCIDENTAL ERASURE

* BROAD ENVIRONMENTAL APPLICATIONS

Kodak brings surprising capacity to flexible disk drives.

Kodak presents the beginning of a new generation of flexible disk drivesthe Kodak 3.3 MB disk drive.  This 5.25 inch halfheight floppy disk drive is designed for a newgeneration desktop systmes. Utilizing stateoftheart, servoembedded, trackfollowing technology, the Kodak 3.3 MB disk drive provides 2.78 MB of formatted capacity.

The Kodak 3.3 MB disk drive offers a fast 3milisecond tracktotrack access time, 500 Kbit/second transfer rate, brushless  dc spindle motor with auto stop, and 48 and 96tpi (high density) downwardread compatibility.

The highcapacity and randomaccess features offer hard disk capacity with floppy disk convenience and reliability.  So if you're looking  for a primary or abackup device, remember yu can add mas storage removability, flexibility and media interchangeability when yu add a Kodak 3.3 MB disk drive to your system.

Features of the Kodak 3.3 MB disk drive

* High capacity 3.3 MB flexible disk drive

* IMB PC/ATtype interface compatibility

* Downwardread compatibility (48, 96 tpi)

* Fast 3 ms tracktotrack access time

* 500 Kbit/ second transfer rate

* Reliable servoembedded media and absolute vertical clamping for                  guaranteed 100% media and drive interchangeability

* Automatic media detection capability

* Automatic spindle motor power down

SPECIFICATIONS		3.3 MB DISK DRIVE	SPECIFICATIONS		3.3 MB DISK DRIVE

CAPACITY							 ERROR RATES

UNFORMATTED						 SOFT READ ERRORS	1 PER 10(9) BITS

PER DRIVE			3.3MB			 HARDREAD ERRORS	1 PER 10 (12) BITS

PER SURFACE		1.67MB			 SEEK ERRORS		1 PER 10 (6) SEEKS

PER TRACK			10416 BYTES		 MEDIA LIFE		3.5 X 10 (6) PASSES

FORMATTED											PER TRACK

PER DRIVE			2.78MB

PER SURFACE		1.39MB

PER TRACK			8704 BYTES

SECTORS/TRACK		17				SIGNAL AND POWER INTERFACE

TRANSFER RATE		500 KBITS/S		THE KODAK 3.3 MB DRIVE HAS TWO INTERFACE

LATENCY			83MS				CONNECTORS, ONE CONNECTOR (J1) IS FOR ACCESS TIME						DATA SIGNALS.  A SECOND CONNECTOR (J2)

TRACK TO TRACK 	3MS				INTERFACES +5 V DC AND +12 V DC POWER AVERAGE    						SOURCES.

(INCL. SETTLING)	160 MS

SETTLING TIME		18MS

MOTOR START		560 MS

ROTATIONAL SPEED	360/600 RPM

ERASABLE OPTICAL

DISK AND DRIVE TECHNOLOGY ADVANTAGES

ERASABLE

* UNLIMITED WRITE/READ/ERASE CYCLES

* OPTIMUM USE OF DISK SPACE

HIGH PERFORMANCE

* LARGE CAPACITY 3.5" DISK

* RAPID RANDOM ACCESS

* SCSI CONTROLLER

MICRO SIZE

* 3.5" MICRODISK FOOTPRINT

* HALFHEIGHT

REMOVABLE

* INTERCHANGEABLE AND PORTABLE DATA

* MORE CAPACITY PER DRIVE

RELIABLE

* NO HEAD CRASHES

* RESISTS ACCIDENTAL ERASURE

* BROAD ENVIRONMENTAL APPLICATIONS

Kodak brings surprising capacity to flexible disk drives.

Kodak presents the beginning of a new generation of flexible disk drivesthe Kodak 3.3 MB disk drive.  This 5.25 inch halfheight floppy disk drive is designed for a newgeneration desktop systmes. Utilizing stateoftheart, servoembedded, trackfollowing technology, the Kodak 3.3 MB disk drive provides 2.78 MB of formatted capacity.

The Kodak 3.3 MB disk drive offers a fast 3milisecond tracktotrack access time, 500 Kbit/second transfer rate, brushless  dc spindle motor with auto stop, and 48 and 96tpi (high density) downwardread compatibility.

The highcapacity and randomaccess features offer hard disk capacity with floppy disk convenience and reliability.  So if you're looking  for a primary or abackup device, remember yu can add mas storage removability, flexibility and media interchangeability when yu add a Kodak 3.3 MB disk drive to your system.

Features of the Kodak 3.3 MB disk drive

* High capacity 3.3 MB flexible disk drive

* IMB PC/ATtype interface compatibility

* Downwardread compatibility (48, 96 tpi)

* Fast 3 ms tracktotrack access time

* 500 Kbit/ second transfer rate

* Reliable servoembedded media and absolute vertical clamping for                  guaranteed 100% media and drive interchangeability

* Automatic media detection capability

* Automatic spindle motor power down

SPECIFICATIONS		3.3 MB DISK DRIVE	SPECIFICATIONS		3.3 MB DISK DRIVE

CAPACITY							 ERROR RATES

UNFORMATTED						 SOFT READ ERRORS	1 PER 10(9) BITS

PER DRIVE			3.3MB			 HARDREAD ERRORS	1 PER 10 (12) BITS

PER SURFACE		1.67MB			 SEEK ERRORS		1 PER 10 (6) SEEKS

PER TRACK			10416 BYTES		 MEDIA LIFE		3.5 X 10 (6) PASSES

FORMATTED											PER TRACK

PER DRIVE			2.78MB

PER SURFACE		1.39MB

PER TRACK			8704 BYTES

SECTORS/TRACK		17				SIGNAL AND POWER INTERFACE

TRANSFER RATE		500 KBITS/S		THE KODAK 3.3 MB DRIVE HAS TWO INTERFACE

LATENCY			83MS				CONNECTORS, ONE CONNECTOR (J1) IS FOR ACCESS TIME						DATA SIGNALS.  A SECOND CONNECTOR (J2)

TRACK TO TRACK 	3MS				INTERFACES +5 V DC AND +12 V DC POWER AVERAGE    						SOURCES.

(INCL. SETTLING)	160 MS

SETTLING TIME		18MS

MOTOR START		560 MS

ROTATIONAL SPEED	360/600 RPM

RECORDING DENSITY		9908BPI

FLUX DENSITY			9908BPI

TRACK DENSITY			192TPI

TRACKS (PER SURFACE)	160

ENCODING METHOD		MFM

ENVIRONMENTAL LIMITS

AMBIENT

TEMPERATURE			50 TO 115F

RELATIVE HUMIDITY		20% TO 80%

MAX. WET BULB			78F NONCONDENSING

DC VOLTAGE REQUIREMENTS

+ 12V DC + 5%, 100MV MAX. RIPPLE

@ 1.5 A TYPICAL 3.0 A MAX.

+ 5 V dc + 5%, 50 MV MAX. RIPPLE

@ 1.0 A TYPICAL 1.3 A MAX.

MECHANICAL DIMENSIONS

HEIGHT			1.625 IN (41.3MM)

WIDTH			5.75 IN (146.1MM)

DEPTH			8.08 IN (205.2MM)

WEIGHT			2LBS 8 oz (1.12KG)

HEAT DISSIATION

TYPICAL			23W (78.5 BTU/br)

STAND BY			3.2 W (10.9 BTU/br)

RELIABILITY

MTBF				10,000brs poh

TYPICAL USAGE

PM				NON REQUIRED

MTTR				30 MINUTES

COMPONENT LIFE		5 YEARS

Kodak delivers hard disk capacity without the hard disk.

Hard disk performance has finally come to a flexible disk drive.  Presenting the Kodak 12MB cartridge disk drive. This 5.25inch, halfheight intelligent drive incorporates several stateoftheart technologies.  A servo embedded trackfollowing system, voicecoil actuator and embedded SCSI controller work together to get 10.0 MB of formatted storage capacity from a single removable cartridge.

Kodak 12 MB disk drive has performance that rivals many rigid disk drives.  It features an average access time of 65 millionseconds, and an internal transfer rate of 2.2  megabits/second.

So if you're looking for  online storage or a backup device that has lowcost, removable media but still has rigid disk performance and capacity, you're looking for the intelligent 12 MB cartridge disk drive from Kodak.

Features of the Kodak 12 MB disk drive

* 5.25inch halfheight removable cartridge disk drive

* Hard cartridge flexible media

* 10 MB formatted capacity

* Embedded controller with SCSI interface

* Fast 65 ms average access time

* 64bit error correction code

* Powerup diagnostics

* Two writeprotect zones

* Userselectable logical block size

* Driveactivity and cartridgelocked LEDS

SYSTEM

SPECIFICATIONS		12MB DISK DRIVE	REQUIREMENTS		12MB DISK DRIVE

CAPACITY							ENVIRONMENTAL

UNFORMATTED						OPERATIONAL

PER DRIVE			12.68 MB			AMBIENT OPERATING

PER SERVICE		 6.34MB			TEMPERATURE		  50 TO 115F

FORMATTED							RELATIVE HUMIDITY	  20% TO 80%

PER DRIVE			10.004MB			MAX. WET BULB		  78 F NON CONDENSING

PER SURFACE		 5.002MB

TRACK							ALTITUDE			  10,000FT (MAX)

ZONE	0			19.968 BYTES		RELIABILITY

ZONE 1			9,984 BYTES		DRIVE

SECTOR							MTBF				  10,000brs POH

ZONE 0			256 BYTES			CARTRIDGE

ZONE 1			128 BYTES			INSERTIONS		  30,000

SECTORS/TRACK		 78				HARD DATA READ

DATA ORGANIZATION					ERRORS			  1 PER 10 (12)LOGICAL BLOCK SIZE	256,512 OR 		HARD SEEK ERRORS	  1 PER (6) SEEKS

1024 BYTES		MEDIA LIFE		  3.5x (6) PASSES

USER SELECTABLE					  PER TRACK

DATA SURFACES		2

READ/WRITE HEADS	2

RECORDING DENSITY	21,640bpi			SMALL COMPUTER SYSTEM INTERFACE

FLUX DENSITY		21,640 bpi		(SCSI) COMMAND SET

TRACK DENSITY		333tpi			CODE (bex)		COMMAND

CYLINDERS			301				00				TEST UNIT READY

ENCODING METHOD	MFM				01				RECALIBRATE

PERFORMANCE						03				REQUEST SENSE

SCSI DATA TRANSFER					04				FORMAT UNIT

RATE				275KBYTES/S		08				READ

SCSI BURST DATA					0A				WRITE

TRANSFER RATE		600KBYTES			0B				SEEK

ACCESS TIME						12				INQUIRY

TRACK TO TRACK		15MS				15				MODE SELECT

AVERAGE			65MS				16				RESERVE UNIT

FULL STROKE		115MS			17				RELEASE UNIT

LATENCY			50MS				1A				MODE SENSE

1B				START/STOP UNIT

POWER							IC				RECEIVE DIAGNOSTIC

REQUIREMENTS		12MB DISK DRIVE	1D				SEND DIAGNOSTIC

VOLTAGE REQUIRED	5V + 5%			1E				PREVENT/ALLOW MEDIA

12 V + 5%           25				READ CAPACITY

CURRENT REQUIRED					28				READ EXTENDED

+5V OPERATING		1.4 AMPS (MAX)		2A				WRITE EXTENDED

+12 V OPERATING	1.4 AMPS (MAX)		2B				SEEK EXTENDED

POWER DISSIPATION					2F				VERIFY

TYPICAL			16W (54.6BTU/BR)	3B				WRITE BUFFER

STAND BY			5W (17.1BTU/BR)	3C				READ BUFFER

THE ELECTRICAL INTERFACE IS SINGLE

ENDED USING

PHYSICAL

HEIGHT			1.625 IN (41.3MM)		A 50 CONDUCTOR NONSHIELDED CONNECTOR WIDTH               5.75 IN (146.1MM)        AS DESCRIBED IN THE PROPOSED ANSI    DEPTH               8.00 IN (203.2MM)    	SPEC. ALL SIGNAL LINES ARE           WEIGHT              3LBS 4 OZ (1.47KG)	     IMPLEMENTED AND ELECTRICALLY CONFORM                                               TO THE SPEC.

Kodak presents a reliable way to put multimegabtes on a floppy disk.

Now you can get 5.24MB of formatted storage capacity on a floppy disk.  Presenting the Kodak 6.6.MB flexible disk drive.  This halfheight drive is designed to work with new generation desktop systmes.  By using stateoftheart, servoembedded trackfollowing technology, the Kodak 6.6 MB disk drive gives yu higher capacity without sacrificing media removability.

The Kodak 6.6 MB disk drive also offers a fast 3millisecond trackto track access time, 500 Kbit/second transfer rate, brushless dc spindle motor with auto stop, and 48, 96 and 19tpi (highdensity) downwardread compatibility.

These highcapacity and randomaccess features give you hard disk capacity with floppy disk convenience and reliability.  So if you're looking for a primary or backup device, remember you can add a "smart" Kodak 6.6 MB disk drive to your system.

Features of the Kodak 6.6 MB disk drive

* high capacity 6.6 MB flexible disk drive

* IBM PC/ATtype interface compatibility

* Downwardread compatiility (48, 96, 192 tpi)

* Fast 3 ms trackto track access time

* Dynamic ramp stepping

* 500 Kbit/second transfer rate

* Reliable servoembedded media and absolute vertical clamping for        guaranteed 100% media and drive interchangeability

* Automatic media detection capability

* Automatic spindle motor power down

SPECIFICATIONS		6.6MB DISK DRIVE	SPECIFICATIONS		6.6MB DISK DRIVE

CAPACITY							ERROR RATES

UNFORMATTED						SOFTREAD ERRORS	1 PER 10 (9) BITS

PER DRIVE			6.6MB			HARDREAD ERRORS	1 PER 10 (2) BITS

PER SURFACE		3.3MB			SEEK ERRORS		1 PER 10 (6) SEEKS

PER TRACK			10416 BYTES		MEDIA LIFE		3.5 x10(6) PASSES

FORMATTED											PER TRACK

PER DRIVE			5.57MB

PER SURFACE		2.78 MB

PER TRACK			8704 BYTES

SECTORS/TRACK		17

TRANSFER RATE		500 KBIT/S

LATENCY			83MS

ACCESS TIME

TRACK TO TRACK		3MS

AVERAGE

(INCL. SETTLING)	225MS

SETTLING TIME		18MS

MOTOR START		560MDROTATIONAL SPEED	  360/600

RECORDING DENSITY	  9908BPI

FLUX DENSITY		  9908BPI

TRACK DENSITY		  384TPI

TRACKS (PER SURFACE)  320

ENCODING METHOD	  MFM

ENVIRONMENTAL LIMITS

AMBIENT

TEMPERATURE		   50 TO 115 F

RELATIVE HUMIDITY	   20% TO 80%

MAX. WET BULB		   78 F NONCONDENSING

DC VOLTAGE REQUIREMENTS

+12V dc + 5%, 100% mV max. RIPPLE

@ 1.5 a typical 3.0 A max

+ 5V dc + 5%, 50mV max RIPPLE

@ 1.0 A TYPICAL 1.5 A MAX

MECHANICAL DIMENSIONS

HEIGHT				1.625 IN (41.3MM)

WIDTH				5.75 IN (146.1MM)

DEPTH				8.08 IN (205.2MM)

WEIGHT				2LBS 80z (1.12KG)

HEAT DISSIPATION

TYPICAL				23W (78.5 BTU/br)

STAND BY				3.2 W (10.9 BTU/br)

RELIABILITY

MTBF					10,000 brs POH

TYPICAL USAGE

PM					NONE REQUIRED

MTTR					30 MINUTES

COMPONENT LIFE			5 YEARS

For use with IBM PC/XT/AT Computers and Compatibles

The Kodak flexible disk controller, FDC1, is designed specifically for OEM's VAR's VAD's and system integrators.  It is a flexible and costeffective controller for both standard drives and Kodak highcapacity 3.3 MB and 6.6 MB flexible disk drives.

Features

* Supports combinations of up to four drives

* Standard (48tpi), Kodak 3.3 MB and 6.6 MB flexible 5 1/4" disk drives

* Standalone or coexists with other flexible disk controllers

* Short slot card

* Switchselectable drive configurations

* Switchselectable board addressing

* Switchselectable DMA and interrupt channel configurations

* External disk drive connector

* PCDOS software driver

Hardware Specifications Physical Dimensions

Width		4.2"

Length		5.75"

Height		 .49"

DC Power Requirements

5V dc @ 1.0A

Environmental Range

Temperature		0to 50 C

Relative Humidity	10% to 90%

noncondensing

Software

includes

Driver (PCDOS)

Format Utility (3.3/6.6

Requires

PC DOS 2.0 or above

Typical Drive Configurations supported

48tpi		3.3MB	6.6MB

___			2		___

1			2		___

2			2		___

___			2		___

___			___		2

1			___		1

2			___		1

___			___		1