#6#CB

Kenwood TS-440 Low Band Computer

Interface Control Program.

Required Equipment:

IBM PC or Compatible with 128K memory (MIN) and a DSDD

floppy disk drive.

CGA or EGA Color Monitor

Chapter 1.  Description

Upon  release   of  the   Kenwood   TS-440   low   band

transceiver,  Kenwood   also   announced   a   computer

interface option.   This  interface allows  control  of

most front  panel aspects of the rig.  The only problem

was that  no software  to control the rig was ever sold

to use the powerful protocol.  That is until now.

TS440ctl.exe allows  the user  complete remote  control

ability from a Personal Computer screen.  The following

are the commands and controls available to the user:

VFO A Frequency

VFO B Frequency

Memory Channel Selection

Tuning

RIT switch, tune, and dial

Band selection

Scan

Split

Lock

Mode

In  addition  to  the  above,  status  of  the  440  is

constantly  updated  on  the  PC  screen.    With  this

program, the  user may  access almost every function of

the 440 from the PC.

Chapter 2,  Setting Up for TS440ctl

As mentioned above, TS440ctl contains a real time clock

that reflects GMT time.  In order for the GMT time to

be correct, you must first make sure that your system

clock is set correctly for YOUR time zone.  Also, a new

command must me inserted into your autoexec.bat file.

If you are not familiar with this file, consult your

DOS reference manual for details.

TS440ctl needs to know the time zone that it is running

in so that it may determine the correct number of hours

to correct for Greenwich Mean Time.  The program

figures this out by looking for a value called

timezone.   You set timezone to one of the following

values:

EDT  - Eastern Standard Time

CDT  - Central Standard Time

MDT  - Mountain Standard Time

PDT  - Pacific Standard Time

Enter the following line into your autoexec.bat file,

or just enter at the DOS prompt:

set timezone=XXX

where XXX is the timezone where you reside.  The

advantage to entering this into your autoexec.bat is

that is saves the step each time you want to run the

program.  The program defaults to MDT time, for that is

the time zone I live.

The TS-440  communicates with  the PC via an RS-232 EIA

connection.   This is  typically connected to your PC's

COM1 port.   A  3 wire  RS-232 cable  connects  to  the

kenwood   communications    interface.       For   more

information, refer  to the installation manual included

with the interface unit.

Chapter 3,  Using TS440ctl.exe

Start-Up

TS440ctl is started by typing

TS440CTL

at the DOS command line.  If you have a hard disk, make

sure that you are in the directory, or have a path to

the directory in which the program resides.  If you

wish to use the second COM port (COM2), type

TS440CTL 2

When the program begins execution, you should hear the

radio beep several times.  This is normal.  If the

radio does not beep, check to see that all cables are

properly connected and you are using the proper serial

COM port for your computer.

The start up status of the program is VFO A.  What ever

frequency was tuned at the time of last use is the

current frequency, as well as the mode, scan status,

rit status and so on.  This information should

immediately be updated on the screen.

The Screen

The  screen   for  TS440CTL  is  divided  into  several

sections.   Each section  contains related information.

For example, the top of the screen contains the current

status of  all the  functions available,  that is VFO A

frequency, VFO  B frequency, and current memory channel

and channel frequency.  Based on the selected function,

the word  above the  function will  be  illuminated  to

indicate the  current selection.   The  current mode is

also positioned on the first sector of the screen.

The second  sector contains  a "meter"  indicating  the

current band. Based upon the current band, a pointer is

placed under  the  band  position  on  the  meter.  The

current XIT  and LOCK  status are  on the right side of

the screen.

The center  of the  screen contains the current display

frequency, RIT  status,  and  current  RIT  offset.  In

addition, another  function indicator  is  provided  so

that, by  looking at  this sector  alone, the  user can

obtain most  of the  significant information  regarding

the status of the radio.

The next sector contains the current scan status, spilt

status, and  a graphic  indication of  the current  RIT

offset.  A real time clock is also included.

An  lastly,   the  bottom  of  the  screen  contains  a

reference of  all  the  control  commands  used.    The

following sections  describe these  control commands in

detail.

Chapter 4, Keyboard Commands

Several PC  keys are  used to control the various front

panel aspects  of  the  TS-440.    These  commands  are

described here:

Function Keys

F1   -    Decrements the current frequency by the

current step rate.

F2   -    Increments the frequency by the current

step rate.

F3   -    Decrements the RIT offset.

F4   -    Increments the RIT offset.

F5   -    Toggles the status of the RIT (on/ off).

F6   -    Toggles the XIT status.

F7   -    Select frequency for VFO A.

This command will prompt the user for a

frequency to be placed into VFO A.

F8   -    Same as above for VFO B.

F9   -    Select Memory Channel.  This command

prompts the user for the memory channel

desired.

F10  -    Exit, quit the program.

A    -    Switch to VFO A

B    -    Switch to VFO B

C    -    Clear current RIT offset.  This command

allows you to have the radio ignore the

current position of the RIT controller,

and use exclusively the remote control.

Note that if the control is moved on the

rig, the current controller position is

restored, and the program is updated as

to its status.

M    -    Select memory as current function

O    -    Select Mode.  This key scrolls through

all the possible modes of the TS440 (AM,

FM, CW, ect...).

P    -    Toggle split status.

S    -    Toggle Scan Status.

T    -    Transmit/ Receive toggle.  (Careful!)

Alt F7-   Memory to VFO copy.

This command prompts for which VFO to

copy to, and the channel you wish

copied.

Other notes

While using  ts440ctl, you  may notice  that the screen

updates do  not always  track the  status of the radio.

In reality, the problem lies with the radios ability to

relay information  back  to  the  PC.    The  interface

specification states  that  an  update  of  information

takes  place   every   1.5   seconds.   This   can   be

misunderstood to  mean that at any time, information is

updated every  1.5 seconds.   This,  in fact is not the

case.   When activity  takes place on the radio, either

using the  front panel  controls or  the PC,  an update

will not  take place  until the  radio  sits  idle  for

approximately 1.5  seconds.   A good example of this is

spinning the  dial while watching the PC screen running

ts440ctl.   You should  notice an  update taking  place

AFTER the  spin has  stopped.  To counter this example,

placing the  radio in  scan  mode  updates  the  screen

approximately every  1.5 seconds.    From  the  limited

information in  the documentation of this interface, it

is not  possible to determine the exact pattern or rule

chosen for  this, but  the end  result is  a  sometimes

sluggish interface to this program.

During development  of this  program, every  effort was

made to  insure that  status of the screen reflects the

actual status  of the  radio.   If, at  some time,  you

notice that  the screen  does not  reflect that status,

perform some  action on  the keyboard, for example tune

up and  then down.  This change should prompt the radio

to update the status of the PC.

If you  use one  of the many keyboard speed up programs

available, be  warned that if you hold a key down, then

every key  will count.   For  example, if  you tune  by

holding the  F1 key  down, it  may go  further that you

want.

This program was written in Borland International Turbo

C environment.   It  has been  tested on an PC/AT clone

and a Tandy 1000.  If your PC is a close compatible, it

should run  fine.   If  you  encounter  problems,  have

suggestions, or just have questions drop me a line at

Kenneth Byers

or

David Byers (W9HXL)

4813 West Greenway Road

Glendale, AZ   85306

If you find this program useful, please help a starving

computer science  student by  mailing a donation to the

above address.   In  addition to  the charity value, it

will insure  that you  are notified of upgrades to this

program.   Some of  the upgrades I am currently working

on are:

Bulk Memory Transfers which will allow you to keep

libraries of  memory channels  on disk, and upload

and download them to and from the radio.

VFO to  Memory channel  storage which  will  allow

you, at  the touch of a key, to store the contents

of the current VFO to any memory channel.

Utilities that  allow you  to manipulate memories,

as well  as selecting  a memory VIA station ID, or

any other comment field.

Band Tracking,  allowing you  to select only legal

HAM bands,  short wave  bands, or  a user  defined

band pattern.

It took  quite a  while  to  accumulate  the  knowledge

required to  write a  program  of  this  type,  so  the

recognition would  make it  all seem  worth while.    I

would  suggest   a  possible  donation  of  20.00,  but

anything helps.

Thank you, and enjoy.