MicroSpeed ADI Driver Device for AutoCAD

----------------------------------------

Overview

--------

This  document  describes the installation and use of a  FastTRAP

pointing  device  driver  developed by MicroSpeed  for  use  with

AutoCAD by Autodesk.   The driver is usable with all releases  of

AutoCAD  which  support the Autodesk defined ADI (AutoCAD  Device

Interface)  and  allows AutoCAD users to  significantly  simplify

their use of AutoCAD.

Installation

------------

The basic installation process is as follows:

1.   Make sure FastTRAP driver is properly installed

2.   Install AutoCAD for an ADI digitizer

3.   Install  the  FastTRAP  ADI  driver  prior  to  running

AutoCAD

The FastTRAP ADI driver file name is DGTRAP.COM.  As with all ADI

device  drivers,  DGTRAP  is a terminate and stay resident  (TSR)

program  which provides a standardized interface between  AutoCAD

and a device specific software driver,  in this case MicroSpeed's

MAP.SYS  or MAP.COM multi-axis pointer driver.   The  MAP  driver

should  be installed for the system as described in Section 3  of

the  FastTRAP  user's manual prior to installing  the  DGTRAP.COM

driver.

Prior to using the FastTRAP ADI driver you must install AutoCAD's

digitizer  to handle an ADI digitizing device.   This is done  as

follows:

1.   Select   option   5  on  the  AutoCAD  main   menu

("Configure AutoCAD")

2.   Select   option  4  on  the   configuration   menu

("Configure  digitizer").   A  list  of  available

digitizers will be displayed.

3.   Select  "ADI digitizer" from the list of available

digitizers. (If your AutoCAD device driver library

is not in a hard disk sub-directory you will  need

to  have your a diskette containing AutoCAD device

drivers available during the installation).  Refer

to   your  AutoCAD  documentation   for   complete

information on installing device drivers.

After  AutoCAD  device  driver installation,  when you  list  the

AutoCAD  hardware configuration the digitizer should be shown  as

"ADI digitizer".

To  install the DGTRAP driver,  execute the following command  at

the DOS command prompt:

A>DGTRAP <options>

The options,  if specified,  define how DGTRAP will operate  with

AutoCAD.  The options are:

Z=MENU         Define trackwheel to select menu items

Z=GAIN         Define trackwheel to set FastTRAP X-Y

gain

nnn            A number from 1 to 100 setting the

FastTRAP X-Y-Z axis gain

The Z=MENU and Z=GAIN options are mutually exclusive - users must

select  one  or the other.   In the event that both  options  are

specified the last option on the line will be the one that  takes

effect.

The  nnn option can be specified with either the Z=MENU or Z=GAIN

options  by  following the selected option with a space  and  the

desired  gain setting.   Valid gain settings are specified  on  a

linear  scale  from 1 to 100,  with 1 being lowest gain  and  100

being highest gain.   If the nnn option is not specified the gain

is set to a default value of 25.

The default settings for the DGTRAP command is Z=MENU and gain=25

if no options are specified.  This is equivalent to the command:

A>DGTRAP Z=MENU 25

Examples:

A>DGTRAP

Set trackwheel for menu select and gain for X-Y-Z axes to 25

A>DGTRAP Z=GAIN

Set trackwheel for gain select with an initial gain setting

for X-Y-Z axes of 25

A>DGTRAP 50

Set trackwheel for menu select and gain for X-Y-Z axes to 50

A>DGTRAP Z=GAIN 50

Set trackwheel for gain select with an initial gain setting

for X-Y-Z axes of 50

NOTE:      The  DGTRAP driver must be installed prior to  running

AutoCAD.   You  may want to set up a batch file to  automatically

load the DGTRAP driver and then load AutoCAD.

OPERATION

---------

The  ADI  driver utilizes all three of the FastTRAP's input  axes

and  all three of it's input buttons to give AutoCAD  users  more

efficient control over commonly used pointing functions,  thereby

increasing   user  productivity.   The  ADI  driver  assigns  the

FastTRAP's hardware resources as follows:

Trackball:          Select pointer position

Trackwheel:         Select menu items or set X-Y gain

(see below)

Left button:        Pick item

Middle button:      Select modifiers menu

Right button:       Enter command

The  trackwheel  function can be defined to  either  select  menu

items or to set the gain of the FastTRAP trackball X-Y axis gain.

For  most users the select menu option (the default) will be  the

most useful.

The  ADI driver functional configuration is optimized to  improve

user  productivity  compared  to  other  commonly  used  pointing

devices.  For example, when using AutoCAD with a conventional two

button  mouse all menu items must be selected by first moving the

pointing  cursor  from  the  active  drawing  area  to  the  menu

selection bar and then picking the menu item; selecting modifiers

for  the  various menu options requires  additional  picking  and

pointing.   In  many  drawing applications the  continual  cursor

positioning  and  repositioning significantly  increases  drawing

time.

Using the new FastTRAP ADI driver,  menu items and modifiers  can

be  selected directly using the trackwheel and the middle button;

there  is no longer any requirement that the pointing  cursor  be

moved  from the active drawing area.   This reduces the number of

required pointer positioning and repositioning operations with  a

subsequent increase in user productivity.

For questions, contact MicroSpeed technical support: 415/490-1403

AutoDesk, AutoCAD, and ADI are registered trademarks of AutoDesk,

Inc.

FastTRAP, MicroSpeed, and trackwheel are trademarks of

MicroSpeed, Inc.

#DI ar