RBcomm v3.40  Copyright (c) 1989-1993 Ralf Brown  All Rights Reserved

You may redistribute this program provided that you provide unmodified

copies of all files, and do not charge any fee for making the copy.

--------------------------------------------------------------------------

RBcomm is a lean and mean comm program which will run in 46K without

file transfer capability or 65K with file transfer capability. Since it

is so lean, you will not get a lot of the fancy features of other comm

programs, though there are plenty of features to let you get your work

done (I've been using it exclusively for over five years, and have

added features as I've found that I could be more productive with them

than without).

Features:

small (runs in as little as 46K [65K with DSZ, 66K with Puma/MPt])

DESQview-aware

pop-up menus

'type' a file to the remote system, optionally expanding blank lines

seamless file transfer using DSZ, PCZ, or Puma/MPt

Zmodem and Puma/MPt autodownload (others easily added)

shell to DOS, using well under 1K while shelled

keyboard reassignment, powerful keystroke macros, "DOORWAY" mode

250-number dialing directory

ANSI/VT102, VT52, and AVATAR level 0 terminal emulations

[UnixWindows and AVATAR level 1 partially implemented]

ANSI music

132-column support

supports the 16550A UART's FIFOs and speeds to 115,200 bps

(a 4.77MHz 8088 can handle 19200)

scrollback buffer

Registration:

Continued use of DSZ requires registration with Omen Technology, Inc.

See the DSZ documentation for details.

Continued use of Puma/MPt requires registration with Matthew Thomas.

See the Puma or MPt documentation for details.

If you like RBcomm, send me a picture postcard of some sight in your

area.  If you don't like RBcomm, feel free to send me a postcard

anyway, telling me what you don't like.	 I just might change the

thing you don't like in the next release.

Support:

None (after all, I'm not getting any money for this).  I will try to

fix bugs as time allows.  When reporting a suspected bug, please

include as much detail as possible (such as a verbose log if it

involves the terminal emulation).

DISCLAIMER:  This software is distributed AS IS and without any express

or implied warranties.	The author disclaims all responsibility

for any damages which might be incurred as the result of using

or misusing* the program.  Although widely used by many people,

the software is not guaranteed to function on any system other

than the author's own.

*RBcomm has the ability to overwrite or delete files, which can

result in data loss if used indiscriminately.

Ralf Brown			[valid until November 1, 1993]

813 Copeland Way, Suite 26

Pittsburgh, PA 15232

Internet: ralf+@cs.cmu.edu

UUCP: {harvard,ucbvax,uunet}!cs.cmu.edu!ralf

BIT:  ralf%cs.cmu.edu@cmuccvma

FIDO: Ralf Brown 1:129/26.1

Files in the RBcomm distribution archive:

RBCOMM.DOC	this file

COMM.COM	the RBcomm main program

RBCONFIG.COM	the configuration program

MACRO.COM	the keyboard macro compiler

DVPWIDTH.COM	program to set maximum width in DESQview .DVP files

R?-PIF.DVP	DESQview program information files

*.MAC		keyboard macro definition sources

*.HLP		help screens for corresponding macro files

TERMCAP		Unix "termcap" entry for RBcomm

Availability:

The newest version is always available on:

SoundingBoard 1:129/26 File Requests

(412)621-4604

24 hours, USR HST 14.4

CS.CMU.EDU [128.2.222.173]

directory /afs/cs.cmu.edu/user/ralf/pub

You must change directly to this directory with a single

command due to the way our anonymous FTP works.

New versions will also be available here within a few days of release:

Rosedale Dataline

(301)866-4554

24 hours, USR HST 9600

TP Board

Fidonet nodes participating in DVNet

WSMR-SIMTEL20.ARMY.MIL [26.2.0.4]

directory PD:<MSDOS2.MODEM>

System Requirements:

IBM PC or close compatible

at least 46K free memory (65K for file transfers)

one or more serial ports

DOS 2.0 or higher

48-128K disk space or EMS or XMS memory for swapping, depending

on configuration

-------------------------------------------------------------------------------

Installation

------------

Before you use RBcomm for the first time, you need to tell it where to find

its support files and how to talk to the modem.	 To do so, copy the RBcomm

files to the directory in which you wish to install them, change to that

directory, and type

RBCONFIG CONFIG COMM.COM

(if COMM.COM is not in the current directory, use the full path, i.e.

C:\COMM\COMM.COM, or RBCONFIG will not be able to find it)

Please do not use your original copy.  If you forget to run RBCONFIG, you

will be told to do so when you attempt to run COMM.COM.

You will now be asked (via a menu) to fill in several groups of information.

Note that you may use either forward slashes ('/') or backward slashes

('\') to separate directories in a pathname.

Press 'D' to set the directories and extensions to use.

RBcomm dir:	where the keyboard macro files and dialing directory are stored

Swap directory: where to store the swap file when running DSZ or COMMAND.COM

and neither EMS nor XMS is available.  A RAMdisk is ideal for

storing the swap file, which will be about 64K (the exact size

depends on your setup).

Use XMS if available:  if set to N, RBcomm will always swap to disk in the

directory specified by the previous item, or EMS if enabled.

If set to Y (default), RBcomm will swap to XMS memory if there

is enough available.

Use EMS if available:  if set to N, RBcomm will always swap to either XMS

(if available) or to disk.  If set to Y (default), RBcomm will

swap to EMS memory if XMS is unavailable and there is enough

free EMS.

Macro file extension:  default extension to apply to keyboard macro files

Default macro file:	  keyboard macro file to load on startup and hangup

if this file does not exist, RBcomm will use a built-in set

of default key bindings

Press 'S' to select the serial port to use.  You may setup

configurations for "COM1" through "COM4", as well as the default port to

use when not otherwise indicated in the dialing directory. The values

given for "COM3" and "COM4" need not bear any relationship to the

numbering of the actual serial ports in your system (you could, for

example, set them up to be the same as "COM1" except for the length of

the break signal).  RBCOMM does check that "COM1" and "COM2" exist

according to the BIOS data area, however, to avoid running in a DESQview

window which has incorrect settings for supporting serial

communications.	 Break length specifies the length of the signal that is

sent when you give the break command (default Alt-B) in multiples of the

standard clock tick of 55ms.  You may specify a separate setup string

for each port.	The setup string needs to ensure that the modem echos

back any commands it is sent, and asserts carrier detect only while

actually connected with another modem (for Hayes-compatible "AT" command

sets, "E1&C1").

Press 'M' to define the modem setup.  In this section, you need to enter

several strings to be sent to the modem.  Note that you may enter control

characters such as ^M (carriage return) by pressing control-Q followed

immediately by the desired control character.  You can enter a DEL (ASCII

127) by pressing Alt-D.  Since most modems nowadays use the Hayes 'AT'

command set, the only entry you are likely to change is "dial prefix",

which will be "ATDT" for touch-tone dialing or "ATDP" for pulse dialing.

Press 'R' to define the modem's responses.  These should be the minimum

substring that uniquely identifies the response.

Press 'P' to setup dialing parameters.	You may specify the name of the

dialing directory, how long to try before declaring a time-out, and how long

to wait before trying again.

Press 'Z' to set the name of the program to run for Xmodem, Ymodem, and

Zmodem transfers, and the parameters for the various types of file

transfers.  The parameter entries all allow replaceable parameters

introduced by a percent sign--see the EXEC macro command for details.

An additional replacement of %F is available which expands to the

name(s) of the file(s) to transfer.  Note that the default "portx" will

cause DSZ to report the port as COM9, but it will still use the correct

serial port.  You need to use "portx" if you will be using more than

one serial port.

Press 'F' to setup Puma/MPt file transfers.  You may specify the name of

the program to run for such transfers, and the upload and download

parameters.

Press 'C' to set the colors you want RBcomm to use.  You may select the

default and "underlined" colors, as well as the colors to use on menus

and the scrollback viewer.

Press 'V' to determine whether 132-column mode should be enabled, whether

to start in 132- or 80-column mode, and the register values needed to set

132-column mode.  When running under DESQview, RBcomm can tell DESQview to

set the virtual screen size to 132 columns if you specify zero for AX.

DESQview will then display as much as it can at one time; this is useful

if you need a 132-column display but do not have a video board which

supports 132 columns.

Press 'T' to adjust a number of toggles.  Many of these may also be changed

from within RBcomm.

"Local echo" allows you to turn on half-duplex operation by default.

"Verbose" sets whether to include terminal control sequences in a log file.

"Strip parity bit" specifies whether the high bit should be stripped from

all incoming characters, even when parity is set to None.

"Visual bell" specifies whether to flash the screen instead of sounding a

beep when a ^G is received (internally-generated beeps always use sound).

"Should backspace send delete" specifies whether the values sent by

backspace and control-backspace should be exchanged.

"Check for enhanced keyboard" determines whether RBcomm will use the

enhanced keyboard BIOS calls if it thinks those calls exist.  Some

systems incorrectly indicate support for those calls.  Set this to

'N' if RBcomm appears to hang your system.

"May RBcomm to change NumLock" determines whether RBcomm will change the

state of the NumLock key when it receives the "keypad numeric mode" or

"keypad application mode" commands.  Disabling the NumLock changes is

useful for laptop users without a separate number pad.

"Save screen" specifies whether the current screen is saved prior to

executing external programs (except for file transfers) and restored

afterwards.  If the screen is saved, you will be asked to press a

key before the screen is restored.

Finally, press 'O' for miscellaneous options.

"Heap size" determines how much memory RBcomm allocates for macro files,

macro execution, and some of RBcomm's own processing.  This must be

set to at least 800 bytes more than the size of the largest macro

(.RBM) file.  A larger setting will not harm, but will needlessly

use more memory.	Should you get an "out of memory" or "stack full"

error message, you will need to increase this value.  When set to

zero, RBcomm will grab as much heap space as it can (about 26K).

"Number of screens" specifies how many virtual terminals to allocate

space for.  This will become useful for the UnixWindows protocol;

most users will want to set this value to 1.

"Time between sends in idle mode" determines how often Idle mode (see

Alt-I) sends some characters to keep the connection alive when you

are not actively using RBcomm.

"String idle mode sends" specifies what characters to send to keep the

connection alive.

"Answerback message" allows you to specify the string which RBcomm will

send when it receives a ^E.  The ANSWERBACK macro command can override

this setting.

"Default pace character" determines which character RBcomm will wait for

after sending each line of text from the file being typed to the

remote system.  Setting this value to ^@ means that RBcomm will not

pause after each line.  This setting may be changed while RBcomm is

running by using the PACECHAR macro command.

"Use EMS for scrollback" determines whether RBcomm will allocate 32K

of expanded memory (if available) for the pager and scrollback

buffers.	If set to Y and at least 32K of EMS is available, RBcomm

will give you a 6K pager buffer and 26K scrollback buffer regardless

of the next two settings.

"Size of non-EMS pager buffer" determines how much memory RBcomm will

allocate for the file pager's buffer when EMS is not available or

has been disabled.  Larger values can improve the pager's performance

on files with long lines, but values greater than 6-8K will not have

much of an effect unless you have an extremely large screen.  Values

smaller than the number of characters on the screen will cause

significant slowdowns when viewing files or displaying a directory,

due to thrashing as RBcomm constantly re-reads data.

"Size of non-EMS scrollback buffer" determines how much memory RBcomm

will set aside for storing text received from the serial port.  As

new text comes in, the oldest will be discarded.	Whatever text is

in the scrollback buffer may be viewed with Alt-O.

On choosing "Quit" from the menu, you will be asked whether to save the

new configuration to disk.  If you specify that you want to save the changes,

the executable on disk will be updated.

---------------------

DESQview Installation

---------------------

After performing the file copy and configuration described in the

previous section, you need to install RBcomm on the DESQview Open

Window menu.  In DESQview, tap the Alt key to bring up the DESQview

menu, then press "O" for the Open Window menu, and "AP" to add a

program.  Select "Other", then fill in the directory in which you've

placed the RBcomm files and press Enter.  You will be given the choice

of RBcomm, RBcomm plus DSZ, and RBcomm 132col; select one or more and

press Enter.  Now use "CP" from the Open Window menu to change the

paths and/or the keys for starting RBcomm.

The only difference between "RBcomm" and "RBcomm + DSZ" is that the

former allocates the minimum amount of memory for RBcomm to run (46K),

which is not enough to invoke DSZ, but does save 19K when you do not

need file transfer capability.  Should RBcomm complain about

insufficient memory (the window exits almost instantly), decrease the

heap size on the "Other options" menu in RBCONFIG.  If your environment

is particularly large, you may need to use Change a Program to increase

the window sizes by a K or two.  With certain screen sizes, you may

also need to give RBcomm 1K of system memory (on the advanced options

screen).

To get a 132-column display under DESQview, configure RBcomm to use

132-column mode and then make a .DVP file using "Change a Program"

which has all fields except for "maximum width" set to the desired

values.  Change a Program only allows 127 columns in the window, so

choose an arbitrary size smaller than that.  After pressing Enter to

save the .DVP, run the included DVPWIDTH program to change the maximum

width to 132 columns, i.e.

DVPWIDTH 132 RZ-PIF.DVP

You must give the full name of the .DVP file, including the extension.

Note that Change a Program will force you to change the window's width

if you have set the width greater than 127 and run Change a Program

again.

If you do not wish to use the DVPWIDTH program, or your version of

DESQview experiences difficulties with a 132-column setting in the .DVP

file, RBcomm can still use 132-column windows through an alternate

method.  For this alternative, you must give it enough "system memory"

to store the entire maximum-size screen.  Whatever memory is used to

store the initial screen size set under "Window Position" is then

wasted, so that size should be set fairly small (such as 15 columns by

the desired number of rows).  If you are not interested in the reason,

skip the rest of this paragraph.  DV normally reuses the same section

of memory when resizing the virtual screen.  However, resizing it

beyond the maximum defined by the .DVP causes an entire new buffer to

be allocated.  Thus, minimizing the "maximum screen size" also

minimizes the DESQview overhead by reducing the size of the screen

buffer which gets discarded.

RBcomm is sufficiently DESQview-aware to ask DV for the size of the

screen. If you want a 120x60 screen, just set the maximum window size

to 120 columns and 60 rows.  DESQview will display as much as it can on

your screen, but RBcomm will use the full size.  If you have enabled

132-column mode with RBCONFIG, RBcomm will also switch to 132 columns

by the given number of rows when it receives the sequence "<Esc>[?3h".

RBcomm refuses to load itself twice on the same serial port when

running under DESQview.  However, when shelled to DOS, you can load

another copy in a different window, but make sure to exit the second

copy before returning from the DOS shell, or the first copy will abort.

The method RBcomm uses is compatible with ONLY1.SHP using the names

"COM1" through "COM4" (depending on the serial port), so you can also

keep RBcomm from conflicting with other programs that use a serial

port, such as BBS mailers.

Finally, RBcomm returns the rest of its time-slice to DV if it doesn't

need a full slice.  This improves the performance of programs in other

windows, and makes supplemental programs such as TAME unnecessary (as

phrased by TAME's author, RBcomm is "DV-polite").

-----------------------------------------------------------------------------

Dialing Directory

-----------------

The dialing directory is a specially formatted data file (rather than a

plain text file) containing up to 250 entries.  This data file may be

created and maintained with RBCONFIG (see Editing the Dialing Directory

below).

For the convenience of those using RBcomm version 3.31 or earlier, or

those wishing to distribute dialing entries for other RBcomm users,

RBCONFIG has the capability to compile a plain text file into an RBcomm

dialing directory.  In the plain text file, each dialing entry consists

of two lines, looking like this, with no blank lines between entries:

Doctor's WOC Inn

14126214604|4800N81|B|OPUS|password|\\N2

The first line contains the description which will be used on the dialing

directory screen and when dialing.  The second line contains six fields

separated by vertical bars.  These fields are

1. number to dial (direct connection if empty)

2. modem parameters, containing in order

baud rate (110, 150, 300, 600, 1200, 2400, 4800, 9600, 19200, 38400,

57600)

parity    (N for none, E for even, O for odd, S for space,

M for mark)

data bits (5, 6, 7 or 8)

stop bits (1 or 2)

handshake to use when receive buffer is full

(H for hardware RTS/CTS, X for software XON/XOFF)

(optional, default is hardware)

timed call flag

(if handshake present and the character after the handshake

is T, a short "bip" will sound about 50 seconds into each

minute after establishing a connection.  Every five

minutes, the "bip" sounds twice)

serial port number

(if handshake is present, following either the handshake

character or the "T" timed call flag with a comma and

the number of the serial port will tell RBcomm to switch

to that port before dialing)

3. terminal emulation

A for ANSI with VT102 extensions

B for ANSI-BBS (same as ANSI, but clearing screen also homes cursor,

character set switching is disabled, and the AVATAR

level 0 command set replaces two RBcomm private

commands)

V for VT52 with H19 extensions

4. keyboard macro file to load

5. password for this system (used by PASSWORD macro command).  Display of

passwords in the dialing directory may be turned on or off with

RBCONFIG.

6. optional modem setup string, minus command lead-in defined by RBCONFIG

(may include vertical bars).	In the example above, the \\N2 tells

my modem to use MNP error correction.

If the keyboard macro file field is non-empty, the specified file will

be loaded immediately upon successfully connecting with the remote

system.  Since each entry in the dialing directory can specify an

independent set of keyboard macros, you can have a different set of

keyboard bindings for each system. If the macro file specifies a

binding for the ONLOAD "key", that macro will be executed immediately.

Further, if the macro file specifies a binding for the AUTO "key", that

macro will be executed immediately after the ONLOAD macro (if any).

The setup string may contain the following special sequences:

~    pause for half a second

\n   send a line feed

\r   send a carriage return

\f   send a form feed

\t   send a horizontal tab

\a   send a ^G

\b   send a backspace

\!   send a break

\~   send a tilde

\\   send a backslash

Example modem parameters:

1200E71X	 default COM port, 1200 bps, even parity, seven data

bits, one stop bit, use Xon/Xoff handshake when

receive buffer fills up

19200N81HT,2	 COM2, 19,200 bps, no parity, eight data bits, one

stop bit, use hardware handshake, and sound off

every minute

Compiling the Dialing Directory

-------------------------------

Once you have created a plain-text dialing file, you must compile it

into the binary form used by COMM.COM.  The directory compiler is

invoked with

RBCONFIG DIAL [-C<port>] <filename>

where <filename> is the name of the dialing directory to compile and

<port> is a number from 1 to 4 specifying which serial port is the

default port when none is explicitly given in the file.  Any extension

given with the filename is ignored; the text file always has the

extension .DIR and the compiled binary file has the extension .RBD.

If you want to add entries from a dialing file to the dialing directory

rather than creating a dialing directory from scratch, you can append

new entries to an existing directory with

RBCONFIG DIAL [-A<port>] <dialdir> <filename>

Editing the Dialing Directory

-----------------------------

To interactively edit a dialing directory, use

RBCONFIG DIAL -E <dialdir>

You may wish to read the section on the format of a plain-text dialing

file for information on the various options, and the special characters

you may use in the initialization string.

-----------------------------------------------------------------------------

Environment Variables

---------------------

RBcomm uses the following variables in its environment:

COMSPEC	  specifies which program to load for Shell-to-DOS (Alt-D) and

Execute-Program (Alt-G).

RBCOMM	  specifies default commandline options (which may be overridden by

the commandline).  See below for the valid options.  Note that only

the first 160 bytes will be used due to internal space limitations.

SWAPDIR	  overrides the default swap directory, where RBcomm stores itself

when swapping to disk.

-----------------------------------------------------------------------------

Commandline Options

-------------------

RBcomm may be invoked with one or more options, described below.  These

options are processed from left to right; if an option is present

multiple times, the rightmost instance will be the one in effect.  The

options in the RBCOMM environment variable (if present) are processed

before the commandline, and thus are overridden by the commandline.

Options are not case-sensitive.

-B	use BIOS calls to display characters received from the

serial port (aka 'port check writes').  RBcomm's own

menus and messages will still use direct screen writes.

This option is primarily for the convenience of people

using screen readers.

-Cn	select COMn, where 'n' may be 1 to 4.  This option

overrides the default set with RBCONFIG.

-Ddir	set the RBcomm directory to 'dir'.  This option overrides

the RBCONFIG default.

-Mfile	set the default macro file to use whenever not connected

to a remote system.  This option overrides the RBCONFIG

default.  Only the first 8 characters are used.

-Nfile	set the RBcomm dialing directory to 'file'.  This option

overrides the RBCONFIG default; it replaces the RBCDIAL

environment variable used by prior releases.  Only the

first 12 characters are used; if no extension is given,

it defaults to .RBD.

-Pparm	force default parameters for the current serial port.

Instead	of reading the port's parameters, RBcomm will

set them to the value specified here.  'parm' is in the

same format as used in the dialing directory, except

that the port number is not allowed.  You may intersperse

-C and -P options to set forced parameters for more than

one port.

numlst	a list of numbers to dial as if typed at the dialing

directory (see Alt-Q below) prompt.  Each number must

be separated from the next by a comma.  No blanks are

allowed, as they signal the end of an argument on the

command line.

-----------------------------------------------------------------------------

Memory Requirements

-------------------

RBcomm's memory usage is approximately

39.3K + heap size + screens + pager buffer + scrollback buffer

where

screens = number_of_screens * (max_size + 352)

max_size = greater of (2*rows*columns) for 80 and 132-column modes

pager buffer and scrollback buffer are both 0 if allocated in EMS

If there is insufficient memory, the scrollback buffer is shrunk; if there

is still not enough memory, the pager buffer is shrunk as well.

-----------------------------------------------------------------------------

Keyboard Commands

-----------------

The commands described here may be bound to other keys, but the default

keystroke named here may always be used when preceded by Alt-= (hold down

the Alt key and press the equal sign at the upper right of the typewriter

section of the keyboard).

Alt-A	attack dial

Repeatedly dials the number you select from the dialing directory

until a connection is established.  You may also manually enter a

number to attack-dial.

Alt-B	send break

Alt-C	call a number

Dial the number you select from the dialing directory (once only).

You may also manually enter a number to dial.  If you are currently

connected to another system, you will be prompted whether to hang

up.  Press Esc to abort the dial and return to the current session.

Press "Y" to disconnect and then dial a new number, or "N" to maintain

the current connection and load new macros and parameters without

dialing.

Note that the modem parameters always have an "H" appended when neither

"H" nor "X" is present in the dialing directory, since the handshake

defaults to hardware.

See also Alt-Q.

Alt-D	DOS shell

Temporarily exit to DOS.  Type "EXIT" to return to comm session.

RBcomm will swap itself out of memory, leaving only 448 bytes plus

its copy of the environment in memory (464 bytes when swapping to

EMS).  When swapping to disk, a 46K-96K swap file will be created

in the directory you specified with RBCONFIG, and will be deleted

when you return to the comm session.  File transfers also create

the temporary swap file.

Alt-E	toggle local echo

By default, RBcomm assumes that the remote system will echo any

characters you need to see.  After pressing Alt-E once, RBcomm will

display any characters you type without requiring the remote system

to echo them.  After pressing Alt-E a second time, RBcomm will once

again assume that the remote system will echo characters.

Note: you may also toggle local echo by pressing 'L' on the Alt-P

parameters menu.

Alt-F	list files

Display a three-up listing of the files matching a given filespec.

This command uses the pager, whose commands are	listed in a separate

section below.	Note that while the pager can back up past the

beginning of its buffer, doing so can be slow for large directories.

Please be patient if the screen is not updated instantly while backing

up.

Alt-G	Go execute a program

You will be prompted to enter a command.  Everything up to the first

blank is the program to execute, and the rest of the line is the

command tail to pass to the program.  If you do not specify an explicit

path, RBcomm will search your PATH for you; similarly, if you do not

give an explicit extension, RBcomm will look for both .COM and .EXE

files.

(to execute a batch file or a COMMAND.COM internal command, you should

use

"%VCOMSPEC% /c cmd"

as the implicit COMMAND.COM invocation from earlier releases has been

removed)

Alt-H	hang up

Terminate the connection.  First drops DTR, and if Carrier Detect is

still active, sends the modem's hangup string.	The default macro set

is reloaded from disk (if present) or from the built-in defaults.

If the CLEANUP "key" has a macro binding, it is executed before

hanging up; if it includes the LOWER_DTR command, DTR will remain

dropped.

Alt-I	idle

This command allows you to keep a connection alive if you need to step

away from your computer for a while.  Sends a space followed by a

backspace every two minutes (configurable with RBCONFIG) until you

press a key to cancel the command.

Alt-J	jump to directory

Pops up a window displaying the current directory.  You may switch to

a different directory by entering it in place of the displayed

directory.  Relative paths without a leading slash or backslash will

change to a subdirectory of the current directory.  If you specify a

drive letter, that drive will become the current drive as well.

Alt-K	unused

Alt-L	toggle logging to file

When pressed the first time, you will be asked for the name of a file

to which all characters received from the remote system will be

appended (percent signs introduce variable expansions--see the EXEC

and OPEN_LOG commands below).  When pressed the second time, RBcomm

will no longer append received characters to that file.	 If verbose

(see Alt-Y) is active, all characters will be appended exactly as

received, otherwise any command sequences embedded in the received

data are stripped out.

Alt-M	learn macro string

When you press Alt-M, you will be asked to enter a text string of up

to 79 characters, using the same editing keys used elsewhere.  This

text string may be replayed with Alt-N.

Alt-N	play back macro string

Plays back the last string recorded with Alt-M, sending it to the

remote system.

Alt-O	scrOllback

This command pops up the scrollback pager, allowing you to review

text which has already scrolled off the screen.	 The keystrokes

which the pager understands are listed in a separate section below.

In addition, 'M' marks the top of the currently displayed screenfull

as the beginning of the region to write when 'W' is pressed.  After

pressing 'W', you will be prompted for a filename.  The section of

the scrollback buffer between the marked top (default: beginning of

buffer) and the bottom of the currently displayed screen is appended

to the specified file.

Alt-P	set parameters

This command pops up a menu which allows you to set the following

parameters:  COM port, speed, parity, parity stripping, handshake,

backspace translation, visual bell, and terminal emulation.  You may

also reset the terminal emulation to its initial state, in case the

remote system leaves you stranded in a strange condition.  Press Esc,

Enter, or 'R' to exit this menu.

Note that Alt-P will discard any characters which have been received

but not yet processed, because it always reinitializes the serial

port when it pops down.

Alt-Q	dial queue of numbers

Repeatedly dials each number you specify until a connection is

established with one of the numbers.  The numbers are dialed in

round-robin fashion until a connection is established.	A beep will

sound to notify you of the connection, and the name of the remote

system and the modem's connect response will be displayed.

If there were multiple numbers in the previous dialing attempt,

Alt-Q will immediately continue dialing the numbers which to which

RBcomm has not yet successfully connected.

You specify the entries to dial by giving the numbers as they appear

in the dialing directory, separated by commas or blanks.  A number

may be specified more than once, and will be dialed more than once

during a round of dialing attempts.  As a shortcut, you may separate

two numbers by a dash to dial all numbers in the range, inclusive.

Alt-R	receive file

Pops up a menu allowing you to select the protocol with which you wish

to receive a file from a remote system.	 Press ESCape if you do not

wish to receive a file.	 Note that Zmodem features automatic download,

so you do not need to press Alt-R unless you wish to give DSZ

additional parameters.	You may also add additional autodownloading

protocols--see the WHEN macro command in the next section.

Not listed on the menu:	 ^G, ^Y, and ^Z pop up a parameter prompt,

then proceed with the download specified by the corresponding

non-control letter.

Alt-S	send file

Pops up a menu allowing you to select the protocol with which you wish

to send a file to a remote system.  Press ESCape if you do not wish to

send a file.  After selecting a protocol, you will be asked to enter

the name of the file to send (Ymodem, Ymodem-G, and Zmodem allow

wildcards and multiple filespecs).

If you press the control character corresponding to the desired

protocol, the file(s) will be deleted after a successful transfer.

For batch transfers, only the files corresponding to the first

filespec are deleted.

Alt-T	type ASCII file to other system

sends an ASCII file to the remote system, pausing after each line

for the return to be echoed.  The send may be aborted at any time

by pressing Escape.  The EXPAND_BLANK macro command may be used

to turn on blank-line expansion (send a single space instead of

blank lines).  This feature is off by default.

Alt-U	load/save user interface as defined in keyboard macro file

Note that keyboard macro files are always stored in the RBcomm

directory as defined by RBCONFIG.

WORDSTAR.RBM and EMACS.RBM contain the keyboard definitions for using

WordStar (tm) and Emacs cursor movement commands on the cursor pad

(i.e. Home goes to the start of the line, etc.)	 EMACS.RBM also binds

all Alt-letter keys to be Esc-letter, making the Alt key into a true

Emacs meta-key.	 You will need to use the Alt-= override to use the

RBcomm commands while EMACS.RBM is loaded.

Alt-V	View file

(toggle verbose, which used to be on Alt-V, is now on Alt-Y)

Page through a text file.  This uses a simple file lister which

only understands a few keystrokes; they are listed in a separate

section below.

Alt-W	select Which screen is visible

Displays a menu of the available screens.  Press the indicated

number to make that screen the visible screen.	If UnixWindows

is not enabled, the selected screen will also be made the

active screen (characters received from the serial port are

displayed on the active screen).  Under UnixWindows, the

visible and active screens may be different, as the active

screen is set by commands from the remote system; thus, only

the visible screen is set by this command.

Alt-X	exit

Terminate RBcomm.  If you are still connected to the remote system

(Carrier Detect asserted), you will be asked whether or not to hang

up.

Alt-Y	toggle verbose

(previously Alt-V)

Toggles the verbose mode used with Alt-L.  When not logging, the

control characters which are not used for terminal commands will be

displayed as ^X when received from the remote system.  When logging to

a file, terminal command sequences will only be stored in the log file

if verbose mode is ON.

Alt-Z	send file with Zmodem

This command will send the specified file(s) using the Zmodem

protocol.

Alt-1,2,3,4,5,6,7,8,9,0	 dial first ten numbers in dialing directory

Alt-F1 through Alt-F10	 dial 11th through 20th number in dialing directory

F1	pop up help screen

Displays the contents of the help file for the current macro file

(if that file is not found, RBcomm attempts to use RBCOMM.HLP).

Press an Alt or function key to execute the default binding for the

key.  Escape exits immediately, and any other key pages through the

help file (if more than 20 lines long).

Alt--	cancel any pending WHEN, DELAYED, or WINDOW commands

Alt-=	use default binding of next keystroke

If the keyboard has been redefined, pressing Alt-= will allow you to

access the built-in default definitions (as listed in this section).

Press Alt-= and then immediately press the key whose default

definition you want to use.  As a result, Alt-= is the only keystroke

which may not be redefined.

-----------------------------------------------------------------------------

Line Editor

-----------

Whenever you are prompted for a line of input, you may use the following

keys to edit the line:

Esc	abort

Return	finish input

Home	move to start of line

^A	move to start of line

End	move to end of line

Right	move a character right

^D	move a character right

Left	move a character left

^S	move a character left

^Right	move word right (to next blank)

^Left	move word left (to previous blank)

Del	delete the character under the cursor and shift remainder of

line left

Backsp	delete the character to the left of the cursor, and shift the

remainder of the line left

Alt-G	delete the word to the right of the cursor

Alt-H	delete the word to the left of the cursor

^End	delete from the cursor to the end of the line

^T	transpose the two characters to the left of the cursor

Alt-D	insert an ASCII DEL (127) character

Alt-P	insert the contents of the cut buffer

^Q	treat the next keystroke as a literal character to insert

-----------------------------------------------------------------------------

Pager

-----

The pager is used to display directories, files, help screens, and the

scrollback buffer.  It is fairly simple, supporting the following

keystrokes:

Up	move up one line

Down	move down one line

PgUp	move up one screenfull

PgDn	move down one screenfull

Space	move down one screenfull, exiting if already at end of

file/directory/buffer.

Right	(files only) shift the left margin right by eight spaces,

allowing you to view lines extending past the right

edge of the screen.  When the left margin is other than

zero, the indent is shown at the right edge of the status

line.

Left	(files only) shift the left margin left by eight spaces if

it is not already zero.

Home	restart the pager.  For files and directories, you will

be put back at the beginning of the file; for the scroll-

back buffer, you will be placed at the end of the buffer.

Esc	exit from the pager.

The scrollback pager adds the following keys to the above standard keys:

M	mark the top of the region to write to a file

W	write the marked region to a file

The dialing directory adds the following keys:

0-9	start entering the number(s) to dial

M	specify a telephone number to dial manually

R	recall the last list of numbers to dial

-----------------------------------------------------------------------------

Error Messages

--------------

file not found

RBcomm was unable to open the requested file.  This may be due to

wildcards or a misspelling in the filename, an invalid path,

a lack of file handles (see FILES= in your DOS manual), or a file

sharing conflict.

invalid macro file

the macro file you wanted to load is either corrupted or from an

different version of RBcomm which uses an incompatible format for

macro files.  Try recompiling the macro file with the same version

of the compiler as the version of RBcomm you are using.

MODEM NOT RESPONDING

RBcomm did not get an acknowledgement from the modem within four

seconds of the last command.  If you only get this message on attack

or list dialing, you need to increase the delay before redialing

with RBCONFIG (under "dialing parameters")--some modems take longer

to reset after a hangup command than others.

--out of memory--

There was not enough memory to pop up the requested menu or prompt.

Increase the heap size with RBCONFIG and try again.

STACK FULL

Your macro was so deeply nested that it ran out of space.  This can

be caused by an infinite recursion in your macro.  Increase the heap

size with RBCONFIG (under "Other options") and try again.

-----------------------------------------------------------------------------

Warnings

--------

Don't try to load a TSR while shelled to DOS.  RBcomm won't be able to swap

itself back in and will have to abort.	Your connection will not be broken,

however.

Don't mess with the swap file while shelled.  You'll be sorry....  (RBcomm

is able to detect a missing or truncated swap file on returning, but not

other changes)

IMPORTANT:  if you are swapping to a floppy drive, DO NOT UNDER ANY

CIRCUMSTANCES replace that floppy disk while shelled to DOS or running a

file transfer.	You will definitely trash the file allocation tables and

directory of the second disk.  This is a problem with DOS itself (at least

through 3.10, later versions may have corrected it) when faced with a

diskette change while a file is open on the diskette which gets removed.

The swap file is such an open file.

-----------------------------------------------------------------------------

Known Bugs and Limitations

--------------------------

If you are using a serial port which the BIOS does not recognize, RBcomm

will most likely read garbage parameters from the port at startup, requiring

you to set the parameters by hand with Alt-P or the -P commandline option.

The pager has trouble with extremely long lines (>500 columns or so).

When encountering such a line, you may not be able to move up or down by

single lines.  PgUp and PgDn will still work, however.

RBcomm requires that the EMS page frame contain at least three pages

in order to use EMS for the scrollback and pager buffers, and at least

one page in order to swap to EMS.  This is important when running under

memory managers such as QEMM-386 which allow page frames smaller than

the standard four pages.

-----------------------------------------------------------------------------

Macro Compiler

--------------

Syntax:	 MACRO srcfile [srcfile ...]

compiles each of the specified source files in turn to a macro file

with the same name and the extension .RBM.  If no extension is

specified for a source file, the default of .MAC is used.

Important Note:

RBcomm v3.4 and later macro files are INCOMPATIBLE with

earlier versions.  You must recompile all your macro files

if RBcomm complains about invalid macro files.

-------------------

Macro File Commands

-------------------

Each command has the following form:

keyname	 commandword   [args]

where keyname specifies to which key the command is to be bound.  Keynames

(which are not case-sensitive) are:

F1 through F12 for the function keys

+F1 through +F12 for the shifted function keys

^F1 through ^F12 for the control function keys

@F1 through @F12 for the alt-function keys

@A through @Z for the alt-letter keys

@1 through @0 for the alt-digits

Gray+, Gray-, Gray*, Gray/ for plus, minus, star, slash keys on keypad

@Plus and @Minus, @Gr*, @Gr/ for Alt-Gray+, Alt-Gray-, Alt-Gray* and

Alt-Gray/

Left, Right, Up, Down for the cursor keys on the numeric pad

Home, End, PgUp, PgDn, Ins, and Del for the other keypad keys

KP5 for the '5' key on the number pad (enhanced keyboard)

^Home, etc. for the control versions of the numberpad keys

CP_Home	 Home key on gray cursor pad

CP_Up	 Up arrow on gray cursor pad

CP_Down	 Down arrow on gray cursor pad

CP_Enter	 Enter key on cursor pad

etc.

@Left, @Right, @Up, @Down, @Home, @End, @PgUp, @PgDn, @Ins, @Del for

Alt- versions of the cursor pad keys

^Left, ^Right, etc for Ctrl- versions of the cursor pad keys

^Break	 for control-break

@- @= @[ @\ @] @; @' @, @. @/ @` @* for various other Alt-keys

There are also some pseudo-keys to which you may assign a macro:

"OnLoad" is executed any time the macro file is loaded into memory

"AutoDL" is executed whenever the autodownload mechanism is

initialized.  See the AUTO_XFER command for details.

"Auto" is executed automatically when a connection is established

"Reconnect" is executed automatically when reconnecting without

hanging up and redialing

"CleanUp" is executed when hanging up if carrier detect is active

"OnAbort" is executed whenever a macro other than OnAbort aborts and

displays the **ABORTED** message, just prior to displaying

the message

"Alarm" is executed whenever a connection is established with a remote

system or when a file transfer (including TYPEing a file)

completes.  If not bound, a simple beep is generated.

And there is a no-operation pseudo-key for use by FORCE_CLEANUP:

"Null" does nothing and returns immediately

Finally, there are two sets of pseudo-keys which can be used to

override the default settings for file transfers.

"#upload_x" will override the command string for the upload

protocol with letter 'x' on the upload menu.

Format:  #upload_z "upload command string"

"#download_x" will override the command string for the download

protocol with letter 'x' on the download menu.

Format:  #download_z yes|no "download command string"

The first parameter indicates whether or not RBcomm

should prompt the user for a filename or parameters

(which may be inserted in the download command using

the %F substitution--see EXEC).

You may assign macros to control keys by specifying "^x" as the key name,

where "x" is the control key you want to use.  Similarly, you may assign a

macro to a regular key by preceding the character representing the key by a

backslash (i.e. \A for capital 'A', \a for lowercase 'a'). Note that macros

are never in effect when you are prompted for information by RBcomm; they

only affect what (if anything) gets sent to the remote system.

Finally, you may use "#nnn" where nnn is the decimal scancode of the key

to which you wish to assign the macro (values of 224 through 255 are reserved,

167 through 223 are currently unused by any key combinations supported by

the IBM ROM BIOS).

Note that the cursor pad keys will execute the binding for the

corresponding number pad key if not specifically bound (but not vice

versa).	 CP_Enter defaults to the binding for @Enter.

The commandword is not case-sensitive, but you must include the underscore

if present and use the entire commandword (abbreviations are not

supported).

With the exception of the ANSWERBACK, TEST, TEXT, WAITFOR, and WHEN

commands, all commands which take a string argument pop up a prompt if the

string is empty (i.e. "").  Strings may include the following special

sequences:

^x   specified control character

^?   ASCII DEL (127)

\a   ^G (bell)

\b   backspace

\e   escape

\f   form feed

\n   line feed

\r   carriage return

\t   horizontal tab

\v   vertical tab

\\   backslash

\^   carat

\0   ASCII NUL (for TEXT command only)

For character arguments, you may specify either

'c'

where the character c is interpreted as for a string, or you may specify its

ASCII value.

Everything from a semicolon (unless it is in a string) to the end of the line

is considered a comment, as are blank lines.

If a line starts with #include rather than a key name, the specified file

will be compiled into the macro file as if it were part of the current text

file.  Note that the string is processed just like any other strings, so

that you must double any backslashes (or use forward slashes, instead).

Format:

#INCLUDE  "d:/path/filename"

an extension of .MAC is assumed if no extension is given, and the drive

letter and path are optional (default is current drive and directory).

#INCLUDEs may be nested up to twelve deep.

If a line starts with #ignore rather than a key name, any subsequent

occurrences of the keyname following the #ignore will be ignored, just as

if you had already defined a binding for the key.  This is useful when

#include'ing a file if you do not want all the bindings in the included

file.

Format:

#IGNORE	<keyname>

For readability, you may give names to extended keys with the

#DEFKEY <newname> <keynumber>

command.  For example, "#defkey Proc1 199" will let you refer to

"Proc1" and "#199" interchangeably.

ABORT

abort the current macro or operation as if the user had pressed Esc.

ABORT_UNTIL

Abort any currently executing UNTIL command after the command(s) it

controls complete.  If multiple UNTIL commands are nested, each is

aborted as the commands it controls complete, until all pending

UNTILs have been aborted.  Useful mainly in conjunction with the

DELAYED command to provide a timeout on UNTIL loops.

See also DELAYED, UNTIL.

ALARM

Execute the macro bound to the "Alarm" pseudo-key.  If Alarm is not

bound, this command is equivalent to BEEP.

See also BEEP, SOUND.

ANSWERBACK "string"

sets the answerback message to the specified string.  If the string is

empty (i.e. ""), answerback is disabled (making ^E a cursor-positioning

command), otherwise, answerback is enabled.

AT hh:mm:ss

execute the following line (or lines if a MULTI) at the specified

time.  If the specified time is less than the current time, the

command(s) will be executed the next day.  The seconds may be

omitted, in which case they default to 0.

Note: the command(s) may actually execute later than requested if

RBcomm is busy processing incoming characters at the time the

command should execute.  The command will execute as soon as RBcomm

empties its receive buffer.

See also DELAYED.

AUTO_XFER

reenable autodownloads/autouploads if they have been canceled by

an ENDWHEN ALL_XFER.  If the AutoDL pseudokey has a binding, it

is executed; otherwise, a default routine equivalent to

WHEN 0 "**^XB00"

RECEIVE 'Z'

is executed.  The command ENDWHEN ALL is implemented as an

ENDWHEN ALL_XFER followed by AUTO_XFER.

Further, the dialer does an ENDWHEN ALL_XFER followed by an

AUTO_XFER, so the AutoDL pseudokey will allow autodownloads to

be preserved across a dial attempt which does not cause a new

macro file to be loaded.

The autodownloads are also reenabled with AUTO_XFER when hanging

up and immediately after loading a new macro file.

Note: Puma autodownload may be enabled with the following lines

as part of the AutoDL pseudokey:

WHEN 0 "^X^H^XPuma^X^H^X"

RECEIVE 'P'

Although I have not yet seen MPt (the renamed Puma), I understand

that its autodownload string has changed, requiring

WHEN 0 "^V^H^VMPt ^V^H^V"

RECEIVE 'P'

WARNING: do not invoke while autodownload is enabled, as you will wind

up invoking a download multiple times for one download request....

See also ENDWHEN, WHEN.

AVATAR ON|OFF

Specify whether AVATAR command sequences should be honored by

the terminal emulator.

See also RBCOMM_CMDS.

AWAITKEY

pause until a key is pressed.  The next function which reads a key

will read the key which was pressed.  Returns immediately if there

was any typeahead (including RBcomm's key stack).

See also KFLUSH.

BEEP

sound the bell.	 Useful mainly in conjunction with the MULTI or

DELAYED commands (see below).

See also ALARM, SOUND.

BREAK

send a break to the remote system

CALL keyname

execute the macro (if any) bound to the specified key and then continue

executing the current macro (if inside a MULTI).

See also GOTO_KEY, PUSHKEY.

CANCEL_DELAYED which

cancel the specified delayed action.  Currently supported:

ALL    cancel all delayed actions

LAST   cancel the last executed DELAYED.	 If called a second

time without an intervening DELAYED, the second call

is ignored.

See also DELAYED.

CANCEL_NOTIFY

remove the notify window immediately instead of at the end of the

configured period of time.

CHDIR "dir"

change the DOS default directory to the specified directory.

CLOSE_LOG

close the current log file.  If logging is off, this command has no

effect.

See also OPEN_LOG, TOGGLE_LOG.

CUT num dir "search"

search the screen backwards from the current cursor position until

the search string (case sensitive) is found, then copy "num"

characters in the indicated direction (AFTER or BEFORE) to the cut

buffer.	 The contents of the cut buffer remain unchanged if the search

string is not found (in which case the completion status is set to

FAILED for testing by IF or UNTIL).  The cut buffer may be sent to

the remote system with the PASTE command, inserted into a line

being edited by pressing Alt-P, or inserted with the %C variable

expansion.

Note that trailing blanks are deleted from the cut buffer.  In

addition, the CUT command treats the entire screen (even if a WINDOW

command was used to restrict output to a portion of the screen) as a

single long line, ignoring all line wrapping for both search and cut.

Examples:

Screen contains "1234test5678cursor here->" and the cursor is as

indicated.  Then

CUT 3 BEFORE "test"

places "234" into the cut buffer, while

CUT 6 AFTER "test"

places "5678cu" into the cut buffer.	 Further,

CUT 7 BEFORE ""

will place " here->" into the cut buffer.

See also PASTE, TRIM.

DELAYED time

execute the macro on the next line (or lines if it is a MULTI) the

given amount of time from now.	You may have up to six delayed

commands active at any given time.  The maximum delay time is 18

hours; the macro compiler will complain if you attempt to use a

greater delay.	Times are specified as

seconds

minutes:seconds

or	hours:minutes:seconds

Note: the command may actually execute later than requested if RBcomm

is busy processing incoming characters at the time the command should

execute.  The command will execute as soon as RBcomm empties its

receive buffer.

See also CANCEL_DELAYED, WHEN.

DIAL number redial

dial the specified entry (0-39) from the dialing directory, or pop up

the dialing directory if 'number' is PROMPT.  'redial' specifies how

to dial:

ONCE	make only one attempt

ATTACK	try repeatedly until connected

RECONNECT	make a single attempt if not connected, or set

parameters and password without dialing if already

connected

If the number is PROMPT, the user may enter multiple numbers.  If

'redial' is RECONNECT, all but the first number will be ignored.

Otherwise, the entire list will be tried either once or until a

successful connect.

See also LISTDIAL, MDIAL.

DISPLAY "msg"

Display the variable-expanded version (see EXEC for details) of the

given string at the current cursor position, and update the cursor

position to be at the end of the string.  This is similar to the

MESSAGE command, but it always displays at the current position;

MESSAGE also does not change the cursor position.  The characters

^A through ^F and ^H in the string invoke special actions.  ^C clears

from the current position to the end of the line while ^H backs up

one column if not already at the left edge; the other special

characters are detailed under "Writing your own Help FIles".

See also MESSAGE.

DVEXEC ON|OFF "DVP-command"

Start a program in a separate DESQview window.  The first

parameter specifies whether RBcomm should keep its serial port

routines active; if OFF, the other program may safely perform

serial I/O and RBcomm will wait until it terminates (if ON,

RBcomm will continue immediately, but the new program may not

take over the serial port interrupt without disrupting RBcomm).

The second parameter specifies the full name of the .DVP file

to be launched and optionally commandline parameters to

override those stored in the .DVP.

Example:

DVEXEC OFF "C:/DV/OZ-PIF.DVP /X"

might call the OZSMALL program to perform a CompuServe

B+ file transfer, forcing its commandline to "/X"

(meaning exit on completion).

ECHO ON|OFF

ECHO ON forces half-duplex (local echo on) mode, ECHO OFF forces

full-duplex (local echo off).

(see Alt-E)

See also TOGGLE_ECHO.

ENDWHEN which

cancel the specified WHEN command.  Currently supported:

ALL	       cancel all WHENs except the autodownloads

ALL_XFER    cancel all WHENs including the autodownloads

LAST	       cancel the last executed WHEN.  A second call without

an intervening WHEN acts the same as ENDWHEN ALL.

Note: ENDWHEN ALL is implemented as an ENDWHEN ALL_XFER followed

by an AUTO_XFER.

See also AUTO_XFER, WHEN.

EXEC "command-to-execute"

everything up to the first blank in the string specifies the program to

execute (the PATH will be searched if no explicit path is given, and

both .COM and .EXE will be looked for if there is no explicit

extension), the remainder of the string is the command tail to pass to

the program.  Note that a following IF command will always consider

the EXEC to have succeeded if you use COMMAND.COM to run the program

with the construct

"%VCOMSPEC% /c cmd"

due to a misfeature of COMMAND.COM.

If SAVE_SCREEN is ON, RBcomm will prompt for a key, then restore

the screen to the state it was in before the EXEC.  No message will

be displayed if the program returns an error; however, an error

message will still be displayed if there was an error while attempting

to load the program.

Within the command, a percent sign introduces a variable substitution.

The following sequences are supported:

%a	 hexadecimal I/O base address being used

%C	 contents of the cut buffer (see CUT and PASTE)

%d	 current date, in format mm-dd-yy

%D	 current directory, including drive

%F	 file(s) to transfer; this substitution is only meaningful

for a file transfer parameter string, and has an undefined

action elsewhere

%i	 IRQ number being used (0-15)

%I	 get input from user.  The following characters up to the next

percent sign are used as the prompt and then discarded.

%M	 current macro file name

%N	 name of current remote system (from dialing directory)

%p	 port number (1-4)

%P	 port parameters, in same format as dialing directory

display

%s	 current serial port speed

%t	 current time, in format hh:mm:ss

%V	 get an environment variable.  The following characters up to

the next percent sign are used as the name of the variable

and then discarded.

%w	 current switch character

%%	 a percent sign

%/	 a forward slash unless the last character of the expansion

for the string up to this point is a slash or backslash

%\	 a backslash unless the last character of the expansion

for the string up to this point is a slash or backslash

See also EXECN, IF, SHELL.

EXECN  "command-to-execute"

This command is identical to EXEC, except that no error box is

displayed in the event of an error.  If SAVE_SCREEN is ON, the

screen is restored without pausing for a keystroke.

See also EXEC.

EXIT errorlevel

End RBcomm.  Will prompt you whether or not to hang up if you are

still connected to another system.  Returns the specified errorlevel

to DOS; usually this should be zero, but other values may be used

to control batch file execution.

EXPAND_BLANK ON|OFF

Specify whether RBcomm should send a single blank instead of a

completely blank line when TYPEing a file (see Alt-T and TYPE).

If ON, a blank will be sent to separate two consecutive

carriage returns (TYPE converts both CRLF and LF to CR to

simulate keyboard input); if OFF, no extra blanks will be

generated regardless of the data in the file.

See also TYPE.

FDELETE "filespec"

delete the file(s) named by filespec (which may include wildcards).

Note that there is no confirmation requested, so use this command

with care!!!

See also FILES.

FILES "filespec"

display a three-up list of the files matching the filespec.  This

uses the file pager described above.

See also FDELETE.

FORCE_CLEANUP keyname

execute the following line(s), then execute the macro bound to

"keyname" regardless of how the commands are exited (i.e. normally,

ABORT, or <Esc>).  FORCE_CLEANUPs may be nested, in which case

the innermost one is the only one to execute unless the cleanup

code executes an ABORT itself (then execution gets passed to the one

enclosing it, etc.).  If no cleanup is needed and the FORCE_CLEANUP

is merely being used to prevent a complete abort on Esc, the pseudo-

key Null may be used.  Null is guaranteed to perform no action and

return immediately.

GOTO_KEY keyname

continue execution with the specified macro (if bound).	 Never returns

to the current macro.  If the key is not bound (i.e. GOTO_KEY Null),

execution resumes at the point from which the current macro was

CALLed, effectively producing a return from a nested macro.

See also CALL, PUSHKEY.

HANGUP

hang up.  Executes macro bound to "CleanUp" before actually hanging

up if carrier detect is active.	 After hanging up, the default macro

file is reloaded (thus cancelling any remaining WHENs and DELAYEDs).

See also HANGUP_ONLY.

HANGUP_ONLY

hang up, but do not execute the "CleanUp" macro or reload macro files.

See also HANGUP.

HELP

pop up the help screen.	 If you currently have the macro file FOO

loaded, RBcomm first attempts to read the file FOO.HLP in the RBcomm

directory.  If it is unable to do so, it then attempts to read

RBCOMM.HLP.

While viewing the help screen(s), you may enter an Alt-key command.

This will immediately exit the help system and execute the command

right away.  If you press Alt=, that fact will be remembered, and

the Alt= will be used to quote the next Alt-key command you press

while in the help system.

IDLE

go into idle mode, sending a blank followed by a backspace every 2

minutes.  Press Esc to end idle mode.

Both the interval and the sequence sent by idle mode may be configured

with RBCONFIG.

IF	SUCCESS|FAILED|CONNECTED|OFFLINE|DV|NOT DV

IF SUCCESS and IF FAILED conditionally execute a command based on the

completion status of an immediately preceding EXEC, SHELL, SEND,

SENDFILE, RECEIVE, RECEIVEFILE, or WAITFOR.  The following line (or

lines if a MULTI) is executed only if that prior command was

successful in the case of IF SUCCESS, or if it failed in the case of

IF FAILED.

IF CONNECTED executes the following command if carrier detect is

asserted, while IF OFFLINE executes the following command if

carrier detect is deasserted.

IF DV executes the following command if RBcomm has detected that it

is running under DESQview, while IF NOT DV executes the following

command only if RBcomm is not running under DESQview.

For example, if you want to execute a program only after failing to

see a particular string, you would use

MULTI

WAITFOR 10 "don't run program"

IF FAILED

EXEC "someprog args"

END

See also ABORT_UNTIL, EXEC, SHELL, SEND, RECEIVE, WAITFOR, UNTIL.

KFLUSH

empty the keyboard buffer (including RBcomm's key stack).

See also AWAITKEY, TFLUSH.

LEARN

start learning a macro string.	The recorded string may be sent to

the remote system with RECALL.

See also LOAD_MACRO, RECALL.

LISTDIAL "numbers"

dial the specified numbers from the dialing directory repeatedly until

a connection is established with one of the numbers.  If the empty

string is specified, the dialing directory is popped up and the

previous list of numbers (less the system to which RBcomm

established a connection) is recalled.

Each of the numbers in the list must be separated from the previous

number by one or more spaces or commas.	 For example,  "1,5 7,2 4"

would attempt to dial entry 1, then entry 5, then 7, then 2, and

finally entry 4 before repeating.  Any numbers in the list which are

higher than the highest-numbered entry in the current dialing

directory are silently skipped.

See also DIAL, MDIAL.

LOAD_MACRO "file"

load the specified macro file from the RBcomm directory.  If an empty

filename is given, the user will be prompted for a name, which

defaults to the name of the currently loaded macro file.  The current

macro (if any) and any that it had interrupted are aborted.  However,

if the interruption occurred during a command of extended duration

such as a WAITFOR, that command will complete normally before the

macro is aborted.

See also LEARN, RECALL.

LOG "filename" "message"

append the specified message to the named file.	 Both the filename

and the message may contain variable expansions as described under

EXEC.  If the filename is (or expands to) the empty string "", the

expanded message is appended to the current log file (if any).

See also OPEN_LOG.

LOWER_DTR

requests that DTR remain dropped after the next HANGUP or HANGUP_ONLY

command.  The best place to put this command is in your CleanUp macro.

LTRIM CUTBUFFER "trimchars"

remove from the left (beginning) of the cut buffer (see CUT) the

longest sequence of characters consisting entirely of characters in

"trimchars".  A future version of RBcomm supporting string variables

will permit trimming more than just the cut buffer.

Example: if a previous CUT command has placed "12  A22B  34" into the

cut buffer, the command

LTRIM CUTBUFFER " 1234"

will result in "A22B  34" becoming the contents of the cut buffer,

available for the next PASTE command or @P in the input editor.

See also CUT, RTRIM, TRIM.

MDIAL "number" ONCE|ATTACK

dial the specified phone number as if it had been entered as the

manual number for the dialing directory display.

See also DIAL, LISTDIAL.

MESSAGE row col msg

Display the result of expanding any variables in the specified message

(see EXEC for details) starting at the given row and column on the

screen.	 If either row or column is 255, use the current row or

column.	 Does not change the current cursor position, unlike the

DISPLAY command.  As with DISPLAY, the characters ^A through ^F and ^H

specify special actions.

See also DISPLAY.

MESSAGEBOX msg

if running under TopView or DESQview, a window with the given message

will pop up for three seconds.	This command is ignored otherwise.

The message has any variables expanded, and the first 40 characters

are displayed in the pop up window.  See EXEC for the details of

the variable expansion.

See also NOTIFY, CANCEL_NOTIFY.

MULTI

the following lines, up to but not including a line starting with END,

are assigned to the specified key.  The macros assigned to the key

may not total more than 253 bytes, and the macro compiler will

complain if they do.

The format of the following lines is the same as for a regular macro,

except that you do not specify a keyname.  For example, to make

shift-F2 switch terminal emulation to VT52, use

+F2	MULTI

PUSHKEY 13 0

PUSHKEY 'V' 0

PARAM_MENU

END

You may also use { and } on separate lines as synonyms for MULTI

and END.  The braces are preferred, as MULTI/END will be phased out

in future versions.

MUSIC ON|OFF

specify whether ANSI music should be enabled while using the

ANSI BBS terminal emulation.  This switch is necessary because

the command to play music conflicts with a standard ANSI

control sequence (Esc [ M).

See also PLAY, SOUND.

NOTIFY	msg

if running under TopView or DESQview, and RBcomm determines that it is

in the background, a window with the given message will pop up for

three seconds.	This command is ignored otherwise.

The message has any variables expanded, and the first 40 characters

are displayed in the pop up window.  See EXEC for the details of

the variable expansion.

See also CANCEL_NOTIFY, MESSAGEBOX.

OPEN_LOG  "file"

if logging is on, the current log file will be closed.	Then, the

specified log file will be opened (if no name is given, a prompt is

popped up).  The same variable expansions which are valid for

the EXEC command are applied to the filename which is given (or

entered by the user).

For example, to allow the environment variable RBLOG to specify the

base name of the log file, use

OPEN_LOG "%VRBLOG%.LOG"

See also EXEC, CLOSE_LOG, TOGGLE_LOG, RECEIVE.

PACECHAR 'c'

set the pace character for the TYPE command to 'c'.  RBcomm waits

up to three seconds for the pace character when TYPEing a file to

the remote system.  RBcomm will not wait if the pace character is

ASCII 0 (^@).

See also TYPE.

PARAM_MENU

pop up the "Set Parameters" menu

PASSWORD

send the password defined in the dialing directory.  Does not send

a carriage return or any other terminating sequence.

See also TEXT, TYPE.

PASTE

send the contents of the cut buffer to the remote system.

See also CUT.

PAUSE num-ticks

halt execution for num-ticks/18 seconds.  Useful mainly in conjunction

with the MULTI command (see above).

PLAY "play-string"

play the notes specified by the string.	 The string may contain

the following directives:

An

Bn

Cn

Dn

En

Fn

Gn	play the given note, 'n' specifies duration as

a fraction of a whole note, defaults to that

given by the last L command.  A '#' or '+'

between the letter and 'n' produces a sharp

note, while a '-' produces a flat note.

Ln	specify the default length of notes as 1/n of a

whole note

Mx	specify music type, x=L, N, or S

L)egato: note uses entire period

N)ormal: note uses 7/8 of its time period

S)taccato: note uses 3/4 of its time period

Nn	play note number 'n', from 1 to 95

On	specify octave number, 0 to 9.	O3 starts at

middle C

Pn	rest for the specified period (1/n of a whole

note).	If 'n' is not present, use the length

given by the last L command

Tn	specify tempo, 32 to 255

.	following A-G, N, or P, increases the note's

length by 1/2.

See also BEEP, SOUND.

PUSHKEY key scan

or

PUSHKEY keyname

put the specified key/scancode pair onto RBcomm's internal keyboard

stack, where the last pushed pair will be the next keystroke read

by RBcomm.  The stack is currently eight keystrokes in size, and any

PUSHKEY executed while the stack is full will simply be ignored.

"keyname" may be the name of any keystroke recognized by the macro

compiler.  <scan> is 0 for regular ASCII characters, and 1 for

extended ASCII codes (i.e. function keys).

Useful mainly in conjunction with the MULTI command (see above).

RBCOMM_CMDS ON|OFF

Specify whether the terminal emulator should honor RBcomm

private commands.

See also AVATAR.

RECALL

send the last string recorded with LEARN to the remote system.

See also LEARN.

RECEIVE 'type'

start receiving a file with the specified protocol

0	   prompt for protocol

'A'  ASCII (capture to file--equivalent to OPEN_LOG "")

'X'  Xmodem with parameters

'K'  Xmodem-K with parameters

'Y'  Ymodem

'^Y' Ymodem with parameters

'G'  Ymodem-G

'^G' Ymodem-G with parameters

'Z'  Zmodem

'^Z' Zmodem with parameters

See also IF, OPEN_LOG, SEND, TYPE.

RECEIVEFILE 'type' "parameters"

start receiving using the specified protocol (which may be any of the

ones listed for RECEIVE except 0 or 'A'), passing the specified

parameters to the file transfer module.	 Unlike RECEIVE, this command

will not pop up any messages unless it is unable to load the file

transfer module; in case of an error, the completion status is set

to FAILED.

See also RECEIVE, SENDFILE.

REPEAT times

repeat the command on the next line (or lines if a MULTI) the specified

number of times.

RFLUSH

clear any characters which may still be in the serial port receive

buffer.

See also KFLUSH, TFLUSH.

RTRIM CUTBUFFER "trimchars"

remove from the right (end) of the cut buffer (see CUT) the longest

sequence of characters consisting entirely of characters in

"trimchars".  A future version of RBcomm supporting string variables

will permit trimming more than just the cut buffer.

Example: if a previous CUT command has placed "12  A22B  34" into the

cut buffer, the command

RTRIM CUTBUFFER " 1234"

will result in "12  A22B" becoming the contents of the cut buffer,

available for the next PASTE command or @P in the input editor.

See also CUT, LTRIM, TRIM.

SAVE_MACRO "file"

no longer supported.

SAVE_SCREEN ON|OFF

determine whether or not to save the current screen while executing

external programs (other than file transfer).  The state of this

flag slightly modifies the behavior of EXEC, EXECN, and SHELL.

See also EXEC, EXECN, SHELL.

SCROLLBACK

bring up the scrollback pager.	In addition to the standard keystrokes

understood by the pager, 'M' marks the top of the currently displayed

screenfull as the beginning of the region to write when 'W' is pressed.

After pressing 'W', you will be prompted for a filename.  The section

of the scrollback buffer between the marked top (default: beginning of

buffer) and the bottom of the currently displayed screen is appended

to the specified file.

SELECT_SCREEN num

select a new screen to be the visible and active screen.  If "num" is

PROMPT, displays a menu of the available screens.

SEND 'type'

prepare to send a file with the specified protocol

0	   prompt for protocol

'A'  ASCII

'X'  Xmodem

'K'  Xmodem-K

'Y'  Ymodem

'G'  Ymodem-G

'Z'  Zmodem

control character versions of any of the above ('^X', '^Y', etc) will

delete the file(s) corresponding to the first filespec after sending.

See also IF, SENDFILE, RECEIVE, TYPE.

SENDFILE 'type' "file"

send the specified file with the given protocol, which may be any of

the ones listed for SEND except 0 or 'A'.  Unlike SEND, this command

will not pop up any messages unless it is unable to load the file

transfer module; in case of an error, the completion status is set

to FAILED.

See also RECEIVEFILE, SEND.

SHELL

shell to DOS.  If SAVE_SCREEN is ON, the original screen will be

restored when you exit from the subshell.

See also EXEC, IF.

SOUND freq dur

Generate a tone with a frequency of <freq> Hertz for <dur> clock

ticks (max 255).  If the frequency is 0, a musical pause of the

specified duration is created.	If both frequency and duration are

zero, all queued tones are discarded (applies only to DESQview at

this time).

See also BEEP, PLAY.

TEST EXISTS "filespec"

determine whether any files matching the filespec exist.  Sets the

status to SUCCESS if at least one exists, FAILED if none.  The status

may be checked with IF or UNTIL commands.

See also TEST WHEN, IF.

TEST WHEN "str"

determine whether a WHEN with the specified search string is currently

active.	 Sets the status to SUCCESS if at least one exists, FAILED if

there are no WHENs with the specified string.  The status may be

checked with IF or UNTIL commands.

See also IF, WHEN, TEST EXISTS.

TEXT "msg"

send the specified characters out the comm port.  "msg" is limited to

253 characters.

TFLUSH

discard any characters queued for transmission to the remote system

which have not yet been sent.

See also KFLUSH, RFLUSH.

TOGGLE_ECHO

toggle the state of local echoing

See also ECHO.

TOGGLE_LOG

if logging is currently on, turns off logging received data to file

if logging is off, pops up a prompt for the name of the capture file

See also OPEN_LOG, CLOSE_LOG.

TOGGLE_VERBOSE

toggle the state of verbose mode (see Alt-Y)

See also VERBOSE.

TRANSLATE_BS ON|OFF

TRANSLATE_BS ON turns on backspace/delete swapping, TRANSLATE_BS OFF

turns it off.

TRIM CUTBUFFER "trimchars"

remove from both beginning and end of the cut buffer (see CUT) the

longest sequence of characters consisting entirely of characters in

"trimchars".  A future version of RBcomm supporting string variables

will permit trimming more than just the cut buffer.

Example: if a previous CUT command has placed "12  A22B  34" into the

cut buffer, the command

TRIM CUTBUFFER " 1234"

will result in "A22B" becoming the contents of the cut buffer,

available for the next PASTE command or @P in the input editor.

See also CUT, LTRIM, RTRIM.

TYPE "file"

send the specified file to the remote system as a stream of ASCII

characters.  Pauses at the end of each line and waits for the pace

character to be echoed (will continue after three seconds even if

the echo is not seen).	Equivalent to

SEND 'A'

if no filename is given.

See also EXPAND_BLANK, PACECHAR, SEND.

UI_MENU

No longer supported.  Use LOAD_MACRO "" instead.

See also LEARN, LOAD_MACRO.

UNTIL SUCCESS|FAILED|CONNECTED|OFFLINE

repeatedly executes the following line (or lines if a MULTI) until

the status of the last action is either success for UNTIL SUCCESS

or failure for UNTIL FAILED.   UNTIL CONNECTED repeatedly executes

the following command until carrier detect becomes active, while

UNTIL OFFLINE repeats until carrier detect drops.

See also ABORT_UNTIL, IF.

VERBOSE ON|OFF

VERBOSE ON turns on verbose mode, VERBOSE OFF turns off verbose mode.

(see Alt-Y)

See also TOGGLE_VERBOSE.

VIEW "filename"

display the specified file one screenfull at a time.  Pops up a prompt

if the null filename is given.

Pressing the space bar displays the next screen, Escape exits the

file view.

WAITFOR timeout "wait-string"

wait up to timeout seconds for the specified string of characters to

arrive over the comm port.  Useful mainly in conjunction with the

MULTI command (see below).

You may test whether the string was actually received by following

the WAITFOR with IF SUCCESS or IF FAILED (unlike versions prior to

3.11, a failed WAITFOR does not abort the MULTI it is a part of).

See also IF.

WHEN	iterations "when-string"

execute the command on the following line (or lines if it is a MULTI)

whenever the "when-string" is encountered in the data stream coming

from the remote system.	 After the string has been encountered

"iterations" times, the WHEN is automatically canceled (if iterations

is zero, the WHEN must be explicitly canceled with ENDWHEN).

You may have up to six WHENs active at any time (eight if you cancel

the autodownload [see ENDWHEN above]).	All WHENs are canceled on

hanging up or dialing a number, and autodownload is re-enabled.

WHENs are tested in the order in which they were executed, so multiple

WHENs with the same when-string will trigger in the order in which

they were asserted.

This can be an extremely powerful command.  For example, you can

implement another autodownloading (or uploading!) protocol simply by

adding

WHEN 0 "start-string"

EXEC "xfer-program parms"

to the AUTO or OnLoad macro.  On CompuServe, you can make Ymodem

autodownloading by adding

WHEN 0 "initiate YMODEM receive"

RECEIVE 'Y'

to the OnLoad macro.

The built-in Zmodem autodownload corresponds to

WHEN 0 "**^XB00"

RECEIVE 'Z'

while Zmodem auto-upload can be added with

WHEN 0 "**^XB01"

SEND 'Z'

The standard Puma autodownload is equivalent to the lines

WHEN 0 "^X^H^XPuma^X^H^X"

RECEIVE 'P'

[thanks to Matthew Thomas for publishing his autodownload string!].

See also DELAYED, ENDWHEN.

WINDOW	row col width height

limit the terminal emulator to a portion of the full screen.

WINDOW 0 0 255 255 will restore use of the full screen regardless of

the screen size.

-----------

Time Limits

-----------

The maximal time intervals which may be specified for various commands are:

255 ticks (~14 seconds)		PAUSE, SOUND

255 seconds (4.25 minutes)	WAITFOR

18 hours (64800 seconds)	DELAYED

-----------------------------------------------------------------------------

Ready-Made Macro Files

----------------------

The RBcomm distribution includes the following macro files:

AVATAR.MAC	for use with a BBS that supports AVATAR emulation in

its full-screen editor.  This file implements the so-

called "DOORWAY mode", passing through all keystrokes.

To access RBcomm's commands, you must use the Alt=

escape key before the RBcomm command key.

AVATOPUS.MAC	for use with Opus 1.10 or later BBSs, or compatibles

such as Maximus.  Implements DOORWAY mode as AVATAR.MAC

does, but also adds logon scripts for the OPUS CBCS

BAREOPUS.MAC

CPOINT.MAC	logon to Central Point Software's BBS

DDJ.MAC		logon to M&T Publishing's BBS to access the DDJ listings

EMACS.MAC	for use with Unix systems.  Turns your Alt key into a

Meta key.

EXAMPLES.MAC	various sample macros illustrating different features.

LK201.MAC	emulate an LK201 keyboard

OPUS.MAC

QDECK.MAC	logon to Quarterdeck's BBS

RBCOMM.MAC	default macros

VT100.MAC	emulate a VT100 keyboard

VT3270.MAC	emulate a VT100/IBM 3270 keyboard

VT52.MAC	emulate a VT52 keyboard

WORDSTAR.MAC	send WordStar motion commands using arrow keys

-----------------------------------------------------------------------------

Writing your own Help Files

---------------------------

Help files are plain ASCII files with the exception that several control

characters are used to change character attributes within a line (each

line starts with the default attributes).

^A  (A)lternate foreground and background colors, thus turning

reverse video on or off

^B  toggle the (B)old bit

^D  reset to (D)efault attributes

^E  switch to underlined color as defined by RBCONFIG

^F  toggle (F)lashing (blink bit)

-----------------------------------------------------------------------------

Terminal Emulation

------------------

RBcomm supports three terminal emulations.  ANSI provides standard ANSI

escape sequences plus most VT102 and many VT200 escape sequences.  BBS is

identical to ANSI except that clearing the screen with Esc-[-J or Esc-[-2-J

also homes the cursor, character set switching is disabled, and two of

RBcomm's private commands are replaced by the AVATAR level 0 command set.

Finally, VT52 replaces many of the Esc-letter sequences in ANSI with the

actions a VT52 or H19 would perform.  In addition, in all of these modes,

RBcomm has its own, much more compact command set (which may optionally

be disabled to permit display of the corresponding control characters).

------------------

Control Characters

------------------

Note: Control characters marked with (RBcomm) may be displayed if the

terminal emulation is set to "BBS" and RBcomm private commands are

disabled.

^A	UnixWindows command character (see below for sequences)

^B	move cursor right one position (RBcomm)

^C	move cursor down a line (RBcomm)

^D	move cursor left one position, does not wrap to previous line (RBcomm)

^E	send the answerback string to the remote system (ANSI and VT52 only)

^F	special sequences (see below) (RBcomm)

^G	sound bell, or flash screen if visible bell enabled

^H	move cursor left one position, wraps to previous line

^I	move to next tab stop

^J	move cursor down a line, scrolling screen when at bottom

^K	insert a blank line

^L	clear screen and home cursor

^M	move cursor to start of line

^N	"shift out"--switch to G1 character set (VT100)

^O	"shift in"--switch to G0 character set (VT100)

^P	clear rest of line (Note: if AVATAR is in cooked mode, ^P is the

"cook" character, causing the next character to have its high three

bits cleared; two consecutive ^Ps will be needed to give the

clear-line command) (RBcomm)

^R	change to reverse video (RBcomm)

^T	change character attribute to underlined (RBcomm)

^U	clear underlined attribute and reverse video (RBcomm)

^V	start of an AVATAR command (see below)

^W	delete character at cursor position, rest of line shifts left (RBcomm)

^X	insert a blank at the cursor's position, push rest of line

right (RBcomm)

^Y	repeat the following character the number of times specified by

the second character after the ^Y

^Z	delete line cursor is on (RBcomm)

^[	escape sequence (see below)

^\	turn on insert mode (RBcomm)

^]	turn off insert mode (RBcomm)

^^	move cursor up a line (RBcomm)

^_	the following two characters (less 32) specify the new cursor

column and row.	 i.e. ^_-blank-blank homes the cursor. (RBcomm)

----------------

Escape Sequences

----------------

Esc-blank-F  turn off eight-bit control characters

Esc-blank-G  turn on eight-bit control characters.  When eight-bit control

characters are enabled, many of the characters from 80h through

9Fh are equivalent to Esc-<char-40h>.

Esc-(	(VT100) next character ('0', '1', '2', 'A', or 'B') sets G0 character

set

Esc-)	(VT100) next character ('0', '1', '2', 'A', or 'B') sets G1 character

set

Esc-#	(VT100) next character sets character size

only '8' implemented -- fill screen with 'E's for alignment display

Esc-7	save cursor

Esc-8	restore cursor

Esc-<	set terminal emulation to ANSI

Esc-=	set keypad application mode (if allowed to change NumLock by RBCONFIG)

Esc->	set keypad numeric mode (if allowed to change NumLock by RBCONFIG)

Esc-@	turn on insert mode

Esc-A	move cursor up

Esc-B	move cursor down

Esc-C	move cursor right

Esc-D	(ANSI) "index"--move cursor down, scroll if at bottom

(VT52) move cursor left

Esc-E	(ANSI) move to start of next line, scroll if at bottom

(VT52) clear screen

Esc-F	(ANSI) not implemented

(VT52) select graphics character set

Esc-G	(ANSI) not implemented

(VT52) select text character set

Esc-H	(ANSI) set horizontal tab at current position

(VT52) home cursor

Esc-I	(ANSI) horizontal tab

(VT52) reverse linefeed, scrolls down if already on top line

Esc-J	(ANSI) not implemented

(VT52) clear to end of screen

Esc-K	(ANSI) not implemented

(VT52) clear to end of line

Esc-L	(ANSI) not implemented

(VT52) insert a new line at cursor

Esc-M	(ANSI) "reverse index"--cursor up, reverse scroll if at top

(VT52) delete cursor line

Esc-N	(ANSI) not implemented

(VT52) delete char at cursor, rest of line shifts left

Esc-O	(ANSI) not implemented

(VT52) turn off insert mode

Esc-Y	move cursor, next two characters are row + 32, column + 32

Esc-Z	request identification

(ANSI) RBcomm returns the string "rbcommN.NNx" where N.NN is the

version number and "x" is a single byte identifying implemented

capabilities

bit 0: multiple screens available if set

bit 1: file transfer implemented (always set)

bit 6: always set (makes character printable)

bit 7: always clear

remaining bits reserved (zero)

(VT52) RBcomm returns the string Esc-/-Z, indicating a VT100 emulating

a VT52.

Esc-[	ANSI sequence, see below

Esc-]	Operating System Command.  The following characters through an Esc-\

sequence (maximum 80 characters) are skipped, as this command has

no effect.

Esc-^	ANSI Privacy Message.  The following characters through an Esc-\

sequence (maximum 80 characters) are accumulated.  If running under

DESQview, the first 40 are displayed in the notification window;

otherwise, the accumulated characters are discarded.

Esc-_	Application Program Command.  The following characters through an

Esc-\ sequence (maximum 80 characters) are skipped, as this command

has no effect.

Esc-c	reset to initial state

Esc-j	save cursor position	   (Heath H19)

Esc-k	restore cursor position	   (Heath H19)

Esc-l	erase line		   (Heath H19)

Esc-p	turn on bold characters

Esc-q	turn off bold characters

Esc-v	turn on line wrap	   (Heath H19)

Esc-w	turn off line wrap	   (Heath H19)

Esc-z	reset terminal emulation   (Heath H19)

--------------

ANSI Sequences

--------------

All of the sequences listed here consist of Esc-[ followed by zero or more

numbers separated by semicolons followed by the command letter.	 Therefore,

only the command letter will be listed.	 X1, X2, etc refer to the specified

numeric argument, and usually are followed by a default value in

parentheses.  All cursor positioning commands number rows and columns

starting at 1.

blank	extended ANSI sequence (see below)

$	extended ANSI sequence (see below)

@	insert X1 (1) blanks, shifting rest of line to the right

A	move cursor up X1 (1) lines

B	move cursor down X1 (1) lines

C	move cursor right X1 (1) positions

D	move cursor left X1 (1) positions

E	move cursor down X1 (1) lines, scroll if at bottom

F	move cursor up X1 (1) lines, scroll if at top

G	move cursor to position X1 (1) in current line

H	move cursor to row X1 (1), column X2 (1)

I	move to X1st (1) following horizontal tab position

J	case X1 (*):  0	 clear from cursor to end of screen

1	 clear from start of screen to cursor

2	 clear screen

* default is 0 for ANSI and VT52, 2 for ANSI-BBS

K	case X1 (0):  0	 clear from cursor to end of line

1	 clear from start of line to cursor

2	 clear line

L	insert X1 (1) lines at cursor

M	delete X1 (1) lines at cursor

P	delete X1 (1) characters at cursor, rest of line shifts left

S	scroll up X1 (1) lines

T	scroll down X1 (1) lines

U	switch to screen X1 (1) screens later, wrapping back to screen 0

if X1 preceded by an equal sign, switch to specified screen

V	switch to screen X1 (1) screens prior, wrapping to last screen

X	erase X1 (1) characters starting at cursor position, cursor stays put

Z	move cursor to X1st (1) preceding horizontal tab position

`	move cursor to position X1 (1) in current line

a	move cursor X1 (0) positions from current position in line

c	device attribute report

responds by sending Esc-[-?-6-c (VT102)

d	move cursor to line X1

e	move cursor X1 (0) lines from current line

f	move cursor to row X1 (1), column X2 (1)

g	case X1 (0):  0	  clear horizontal tab at current position

3	  clear all horizontal tab stops

'>' clear all horizontal tab stops, then set tabs every

N positions (i.e.  Esc-[->-5-g sets tabs every five

columns)

h	select mode

case X1 (0):  ?2  set emulation to ANSI

?3  select 132-column mode (if enabled by RBCONFIG)

4  turn on insert mode

?5  turn on inverted video

?6  turn on scrolling-region-relative cursor positioning

(origin mode)

?7  turn on wrap mode

12  turn on local echo

20  turn on newline mode (send CRLF when CR pressed)

?25  turn cursor on (make visible)

i	media copy

case X1 (1):   1  dump entire screen to printer

?1  dump screen line containing cursor to printer

4  exit printer controller mode [not yet implemented]

?4  stop echoing to printer [not yet implemented]

5  enter printer controller mode [not yet implemented]

?5  start echoing to printer [not yet implemented]

l	reset mode

case X1 (0):  ?2  set emulation to VT52

?3  select 80-column mode

4  turn off insert mode

?5  turn off inverted video

?6  set top of scrolling region to topmost line, bottom

to bottom of screen, and turn off relative cursor

positioning (absolute cursor positioning)

?7  turn off wrap mode

12  turn off local echo

20  turn off newline mode (send only CR when CR pressed)

?25  turn off cursor (make invisible)

Note: may not work on all systems or in all video

modes

m	select graphic rendition

for each Xn in order,

0	 reset attributes to white on black, turn off reverse

video

1	 set bold

4	 set underlined

5	 set blinking

7	 set reverse video

8	 set invisible (black on black)

21	 turn off bold

22	 turn off bold

24	 turn off underlined

25	 turn off blinking

27	 turn off reverse video

30	 set foreground color

-

37

40	 set background color

-

47

Note: Esc-[-m is equivalent to Esc-[-0-m

n	device status report

case X1 (0):  5	 report terminal status

always sends Esc-[-0-n (terminal OK) to remote

6	 report cursor position

sends string Esc-[-row-;-col-R to remote system

15	 printer status

sends Esc-[-?-1-3-n (no printer) if no printer defined

Esc-[-?-1-0-n if printer is ready

Esc-[-?-1-1-n if printer is not ready

[currently always sends ?13n]

25	 report User Definable Key status

always sends Esc-[-?-21-n (UDK's locked)

26	 report keyboard dialect

always sends Esc-[-?-27-;-1-n (US ASCII)

r	set scrolling region to rows X1 (1) through X2 (lines-on-screen)

s	save cursor (may not be nested)

u	restore cursor

x	request terminal parameters (VT100)

case X1 (0):  0	 sends back string indicating bits, parity, speed

1	 sends back string indicating bits, parity, speed

the string sent is

Esc-[-<id>-;-<parity>-;-<bits>-;-<tspd>-;-<rspd>-;1;0x

where <id> is 2 if X1 was 0 and 3 if X1 was 1

<parity> is 1 for none, 2 for space, 3 for mark, 4 for odd,

and 5 for even

<bits> is 1 for 8, 2 for 7, 3 for 6, and 4 for 5 data bits

<tspd> and <rspd> are the transmit and receive speeds:

16 -> 110 bps

32 -> 150 bps

48 -> 300 bps

56 -> 600 bps

64 -> 1200 bps

88 -> 2400 bps

104 -> 4800 bps

112 -> 9600 bps

120 -> 19200 bps

128 -> 38400 bps

136 -> 57600 bps

144 -> 115200 bps

z	reset terminal emulation (Heath H19)

-----------------------

Extended ANSI sequences

-----------------------

If the command character for an ANSI sequence is a blank, the NEXT character

specifies the actual operation.

@	scroll left X1 (1) columns

A	scroll right X1 (1) columns

If the command character for an ANSI sequence is a dollar sign, the NEXT

character specifies the actual operation.  These are DEC VT2xx extensions.

p	  ANSI mode control state request

always returns CSI X1 ; 0 $ y	   (unknown mode)

u	  terminal state request

always returns DCS 1 $ ST	   (no state information returned)

---------------------

UnixWindows sequences

---------------------

When RBcomm receives a ^A, the following character specifies the actual

command.  If UnixWindows is enabled, a ^A introduces a UnixWindows command

at ANY time, even in the middle of another command; when disabled, ^A is a

normal command sequence introducer.  This distinction is necessary because

both AVATAR and RBcomm private commands may include ^A within the sequence.

The UnixWindows commands are encoded as follows:

+---+---+---+---+---+---+---+---+

| 0 |dir|  function | parameter |

+---+---+---+---+---+---+---+---+

where "dir" is 0 for commands being sent to RBcomm from the remote system

and 1 for commands sent by RBcomm.  RBcomm currently supports the following

functions:  [Note: this is not yet enough to successfully run UnixWin]

0  NEWW	  [not yet implemented]

1  KILLW  [not yet implemented]

2  SELIN  The parameter of this function specifies which "window" will

display characters received from the serial port.  UW windows

are numbered from 1 to 7 and mapped to RBcomm screens 0 through

6.  UW window 0 does not exist; if function 2 is invoked with

parameter 0, the command is ignored.

3  SELOUT This command is never sent by the remote host, and is thus

ignored

4  WINOPT [not yet implemented]

5  META	  If the parameter is 0, set the high bit of the next received

character before processing it.  If the next character is a ^A

UnixWindows command character, this command will apply to the

next non-UW character (or the control character produced by

function 6 below)

If the parameter is one of the following, act as if the specified

character had been received; otherwise, ignore the command.

parameter 1 = char 129

parameter 2 = char 145

parameter 3 = char 147

6  CTRL	  if the parameter is one of the following, act as if the specified

control character had been received; otherwise, ignore the command.

If function 5 was received immediately prior to this command, the

high bit will be set.

1   ^A

2   ^Q

3   ^S

7  MAINT  the parameter specifies one of a number of maintenance functions:

0   ENTRY  sent by UW server on startup.  RBcomm enables the

UnixWindows protocol on receiving this command.

2   ASKPCL remote system requests the start of protocol negotiation.

RBcomm responds with CANPCL 1 (the most basic protocol).

3   CANPCL remote system specifies a protocol it is capable of

supporting in the following byte (1Fh is added to the

protocol number to make it printable).  If protocol 1

specified, RBcomm responds with SETPCL 1, otherwise it

responds with CANPCL 1.

4   SETPCL remote system specifies a protocol which both ends are

to use.  The protocol is sent in the next byte, with 1Fh

added to make it printable.  As RBcomm only supports

protocol 1, this function is currently ignored.

7   EXIT   sent by UW server on shutdown.  RBcomm disables the

UnixWindows protocol on receiving this command.

------------------------

Special RBcomm sequences

------------------------

When RBcomm receives a ^F, the following character specifies the actual

command.

^x	display the IBM PC screen character corresponding to the control

character

0	turn off visual bell, ^G will beep

1	turn on visual bell, ^G will flash the screen, but internally

generated beeps still sound

2	flash the screen

3	beep even if visual bell turned on

4	fill area.  Identical to AVATAR ^V^M (see below) but does not cancel

insert mode.

5	clear from cursor to end of screen

6	repeat pattern.	 Identical to AVATAR ^V^Y (see below)

:	disable the UnixWindows protocol

;	enable the UnixWindows protocol

<	set terminal emulation to VT102/ANSI.  Does not affect any other

settings

=	set terminal emulation to ANSI-BBS.  Does not affect any other

settings.

>	set terminal emulation to VT52.	 Does not affect any other settings.

?	query terminal emulation type.	Sends back <127><type> where type is

'A' for VT102/ANSI, 'B' for ANSI-BBS, or 'V' for VT52.  The type

is converted to lower case if the UnixWindows protocol is enabled.

@	send identification (see Esc-Z)

A	[obsolete--this command no longer does anything]

B	if next character is '0' through '8' switch to the specified

screen, provided that memory was allocated for it at startup.

Sends a response of 127-<digit>-<status> to the remote system,

where the digit is the screen number from the ^FB command, and

status is 'Y' if the screen exists or 'N' if it doesn't.  It is

the remote system's responsibility to redraw the screen if the

response is 'N'.

C	if next character is '0' through '8' and the specified screen was

allocated at startup, clear that screen to blanks.

------------------------

AVATAR command sequences

------------------------

^V^A	set screen attribute to low seven bits of following character

^V^B	set blink

^V^C	move cursor up a line

^V^D	move cursor down a line

^V^E	move cursor left one space

^V^F	move cursor right one space

^V^G	clear from cursor to end of line

^V^H<r><c>  move cursor to row <r> and column <c>, where the upper left corner

is 1,1

^V^I	turn on insert mode until next AVATAR command (except ^Y and ^V^Y)

^V^J	scroll area up.	 Next five characters specify number of lines to

scroll, top margin, left margin, bottom margin, and right margin (all

margins are based on 1,1 being the upper left corner of the screen)

^V^K	scroll area down.  Next five characters are as for ^V^J

^V^L	clear area.  Next three characters specify screen attribute for cleared

area, number of lines less one, and number of columns less one.	 The

blink bit of the attribute is ignored, and the current display attribute

is set to the attribute of the cleared area.  If the requested area

extends beyond the current window limits, it will be truncated to fit.

^V^M	fill area.  Next four characters specify screen attribute for filled

area, character to fill with, number of lines less one, and number of

columns less one.  The current display attribute is set to the filled

attribute with blinking turned off.  If the requested area extends

beyond the current window limits, it will be truncated to fit.

^V^N	delete character at cursor position, shifting the remainder of the

line.

The following commands (except for ^V^Y) are Level 1 extensions.  They

are not fully implemented, and are not expected to be completely

correct, as the official specifications have just (1/20/91) been

published, and I have not had time to implement and test everything.  In

cases where the official specs differ from the sketchy information I had

to work with, the behavior will be incorrect.  Those commands marked

[not implemented] will merely be skipped without any further processing.

^V^O	turn clockwise mode on [not implemented]

^V^P	not used

^V^Q	query.	Next character specifies the type of query

^Q  get driver version info.  Returns the string

"AVT0,rbcommN.NNc\r"

where N.NN is the RBcomm version and c is the capability byte

(see Esc-Z).  This identifies RBcomm as complying with the AVATAR

level 0 specification, since level 1 support is still incomplete.

^V^R	reset AVATAR [not implemented]

^V^S	make a sound.  The next three characters specify the note number,

octave, and duration in tenths of a second.  The note is computed as

(note-'A')*2 + sharp

The current implementation does not queue any tones unless RBcomm is

running under DESQview (which can queue notes).

^V^T	highlight character at cursor position.	 Next character specifies new

attribute.

^V^U	highlight window.  Next two characters specify the window handle and

new attribute.

^V^V	define window.	Next six characters specify the window handle, default

attribute, top margin, left margin, bottom margin, and right margin.

The default attribute is also the initial current attribute for the

new window.  Note: window 0 can not be redefined.

^V^W	switch to window.  The next character specifies the handle of the

window to switch to.

^V^X	flush input [not implemented]

^V^Y	repeat pattern.	 The following character specifies the length of the

pattern to be repeated, followed by the pattern, and finally followed

by a single character indicating the number of times to repeat the

pattern.  The pattern may contain command sequences, but is limited

to 80 characters (longer patterns are truncated to 80 characters).

^V^\	go to bed [not implemented]

^V^]	wake up [not implemented]

^V^^	start vertical output [not implemented]

^V^_	start horizontal output [not implemented]

^V!	poke char/attr to physical screen.  The next four characters specify

the character and attribute to poke, and the row and column at which

to display that character and attribute.

^V"	turn off line wrap

^V#	wrap in zigzag mode [not implemented]

^V$	turn on line wrap

^V%	reverse direction of linefeeds [not implemented]

^V&	linefeeds move in normal direction [not implemented]

^V'	set cursor type.  The next character specifies the cursor's shape:

^A return to default (startup) cursor shape

^B make cursor invisible (does not work on all systems in all modes)

^C block cursor, covering entire character cell

otherwise, this command is ignored

^V(	output in forward direction [not implemented]

^V)	output in reverse direction [not implemented]

^V*	system pause.  The next character specifies the	duration in tenths of

a second.  Contrary to the official AVATAR specs, RBcomm treats zero

as 256 tenths seconds, rather than a full hour.	 This is both more

consistent and more secure, as neither keyboard nor serial port

activity is possible during a system pause.

^V+	insert a line

^V,	insert a blank column at the current cursor position

^V-	delete current line

^V.	delete current column

^V/	set/reset static mode [not implemented]	 The next character specifies

whether or not the cursor should be advanced after outputting a

character to the screen.

^V0	highlight from cursor to end of line.  The next character specifies

the new attribute to be applied to the rest of the line.

^V1	highlight from start of line to cursor.	 The next character specifies

the new attribute to be applied to the beginning of the line.

^V:	keyboard mode.	The next character specifies the mode.	RBcomm

always returns ^V:0 (default mode), as it does not support this

command.

^V<	scroll left.  The next five characters specify the number of columns

to scroll, the top margin, left margin, bottom margin, and right

margin of the area to scroll.

^V=	set parser mode.  If the next character ANDed with 1Fh is ^R or ^C,

set the mode to raw or cooked, respectively.  In cooked mode, the

character immediately following a ^P is ANDed with 1Fh, and the result

is treated as if it had arrived from the remote system instead of the

^P sequence.

^V>	scroll right.  The next five characters are as for ^V<

^V?	peek at physical screen.  The next two characters specify the row and

column at which to peek.  RBcomm returns the poke command (see ^V!)

needed to restore the given character position to its current state.

-----------------------------------------------------------------------------

Acknowledgements

----------------

Thanks to Thomas Zerucha for his numerous comments and suggestions, many of

which have been implemented.

Thanks to Walter Cox for his comments and suggestions on v2.81, one of the

included keyboard bindings, and his patience in testing new versions.

Thanks to Mike Weaver for his comments and suggestions, some of which have

been implemented.

Thanks to Dave Doren for banging on the macro language and reporting bugs

and possible enhancements.

-----------------------------------------------------------------------------

Program History

---------------

v2.72  9/3/89	first public release

v2.81  10/4/89	second public release

v3.01  1/6/90	third public release

v3.12  4/28/90	fourth public release

v3.21  7/29/90	fifth public release

v3.22  9/9/90   switched from spawnlo() to spawnlpo(), adjusted Alt-G

added DISPLAY command, %N variable expansion

stubbed out AVATAR level 1 support

v3.23  9/23/90	enhanced input line editor

fixed subtle bug in environment reading

added MDIAL command, %w variable expansion

continued adding AVATAR level 1 support

v3.24  10/6/90	size optimizations

added PACECHAR command, SWAPDIR environment variable

v3.25  10/21/90 added more AVATAR level 1 functions, swap to EMS

fixed scrolling-region/cursor-move interaction bug

internal changes

v3.26  11/25/90 more internal changes and size optimizations

OPEN_LOG in OnLoad macro now works in all cases

v3.27  12/9/90	added SELECT_SCREEN command on Alt-W

more UnixWindows functions

v3.28  12/21/90	added commandline options

some more AVATAR level 1 functions

new DVPWIDTH program for 132 column DESQview windows

v3.30  1/13/91	completely rewrote file browser, implemented scrollback

FILES command now uses new browser

added SCROLLBACK, SAVE_SCREEN commands

v3.31  2/3/91	bugfixes to scrollback

some more AVATAR level 1 functions

(sixth public release)

v3.32  2/17/91  restored command entry from help screen

added %/ and %\ expansions

dialing dir now uses pager, capacity increased to 40 entries

v3.33  3/17/91	removed SAVE_MACRO/UI_MENU, saving 380 bytes

simplified macro learning to single key, saving 330 bytes

macro file format changes

current macro file now kept if new file not found or invalid

new compiled dialing directory allows up to 250 entries

v3.34  4/10/91	directory pager can now back up on very large directories

scrollback buffer can now be written to a file

added ALARM, MUSIC, PLAY, SOUND, TFLUSH commands

v3.35  3/15/92	rearranged RBcomm private terminal functions

fixed scrollback and pager problems

added AVATAR, EXPAND_BLANK, and RBCOMM_CMDS commands

added optional BIOS writes for serial input

now supports 115200 bps

no longer randomly drops behind actual end of serial input at

extremely high speeds

v3.36  8/23/92	now hide DESQview mouse pointer while inside RBcomm window

speed optimizations

reworked RBCONFIG for greater ease of use

enhanced DIALDIR and merged it into RBCONFIG

v3.40  1/3/93	fixed RBCONFIG serial-port-info update & dialdir Move bugs

added DVEXEC, TRIM, LTRIM, and RTRIM commands

added #DOWNLOAD_x and #UPLOAD_x metacommands, %F expansion

the elusive dialing rollover bug has been squashed

the number pad with NumLock on can now be distinguished

from the typewriter number keys using the NP_n keynames

MACRO.COM can no longer decompile, but can compile

multiple macro files at one time

(seventh public release)