If you want your own modem to fit inside the TERMINAT.DOC you can create a

file containing data about how to setup your modem.

Line  Describtion

����  ��������������������������������������������������������������������

1   Name that will be shown in menu

2   Baudrate to use

3   Not used

4   Not used

5   Modem string delay milliseconds

6   Initstring

7   Dial prefix

8   Dial suffix

9   Connect string

10   No connect string 1

11   No connect string 2

12   No connect string 3

13   No connect string 4

14   Dial cancel string

15   Hangup string  DTR = Toggle DTR, else use string like ATH0

16   Ring string

17   Not used

18   Auto-answer string

19   Not used

20   Not used

21   CTS/RTS hardware flow control   0=Off  1=On

22   Xon/Xoff software flow control  0=Off  1=On

23   Breaktiming

24-  Lines starting with:

$ Information shown before sending commands to modem

^ Commands to send to modem

* Information shown after sending commands to modem

You can use up to 10 lines for $,^,* statement, they must come in that

order, se example.

Just use this as standard and then change the options you need, remove

the line and numbers to the right.

Line

------

@ZyXEL U-1496, U-1496B, U-1496E V.32bis V.42bis                             1

57600                                                                       2

3

4

10                                                                          5

ATZ^M                                                                       6

ATDT                                                                        7

^M                                                                          8

CONNECT                                                                     9

NO DIAL TONE                                                               10

NO CARRIER                                                                 11

BUSY                                                                       12

VOICE                                                                      13

^M                                                                         14

DTR                                                                        15

RING                                                                       16

ATS0=0H0^M                                                                 17

ATA^M                                                                      18

OK                                                                         19

ATM0H1^M                                                                   20

1                                                                          21

0                                                                          22

500                                                                        23

$There is no switch to set, just let the program to write the

$setting to V-RAM.  If you do not have 16650 UART installed in

$your I/O card, please use 38400 buad for reliable operation.

^AT&F^M

^ATQ0M1X6V1S7=60^M

^AT&K4&B1&C1&D2&H3*E0&N0&W^M

*Your ZyXEL is now configured.  To get the most out of it,

*be sure to set all your dialing directory entries to 57600 baud.