Gport

C Language Game Port Library

Version 1.01

1 September 91

Copyright (c) 1991  Bri Productions

1

Table of Contents

Introduction

General Description.....................3

Disclaimer..............................3

Registration............................3

Contacting the Author...................3

A typedef...............................3

General Functions

Description.............................4

GamOpen.................................5

GamClose................................6

GamAxis.................................6

GamAxes.................................7

GamButton...............................8

Calibration Functions

Description............................10

GamRawAxis.............................11

GamCalAxis.............................12

GamCenter..............................13

Appendix A - Things to Look Out For............15

Appendix B - Joystick Calibration/Centering....16

Appendix C - Registration Form.................18

2

General Description.

Gport is a C language, high speed, high resolution game port

library for IBM and compatible computers. Gport is compatible

with most if not all DOS based C compilers.

Some of Gport's features are:

Optional joystick calibration.

Yields higher resolution on faster computers.

Written in assembly language for optimum speed and efficiency.

DISCLAIMER.

Gport is provided AS IS. Bri Productions specifically disclaims

any and all warranties, expressed or implied, including fitness

for a particular purpose. Use this product at your own risk.

Registration.

Gport is a user supported software product. Distribution of

this product, unaltered and without charge, is encouraged. If

you find this product useful, you are encouraged to register

your copy. This allows Bri Productions to continue to provide

and support low cost, high quality shareware.

The registration fee is $25.00 US dollars and includes libraries

for small, medium, compact and large memory models and complete

source code. A registration form is provided in Appendix C.

Contacting the author.

Bri Productions may be contacted by any of the following means:

U.S. mail       -       Bri Productions

P.O.Box 7121

Fremont, CA 94537-7121

Phone           -       (510) 794-0616

CompuServe      -       76635,2246

CompuServe is a trademark of CompuServe Inc.

A typedef.

You will see the data type 'byte' throughout the Gport library.

This is a typedef defined in gport.h. Byte is an unsigned

character used for byte size values.

3

General functions

There are two functions that are required before other game

port functions can be called. They are GamOpen() and GamClose().

GamOpen() initializes the game port and installs an interrupt

handler for monitoring the buttons. GamClose() releases the

interrupt handler. Failing to call GamClose() prior to the

program's termination will most certainly cause an operating

system crash.

GamAxis() and GamAxes() fetches the position(s) of one or

more joystick axes. A full up or right joystick returns a value

of approximately +1000 and full down or left returns a value of

approximately -1000. GamButton() fetches the status of the game

port's buttons. Information on a button pressed since the last

call to GamButton() as well as a presently pressed is available.

4

----------------------------------------------------------------

GamOpen

----------------------------------------------------------------

Function

Initializes the game port and installs the interrupt

handler.

Syntax

#include "gport.h"

int GamOpen(void);

Remarks

GamOpen initializes the game port. If an axis is found to

be valid, it's corresponding bit is set in the returned

status word. After a call to GamOpen, a subsequent call

to GamClose is required. Failure to do so will most

definitely cause the operating system to crash.

Return Value

GamOpen returns a valid axis status word. The bits/values

are defined as follows:

JAX_VAL  (0x1) - Joystick A, x axis valid

JAY_VAL  (0x2) - Joystick A, y axis valid

JA_VAL   (0x3) - Joystick A, both axes valid

JBX_VAL  (0x4) - Joystick B, x axis valid

JBY_VAL  (0x8) - Joystick B, y axis valid

JB_VAL   (0xC) - Joystick B, both axes valid

See also

GamClose

Example

Attempt to open the game port and check Joystick A.

{

int valid_axes;

valid_axes = GamOpen();

if((valid_axis & JA_VAL) == JA_VAL)

{

...Joystick A OK

}

}

5

----------------------------------------------------------------

GamClose

----------------------------------------------------------------

Function

Releases the interrupt handler.

Syntax

#include "gport.h"

void GamClose(void);

Remarks

After a call to GamOpen, a subsequent call to GamClose

is required. Failure to do so will most definitely cause

the operating system to crash.

See also

GamOpen

Example

Close the game port prior to terminating.

{

...closing code

GamClose();

exit(0)

}

----------------------------------------------------------------

GamAxis

----------------------------------------------------------------

Function

Gets the coordinate of a joystick axis.

Syntax

#include "gport.h"

signed GamAxis(byte axis);

6

Parameters

axis  - Constant that defines which axis to get the

coordinate of. Possibilities are:

JAX  (0) - Joystick A, X axis

JAY  (1) - Joystick A, Y axis

JBX  (2) - Joystick B, X axis

JBY  (3) - Joystick B, Y axis

Remarks

GamAxis fetches the coordinate of a single joystick

axis. The coordinate is returned as a signed integer.

Return value

GamCoord returns the coordinate of the specified axis.

The values will range from -1000 to +1000.

See also

GamAxes GamButton

Example

Update the current positions of the X & Y axes for

joystick A.

{

YourXaxisFunction( GamAxis(JAX) );

YourYaxisFunction( GamAxis(JAY) );

}

----------------------------------------------------------------

GamAxes

----------------------------------------------------------------

Function

Gets the both axes coordinates of a joystick.

Syntax

#include "gport.h"

void GamAxes(byte stick, signed *x, signed *y);

7

Parameters

stick - Constant that defines which joystick to get

the coordinates of. Possibilities are:

JA  (0) - Joystick A

JB  (1) - Joystick B

x - pointer to the signed integer where the x axis

coordinate will be stored.

y - pointer to the signed integer where the y axis

coordinate will be stored.

Remarks

GamAxes fetches the coordinates of both axes for

the specified joystick. The coordinates are stored

at the pointers passed to GamAxes. The values will

range from -1000 to +1000.

See also

GamAxis GamButton

Example

Update the current positions of the X & Y axes for

joystick A.

{

signed x,y;

GamAxes(JA, &x, &y);

YourXaxisFunction( x );

YourYaxisFunction( y );

}

----------------------------------------------------------------

GamButton

----------------------------------------------------------------

Function

Gets the status of the joystick buttons.

8

Syntax

#include "gport.h"

byte GamButton(void);

Remarks

GamButton returns the current status of the joystick

buttons as well as any button status since the last

call to GamButton.

Return value

GamButton returns the status of the joystick buttons.

If the button is currently pressed, it's current bit

will be set. If a button has been pressed since the

last call to GamButton, it's pending bit is set.

The bit constants are as follows:

BA1_CURR (0x01) - Joystick A, button 1 current.

BA2_CURR (0x02) - Joystick A, button 2 current.

BB1_CURR (0x04) - Joystick B, button 1 current.

BB2_CURR (0x08) - Joystick B, button 2 current.

BA1_PEND (0x10) - Joystick A, button 1 pending.

BA2_PEND (0x20) - Joystick A, button 2 pending.

BB1_PEND (0x40) - Joystick B, button 1 pending.

BB2_PEND (0x80) - Joystick B, button 2 pending.

See also

GamAxis GamAxes

Example

Check if the player has fired (button 1). If so

call the fire update function.

#define FIRE  (BA1_PEND) | (BA1_CURR)

{

if(GamButton() & FIRE)

{

YourFireFunction();

...more code

}

}

9

Calibration Functions

The performance of the joystick can be significantly improved

by calibrating it, especially if the joystick is misaligned.

This is achieved by collecting the raw value of the position of

the joystick extremes; full up, down, left, and right; and

recalculating scaling math. GamRawAxis() fetches the raw

value of the position of the joystick, which is passed to the

function GamCalAxis(). GamCalAxis() then recalculates the

scaling math.

The calibration is done in two steps in order to leave open

the option of saving the raw values to disk for future

reference. This saves the step of having to recalibrate

every time a program is run.

GamCenter() sets the centering mode. There are two ways to

determine where the center of a joystick is. The REAL center

is defined as where the joystick comes to rest. The MEAN

center is defined as the mean between the joystick extremes.

For a more detailed description on calibration, see Appendix B.

10

----------------------------------------------------------------

GamRawAxis

----------------------------------------------------------------

Function

Gets the raw value of a joystick axis.

Syntax

#include "gport.h"

unsigned GamRawAxis(byte axis);

Parameters

axis  - Constant that defines which axis to get the

coordinate of. Possibilities are:

JAX  (0) - Joystick A, X axis

JAY  (1) - Joystick A, Y axis

JBX  (2) - Joystick B, X axis

JBY  (3) - Joystick B, Y axis

Remarks

GamRawAxis returns the raw value of a joystick position

to be used for calibrating the joystick . Smaller values

are to the left and up while larger values are to the

right and down. The actual value will vary from machine

to machine.

Return value

GamRawAxis returns the raw value for the position of

the selected joystick axis.

See also

GamCal GamCenter

Example

Calibrate the X axis on joystick A.

{

int raw_ax_right;

int raw_ax_left;

puts("Hold joystick full right and press any key");

getch();

11

raw_ax_right = GamRawAxis(JAX);

GamCalAxis(JAX, raw_ax_left);

puts("Hold joystick full left and press any key");

getch();

raw_ax_left  = GamRawAxis(JAX);

GamCalAxis(JAX, raw_ax_right);

}

----------------------------------------------------------------

GamCalAxis

----------------------------------------------------------------

Function

Calibrates a joystick axis extreme from a raw value.

Syntax

#include "gport.h"

void GamCalAxis(byte axis, unsigned raw_value);

Parameters

axis  - Constant that defines which axis to get the

coordinate of. Possibilities are:

JAX  (0) - Joystick A, X axis

JAY  (1) - Joystick A, Y axis

JBX  (2) - Joystick B, X axis

JBY  (3) - Joystick B, Y axis

raw_value - Raw value returned from a call to GamRaw

on which to calibrate.

Remarks

GamCalAxis calibrates a joystick extreme from a raw

value from a previous call to GamRawAxis. Notice that

it is not necessary to specify the extreme (left/right

or up/down) in question.

See also

GamRawAxis GamCenter

12

Example

Calibrate the X axis on joystick A.

{

puts("Hold joystick full right and press any key");

getch();

GamCalAxis(JAX, GamRawAxis(JAX));

puts("Hold joystick full left and press any key");

getch();

GamCalAxis(JAX, GamRawAxis(JAX));

}

----------------------------------------------------------------

GamCenter

----------------------------------------------------------------

Function

Sets the centering mode.

Syntax

#include "gport.h"

void GamCenter(byte mode);

Parameters

mode - Constant defining the centering mode to be used.

Possibilities are:

*  REAL (0) - real centering mode.

MEAN (1) - mean value centering.

* default

Remarks

GamCenter sets the centering mode to be used. In real

mode, the center of an axis will be where the joystick

is at rest. In mean mode, the center is calculated as

the mean between the two previously calibrated extremes.

For a more detailed description of the centering

modes see Appendix B.

See also

GamRawAxis GamCalAxis

13

Example

Set the centering to mean mode.

{

GamCenter(MEAN);

}

14

Appendix A

Things to look out for.

Some faster machines will exhibit a "fold back" on the down

and right strokes of the joystick. Special AT game ports

are available which usually resolve this problem.

15

Appendix B

Calibration

When the function GamOpen() is called, some assumptions are made

about the raw values of the positions of the joystick extremes;

left, right, up and down. The assumptions are based on the raw

value of the joystick axes at the time of the call to  GamOpen()

(at rest at the center). In most cases the joystick will perform

reasonably well providing the alignment of the joystick is at

least fair.

The performance of the joystick can be greatly improved if it

is calibrated. Even a poorly aligned joystick can be made to

appear to perform quite well. In order to accomplish this the

assumptions about the joystick extremes must be replaced by

their actual raw values. This requires the cooperation of the

user as well as the functions GamRawAxis() and GamCalAxis().

Typically the user will be prompted to hold the joystick to

each of the four extremes and hit a key on the keyboard. For

each extreme, the raw value of the joystick position is read

by a call to GamRawAxis() and the returned value is passes to

GamCalAxis(). GamCalAxis will then make some recalculations

and replace the previous assumptions (or calibration).

Notice that the calibration is done in two steps. First the

raw value is fetched and then it is passed. This allows the

program to store the raw values for future reference. Next

time the program is run, the values can be retrieved and

passes directly to GamCalAxis(). This saves the step of

having to interface with the user and calling GamRawAxis()

every time the program runs.

Centering

There are two ways to interpret where the center of the

joystick is . One way is to define the center as being where

the joystick comes to rest. Gport port refers to this as REAL

mode. The second way is to define the center as being

equidistant, or at the mean, from all of the joystick extremes.

Gport refers to this centering mode as MEAN mode.

REAL mode has the obvious advantage that the joystick will

always return approximately to the center. This will preclude

any drifting. The disadvantage of REAL mode is that there is

no evidence of how well the joystick is actually aligned.

The advantage and disadvantage of MEAN mode is the complement

of those of REAL mode. Drift will occur if the joystick is

not aligned well. But since there will be the evidence of

the joystick's misalignment, MEAN mode could be used to

actually align a joystick.

16

Recall that MEAN mode refers to the joystick extremes. Also

recall that the joystick extremes are assumed until the

joystick is calibrated. For this reason there is little

difference between the two centering modes until the joystick

is calibrated. Calculating the mean center using the assumed

extremes will result in the same center since the reverse

process is used to calculate the assumed extremes.

17

Appendix C

Gport Registration Form

Name: ______________________________________________________

Company: ___________________________________________________

Address: ___________________________________________________

City: ____________________________  State: _________________

Zip code: _________________  Phone # : _____________________

CompuServe # : _____________________________________________

How did you acquire Gport ? _________________________________

____________________________________________________________

What functions do you find most useful ?____________________

____________________________________________________________

What functions do you find least useful ?___________________

____________________________________________________________

What functions, not in Gport, would you like to see ?_______

____________________________________________________________

What other types of libraries would you find useful ?_______

__ Serial Communications  __ Expanded Memory  __ IEEE/GPIB

Others: ____________________________________________________

registration                                      $ 25.00

California residents add sales tax (  8.25% )     $________

shipping inside continental U.S.   ( $ 2.00 )

shipping outside continental U.S.  ( $ 5.00 )     $________

Total:   $________

All Payments Must be in U.S. Dollars

Make check or money order payable to:    Bri Productions

P.O. Box 7121

Fremont, CA 94537-7121

18