PRESS [PAGE UP] AND [PAGE DOWN] TO SCROLL THROUGH HELP PAGES

PRESS ENTER TO RETURN TO THE MAIN MENU

-------------------------------------------------------

PAGE  1:  THIS INDEX

PAGE 10:  INFORMATION ABOUT THIS PROGRAM

PAGE 12:  INSTRUCTIONS FOR USING OPTIMAX

PAGE 13:  OPERATION OF THE PROGRAM

PAGE 14:  EYEPIECE AND IMAGE SIZE CHART

PAGE 14:  DIAGONAL SIZE CHART (AUTOMATIC MODE)

PAGE 17:  CHART SHOWING RELATIVE AREAS

PAGE 18:  DIAGONAL SIZE CHART (MANUAL MODE)

PAGE 20:  USING THE "SLASH" COMMANDS

PAGE 21:  FILE STRUCTURE

PAGE 22:  DISPLAY PROBLEMS

Press ESCAPE to exit Help.

================================================================

OPTIMAX                                           Page 10

Version 2.2

(C) COPYRIGHT  1992 by Max E. Klinger

All Rights Reserved

Program written by Max E. Klinger

NOTICE: This product is copyrighted and is not public

domain software.

If you would like to contact the author with questions or

comments about the program, write:

Max E. Klinger

3142 Folsom Street

Boulder, CO 80304

Page 11

This program consists of five files:

OPTIMAX.EXE

OPTIMAX.WKB    (Data file)

OPTIMAX4.HLP

OPTIMAX.DOC    (Copy of these Help Screens)

OPTIMAX.TXT    (Detailed text and manual)

To print the documentation, for example from a floppy in the

A: drive, enter at a system prompt: "TYPE A:OPTIMAX.DOC >PRN"

and then press [ENTER].  To print the detailed text and

manual, for example from a hard drive labeled "C," enter at

the system prompt:  'TYPE C:\OPTIMAX.TXT >PRN" and then press

[ENTER].

================================================================

Page 12

Instructions for Using OPTIMAX

This program is a compiled spreadsheet with two different

command systems.  Most basic functions in the program can be

performed with one of three commands.

ALT-M     invokes a menu system that controls most of

the program features.

ALT-C     when in the Automatic diagonal calculation

screen, after entry of the data, will

calculate the diagonal size.

ALT-D     when in the Manual diagonal calculation

screen, after entry of the data, will

determine the results of the diagonal

entered.

Page 13

OPERATION OF THE PROGRAM

Basic operation of the program consists of four different

components.

1.  Eyepiece and Image Size Chart

2.  Diagonal Size Chart (Automatic Mode)

3.  Chart Showing  Relative Area of Diagonals

4.  Diagonal Size Chart (Manual Mode)

Component # 4 is an alternative to ## 2 & 3.

1.  Eyepiece and Image Size Chart

This chart will determine the four characteristics of

eyepiece performance shown in the four columns on the right side

of the screen for up to 17 different eyepieces.  To use, merely

[Press Page Down for continuation]

==================================================================                                                        Page 14

insert the size of the objective mirror (in inches) and the

focal ratio at the top of the chart and the focal length and

apparent field size for each eyepiece in the two left columns.

When this data has been entered, entering the information

called for in the first two columns on any line will

cause the characteristics in the last four columns to be

calculated automatically.

2.  Diagonal Size Chart (Automatic Mode)

In this mode, the program will make a series of

determinations about the optimum diagonal size as well as about

the performance of various readily available commercial

diagonals.  To use this feature, enter the objective mirror size

in inches, focal ratio, desired linear image size in inches, and

the desired distance from the center of the diagonal (optical

axis) to the focal plane (in inches).  Then press ALT-C.

Page 15

The method of calculating the diagonal size is based upon

the method described by William Peters and Robert Pike in "Sky &

Telescope," March 1977 p. 220.  Under this method, the "%

Illumination" represents the amount of vignetting at the edge of

the field as a percentage of full illumination.  When this

percentage is 63%, it represents a .5 magnitude light loss at

the edge of the field, which is usually unnoticeable for

most purposes under the Peters-Pike method.  An asterisk in the

column marked "Unsuitable Diag." means that the diagonal size

shown is smaller than the theoretical minimum size diagonal.

For diagonal sizes or F/ ratios that are at the extremes

for a given configuration, the formulas used for these

calculations to become indeterminate.  This has several

consequences.  For extreme F/ ratios (e.g., smaller than F/3 or

larger than F/12) with small mirrors, the formulas may produce

[Page Down to Continue]

==================================================================                                                        Page 16

odd results.  Similarly, for diagonal sizes that are much

larger or smaller than the optimum size, the results may

occasionally be spurious.  None of these problems arises

with reasonable and usual F/ ratios and diagonal sizes.

When the results for a given diagonal size appear as

"Indeterminate -- Indeterminate" this indicates a diagonal

size that is smaller than the optimum size, and the formulas

indicate that the % of Illumination is negligible while the

magnitude loss becomes extreme.

For comparison purposes, the diagonal size determined under

a more traditional formula can be determined on the screen

for MANUAL DETERMINATION of the diagonal size.  After pressing

ALT-M, select "M" or "Manual Diagonal."

Page 17

3.  Chart Showing  Relative Area of Diagonals

After using part 2, go to the Chart in Part 3.  This will

show, for the diagonals determined in Part 2, the relative area

of each diagonal in relation to the size of the objective mirror

Optimally, a diagonal larger than about 5-6% of the area of the

objective mirror may increase diffraction and cause a loss of

contrast.

==================================================================

Page 18

4.  Diagonal Size Chart (Manual Mode)

This part allows the manual determination of the

performance of a given diagonal size or a manual search for the

optimum diagonal size.  To use this feature, enter the objective

mirror size in inches, focal ratio, desired linear image size in

inches and the desired distance from the center of the diagonal

(optical axis) to the focal plane (in inches).  Then enter a

diagonal size in the space shown. Press ALT-D and the program

will display the results.

The percentage illumination and magnitude loss (as

explained above) as well as the relative area of the selected

diagonal are shown.  To determine manually the optimum diagonal

size under the Peters-Pike method mentioned above, keep

substituting different diagonal sizes (each time pressing ALT-D)

until the magnitude loss equals .5.

Page 19

The warning that a diagonal is "too small" means that

the diagonal size entered is smaller than the theoretical

minimum diagonal, without regard to considerations of

vignetting.  Under no circumstances would such a diagonal

be suitable, no matter what size magnitude loss is

shown.

The listing for Diagonal Size (Traditional) shows

the diagonal size determined under one of the traditional

methods, here using the formulas developed by Ralph Dakin.

In using this chart, it is important to remember the

limitations imposed by the mathematics used for these

calculations.  See  Pages 15-16 above.

==================================================================

Page 20

"SLASH" COMMANDS

To invoke the menus and options indicated across the

top of the screen, press the Forward Slash key [/].

Then select the desired item from the list.  While

using these "slash" commands will give you greater control

over the program, it is not necessary to use any of these

commands in order to take advantage of the features of the

program.

These "slash"  commands are similar to those used in

LOTUS 1-2-3, but not all of the features listed in the

menus will be available in this program.  These commands

can be used, for example, to control printer settings, such

as margins and the like.  DO NOT attempt to add or delete

rows or columns or otherwise to change the basic layout of

the charts.  The program may freeze if these are attempted.

[Page Down to Continue]

Page 21

FILE STRUCTURE

You may save the data you have entered into the program in

a data file (with .WKB extension) by entering a new file name

when prompted by the "save" menu option or when you press

/fs.  These data files can be reloaded into the program by

using the "file get" [/fg] commands under the File menu.

To clear all data from the program, press slash (/) then

select "worksheet" and "erase" from the menus.  Answer "yes"

when prompted.  Then press F-9 to clear the screen.  All data

will be cleared and lost, unless you have first saved it in

a new data file.

==================================================================                                                        Page 22

DISPLAY PROBLEMS

If your monitor is not displaying the program correctly,

try selecting the video option after pressing the forward slash

[/].  From this menu you may select from one of three different

display modes, i.e., color, black & white, monochrome, and to

change the size of the display.

Press [Page Up] to reread the Help Screens

Press [Escape] to exit from Help

[ END OF HELP PAGES ]