Publication Part Number W0001-030

Sheet  1 of 40

Title:  IBM PC, MS DOS

CalComp Protected Mode ADI driver

Version 2.0

Publication Part Number W0001-030

Sheet  2 of 40

CONTENTS

System Description.............................................  3

AutoCAD ADI Driver Plot Size Limit.............................. 3

Installation.................................................... 4

Configuring AutoCAD Release 11  386............................. 5

Configuring AutoCAD Release 10  386.............................11

Configuring CalComp Plotters....................................17

1023 Artisan...............................................18

1025 Artisan...............................................18

1026 Artisan...............................................18

1043.......................................................21

1044.......................................................21

1073,1075,1077.............................................22

2024.......................................................23

2026.......................................................23

52224......................................................27

52236......................................................27

52424......................................................27

52436......................................................27

67436......................................................27

68436......................................................27

68444......................................................27

57424......................................................29

57436......................................................29

57444......................................................29

58424......................................................29

58436......................................................29

58444......................................................29

5912.......................................................31

5913.......................................................31

5902A......................................................31

6603 VRC...................................................33

6613 VRC...................................................33

System Requirements.............................................37

Recommended Documents...........................................38

List of Materials...............................................39

Pen Definitions.................................................40

Publication Part Number W0001-030

Sheet  3 of 40

GENERAL DESCRIPTION

CCP386 is a protected mode ADI (Autodesk Device Interface) driver

for AUTOCAD release 10 386 or version 11 386 which will produce

CalComp 907/PCI/CCGL plot data.

The CCP386 has several advantages over other CalComp plotter

drivers.

- CCP386 supports all current CalComp plotters and Vector

printers.

- Outputs Serially at 8 different baud rates.

- Can use 7,e,1 or 8,n,1 parity.

- Can use XonXoff, Hardware, or CalComp AckNak flow control

protocols.

- Outputs Centronics Parallel by bypassing the bios which will

allow for at least a 2 to 1 increase in through put.

- Outputs to a user named plot file.

- Can download pen definitions with the plot data to relieve

user from pen mapping on the plotter.

- Can download a copy command to the plotter, sending the plot

only one time and producing up to 63 copies.

- User can tailor CalComp specific data parameters such as sync

codes, EOB codes, and checksumming to match an existing plotter

setup.

- Smaller file size (typically about one-half)

CDCL setup information can be included with the plot to configure

the plotter.  Calcomp Device Control Language is a way to

re-configure the plotter to match the incoming plot file.

Autocad ADI driver Plot Size Limit.

When the user selects the step size and plot size using Autocad's

"Configure Plotter" function,  the total X or Y steps in a plot

(plot size multiplied by the stepsize) cannot exceed 65535.

CCP386 handles this problem automatically.

Publication Part Number W0001-030

Sheet  4 of 40

INSTALLATION

Step 1. Check the CalComp shipped diskette.

Insert the CalComp shipped diskette in Drive A or B.    Type DIR

A: or B:.  Check your diskette and compare the files with the

Section "list of Materials".  Also read  "System Requirements"

before you begin.

Step 2. Copy files to hard disk.

You may copy CCP386.EXP wherever you wish.  Just remember where

you put it,  you will need to point to it later in this

installation.  As an example you might want to create a

subdirectory called CALCOMP.

md CALCOMP

Make CALCOMP the current subdirectory.

cd CALCOMP

Copy CCPADI.EXP to the hard disk and also this document if you

wish.

copy A:CCP386.EXP

or

copy B:CCP386.EXP

copy A:CCP386.DOC

or

copy B:CCP386.DOC

Copy A:SETUP.EXE      Setup program for the Colorview

or               and Plotmaster

Copy B:SETUP.EXE

Step 3:  Locating CCP386 for Autocad.

In order for AUTOCAD to find the Protected Mode ADI drivers, ACAD

has allocated an environment variable called PLPADI.  Environment

variables are initialized by the DOS SET command.  Using our

example from above:

SET PLPADI=C:\CALCOMP\CCP386.EXP

Without the PLPADI environment variable AUTOCAD will not be able

to find the protected mode driver during configuration or plot

time.  This set command is best added to your AUTOEXEC file so it

is automatically done at boot up time.

Publication Part Number W0001-030

Sheet  5 of 40

CONFIGURING AUTOCAD RELEASE 11

Start up Autocad and answer each question prompted by AutoCAD as

follows:

NOTE:  <cr> is the same as <return>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Main Menu

0.  Exit AutoCAD

1.  Begin a NEW drawing

2.  Edit an EXISTING drawing

3.  Plot a drawing

4.  Printer Plot a drawing

5.  Configure AutoCAD

6.  File Utilities

7.  Compile shape/font description file

8.  Convert old drawing file

9.  Recover damaged drawing

Enter selection: 5 <cr>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Current AutoCAD configuration

Video Display:           IBM Enhanced Graphics Adapter

Digitizer:               Microsoft Serial or Bus Mouse

Plotter:                 ADI P386 plotter

CCP386 CalComp P386 ADI Driver Ver 2.0

58436/68436

Printer plotter:         None

Press RETURN to continue:  <cr>

Publication Part Number W0001-030

Sheet  6 of 40

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 5 <cr>

Your current plotter is ADI P386 plotter

Do you want to select a different one? <N>

If your current plotter is ADI P386 plotter then skip over available

plotter/driver selection.

Y <cr>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Available plotters:

1.  None

2.  ADI P386 plotter

3.  Alpha Merics (obsolete)

4.  Autodesk Device Interface

5.  CalComp 906/907/PCI

6.  Canon Laser Printer

7.  Hewlett-Packard

8.  Houston Instrument DMP Series

9.  IBM 7300 Series

10.  PostScript device

11.  UNIX Plot device

Select device number of ? to repeat list <1>: 2 <cr>

Publication Part Number W0001-030

Sheet  7 of 40

At this point AUTOCAD will look for the environment variable PLPADI in

order to locate the required Protected ADI driver.  If PLPADI is not

found AutoCAD will display an error message and allow you to name the

P386 ADI driver.  It is best to remember to issue the SET PLPADI

command as listed in Installation.

Once the Driver issues have been solved CCP386 takes over and requests

you to select a supported plotter:

Supported models:

1.  1023

2.  1025

3.  1026

4.  1043

5.  1044

6.  1073/1075/1077

7.  2024 pacesetter

8.  2036 pacesetter

9.  52224/52424/57424

10.  52236/52436/57436/67436

11.  57444

12.  58424

13.  58436/68436

14.  58444

15.  5912 colorview

16.  5913 colorview

17.  6603 colormaster plus vrc

18.  6613 colormaster plus vrc

19.  5902A plotmaster

Enter selection, 1 to 19 <3>: 12 <cr>

Would you like to calibrate your plotter? <N> N <cr>

Write the plot to a file? Y or N <cr>

Size Units (Inches or Millimeters) <I> I or M <cr>

Plot origin in Inches <0.00,0.00>: 0,0 <cr>

Publication Part Number W0001-030

Sheet  8 of 40

Standard values for plotting size

Size    Width   Height

A       10.50     8.00

B       16.00    10.00

C       21.00    16.00

D       33.00    21.00

MAX    163.80    23.50

Enter the Size of Width,Height (in Inches) <MAX>: D <cr>

Rotate plot 0/90/180/270 <0>: 0 <cr>

Pen width in Inches <0.010>: <cr>

Adjust area fill boundaries for pen width? <N> <cr>

Remove Hidden lines? <N> <cr>

Specify scale by entering:

Plotted Inches=Drawing Units or Fit or ? <F>: <cr>

THE FOLLOWING OPTIONS ARE CALCOMP PLOTTER SPECIFIC.

Use the following options to match an EXISTING plotter.

****** Configure CalComp 58424 ******

Enter decimal sync code value, 2 to 127 <22>:

Enter number of sync codes, 1 or 2 <2>:

Enter decimal EOB code value, 2 to 31 <13>:

Enable Checksumming? <Y>

0 = 95

1 = 59

Enter number corresponding to required radix, 0 or 1 <0>:

This option will allow you to lower the numbering system used in

the plot data which will stop ASCII to EBCDIC translation

problems if the plot is to be uploaded to an EBCDIC computer for

plotting.

Some supported plotters can accept data Serially or Parallel.  If

you plotter is one of them this prompt will appear

This plotter will accept data two ways.

0 = Serial

1 = Parallel

Enter number corresponding to required output, 0 or 1 <0>:

Publication Part Number W0001-030

Sheet  9 of 40

Available Serial or Parallel Ports:

1 = COM1   or   LPT1

2 = COM2   or   LPT2

Enter number corresponding to required COM/LPT port, 1 or 2 <1>:

If operating serially the following 3 prompts will appear:

Available Baud Rates:

0 = 300

1 = 600

2 = 1200

3 = 2400

4 = 4800

5 = 9600

6 = 19200

7 = 38400

Enter number corresponding to required baud rate, 0 to 7 <6>:

Available Character Framing:

0 = 8,n,1

1 = 7,e,1

Enter number corresponding to required Character Framing, 0 or 1 <1>:

Available handshaking:

0 = xonxoff

1 = hardware/direct control

2 = ACKNAK

Enter number corresponding to required handshaking, 0 to 2 <1>:

If your plotter has configurable stepsizes:

This plotter has several stepsizes.

0 = 200

1 = 300

2 = 400

3 = 2032

Enter number corresponding to required stepsize, 0 to 3 <2>:

Publication Part Number W0001-030

Sheet 10 of 40

If your plotter is multicopy capable:

Do you wish to be prompted at plot time for extra copies? <N>:

If your plotter is capable of accepting color pen definitions:

Do you wish to download default pen color definitions? <N>

If your plotter is a roll feed pen plotter:

Plotter Modes

0 = Cut Sheet

1 = Roll

Enter number corresponding to plotter mode, 0 or 1 <0>:

If your plotter is CDCL compatible:

Do you wish to download CDCL setup information? <N>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 0 <cr>

If you answer N to the following question, all configuration

changed you have just make will be discarded.

Keep configuration changes? <Y> Y<cr>

FINISHED  The CalComp CCP386 driver is now configured.

Publication Part Number W0001-030

Sheet 11 of 40

CONFIGURING AUTOCAD RELEASE 10

Start up Autocad and answer each question prompted by AutoCAD as

follows:

NOTE:  <cr> is the same as <return>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Main Menu

0.  Exit AutoCAD

1.  Begin a NEW drawing

2.  Edit an EXISTING drawing

3.  Plot a drawing

4.  Printer Plot a drawing

5.  Configure AutoCAD

6.  File Utilities

7.  Compile shape/font description file

8.  Convert old drawing file

Enter selection: 5 <cr>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configure AutoCAD.

Current AutoCAD configuration

Video Display:              IBM Enhanced Graphics Adapter

Digitizer:                  Microsoft Serial or Bus Mouse

Plotter:                    ADI P386 plotter

CCP386 CalComp P386 ADI Driver Ver 2.0

58436/68436

Printer plotter:            None

Press RETURN to continue:  <cr>

Publication Part Number W0001-030

Sheet 12 of 40

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 5 <cr>

Your current plotter is ADI P386 plotter

Do you want to select a different one? <N>

If your current plotter is ADI P386 plotter then skip over

available plotter/driver selection.

Y <cr>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Available plotters:

1.  None

2.  ADI P386 plotter

3.  Alpha Merics (obsolete)

4.  Autodesk Device Interface

5.  CalComp 906/907/PCI

6.  Hewlett-Packard

7.  Houston Instrument DMP Series

8.  IBM 7300 Series

9.  PostScript device

10.  UNIX Plot device

Select device number of ? to repeat list <1>: 2 <cr>

Publication Part Number W0001-030

Sheet 13 of 40

At this point AUTOCAD will look for the environment variable

PLPADI in order to locate the required Protected ADI driver.  If

PLPADI is not found AutoCAD will display an error message and

allow you to name the P386 ADI driver.  It is best to remember to

issue the SET PLPADI command as listed in Installation.

Once the Driver issues have been solved CCP386 takes over and

requests you to select a supported plotter:

Supported models:

1.  1023

2.  1025

3.  1026

4.  1043

5.  1044

6.  1073/1075/1077

7.  2024 pacesetter

8.  2036 pacesetter

9.  52224/52424/57424

10.  52236/52436/57436/67436

11.  57444

12.  58424

13.  58436/68436

14.  58444

15.  5912 colorview

16.  5913 colorview

17.  6603 colormaster plus vrc

18.  6613 colormaster plus vrc

19.  5902A plotmaster

Enter selection, 1 to 19 <3>: 12 <cr>

Would you like to calibrate your plotter? <N> N <cr>

Write the plot to a file? Y or N <cr>

Size Units (Inches or Millimeters) <I> I or M <cr>

Plot origin in Inches <0.00,0.00>: 0,0 <cr>

Standard values for plotting size

Size    Width   Height

A       10.50     8.00

B       16.00    10.00

C       21.00    16.00

D       33.00    21.00

MAX    163.80    23.50

Enter the Size of Width,Height (in Inches) <MAX>: D <cr>

Publication Part Number W0001-030

Sheet 14 of 40

Rotate 2D plots 90 degrees clockwise? <N>: N <cr>

Pen width <0.010>: <cr>

Adjust area fill boundaries for pen width? <N> <cr>

Remove Hidden lines? <N> <cr>

Specify scale by entering:

Plotted Inches=Drawing Units or Fit or ? <F>: <cr>

THE FOLLOWING OPTIONS ARE CALCOMP PLOTTER SPECIFIC.

Use the following options to match an EXISTING plotter.

****** Configure CalComp 58424 ******

Enter decimal sync code value, 2 to 127 <22>:

Enter number of sync codes, 1 or 2 <2>:

Enter decimal EOB code value, 2 to 31 <13>:

Enable Checksumming? <Y>

0 = 95

1 = 59

Enter number corresponding to required radix, 0 or 1 <0>:

This option will allow you to lower the numbering system used in

the plot data which will stop ASCII to EBCDIC translation

problems if the plot is to be uploaded to an EBCDIC computer for

plotting.

Some supported plotters can accept data Serially or Parallel.  If

you plotter is one of them this prompt will appear

This plotter will accept data two ways.

0 = Serial

1 = Parallel

Enter number corresponding to required output, 0 or 1 <0>:

Available Serial or Parallel Ports:

1 = COM1   or   LPT1

2 = COM2   or   LPT2

Enter number corresponding to required COM/LPT port, 1 or 2 <1>:

Publication Part Number W0001-030

Sheet 15 of 40

If operating serially the following 3 prompts will appear:

Available Baud Rates:

0 = 300

1 = 600

2 = 1200

3 = 2400

4 = 4800

5 = 9600

6 = 19200

7 = 38400

Enter number corresponding to required baud rate, 0 to 7 <6>:

Available Character Framing:

0 = 8,n,1

1 = 7,e,1

Enter number corresponding to required Character Framing, 0 or 1 <1>:

Available handshaking:

0 = xonxoff

1 = hardware/direct control

2 = ACKNAK

Enter number corresponding to required handshaking, 0 to 2 <1>:

If your plotter has configurable stepsizes:

This plotter has several stepsizes.

0 = 200

1 = 300

2 = 400

3 = 2032

Enter number corresponding to required stepsize, 0 to 3 <2>:

If your plotter is multicopy capable:

Do you wish to be prompted at plot time for extra copies? <N>:

If your plotter is capable of accepting color pen definitions:

Do you wish to download default pen color definitions? <N>

Publication Part Number W0001-030

Sheet 16 of 40

If your plotter is a roll feed pen plotter:

Plotter Modes

0 = Cut Sheet

1 = Roll

Enter number corresponding to plotter mode, 0 or 1 <0>:

If your plotter is CDCL compatible:

Do you wish to download CDCL setup information? <N>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 0 <cr>

If you answer N to the following question, all configuration

changed you have just make will be discarded.

Keep configuration changes? <Y> Y<cr>

FINISHED  The CalComp CCP386 driver is now configured.

Publication Part Number W0001-030

Sheet 17 of 40

CONFIGURING CALCOMP PLOTTERS

You have the option of configuring CCP386 to match an existing plotter

setup or you can take the defaults of CCP386 and configure the plotter

to match.

Publication Part Number W0001-030

Sheet 18 of 40

Configuring the 1023, 1025, or 1026 (ARTISAN) plotter

In the following prompts there will be several lines of instructions

for each question.  The ones preceded *D are the CCP386 default

answers.  The ones preceded with *U are user definable and should be

written down so that CCP386 can be configured to match.  If you are

going to configure CCP386 to match the plotter setups,  do not execute

the *D instructions.

*D    CCP386 defaults

*U    User defined

Follow the instructions in Chapter 1 of the ARTISAN User's Guide to

load media.  Select the 'Manual' softkey.  When the plotter finishes

sizing the media,  the display will show:

User#1     OrgRtrn

Online      Replot

Press the 'Next' key twice.  The display will show:

Set-up        Test

Size          Time

Press the 'Set-up' softkey.  The display will show:

Scale      PenParm

Host    Form Align

Press the 'Host' softkey.  The display will show:

Format        Baud

Data     Handshake

Press the 'Format' softkey.

*D  Use the lower left softkey to step through the list of data

Formats.  Stop when the display shows:

*U        Data format

[PCI]

Press the 'Exit key to return to Host menu.

Format        Baud

Data     Handshake

Publication Part Number W0001-030

Sheet 19 of 40

Press the 'Baud' softkey.

*D  Press the lower right softkey to step through the list of baud

rates.  Stop when the display shows:

*U        Baud Rate

[19.2K]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Data' softkey.

*D  Press the upper left softkey to change the number of data bits.

Stop when the number changes to [7 bit]

*D  Press the upper right softkey to change the number of stop bits.

Stop when the number changes to [1 stop]

*D  Press the lower left softkey to change the parity type.  Stop when

the parity reads [Even Parity].

*U        [7 bit]   [1 stop]

[Even Parity]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Handshake' softkey.

*D  Press the lower left softkey to step through the list of handshake

protocols.  Stop when the display shows:

*U        PCI Handshake

[DirCntrl]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Next' key.  The display will show the Sync menu:

Sync        End Msg

Chksum

Publication Part Number W0001-030

Sheet 20 of 40

Press the 'Sync' softkey.

*D  Press the upper left key to change the # of sync to 2

*D  Press the '2' key twice to change the Sync Char to 22.

The display will show:

*U          # of sync [2]

Sync char.     22

Press the 'Exit' key to return to the Sync menu.

Sync        End Msg

Chksum

Press the 'End Msg' softkey.

*D  Press the '1' key and then the '3' key to change the End of Msg

char to 13.

The display will show:

*U   End of Msg Char

13

Press the 'Exit' key to return to the Sync menu.

Sync        End Msg

Chksum

Press the 'Chksum' softkey.

*D  Press the lower left softkey until PCI mode chksum changes to

ENABLED.  The display will show:

*U        PCI mode chksum

[ENABLED]

If you have changed any of the above options, press the 'Exit' key 3

times.  The display will show:

Save as User n?

n is current user number

Yes             No

Press the lower left softkey to save configurations.

If you were getting the current setups press the 'Exit' key 4 times to

return to the original menu.  The display will show:

User#1     OrgRtrn

Online      Replot

Press the 'Online' softkey to plot.

Publication Part Number W0001-030

Sheet 21 of 40

Configuring the 1043 or 1044 Pen Plotter

Set the switch on the back of the 1043/1044 plotter:

Bit 1 2 3 4 5 6 7 8

0 1 0 1 0 0 0 0   For Hardware handshaking but will work with

any protocol.

The Serial cable from the PC serial port should be attached to 'TO DTE

HOST' of the 1043/1044 plotter.

Power the plotter on.  Press the 'Setup' key, then the number 7 and

then press the 'Enter Key'.  The display will show 'Host set up

number'.  Press 'Next', 'Enter', make changes if configuring the

plotter to match CCP386 defaults, otherwise step through the Host

setups and write down what you find in order to Configure CCP386 to

match.  Repeat the process for 8 HOST set up modes ('Next' and

'Enter').

For CCP386 Defaults change to the following:

Host Set Up Mode      Top              bottom

0-Parity & bits/char  2 Even Parity    7 Bits/char

1-Stop bits & clock   1 Stop Bit       0 Int clock

2-Interface           0 Serial        00 IEEE Address

3-Host Baud           19200            0 PCI Mode

4-Muting & Checksum   NO Term muting   YES checksum

5-Isochronous & EOM   NO Isochronous   13 End of Mess Char

6-Direct Cntrl-Xon    YES Direct Cntrl NO XON/XOFF

7-Term Baud & Duplex  19200            0 Full

8-Sync Code & Value   2 Sync Code      22 Sync Code Value

Press 'Save', Enter user number and press 'Enter' to save any changes

you might of made.

Publication Part Number W0001-030

Sheet 22 of 40

Configuring the 1073, 1075, 1077 pen plotters.

There are two types of controllers available for these plotters, the

PCI and the PLOT MANAGER.  It is best if Field Service is called in

order to determine what controller is present and how to set the

switches, jumpers or thumb wheel switches.  If you wish to attempt the

setup you will need the appropriate manual.

Publication Part Number W0001-030

Sheet 23 of 40

Configuring the 2024 or 2036 Pacesetter

The Pacesetter plotters are unique in that they are the only CalComp

pen plotter capable of accepting data both serially or parallel.  The

setup procedures for the two are similar in most respects.

The following paragraphs contain the procedures for changing the

communications parameters.  Each procedure is self-contained.

To Change parameters, proceed as follows:

1.  Power-on the plotter

2.  Press the Menu key.

NOTE

The Port Type menu display only appears when the optional

Centronics interface is installed.  Skip this step and

proceed to step 4 if the plotter only has a RS-232C (serial)

port.

3.  Press the up or down arrow keys until the display shows:

Port Type:

Use the left or right arrow keys to select Port Type:

If serial                             If Parallel

Port Type:                            Port Type:

RS-232C                               Centronics

4.  This step is only for serial communications.

Press the the down arrow key until the display shows:

Protocol:

Use the left or right arrow keys to select:

Protocol:

PCI

If using parallel communications, skip to step 8.

Publication Part Number W0001-030

Sheet 24 of 40

5.  Use the down arrow key to display:

Baud Rate:

Use the left or right arrow keys to select the correct baud

rate configured in AutoCAD.  CCP386 defaults to:

Baud Rate:

19200

6.  Use the down arrow key to display:

# of bits-parity

Use the left or right arrow keys to select the correct number

of bits per character and the type of parity configured in

AutoCAD.  CCP386 defaults to:

# of bits-parity

7-even

7.  Use the down arrow key to display:

Handshake:

Use the left or right arrow keys to select the correct

handshaking configured in AutoCAD.  CCP386 defaults to:

Handshake:

Hardware

8.  Use the down arrow key to select:

Sync # EOB CHK

There are three pre defined configurations for this options

labeled A, B, or C.  Use the left or right arrow keys to display

each.  If one of the three selections match the configurations

stored in Autocad, display it and proceed to step 14.  CCP386

defaults to:

Sync # EOB CHK

A:  22   2   13   ON

Publication Part Number W0001-030

Sheet 25 of 40

9.  If none of the pre-defined configurations match your needs

select:

Sync # EOB CHK

Special Format-1 or 2

10. Then press the down arrow key.  The following will be

displayed:

PCI-# of sync

2

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

11. Then press the down arrow key.  The following will be

displayed:

PCI-Sync Char

22

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

12. Then press the down arrow key.  The following will be

displayed:

PCI-EOM Char

13

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

13. Then press the down arrow key.  The following will be

displayed:

PCI-Checksum

On

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

Publication Part Number W0001-030

Sheet 26 of 40

14. Then press the down arrow key until the following is

displayed:

Save as User:

NO

Use the left or right arrow keys to select the user number

under which these value will be stored.

15. Then press online.  The plotter is now ready for operation.

Publication Part Number W0001-030

Sheet 27 of 40

Configuring the 52224, 52424, 52236, 52436, 67436, 68436, or 68444

These setup instructions assume that a communications terminal is

attached to the TERM port located on the back of the plotter.

Press <CTRL-P> and enter user PIN or press return.

If you only wish to determine the present configuration of the plotter

use the DISPLAY command to display them.

Determine which port is connected to your computer by tracing the

cable.  The 52224, 52236, 52424, and the 52436 have 1 parallel port

called 'PAR' and three serial ports called 'COM_1  COM_2  COM_3'.  The

67436, 68436 and the 68444 have two parallel and two serial ports

called PAR_1, PAR_2, COM_1, and COM_2 respectfully.

If your PC is connected to a Parallel port type:

DISP PAR on the 52224, 52236, 52424, 52436 to display the parallel

parameters or DISP PAR_n, where n is the parallel port number, on the

67436, 68436, or the 68444.

To changed the Parallel parameters, TYPE:

PAR  or PAR_n

Input Monitor Dump                NO

Host to Plotter CDCL              CENT

Dead Job Time Out                 30

Time Out Action                   Force

Plot Command Language             907

Steps Per Inch                    200 for the 52224 or 52236

400 for the 52424, 52436,

67436, 68436, 68444

Beginning of Record Sync Char    $16 hex

Number of Sync Chars              2

End of Record char                $D

Checksum                          Y

Print                             N

Publication Part Number W0001-030

Sheet 28 of 40

If your PC is connected to COM_1, COM_2, or COM3 type

COM_n                            n is either 1, 2, or 3

Input Monitor dump                NO

Baud Rate                         19200

Character Framing                 7,e,1

Host to plotter CDCL              CTS

Plotter to Host CDCL              CTS

Dead Job Time Out                 30

Time out action                   Force

Plot Command Language             907

Steps per inch                    200 for the 52224 or 52236

400 for the 52424 or 52436

67436, 68436, 68444

Plot Data Protocol                CTS

Beginning of Record Sync Char    $16

Number of sync Chars              2

End of Record Char                $D

Checksum                          Y

To save the setups type:

SAVE n ALL    n is the user number.   The power on default user

number is 1.

To retrieve a user setup type:

USER n all    n is the user number.

If the configurations are not saved they will be lost when the plotter

is powered off.

Publication Part Number W0001-030

Sheet 29 of 40

Configuring the 57424, 57436, 57444, 58424, 58436, or 58444

An example of configuring the 908K or XP board thru OPCOM.

Refer to CalComp plotter users guide OPCOM (Operator

Communication) operations with the optional attached terminal.

These setup instructions assume that a communications terminal is

attached to the TERM port located on the back of the plotter.

PARALLEL COMMUNICATIONS:

(Framing: bits, parity and stop bits are not applicable for

Centronics interface).

<CTRL-P>

To change the communications parameters type:

COM

MODE                             CENT

CSUM                             Y

FRAME                             8,n,1

EOB                              $D  or  13

TACODE                            0

CLOCKING                          ASYNC

SYNC                             $16,$16  or  22,22

To save the parameters type:

SAVE n ALL   n is a user number

To view the existing communications setups type:

DISP COM

Publication Part Number W0001-030

Sheet 30 of 40

If OPCOM is used,  these switches may be ignored.  This is provided as

reference for CalComp Customer Service and Field Analyst.

An example of 908K Board Switch setting with CCP386.

Bits

Switches  8 7 6 5 4 3 2 1

S3      0 0 0 1 1 1 0 1  (bit 8    diagnostic mode disabled

bit 7    units = inches

bit 6-4  plotter width = 36 inches

bit 3-2  resolution = 400 dpi

bit 1    indicates color)

S4      1 1 0 0 0 0 0 0  (bit 8-7  disk type: 70 megabyte

bit 6    single disk installed

bit 5    tape bits/character: 8 bits

bit 4    tape density: 1600 bpi

bit 3-2  most significant strip

bit 1    pen plotter limits emulation

disabled)

S5      0 0 1 0 0 0 1 0   bit 8-5  DIRECT CONTROL MODE

0 0 1 1 0 0 1 0            CENTRONICS

bit 4-1  AutoCAD stepsize:  400/inch

S6      0 1 1 0 1 1 0 1  (bit 8-4  End of buffer char = '13 $D'

bit 3-1  character framing: 8 bit

no parity, 1 stop bit

S7      0 1 0 0 0 0 0 0  (bit 8    terminal muting disable

bit 7    checksum enable

bit 6    X16 clocking

bit 5-1  half duplex turn around

character not used

S8      1 0 0 1 0 1 1 0  (bit 8    double sync character

bit 7-1  sync character = '22 $16'

- Set Switch S1 to NORMAL and LOCAL

- Set rotary switches S10 and S11 according to the host baud rate

and terminal baud rate, respectively.

Publication Part Number W0001-030

Sheet 31 of 40

Configuring the 5912, 5913 ColorView, or the 5902A Plotmaster

Enclosed on the floppy with CCP386 is a program called SETUP.EXE.

This program is used to set the communication parameters.

SETUP

CALCOMP Color Thermal Plotter Configuration

Copyright (C) Calcomp 1988 Rev. x.x

Use the left/right arrow keys to scroll thru the options

Use the up/down arrow keys to scroll thru the menu

When the correct information is displayed use the <enter> key

Model Type ...............  Model 5912

Interface Mode ...........  PCI HCBS

F5 key exits the program and returns to DOS

ESC key restarts the program

HOME key moves the cursor to the top selection on current screen

END key moves the cursor to the last selection on current screen

When New: is displayed enter the data with the numeric keys

The initial screen will ask for either the 5912, 5913, or the

Plotmaster.  Use the arrow keys to select.  The interface mode is for

either PCI HCBS or PCI AUTOCAD.  If CCP386 defaults are to be accepted

select PCI AUTOCAD.

Publication Part Number W0001-030

Sheet 32 of 40

Model 5912 Configurable Parameters

Port connected to                COM1  (use arrow key for

other ports)

Baud Type                        19200

Parity Enable                    Enabled

Parity Type                      Even

Data Bits                        7

Stop Bits                        1

PCI DATA

Number of Sync characters        2

Sync character code              22

Checksum enable                  Enabled

Handshake type                   Direct Control

End of Message Character         13

Turnaround Code                  0

Scale factor numerator           1

Scale factor denominator         1

X axis Origin Offset             0

Y axis Origin Offset             0

Print Timeout Value              300

Plotmaster emulation             NO

Change Screen with PgUp or PgDn

Send Data (F2), Default (F4), System (F5), Restart <ESC>

Change the parameters to look like the above if you are using

CCP386 defaults.

F2 will send the parameters to the Colorview and save them in EEPROM.

F5 will exit the program.

Publication Part Number W0001-030

Sheet 33 of 40

Configuring the 6603 or 6613 Colormaster Plus VRC

The setups on the VRC unit us accomplished from the control panel.

Which after initialization should look something like this:

ONLINE         Reset

Setups         Tests

Take the printer OFFLINE by placing the cursor on ONLINE and pressing

ENTER.  The following menu is displayed:

OFFLINE        Reset

Setups         Tests

Position the cursor on Setups and press ENTER.  The following is

displayed:

SETUPS

User          Factory

Position the cursor on User and press enter.  The following is

displayed:

USER        DataFormats

Printer           Ports

Position the cursor on DataFormats and press enter.  The following is

displayed:

DATA FORMAT

CCGL     VDI   Epson

Position the cursor on CCGL and press enter.  The following is

displayed:

CCGL

Numerator  : 02430

Use the cursor to select each digit and change this value to read

00001.  Press enter when completed.  The following will be displayed:

CCGL

Denomintr  : 70633

Publication Part Number W0001-030

Sheet 34 of 40

As with the Numerator used the arrow keys to select each digit and

change this value to read 00001.  Press enter when completed.  The

following will be displayed:

CCGL

X Offset  :+00000

Press enter and the following will be displayed:

CCGL

Y Offset  :+00000

Press enter and the following will be displayed:

CCGL

# of Syncs    :1

Use the arrow keys to change this value to 2 if using CCP386 defaults.

Press enter when finished and the following will be displayed:

CCGL

Sync Code      :2

Use the arrow keys to change this value to 22 if using CCP386

defaults.  Press enter when finished and the following will be

displayed:

CCGL

End Code       :3

Use the arrow keys to change this value to 13 if using CCP386

defaults.  Press enter when finished and the following will be

displayed:

CCGL

Checksum:   Disable

Use the arrow keys to change this value to enable if using CCP386

defaults.  Press enter when finished and the following will be

displayed:

CCGL

TimeOut:    0010 S

Use the arrow keys to change this value to 300 if using CCP386

defaults.  Press enter when finished and the following will be

displayed:

CCGL

5902A Pens:  Yes

Publication Part Number W0001-030

Sheet 35 of 40

Use the arrow keys to change this value to NO if using CCP386

defaults.  Press enter when finished and the following will be

displayed:

CCGL

Param Set: HCBS

You have now set all the CCGL Setup parameters.  To go to the USER

submenu, press CLEAR.

USER        DataFormats

Printer           Ports

Select 'Ports' and press enter.  The following will be displayed:

SERIAL PORT

Baud Rate: 9600

Use the arrow keys to change this setting to 19200 if you are using

CCP386 defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Data Bits: 8

Use the arrow keys to change this setting to 7 if you are using CCP386

defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Stop Bits

Use the arrow keys to change this setting to 1 if you are using CCP386

defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Parity:  None

Use the arrow keys to change this setting to even if you are using

CCP386 defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Handshk:  CCGL

Press enter when finished and the following will be displayed:

SERIAL PORT

Handshk:  XON/XOFF

Publication Part Number W0001-030

Sheet 36 of 40

Use the arrow keys to change this setting to DTR/RTS if you are using

CCP386 defaults.  Press enter when finished and the following will be

displayed:

USER        DataFormats

Printer           Ports

Press the Clear button until the following is displayed:

ONLINE         Reset

Setups         Tests

Publication Part Number W0001-030

Sheet 37 of 40

SYSTEM REQUIREMENTS

IBM PC or compatible with one disk drive and a hard disk

Operating system  DOS 2.0 or higher

RS232C Serial port or Centronic parallel port

A working version of ACAD 10 386  or  ACAD 11 386.

Special requirement for the IBM PC AT

In order to use an existing 25-pin cable with the 9-pin configuration

the serial port of the AT,  construct or buy an adapter as shown below:

9 PIN FEMALE   25 PIN MALE

1 -------------------- 8         Carrier detect

2 -------------------- 3         Receive Data from plotter

3 -------------------- 2         Transmit data to plotter

4 -------------------- 20        Data Terminal Ready

5 -------------------- 7         Ground

6 -------------------- 6         Data Set Ready

7 -------------------- 4         Request to Send

8 -------------------- 5         Clear to Send

9 -------------------- 22        Ring Indicator

Publication Part Number W0001-030

Sheet 38 of 40

RECOMMENDED DOCUMENTATION

IBM PC Disk Operating System (DOS) manual

Plotter operations manual.

Publication Part Number W0001-030

Sheet 39 of 40

LIST OF MATERIALS

Each order should contain the following materials:

CalComp ADI Driver software package contained on a double sided

double density (360K) 5-1/4" floppy diskette which includes the

following files:

CCP386.EXP  The protected mode ADI driver.

CCP386.DOC  This Document

SETUP.EXE  A setup program for use with the 5912, 5913 or

Plotmaster.

Publication Part Number W0001-030

Sheet 40 of 40

DEFAULT PEN DEFINITIONS

PEN      COLOR         WIDTH in inches

1       RED            0.01

2       YELLOW         0.01

3       GREEN          0.01

4       CYAN           0.01

5       BLUE           0.01

6       MAGENTA        0.01

7       BLACK          0.01

8       ORANGE         0.01

9       LIGHT RED      0.01

10       BROWN          0.01

11       LIGHT GREEN    0.01

12       LIGHT CYAN     0.01

13       LIGHT BLUE     0.01

14       LIGHT MAGENTA  0.01

15       1/2 BLACK      0.01

PEN WIDTH TABLE

PLOTTER RESOLUTION

WIDTH     200 DPI        300 DPI        400 DPI

IN   MM        IN    MM       IN   MM

1   .005   .13    .00375   .10    .0025   .06

2   .010   .25    .00750   .19    .0050   .13

3   .015   .38    .01125   .29    .0075   .19

4   .020   .51    .01500   .38    .0100   .25

5   .025   .64    .01875   .48    .0125   .32

6   .030   .76    .02250   .57    .0150   .38

7   .035   .89    .02625   .67    .0175   .44

8   .040  1.02    .03000   .76    .0200   .51

9   .045  1.14    .03375   .86    .0225   .57

10   .050  1.27    .03750   .95    .0250   .64

11   .055  1.40    .04125  1.05    .0275   .70

12   .060  1.52    .04500  1.14    .0300   .76

13   .065  1.65    .04875  1.24    .0325   .83

14   .070  1.78    .05250  1.33    .0350   .89

15   .075  1.91    .05625  1.43    .0375   .95

16   .080  2.03    .06000  1.52    .0400  1.02