NETWORK.DOC

Page 1 of 8

PROTECTED MODE ADI - NETWORK PLOT SPOOLING

The CalComp protected mode ADI driver follows AutoCAD conventions

and provides the same network plot spooling support as an Autodesk

in-line driver.  There are basically two methods which can be used

on Novell networks:  plot to file with I/O redirection and plot to

file with AUTOSPOOL.  Since most network system software has

similar utilities, similar methods can be used on other networks.

These methods, including setup information, are described in the

following sections.

Before proceeding, verify the following:

The CalComp protected mode ADI driver is installed and has

been tested with a CalComp plotter in a local configuration.

A workstation is connected and logged into the network.

Print servers and print queues are setup and operational.

THE NOVELL CAPTURE UTILITY

The Novell CAPTURE utility can be used to redirect the output of a

logical DOS LPT device, such as LPT1, to a network print queue and

on to the plotter.

Setup:

1.  Start the CAPTURE utility before starting AutoCAD.  This can

be done at the DOS prompt or, for regular use, it can be

included in a login script or batch file.

2.  Start AutoCAD and configure for plot to file from the

configure plotter menu.  To make this the default, answer YES

when you see the following question during configuration:

Write the plot to a file?

Plot to file is also an option which can be setup at plot

time.  Since the ADI driver ignores the handshaking selection

when plotting to a file, no other changes need to be made from

the plotter configuration menu.

3.  Configure the plot file name for LPT1 (assuming that CAPTURE

is redirecting the output of LPT1).  From the operating

parameter menu, select option 3, default plot file name.  When

prompted, enter the following:

LPT1

For occasional use, you can also enter LPT1 as the file name

at plot time.

NETWORK.DOC

Page 2 of 8

4.  Make sure that the plotter is connected to the print server.

Check the plotter setup and, if the connection is serial,

ensure that the baud rate, character framing and handshaking

(xon/xoff or hardware) match the Novell printer configuration

in PCONSOLE.

This completes the setup for plot spooling using the CAPTURE

utility.

NOTES ON USING CAPTURE

If the Autoendcap option is enabled (default), captured plots will

be queued when AutoCAD completes the plot job.  This is useful if

you want to send single plot jobs to individual print queue files

and plot without exiting AutoCAD.  A new capture session starts

automatically with each plot job.

If the Auotendcap option is disabled, captured plots will be

queued when you exit AutoCAD and type ENDCAP at the DOS prompt.

This is useful if you want to append multiple plot jobs in a

single print queue file.  ENDCAP also terminates the capture of an

LPT device.

The Timeout option should be disabled (default) since it can cause

unpredictable results.

Typical captured plots appear as "LPT1 Catch" in the print queue

job description.

The following CAPTURE options should be used to prevent any

extraneous data from being sent to the plotter.  Instead of

including these in the command line, they can be set up as

defaults in PRINTCON.

NoTabs            NoBanner        NoFormFeed

For additional information please refer to the Novell Print Server

Manual and the AutoCAD Installation and Performance Guide.

THE AUTOCAD AUTOSPOOL FEATURE AND THE NOVELL NPRINT UTILITY

The AutoCAD AUTOSPOOL feature can be used to output plots to

unique files in a user defined plot spooler directory.  Once the

plot files are in the spooler directory, it is the responsibility

of a spooler program, such as the Novell NPRINT utility, to move

the files to a network print queue and on to the plotter.  AutoCAD

provides a hook with which to automate spooler execution in the

form of an environment variable, ACADPLCMD.  Once AutoCAD finishes

writing the plot file, it generates a command using the string

defined in ACADPLCMD, substituting a unique file name for the "%s"

in the string, and sends it to DOS for execution.  AutoCAD then

returns to the drawing editor or main menu, depending on where the

plot job was started.

NETWORK.DOC

Page 3 of 8

Setup:

1.  Make a directory or identify an existing directory for plot

spooling.  This directory can be on a local or network drive.

Both AutoCAD and NPRINT will need read/write access to this

directory.  For this example we will use Z:\PLOTS.

2.  Define the environment variable, ACADPLCMD, for the spooler

program (NPRINT) using the SET command.

SET ACADPLCMD=NPRINT %s

Note that the "s" is lower case and there are no spaces around

the equal (=) sign and no trailing space at the end of the

line.

If you want the plot files deleted from the spooler directory

once they're in the queue, use the Delete option with NPRINT.

For example:

SET ACADPLCMD=NPRINT %s D

For regular use, you can add the appropriate SET statement to

the AUTOEXEC batch file.  If you do, be sure to use two

percent (%) signs, since DOS batch files use the percent sign

to indicate parameters passed to the program.  For example:

SET ACADPLCMD=NPRINT %%s

Set ACADPLCMD=NPRINT %%s D

3.  Start AutoCAD and configure for plot to file from the

configure plotter menu.  To make this the default, answer YES

when you see the following question during configuration:

Write the plot to a file?

Plot to file is also an option which can be setup at plot

time.  Since the ADI driver ignores the handshaking selection

when plotting to a file, no other changes need to be made from

the plotter configuration menu.

4.  Configure the plot file name for AUTOSPOOL.  From the

operating parameter menu, select option 3, default plot file

name.  When prompted, enter the following:

AUTOSPOOL

For occasional use, you can also enter AUTOSPOOL as the file

name at plot time.

NETWORK.DOC

Page 4 of 8

5.  Configure the plot spooler directory.  From the operating

parameter menu, select option 4, plot spooler directory.  When

prompted, enter the path and name of the directory created in

step 1, including a trailing backslash.  For example:

Z:\PLOTS\

6.  Make sure that the plotter is connected to the print server.

Check the plotter setup and, if the connection is serial,

ensure that the baud rate, character framing and handshaking

(xon/xoff or hardware) match the Novell printer configuration

in PCONSOLE.

This completes the setup for plot spooling using AUTOSPOOL and the

NPRINT utility.

NOTES ON USING AUTOSPOOL AND NPRINT

Spooled plot files appear in the spooler directory and print queue

job description with unique file names.  Spooled plot files are

given unique names by AutoCAD in the format VXXXXXXX.YYY, where V

indicates a vector plot file, XXXXXXX is based on the current

time, and YYY is the network node name.  For example:

V1234567.RAY.  The default network node name is AC$ and can be

changed from the operating parameter menu, option 6.

The following NPRINT options should be used to prevent any

extraneous data from being sent to the plotter.  These should be

set up as defaults in PRINTCON.

NoTabs            NoBanner        NoFormFeed

If the spooler program doesn't delete the old plot files from the

spooler directory, you will need to do this manually to prevent

filling up the hard disk.

Besides NPRINT, you can also use your favorite spooler program.

Just define it in ACADPLCMD.  ACADPLCMD can also be used to start

a batch file, which in turn can perform some setup before starting

the spooler.  Or, instead of using ACADPLCMD, you can have a

spooler program running on a remote node, queueing up plot files

from the spooler directory.

For additional information please refer to the Novell Print Server

Manual and the AutoCAD Installation and Performance Guide.

NETWORK.DOC

Page 5 of 8

REAL MODE ADI - NETWORK PLOT SPOOLING

The CalComp real mode ADI driver provides network plot spooling

support using plot to file with I/O redirection.  There is

basically one method which can be used on Novell networks.  Since

most network system software has similar utilities, a similar

method can be used on other networks.  This method, including

setup information, is described in the following sections.

Before proceeding, verify the following:

The CalComp real mode ADI driver is installed and has been

tested with a CalComp plotter in a local configuration.

A workstation is connected and logged into the network.

Print servers and print queues are setup and operational.

Plot to file with AUTOSPOOL is not supported.

THE NOVELL CAPTURE UTILITY

The Novell CAPTURE utility can be used to redirect the output of a

logical DOS LPT device, such as LPT1, to a network print queue and

on to the plotter.

Setup:

1.  Configure the ADI driver for plot to file.  Run the

configuration program, CCPCFG.  From the communications menu,

select the File option, save the settings and exit.  There is

no need to reload the driver, since it always reads its

configuration file before plotting begins.

2.  Start the CAPTURE utility before starting AutoCAD.  This can

be done at the DOS prompt or, for regular use, it can be

included in a login script or batch file.

3.  Make sure that the plotter is connected to the print server.

Check the plotter setup and, if the connection is serial,

ensure that the baud rate, character framing and handshaking

(xon/xoff or hardware) match the Novell printer configuration

in PCONSOLE.

4.  Start AutoCAD and select a drawing to plot.  Configuring the

plot file name for LPT1 (assuming that CAPTURE is redirecting

the output of LPT1) is done at plot time.  Enter LPT1 when the

final prompt appears:

Enter output file name <CCPADI.PLT>  ==>

CCPADI.PLT is the initial default plot file name.  Subsequent

default plot file names will be the last file name used, in

this case LPT1.

This completes the setup for plot spooling using the CAPTURE

utility.

NETWORK.DOC

Page 6 of 8

NOTES ON USING CAPTURE

If the Autoendcap option is enabled (default), captured plots will

be queued when AutoCAD completes the plot job.  This is useful if

you want to send single plot jobs to individual print queue files

and plot without exiting AutoCAD.  A new capture session starts

automatically with each plot job.

If the Auotendcap option is disabled, captured plots will be

queued when you exit AutoCAD and type ENDCAP at the DOS prompt.

This is useful if you want to append multiple plot jobs in a

single print queue file.  ENDCAP also terminates the capture of an

LPT device.

The Timeout option should be disabled (default) since it can cause

unpredictable results.

Typical captured plots appear as "LPT1 Catch" in the print queue

job description.

The following CAPTURE options should be used to prevent any

extraneous data from being sent to the plotter.  Instead of

including these in the command line, they can be set up as

defaults in PRINTCON.

NoTabs            NoBanner        NoFormFeed

For additional information please refer to the Novell Print Server

Manual and the AutoCAD Installation and Performance Guide.

NETWORK.DOC

Page 7 of 8

BATCHPLOT - NETWORK PLOT SPOOLING

Batchplot doesn't directly support network plot spooling.

However, Batchplot can output plots to files in a user defined

plot spooler directory.  Once the plot files are in the spooler

directory, it is the responsibility of a spooler program to move

the files to a network print queue and on to the plotter.

When the ADI driver is setup for plot to file, Batchplot forces

the driver to write its files to the AutoCAD plot spooler

directory.  Since Batchplot also generates the plot file names,

plot to file with I/O redirection and plot to file with AUTOSPOOL

are not supported.

Before proceeding, verify the following:

The CalComp protected mode or real mode ADI driver and

Batchplot are installed and have been tested with a CalComp

plotter in a local configuration.

A workstation is connected and logged into the network.

Print servers, print queues and a spooler program are setup

and operational.

Setup:

1.  Make a directory or identify an existing directory for plot

spooling.  This directory can be on a local or network drive.

Both AutoCAD and the spooler program will need read/write

access to this directory.  For this example we will use

Z:\PLOTS.

2.  Configure the ADI driver for plot to file.

A.  Protected mode ADI driver:

Start AutoCAD and configure for plot to file from the

configure plotter menu.  Answer YES when you see the

following question during configuration:

Write the plot to a file?

Since the ADI driver ignores the handshaking selection

when plotting to a file, no other changes need to be made

from the plotter configuration menu.

B.  Real mode ADI driver:

Run the configuration program, CCPCFG.  From the

communications menu, select the File option, save the

settings and exit.  There is no need to reload the driver,

since it always reads its configuration file before

plotting begins.

Start AutoCAD.

NETWORK.DOC

Page 8 of 8

3.  Configure the plot spooler directory.  From the operating

parameter menu, select option 4, plot spooler directory.  When

prompted, enter the path and name of the directory created in

step 1, including a trailing backslash.  For example:

Z:\PLOTS\

Save the settings and exit.

4.  Make sure that the plotter is connected to the print server.

Check the plotter setup and, if the connection is serial,

ensure that the baud rate, character framing and handshaking

(xon/xoff or hardware) match the network spooler printer

configuration.

This completes the setup for plot spooling using Batchplot.

NOTES ON USING BATCHPLOT

Batchplot file naming conventions:

Normal plots:  Drawing file name with .P00 extension.  For

example:

COLUMBIA.P00

Normal plots (multiple copies):  Drawing file name with .PXX

extension, where XX is the copy number starting with 00.  For

example:

COLUMBIA.P00  (first copy)

COLUMBIA.P01  (second copy)

Nested plots:  BATCHPLT with .YYY extension, where YYY is the

sequence number starting with 001.  For example:

BATCHPLT.001  (first nested plot)

BATCHPLT.002  (second nested plot)

AutoCAD default plot file names such as LPT1 and AUTOSPOOL are

ignored.

The AutoCAD spooler automation hook, ACADPLCMD, is not supported.

The spooler program should be transparent to the plot data, i. e.,

it shouldn't send any extraneous data to the plotter.

If the spooler program doesn't delete the old plot files from the

spooler directory, you will need to do this manually to prevent

filling up the hard disk.

For additional information please refer to your spooler program

documentation and the AutoCAD Installation and Performance Guide.