LANDR1.ZIP contains the following Novell server drivers.  The following files

are the lastest certified OS drivers available.

MSM      NLM    15323  2-09-93  2:18p - 4.0 Media Support Module (MSM)

MSM31X   NLM    16118  2-09-93  2:19p - 3.11 Media Support Module (MSM)

ETHERTSM NLM     7418  1-29-93  8:40p - Ethernet Topology Support Module (TSM)

TOKENTSM NLM     7418  1-29-93  8:40p - Token Topology Support Module

RXNETTSM NLM     6202  1-06-93 10:04a - Arcnet Topology Support Module

PCN2LTSM NLM     5691  1-30-93 11:32a - PCN2L Topology Support Module

FDDITSM  NLM     6955  1-29-93  8:44p - FDDI Topology Support Module

NE1000   LAN     4468  1-20-93  3:18p

NE2000   LAN     7294  1-20-93  3:17p

NE2      LAN     4974  1-20-93  3:19p

NE2_32   LAN     5210  1-26-93 10:14a

NE3200   LAN     9719  2-08-93  8:26a

NE1500T  LAN     9963  1-07-93  3:36p

NE32HUB  LAN    12266  1-27-93  9:11a

TOKENDMA LAN     8822  1-29-93  8:45p

TOKEN    LAN     9912  1-11-93 12:25p

NTR2000  LAN     9920  1-11-93 12:23p - Novell NTR2000 TOKEN RING Driver

TRXNET   LAN     3075  1-07-93  1:23p

PCN2L    LAN     4726  1-29-93  8:45p

LSLENH   NLM    11641 11-16-92  8:29a - LSL ENHANCED NLM.

PATCHMAN NLM     9051  4-16-92 11:21a

MONITOR  NLM   117775 10-26-92  9:21a - REQUIRED with new drivers on v3.11

!NVL1201 CFG     7378  3-26-92  9:08a - NE32HUB CFG file

@7151    ADF     2346  7-26-89  5:08p - NE2_32 ADF file

!NVL0701 CFG     2745  9-02-92  1:25p - NE3200 CFG file

!NVL1301 CFG     1169  3-12-92  2:28p - NE32HUB TPE CFG file

!NVL1401 CFG      986  6-10-92  4:02p - NE32HUB PME CFG file

!NVL1501 CFG    11994  1-11-93  3:24p - NE2000 plus CFG file

!NVL0901 CFG     8293 10-04-90 10:42a - NE2100/NE1500T CFG file

@7154    ADF     2657  4-27-92  1:54p - NE2 ADF file

FIRMLOAD COM     1628  1-04-91  8:57a - Use with TOKENDMA to load firmware.

XLOAD    EXE    13872  8-06-91  4:47p

XLOAD    DOC     3472  7-06-92 11:00a

LDR001   PTF    52920 11-06-91  4:55p

LDR001   DOC     3200 07-06-92 11:00a

LANDR1   DOC     (this file)

---------------------------------------------------

Overview:  Server ODI drivers are now composed of 3 different NLMs. For

example when loading the NE2000.LAN both ETHERTSM.NLM and MSM31X.NLM will

autoload.  When loading TOKEN.LAN both TOKENTSM.NLM and MSM31X.NLM will

autoload etc.  The MSM31X.NLM will always load and the TSM is specific to

topology.

LANDR1.ZIP replaces the following in NE386.ZIP.

NE2      LAN    15548 11-11-91 11:19a

!NVL0701 CFG     2585 12-17-91  8:38a

NE3200   LAN    19198  1-15-92 11:24a

NE1000   LAN    11857  7-25-91 12:02p

NE2000   LAN    12389  7-26-91  9:36a

NE2-32   LAN    12185  7-25-91 11:52a

@7151    ADF     2346  7-26-89  5:08p

@7154    ADF     2657 10-03-88  9:42a

NE2100   LAN    16405 10-09-91  4:36p

NE1500T  LAN    16406 10-09-91  4:35p

NE386    DOC     3294  5-29-92 12:28p

The server drivers originally provided in NE386.ZIP fixed the following:

1)   These versions have been rewritten to now use the MSM (Media

Support Module).  The MSM is the new tool (module) available to LAN

driver developers from Novell to develop better LAN drivers.

2)   In some environments these new drivers could help solve

communication problems.

3)   The file server has the potential of locking up under a heavy load

if the file server is a 486 machine and you are using either an

NE2000 or an NE/2.  A race condition exists where the last remote

DMA write cycle of the National Semiconductor DP8390 (on the NIC

card) can be prematurely terminated by writing to the Command

Register with bit "RD2" set before the remote DMA write cycle has

completed (refer to a future IMSP technical bulletin for more

detailed information).  These later versions of the drivers address

the issue.

4)   On certain networks with high traffic the adapter would stop

receiving.

LANDR1.ZIP drivers additionally include the following enhancements:

5)   The default frame type for ethernet drivers is now ethernet_802.2

rather than ethernet_802.3.  However the ETHERTSM.NLM will detect

if the driver is being loaded on Netware 3.11 and default to

Ethernet_802.3.  If the OS isn't Netware 3.11 then the default

frame type will be Ethernet_802.2.

6)   Drivers updated to the latest Novell 4.0 specification.

7)   NE2000.LAN has been enhanced to use shared ram for the NE2000+.

8)   NE2100.LAN has been enhanced to take advantage of the new PCNET

ISA chip set from AMD (performance enhancement).  This will allow

the new clones that have this chipset to run faster.

9)   NE3200.LAN has had some performance enhancements.  This newer

driver in combination with the newer .CFG file is more stable.

When using the NE3200.LAN, Novell support recommends selecting EDGE

TRIGGERED Interrupts when running the EISA configuration and using

the lastest !NVL0701.CFG (DMA timing modified) file supplied in

LANDR1.ZIP.

10)  TOKEN.LAN has been enhanced to use the FAST or QUICK path

available on the IBM NASSAU II FRU #74F9415.  This should increase

performance over early TOKEN.LAN drivers if using these boards

in the file server.  If this driver doesn't load with the IBM

token ring card you have, you may have one of the newer cards

from IBM that requires the IBM supplied driver (for example

TOKENBM.LAN).

11)  TOKENDMA.LAN - This driver is more reliable and better written

than previous drivers provided and distributed.  Also note that

because the card is busmastering and not 32 bit the driver has

to double buffer when writing above 16 meg of RAM.  The issue

doesn't apply to servers that have 16 meg or less of ram.

To install this version of TOKENDMA.LAN:

a. Copy TOKENDMA.LAN, MSM.NLM, and TOKENTSM.NLM and MONITOR.NLM to

the SYSTEM directory.

b. When you load TOKENDMA.LAN it will autoload MSM.NLM and

TOKENTSM.NLM. The lan driver is now more modular and is in 3

separate NLMS instead of being combined as the 3.11 drivers you

have been used to in the past.

c. When loading this driver with 16meg or more on the server, use

the following syntax:  "load tokendma below16"

d. When loading the driver more than once for multiple boards in

the server and the server has 16meg of memory or more, increase

the Reserved Buffers Below 16 Meg in STARTUP.NCF:

"Set Reserved Buffers Below 16 Meg = 50".

If the error

"TokenDMA-NW-073: Unable to allocate memory. Attempt to

reinitialize re-entrant module FAILED 12/30/92 1:20pm: 1.1.39

Cache memory allocator out of available memory."

comes up on the screen when loading the driver, more Reserved

Buffers Below 16 Meg need to be allocated.

* * * * * * * * PLEASE NOTE: * * * * * * * * * * * * * * * * * * *

The loader portion of the Server.exe will need to be modified in

order to properly load the new lan driver.  To make this change in

Server.exe you will need to use XLOAD.EXE and LDR001.PTF.  For more

information about xload and ldr001, read XLOAD.DOC and LDR001.DOC.

XLOAD should always be applied when using interrupts 2 and 9.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

12)  TRXNET.LAN - This driver is tested only with the Novell RX-NET

RX-NET/2 boards.  It may or may not work with other cards or

brands.  From a support standpoint third party vendors should

provide their own drivers for their cards.

LANDR1.ZIP has the following limitations:

1)   LSLENH.NLM may have problems with third party drivers that don't

use the ECB driver workspace properly.  Developers must use this

work space according to the spec.  Recently certified drivers do

so.

2)   PATCHMAN.NLM and LSLENH.NLM provided in LANDR1.ZIP aren't

currently supported on SFT III v3.11.

INSTRUCTIONS TO INSTALL:

A) Rename the old .lan file, monitor.nlm, patchman.nlm.

B) Copy the .lan and all .nlm files into your system directory.

C) Load lslenh.nlm first.  this will auto load patchman.nlm.

D) Load <filename>.lan .this will auto load msm/msm31x and xxxxxTSM.NLM.

* * * * * * * * * * * * * *  PLEASE NOTE * * * * * * * * * * * * * * * * * *

NOTE 1:  USE the included MONITOR.NLM with these new drivers.  If you use

the shipping v3.11 MONITOR.NLM your server could abend when viewing

LAN statistics in MONITOR.NLM with these newer drivers.  This

MONITOR.NLM included is tested and fixed.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

NOTE 2:  loading msm.nlm will generate warnings that cause

the MSM31x.NLM to be auto-loaded.  This is normal.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************