NBSCOM

Version 1.2

01/07/89

Copyright(c) Franklin Antonio, N6NKF

NBSCOM is a program to allow you to accurately set the DOS date/time in your

PC using the National Bureau of Standards Telephone Time Service.

You simply run NBSCOM, and it dials up the NBS, reads the time, sets your

DOS clock, and tells you how far your DOS clock needed to be advanced or

retarded.

Required equipment:  Any PC and a modem.

The NBS Time Service presently uses an ordinary phone number in the 303 area

code.  The only cost of using the service is the cost of a very quick long-

distance call.  The NBS plans to switch to a 900-number in the future.

Instructions..

Take care of two small configuration setup issues, (telling NBSCOM about

your timezone, and your modem), then you just run the program.  See

"Configuration Issues" below.

Running the program..

Just type NBSCOM while at the DOS prompt.

NBSCOM will tell your modem to dial, and all characters exchanged

between the modem and computer will be displayed on the screen.  The NBS sends

one time-stamp every second.  As soon as NBSCOM has correctly received two time

stamps in a row, it will hangup the phone, and tell you the results.  An

example session follows.  At any time during the modem dialog, you can type

any key, and the program will hang up the phone and exit immediately.

A sample session...

--- Modem dialog follows --- Type any key to abort. ---

? = HELP

National Bureau of Standards

Telephone Time Service

D  L D

MJD  YR MO DA  H  M  S ST S UT1 msADV         OTM

47511 88-12-16 06:03:44 00 0 -.1 045.0 UTC(NBS) *

47511 88-12-16 06:03:45 00 0 -.1 045.0 UTC(NBS) *

--- Setting DOS Date & Time to local: 12/15/1988 22:03:44 ---

--- Your DOS time was ahead of NBS by 5.22 seconds ---

--- Info only: Your hardware realtime clock reads: 12/15/1988 22:04:44 ---

--- Done --- Phone call duration was 20.4 seconds. ---

The "Info only" line about your hardware realtime clock is only printed if you

have an IBM AT, or a machine with an AT-compatible BIOS.  NBSCOM reads the AT

RTC, but does not, at this time, attempt to set it.  It makes no attempt to

read or write the large variety of different third-party aftermarket RTCs that

are present in many IBM PC's and XT's.

The average phone call duration is about 20 seconds.  If there is phone line

noise that garbles some of the characters, it will take a little longer.

NBSCOM contains a timeout to prevent phone calls longer than 30 seconds.

The NBS has provided a timeout which hangs up the phone at their end at 55

seconds.

Configuration Issues..

You must set a DOS environment variable to tell NBSCOM what timezone you're in.

The program needs this information so that it can convert the UTC (or GMT, or

ZULU or whatever you like to call it) that NBS speaks to your local time.

The environment variable is "TZ".  (short for TimeZone)  You should put the

command to set it in your autoexec.bat file, so that you never forget.

You must set TZ to your timezone by executing a DOS command like this...

SET TZ=PST8PDT

The first three letters (PST in this case) are the name of a timezone, followed

by an optionally signed number indicating the number of hours difference

between UTC and your timezone, followed by an optional three letter name of a

daylight-savings-time timezone.  If you live in an area where daylight-savings-

time is not used, you should leave off the last three letters.

Other examples..  SET TZ=EST5EDT  or SET TZ=MST7MDT, etc.

If you don't set TZ at all, it defaults to PST.

This program comes with an initialization file, NBSCOM.INI, which specifies

several parameters which may need to be customized for your computer system.

Parameters in this file include...

1.  Which COM port to use.  (default COM1)

2.  The command to dial your modem (default ATDT1-303-494-4774)

3.  The command to hang up your modem (default ATH)

4.  The maximum duration allowed for a phone call.

The default parameters should work for more than 90% of all modems, however,

you should modify the file if it doesn't work for your particular setup.

It is also possible to set the baud rate in the .INI file, however, you should

leave it at 1200.  The NBS only supports 300 and 1200, and uses different

text formats for each.  The program right now only understands the 1200 baud

format.

Copyright statement...

This program (source and object) are Copyright (c) Franklin Antonio, 1988,

All Rights Reserved.  They may be freely distributed and used for non-

commercial purposes only.  If you redistribute this package, you must

distribute all the files (source, object, doc, ini), in their original,

unmodified, form.  You may, additionally, distribute modified versions with

the unmodified versions, but the modified versions must be clearly identified

as modified, with the original copyright statements intact, and the name and

address of the modifier clearly shown.

Why do you need accurate time?

I don't know why *you* need accurate time.  I can tell you why I want accurate

time on my PC.  I use my computer to point antennas at Earth-orbiting

satellites.  Low-Earth-Orbiting satellites move the fastest, and therefore

require the most accurate time.  Consider a satellite orbiting at 250 kilometer

altitude.  (The Russian MIR space station, and the US Space Shuttle orbit at

about this altitude.)  We can calculate how much error we'll have in our

computed antenna pointing angles caused by a certain amount of time error.

These angles change fastest when the satellite is directly overhead, so we'll

compute that case.  It comes out like this...

Time Error	Pointing Error

1 second          1.8 degrees

10 seconds        17.  degrees

1 minute         61.  degrees

Technical Notes:

One of the challenges when writing any program that attempts to dial and

interact with a modem is the very large variety of different brands and

models of modems in use.  They all support a slightly different set of

commands, and handle the various RS232 modem control signals in a slightly

different way.  Furthermore, there are an amazing variety of different ways

that RS232 cables can be wired.

Most commercial communications programs (Crosstalk, MIRROR, ProComm,...) solve

this problem by providing a large number (as many as 20) different config-

uration files which tell the program how to interact with each different kind

of modem.  These differences include the command set, handling of modem control

signals (which are required, and which are provided by the modem), handshake

timing, call progress indications, etc.

I've taken a different approach.

I've written serial i/o routines which are completely oblivious to the state

of the RS232 modem-control signals.  If your modem happens to not provide one

of the modem-control signals, or use one of them in an unusual way, NBSCOM

doesn't care.  (The standard IBM BIOS, on the other hand, requires that the

modem raise DSR and CTS.)  For outgoing modem-control signals, NBSCOM raises

DTR and RTS, which, hopefully will satisfy all modems that care.  (The standard

IBM BIOS raises DTR, but then toggles RTS, depending on whether you are reading

or writing to the serial port.)

I've also provided an .INI file where you can specify the exact character

strings that should be sent to your modem to dial and hangup.  The Hayes-type

commands that i've included by default will probably work with 90% of all

modems, but if you require something different, you can just change it.

NBSCOM also raises DTR and RTS before dialing, and drops them after dialing.

Some modems hang up the phone in response to a falling DTR instead of a

command.

Finally, while most commercial programs attempt to interact with the call

progress indications (ie detection of dial-tone, busy signal, etc) provided

by most modern modems, i have ignored them completely.  I simply dial, then

wait for characters from the modem that look like the NBS format.  This has

the disadvantage that if anything goes wrong during the call (say, for example,

a busy signal) a program that listens for the call-progress info from a modem

would know that that had happened immediately, and tell you.  This program will

instead simply wait 'til the 30 second timeout occurs.  This simpleminded

approach has the advantage that it is compatible with a wide variety of modems,

and it has not yet been an inconvenience.  I have yet to get a busy signal from

NBS!  You can, of course, immediately abort a call at any time manually, by

typing any key on the keyboard.

Disclaimer..

I have no affiliation with NBS.  Comments here do not represent my employer,

or NBS, or anyone but me.  This software is given away, not sold.  I make no

warranty that this software is suitable for any purpose whatsoever.  I have

tested it, and it seems to work for me.  My friends Paul Williamson, KB5MU and

Mike Brock, WB6HHV have tested it, and it seemed to work for them.  Each of us

has a different brand of computer and modem, so there is a good possibility

that this software will work for you too.  Please enjoy it.

Edit History....

Ver 1.0 -- 12/20/88 initial release

Ver 1.1 -- 01/07/88 Bill Taylor & Steve Quigley asked for increased

timeout.  "maxtime" now settable in the .ini file

Ver 1.2 -- fix msg when call times out.

Comments about this program can be sent to...

Franklin Antonio, N6NKF

2765 Cordoba Cove

Del Mar, CA  92014

Compuserve ID: 76337,1365 (HAMNET Forum)

Comments about the National Bureau of Standards Telephone Time Service may be

sent to...

NBS-ACTS

Time and Frequency Division

Mail Stop 52

325 Broadway

Boulder, CO  80303