====================

+----+ |         | |

|    | +--+ ---+ | |

|  -+| |  | +--+ | |

+---++ +--+ +--+ - -

+-

====================

SMALL ARMS EXTERIOR BALLISTICS

for

IBM PC's and COMPATIBLES

__________

Application Programming

and Documentation

by

Benjamin  W. Hartley

Forest Park Estates #103

Jaffrey, NH        03452

---

Voice#:   (603) 532-6248

America OnLine: BHartley

CompuServe:   70033,2612

GEnieMail:    B.HARTLEY1

__________________________________________________________________

_______

____|__     |               (R)

-|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

----|   |   |---------------------

|___|___|    MEMBER

The author is a member of the Association of Shareware Pro-

fessionals, the ASP.   ASP wants to make sure that the shareware

principle works for you.   If you are unable to resolve a share-

ware-related problem with an ASP member by contacting the member

directly, ASP may be able to help.  ASP's Ombudsman can help you

resolve a dispute or  problem with an  ASP member,  but does not

provide technical support for members' products. Please write to

the ASP Ombudsman at 545 Grover Rd., Muskegon, MI 49442-9427 USA

(FAX 616-788-2765, 24 hrs./day) or send a message via CompuServe

mail to: ASP Ombudsman 72050,1433.

__________________________________________________________________

"QBALL" -- source code, executable files, and documentation

are (c) 1991-93 by Benjamin W. Hartley. All rights reserved.

[ QBALL                                                       ii ]

TABLE OF CONTENTS

Dedication, Disclaimer, License, Copyright Information... i

Table of Contents....................................... ii

List of Figures........................................ iii

List of Tables......................................... iii

Introduction............................................ iv

About Shareware.......................................... v

Chapter                                        page

1. Getting Started............................. 1

1.1 Files Required............................ 1

1.2 Will It Work ?............................ 1

1.3 Copying the Program....................... 1

1.4 General Instructions...................... 2

1.5 Starting QBALL............................ 3

2. Computing Ballistic or Gun Data............. 7

2.1 Preliminary Data Entry.................... 7

2.2 Computing and Displaying a Range Table.... 8

2.3 Displaying Graphics...................... 10

2.4 Changing Input Data...................... 15

2.5 Other Ballistics Procedures.............. 17

Point-Blank Range........................... 17

Wind Vector Corrections..................... 18

Slant Angle of Fire......................... 19

Sectional Density........................... 20

Free Recoil Velocity and Energy............. 20

2.6 Bullet Lookup Facility................... 21

3. Inferring Ballistic Coefficients........... 23

3.1 First Run: r1 = 0........................ 24

3.2 Second Run: r1 = 25...................... 25

4. Printed Output............................. 27

4.1 Printer Warnings......................... 27

4.2 Screen Dumps............................. 27

4.3 Printer Setup............................ 27

4.4 Print Functions.......................... 28

4.5 Typical Printed Output................... 28

5. Configuring Display/Default Starting Values 31

5.1 The Utilities and Configuration Menu..... 31

5.2 Toggle Active Switches................... 31

5.3 Configuring Display Colors............... 32

5.4 Using the Values You've Set.............. 33

5.5 Invoking a DOS Shell..................... 34

5.6 Reading Range Tables/Translating Graphics 34

6. Recalling Saved Trajectory Data............ 35

7. File Handling.............................. 37

8. QBALL's Associated Utilities............... 39

8.1 RRGTABLE.EXE............................. 39

8.2 GRFXLATE.EXE............................. 40

Appendices:--

A. Glossary................................... 43

B. Ballistic Coefficient from Tabular Data.... 49

C. Typical Saved Trajectory File.............. 51

D. Selected Bibliography...................... 53

E. Error Analysis............................. 55

F. QBALL and Other Programs................... 57

G. Error Messages............................. 61

H. Default Starting Values.................... 63

[ QBALL                                                       iv ]

INTRODUCTION

QBALL is a menu-driven application for the rapid computation of

ballistic data for small arms projectiles. QBALL will quickly

compute point-of-impact data, velocity and energy levels,

trajectories relative to line-of-sight, and times of flight.

Additional routines permit the calculation of ballistic coeffi-

cients, slant-fire corrections, point-blank range, and gun free

recoil. Printing routines (parallel printers only, LPT1) may be

used to produce hard copy. Starting values and the display to

which the program will default may be changed using the built-in

configuration routines.

You are assumed to have some knowledge of exterior ballistics.

QBALL will not teach you exterior ballistics. Several of the ref-

erences in the Bibliography contain basic introductions to the

subject. Their study will amply repay the effort expended.

Warning!  All QBALL-generated data should be used as a guide to

ballistic performance! You MUST verify QBALL's data with a stan-

dard reloading manual. Several manuals are listed in Appendix C.

If you find QBALL of use, but would like it to do something that

it doesn't do now, or have suggestions to improve what it does

already, please send in a copy of the Enhancement Form. If you are

the first to suggest an improvement which is included in a later

QBALL release, you will receive a free copy of that release.

Should you have problems with QBALL, you may call the author at

603-532-6248; no collect calls, please! Try to call during

reasonable hours. (For guidance: no calls later than 11:00 p.m.

Eastern Time.) You may contact the author by CompuServe mail

message to Ben Hartley, 70033,2612; on America On Line, address

BHartley; or on GEnieMail, address B.Hartley1. If you would prefer

to write, please do so. The author will respond to all requests

help.

The latest registered version of QBALL is available from the

author for a $25.00 registration fee. Registration entitles you to

the latest version with one free upgrade, and a 3-hole-punched,

printed manual. The Professional Development System 7.1 source

code for the current version is available to registered users for

$50.00. You may register and get the source code at the same

time for $60.00.

The most current shareware version of QBALL, which is func-

tionally the same as the same-numbered registered version, is

available direct from the author for a $7.50 fee, and on the

EXEC-PC BBS (Modem: 414 789-4210, 1200/2400 8N1); the LegalEase BBS

(Modem: 509 326-3238, up to 14400 8N1); in the CompuServe Outdoor

Forum, filename "QBLnnn.ZIP";  and on America OnLine, in the

"Sports" Conference, keyword "ballistics." (The author uploads to

these services and BBS. Downloads from other sources may not

include all the files -- caveat emptor.)

QBALL

User's Guide

[ QBALL                                                        1 ]

Chapter 1

Getting Started

"""""""""""""""

1.1  Files Required.

Shareware copies of QBALL are distributed with the following

files in the QBALL220.ZIP/QBL220.ZIP distribution file:

QBALL.EXE       The QBALL executable file;

EGA_SAV.0       EGA format trajectory graphics file;

CGA_SAV.0       CGA format trajectory graphics file;

RRTABLE.EXE     } QBALL utilities, stand-alone and

GRFXLATE.EXE    } callable from within the program;

AMMO.DAT        Data for on-line bullet lookup;

D.NDX, M.NDX    Index files for bullet lookup;

QBALLODD.DOC    Documentation, odd-numbered pages;

QBALLEVN.DOC    Documentation, even-numbered pages;

QBALLENH.DOC    Suggested enhancement form;

QBALLREG.DOC    QBALL registration form;

QBREADME.BAT    Prints short intro on screen;

ONEDOC.COM      Utility to consolidate documentation.

The only absolutely required file is QBALL.EXE.

1.2  Will It Work ?

QBALL is written in and compiled as a stand-alone .EXE file

using Microsoft Professional Development System 7.1 on a Gateway

2000 AT clone (80286 processor), AMI BIOS, MS-DOS 6.0, 4DOS 4.02,

1-5� inch (1.2 meg) and 1-3� inch (1.44 meg) floppy drive, dual

40-megabyte hard disks, and Samsung CM4531 EGA monitor. No co-

processor was used.

QBALL is fully operational with MS-DOS 3.3, 4.01, 5.0, and 6.0,

both with "plain vanilla" COMMAND.COM and 4DOS 4.00/4.02, and

should function properly on any PC-DOS or MS-DOS computer. QBALL

will also function properly in the WINDOWS 3.n "DOS box;" insuf-

ficient experience is available regarding OS/2 to generalize, but

it is known that QBALL will not operate under OS/2 protected mode.

Routines are included to detect CGA or EGA installation; if no

graphics are available, QBALL internally disables the graphics

routines.

1.3  Copying the Program Files.

It would be wise to make a copy of the original QBALL files, and

keep it in a safe place. Run QBALL from copies of the original!

If you have a hard disk, it is recommended that you make a spe-

cial QBALL directory on it. (QBALL will run from a floppy, but it

is s-l-o-o-o-w!) Make sure that you copy QBALL.EXE, RRTABLE.EXE,

and GRFXLATE.EXE to the same directory, or onto the same disk.

QBALL.EXE will look for its associated utility files in the

current directory. Copy AMMO.DAT, the two *.NDX files, and the two

*GA_SAV.0 files to the QBALL directory as well.

[ QBALL                                                        3 ]

1.5  Starting QBALL

Log into the directory or onto the drive where you've put

QBALL.EXE and type:

QBALL (+ [Enter]).

Hint: If you have a monochrome monitor, or you just don't like

colors, start QBALL with: QBALL /b (+ [Enter]). Save the black-

and-white configuration; see Chapter 5.

The next thing you'll see is the opening screen, which looks

something like this:

------------------------------------------------------------

+==========================================================+

|            +=======+  =            =  =                  |

|            |       |  |      ___   |  |                  |

|            |       |  |         |  |  |                  |

|            |       |  +---+ +---+  |  |                  |

|            |    =+ |  |   | |   |  |  |                  |

|            +=====+=+  +---+ +---+  -  -                  |

|                  +=         Reg #N220SW                  |

|                                                          |

|      [ Exterior Ballistics for PC's & Compatibles ]      |

|      [ Using G1 Drag Functions  ---  version 2.20 ]      |

|      [ Copyright (c) 1991-3,  Benjamin W. Hartley ]      |

+----------------------------------------------------------+

Welcome to computer ballistics with QBALL!

QBALL provides small arms trajectory data quickly...

Users are encouraged to verify QBALL data with a...

handbook!  The author will NOT be responsible for ...

misapplication of QBALL-generated data.

EGA

===> press any key <===      Graphics

------------------------------------------------------------

Figure. 1. Opening Screen.

If your computer is graphics-equipped, QBALL reports what kind

of graphics it has found: "CGA," or "EGA."  (VGA graphics "fall

through" to EGA standard.) If no graphics equipment is detected,

QBALL specifies "No Graphics."

The version number are shown on the opening screen. (Here, it's

"version 2.20." If you need to contact the author with comments,

suggestions, or requests for help, it would be helpful if you

include the version number with your communication.

Press any key to continue.

[ QBALL                                                        5 ]

Note, also, that standard metro is shown on the info line. There

are several occasions when you'll need to confirm that QBALL is

indeed using standard metro; make a mental note of what the

standard metro display looks like:

[ Met: 59d 0' 29.53" 78%  ].

One frequent use for QBALL is the production of ballistics

tables for a specific cartridge/load. With the Main Menu shown on

the screen, press <C> to open the menu for computation of ballis-

tic and gun data. More information about using this option is

contained in Chapter 2.

If you wish to infer a ballistic coefficient, then press <B>.

More information is to be found in Chapter 3.

Use <U>tilities... accesses the menu which permits changing the

display colors, the status of the printer, graphics grid, and

noise toggles, and several values which QBALL will use when

started. A "shell to DOS" routine is also included. See Chapter 5

for more information about configuring QBALL. You may also access

"RRTABLE" and "GRFXLATE" from within the Utilities Menu.

<R>ecall Saved Graphics...accesses saved trajectory data, and

allows you to use that data. If there is no trajectory data saved,

this option will not appear. (You did copy the two "*GA_SAV.0"

files from the distribution disk, didn't you?) This option is dis-

cussed in detail in Chapter 6, Recalling Saved Trajectory Files.

<Q>uit... -- press <Q> to return to DOS. QBALL will ask you to

confirm that you indeed wish to quit. Press [Y] to return to DOS

or press "N" to continue using QBALL. If you confirm that you wish

to quit, and you have saved trajectory data, you will also be

prompted:

Retain Saved Data ? ([Y] or N).

The default is [Y] -- that is, keep the saved data. It is strong-

ly recommended that you press <N>: erase the saved data, at least

until you've played with QBALL for awhile.

Let's put QBALL to work...

Press <C> to compute ballistic data, and to do the tutorial

which follows. Go on to Chapter 2, page 7.

[ QBALL                                                        7 ]

Chapter 2

Computing Ballistic or Gun Data

"""""""""""""""""""""""""""""""

2.1  Preliminary Data Entry.

Before doing anything else, we're going to have to enter some

information for QBALL to chew on. The items of information for

which QBALL will prompt you are, in order, (with suggested

practice entries):

a. The ballistic coefficient (C1) for the

projectile in question................ .300

[  For tutorial purposes, enter ballistic ]

[ coefficients and bullet weight by hand, ]

[ and reserve trying QBALL's bullet look- ]

[ up facility for later on.               ]

b. The bullet weight in grains...................  180

[ Automatic entry if data lookup is used. ]

c. Whether or not to use the displayed metro.....  [Y]

[ QBALL  will  default to Standard Metro; ]

[ you can use nonstandard metro, but just ]

[ to keep things simple,  stay with stan- ]

[ dard metro for the present.             ]

d. The muzzle velocity in FPS.................... 2200

Enter values at the appropriate places when prompted. If you try

to use an impossible value, QBALL will respond with an error

message specifying a valid range of values for that parameter. Try

entering some impossible values and see what happens. A complete

list of QBALL's error messages is contained in Appendix G, with a

description of the most common conditions under which they

are displayed.

[ Bullet Lookup Facility:                                        ]

[     For initial ballistic coefficient and bullet weight entry, ]

[ Press [L] while  the appropriate entry box  is displayed.  Use ]

[ PgUp, PgDn, Home, End, and arrow keys  to navigate through the ]

[ list. Press [Enter] to use the highlighted data. Bullet lookup ]

[ sets/resets both  ballistic coefficient and bullet weight when ]

[ used. Maker abbreviations: HDY=Hornady, LYM=Lyman, NOS=Nosler, ]

[ SRA=Sierra, SPR=Speer.   Bullet lookup is displayed in bullet- ]

[ diameter order by default;  press  [M]  to change  to by-maker ]

[ order.                                                         ]

[ QBALL                                                        9 ]

Note that the Range <T>able... option will now display the zero

range.

+=================================+

|  Range <T>able (0 = 250 Yards)  |

+---------------------------------+

Figure 4. Partial Computations Menu, Zero Set

A zero range must be set in order to compute and display a range

table. If no valid zero range is set, QBALL will ask you for one.

You may set or reset the zero range by pressing <Z>; the Range

<T>able... option will display that zero range.

Let's continue with this range table; enter the data shown.

Range to Point of Impact.......... 300

Range Increment...................  50

The following table is displayed:

---------------------------------------------------------------

> Ballistic Computations: Typical Firearm <

Range Table, Zeroed at Highlighted Range

----------------------------------------

Range  R e m a i n i n g  Total  Path/Sight Line  Defl'n  Time

Yards  Velocity   Energy   Drop  M.O.A.   Inches  10 MPH  Flght

-----  -----------------  -----  ---------------  ------  -----

0    2200      1934     0.0   0.0      -1.0     0.0   0.000

50    2068      1708     0.9   7.8       3.9     0.4   0.070

100    1939      1503     3.9   6.8       6.8     1.5   0.145

150    1816      1319     9.1   4.9       7.3     3.5   0.225

200    1700      1155    17.1   2.6       5.2     6.7   0.311

>>250    1589      1010    28.1   0.0       0.0    10.8   0.402<<

300    1485       881    42.4  -2.8      -8.5    15.9   0.499

----------------------------------------

===> press any key <===

---------------------------------------------------------------

Figure 5. Typical Range Table

Range tables are interpreted as follows: for a bullet having the

given ballistic characteristics and fired at the given muzzle

velocity, then, at a range of, say, 200 yards, it will retain 1700

FPS velocity, 1155 ft-lbs of energy, will have dropped 17.1 inches

below the line of departure, will have risen to a maximum of 7.3

inches above the line of sight (at 150 yards), will strike 5.2

inches (2.6 minutes of arc) high, will have been blown 6.7 inches

sideways by a 10 MPH crosswind, and will have taken 0.311 seconds

to travel 200 yards downrange from the muzzle. In similar fashion,

data for other ranges may be read from the table for each of the

listed ranges. Note that the table specifies that Path/Sight Line

is 0.0 at 250 yards, the zero range. That line is highlighted on

the screen display; here, it's shown >> thus <<.

[ QBALL                                                       11 ]

> Ballistic Computations: Typical Firearm <

+--------------------------------------------------------------+

| |                             *                              |

|I|                  *                                         |

| | 5                                      *                 5 |

|N|        *                                                   |

| |                                                            |

|C|-0------------------------------------------------*-------0-|

| *                                                            |

|H|    [draw a smooth curve]                                   |

| |-5  [connecting the "*".]                                -5 |

|E|                                                            |

| |                           YARDS                            *

|S|       50        100        150        200       250        |

+--------------------------------------------------------------+

|     [ --- Remaining Velocity/Energy, Current Round --- ]     |

+--------------------------------------------------------------+

|           >Current<    >Previous<    +=======================+

|Max Ht :  7.4" @ 135                  | *** Graphics Menu *** |

|Min Ht : -8.5" @ 300                  | --------------------- |

|MuzVel :   2200 FPS                   |<G>rid Toggle On/Off   |

|TermVel:   1485 FPS                   |<S>ave Current Data    |

|BCoeff :   .300                       |<R>ecall <0>           |

|ZeroRg :  250 Yards                   | --------------------- |

|Metro  :  Standard                    |<ESC>ape to Choose Next|

+--------------------------------------------------------------+

|        [ --- info line schematically indicated --- ]         |

+--------------------------------------------------------------+

Figure 7. The Graphics Screen

The upper portion of the screen represents your firing range as

viewed from the side. The firing point is at the left; the maxi-

mum range, 300 yards, at the right. Each range increment is indi-

cated by a vertical line, numbered across the bottom. The hori-

zontal lines, indicating distance above or below the line of sight

are numbered at both the left and right. The curved line indicates

the bullet's trajectory relative to the line of sight: the

emphasized line numbered "0-0."  The circle on the curved line

indicates the point at which the maximum height is reached. The

numbers immediately below the trajectory plot are the remaining

velocity and energy at the indicated range for the current round.

The data shown at the bottom left are: the maximum and minimum

heights, and the ranges at which they occur; the muzzle and term-

inal velocities; the ballistic coefficient; the zero range; and

the metro currently in effect.

Press <G> to redraw the screen without the background grid

lines. Tick marks are displayed at the left and right, and on the

sight line; appropriate numbers are shown. Press <G> again to re-

display the grid lines.

<R>ecall... tells you that you have one set of trajectory data

saved to disk. (Yes, that's a "zero;" I'll explain in a bit.)

[ QBALL                                                       13 ]

Now let's play "what if...?"  What would happen to the trajec-

tory if, say, the muzzle velocity is increased from 2200 to 2700

FPS?  Let's find out. Press <ESC> to return to the Computations

Menu, and with the Computations Menu displayed, press <N> to

access the "New or Changed Input Data" Menu.

A complete explanation of the New Input Data Menu can wait for a

bit; this time, when it is displayed, press <V> to change the

muzzle velocity. Note that "2200" is displayed in the entry

window; overtype the new velocity, thus:

Muzzle Velocity         (FPS)  2700

Note that is possible to change more than one item of input data

at a time; this is not recommended, because it is difficult to

determine which change has which effect. Now <ESC> back to the

Computations Menu. Note that your zero range, 250 yards, is still

set. Note that Press <T>, and note that the maximum range, 300

yards, is also set. Press <enter> to accept the displayed range;

then press <enter> again to accept the range increment of 50

yards. After the range table is displayed, press any key. When

presented with the "Choose Next" menu, press <G>. The new graphic

representation of trajectory data looks like Figure 9, page 14.

[ HINT:                                                   ]

[                                                         ]

[   If you plan to save a lot of data, especially range   ]

[ tables, you need all the disk room you can get! Range   ]

[ tables get big, fast. If either of the following is     ]

[ true:                                                   ]

[                                                         ]

[ You're running QBALL from the distribution disk,        ]

[ (tsk,tsk!)                                              ]

[ You're running QBALL on any floppy with other files on  ]

[ it,                                                     ]

[                    then quit QBALL (from here, the      ]

[ keystrokes are: <ESC><ESC><ESC><Q><Y><Y>) and copy all  ]

[ the QBALL*.* files to a fresh (empty) disk. Then re-    ]

[ start QBALL from the new disk.                          ]

[   QBALL is set up to detect a "disk full" error but why ]

[ take the chance? Should a "disk full" error occur,      ]

[ QBALL disables all "save" functions for the duration of ]

[ the current session.                                    ]

[ QBALL                                                       15 ]

> Ballistic Computations: Typical Firearm <

+--------------------------------------------------------------+

| |                             *                              |

|I|                  *                                         |

| | 5                *          *          *                 5 |

|N|        *                               *                   |

| |        *                                                   |

|C|-0------------------------------------------------*-------0-|

| *                                                            |

|H|    [draw smooth curves ]                                   |

| |-5  [connecting the "*".]                                -5 *

|E|                                                            |

| |                           YARDS                            *

|S|       50        100        150        200       250        |

+--------------------------------------------------------------+

|     [ --- Remaining Velocity/Energy, Current Round --- ]     |

+--------------------------------------------------------------+

|           >Current<    >Previous 1<  +=======================+

|Max Ht :  4.6" @ 138    7.4" @ 135    | *** Graphics Menu *** |

|Min Ht : -5.3" @ 300   -8.5" @ 300    | --------------------- |

|MuzVel :   2700 FPS      2200 FPS     |<G>rid Toggle On/Off   |

|TermVel:   1877 FPS      1485 FPS     |<S>ave Current Data    |

|BCoeff :   .300          .300         |<R>ecall <0> thru <1>  |

|ZeroRg :  250 Yards     250 Yards     | --------------------- |

|Metro  :  Standard      Standard      |<ESC>ape to Choose Next|

+--------------------------------------------------------------+

|        [ --- info line schematically indicated --- ]         |

+--------------------------------------------------------------+

Figure 10. Two Sets of Data Graphics Screen

2.4  Changing Input Data.

From time to time, you may wish to change some or all of the

data which you have input to the program. The <N>ew Input Data

Menu, Figure 11, may be accessed from the Computations Menu for

that purpose. Press <N> to display the:

+=================================+

|   *** New Input Data Menu ***   |

|  -----------------------------  |

|  <C>artridge/Bullet Identifier  |

|  Muzzle <V>elocity              |

|  <B>allistic Coefficient        |

|  Bullet <W>eight                |

|  <M>etro (dF 'Alt "Hg  %RH)     |

|  Sight <H>eight                 |

|  Yds/Mets <S>wap (Now Yards)    |

|  -----------------------------  |

|  <ESC> to Computations Menu     |

+---------------------------------+

Figure 11. New Input Data Menu.

Cartridge/Bullet Identifier: You may change the designation of

the cartridge, bullet, or firearm with which you are working.

Enter any string of characters at the prompt, e.g., ".30-'06

Springfield."  Identifiers are limited to a length of 40 charac-

ters; QBALL defaults to "Typical Firearm." You may keep the same

identifier by writing QBALL initialization data as explained in

Chapter 5.

[ QBALL                                                       17 ]

2.5  Other Ballistics Procedures.

Five "other ballistic procedures" are described in this section.

Two, point-blank range and wind vector corrections, directly

affect the range table/graphics displays. The remaining three,

slant angle of fire corrections, sectional density, and gun free

recoil are informational in nature, only. From the Computations

Menu, press <O> to display the "Other Procedures Menu."

+=================================+

|  *** Other Procedures Menu ***  |

|  -----------------------------  |

|  <P>oint-blank Range            |

|  <W>ind Vector Corrections      |

|  Slant <A>ngle of fire          |

|  <S>ectional Density            |

|  <F>ree Recoil of Gun           |

|  -----------------------------  |

|  <ESC> to Computations Menu     |

+---------------------------------+

Figure 12. Other Procedures Menu

Point-Blank Range.

The point-blank range for any weapon/ammunition combination may

be defined as that range for which the trajectory deviates not

more than a specified distance above or below the line of sight.

Press <P> to access the point-blank range determination routines.

Enter the maximum deviation in inches, say, 5, as shown in figure

13.

+==========================================+

|                                          |

|    Determination of Point Blank Range    |

|    ----------------------------------    |

|                                          |

| Maximum Deviation                      5 |

|                                          |

|       Trial Zero Range = 512 Yards       |

|                                          |

+------------------------------------------+

Figure 13. Enter the Maximum Deviation.

After a pause, during which the trial zero range changes several

times, the following is displayed:

+====================================+

|                                    |

| ---------------------------------- |

|    Point Blank Zero Range = 257    |

|         Maximum Range = 304        |

| ---------------------------------- |

|                                    |

+------------------------------------+

Figure 14. Point Blank Range Displayed.

[ QBALL                                                       19 ]

Slant Angle of Fire.

Virtually all firing tables and ballistics computation pro-

grams, QBALL among them, assume that the angle of fire is

relatively close to horizontal. It is known, however, that firing

"upslope" or "downslope" will cause the bullet to hit high

relative to the point of aim. How high? Press <A>, and enter the

following data: maximum range 400 yards and range increment 100

yards. You may enter the "vertical" dimension as an angle, or as a

height in feet above/below the firing point. You'll be prompted

thus:

Use <H>eight above/below firing point or

Vertical <A>ngle above/below horizontal:

For this example, press <A> and enter the vertical angle 25d (the

vertical angle may be entered as positive or negative). The fol-

lowing table is displayed.

+=================================+

|                                 |

|  Hits High for Slant Fire, 25d  |

|    Above or Below Horizontal    |

|  -----------------------------  |

|    Range      True      Hits    |

|    Yards      Drop      High    |

|    -----     -----     -----    |

|       0        0.0       0.0    |

|     100        3.6       0.3    |

|     200       16.2       1.5    |

|     300       41.0       3.8    |

|     400       82.9       7.8    |

|  -----------------------------  |

|     ===> press any key <===     |

|                                 |

+---------------------------------+

Figure 16. Slant Fire "Hits High" Display

The table tells you that the given weapon/ammunition combina-

tion, fired at an angle of 25d above or below the horizontal, will

hit high by the specified distance at each listed range. The

effect is relatively small at short range and smaller vertical

angles, but can become large enough to be of real concern as the

range or the vertical angle increases. Try several examples and

see.

QBALL prompts...

---> More? ([Y] or N) <C>ontinuous  <---

...if there are more data lines than will fit on

one screen. Acceptable values for this routine are as follows:

Range and range increment:     As for Range Tables;

Vertical Angle:                0d to 45d, inclusive; or

Vertical Height:               Height difference expressed in

feet (1200 in the example).

[ QBALL                                                       21 ]

Unfortunately, these figures are relatively meaningless without

some basis for comparison. Here are three for starters, all data

computed using QBALL:

+------------------------------+-------------------+

|                              |       Weapon      |

| Selected Weapon, Caliber,    +----------+--------+

|          and Load            | Velocity | Energy |

|                              |   FPS    | Ft-Lbs |

+------------------------------+----------+--------+

|                              |          |        |

| 1. Ruger M77R (6.75 pounds)  |          |        |

|    .22-250 Remington         |    7.86  |   6.49 |

|       Muz Vel = 3680         |          |        |

|       Bul Wt  =   55         |          |        |

|       Pow Wt  =   36         |          |        |

|                              |          |        |

| 2. Marlin 336CS (7 pounds)   |          |        |

|    .30-30 Winchester         |   10.67  |  12.39 |

|       Muz Vel = 2390         |          |        |

|       Bul Wt  =  150         |          |        |

|       Pow Wt  =   35         |          |        |

|                              |          |        |

| 3. M1 Garand (9.5 pounds)    |          |        |

|    .30-'06 Springfield       |   10.88  |  17.47 |

|       Muz Vel = 2700         |          |        |

|       Bul Wt  =  180         |          |        |

|       Pow Wt  =   50.5       |          |        |

|                              |          |        |

+------------------------------+----------+--------+

Table 1. Free Recoil, Selected Weapons.

The ammunition specified is a "best estimate" using published

Winchester data for velocities and bullet weights; the powder

weights are from the Hornady Handbook. The weapon weights were

taken from the 1991 edition of Gun Digest.

2.6  Bullet Lookup Facility.

Whenever QBALL prompts for a ballistic coefficient or a bullet

weight, you may press [L] to activate the bullet lookup facility.

If the AMMO.DAT file and either of the *.NDX files is present, a

listing of a number of bullets is "windowed" over the prompt screen.

(A sample appears on the next page.) The supplied AMMO.DAT file

contains information about 502 bullets from several makers.

Assuming that D.NDX is available, bullet lookup defaults to a

listing sorted by diameter and weight. If you prefer, and if M.NDX

is available, you may change to a list sorted by maker, diameter and

weight by pressing [M].

You may navigate through the list by using the PgUp, PgDn, Home,

End, and the arrow keys. Press [Enter] to accept the highlighted

bullet, or [ESC] to exit and enter the value manually.

[ QBALL                                                       23 ]

Chapter 3

Inferring Ballistic Coefficients

""""""""""""""""""""""""""""""""

From time to time, you may have need of the ballistic

coefficient of a particular bullet, and the available tables do

not include that data.

Select "Infer <B>allistic Coefficient" at the Main Menu. (The

first time you invoke this routine in a given session, you will be

encouraged to read this section of the manual.)  Press [Y] to

continue with the routine, or "N" to return to the Main Menu.

QBALL will ask you for four pieces of information: two ranges,

and two velocities. The following diagram, a schematic layout of a

firing range viewed from above, will help you visualize the setup.

(Two chronographs are shown. See the note on page 26 if only one

chronograph is available.)

+---------------------------------------------------------+

|                         |  ( > > > >  =  bullet path )  |

|                         |  ( C = chronograph location)  |

|   +---[r1]----+         +-------------------------------+

|   +-----------[r2]----------+                           |

|                                                         |

|   Gun > > > > C > > > > > > C > > > > > > > > > > > >   |

|              v@1           v@2                          |

|                                                         |

|       r1  = range, muzzle to first chronograph          |

|       r2  = range, muzzle to second chronograph         |

|       v@1 = velocity at first range                     |

|       v@2 = velocity at second range                    |

|                                                         |

+---------------------------------------------------------+

Figure 18. Schematic Layout, Infer Ballistic Coefficient.

It is possible for r1 to equal zero; in that case, of course,

v@1 would equal the muzzle velocity. (This is the only time that

QBALL will allow you to enter a range of zero; "zero" is the de-

fault value for r1.)

The routine for inferring a ballistic coefficient allows use of

nonstandard metro, should you have developed your own velocity

data from live firing. You may change the metro conditions to be

used before beginning to enter velocity data. The ballistic coef-

ficient reported, however, is adjusted to standard metro

conditions in all cases.

You may enter as many as twenty-five (25) individual range/vel-

ocity pairs. The average v@1, v@2, and ballistic coefficient are

reported. If r1 is greater than zero, i.e., you are not working

with a muzzle velocity, QBALL reports an inferred muzzle velocity.

Let's work through the process. We'll do this twice: once, with r1

set to 0, and once more with r1 set to 50; r2 will be set at 100

in both cases. (The data is that found in the table on page 9.)

[ QBALL                                                       25 ]

3.2 Second Run: r1 = 50

Enter "first range" = 50 and "second range" = 100. When prompted

for the velocities, enter 2068 and 1939. You should end up with

the following display:

+==============================================================+

|               -----------------------------------            |

|                      Metro: as displayed                     |

| Relative Air Density = 1.000 Relative Sonic Velocity = 1.000 |

|               -----------------------------------            |

|                                                              |

|                                                              |

|                       For Round Number 1                     |

|            First Velocity        (Ft/Sec)      2068          |

|            Second Velocity       (Ft/Sec)      1939          |

|                                                              |

|               For Standard Atmosphere, C1 = .299             |

|                                                              |

|               Calculate another Round? ([Y] or N)            |

|                                                              |

|                                                              |

|                                                              |

|           [ Lower & Upper Ranges (yards)  50 & 100 ]         |

+--------------------------------------------------------------+

Figure 21. Ballistic Coefficient Entry 2.

Press "N"; (you would probably use more than one round in a

"real world" situation.) After a pause, the following is dis-

played:

+=================================================+

|    -----------------------------------------    |

|          Average C1 for 1 round = .298          |

|      Between Mean Velocities 2068 and 1939      |

|        Calculated Muzzle Velocity = 2201        |

|    -----------------------------------------    |

|                                                 |

|             Print C1 Data? ([Y] or N)           |

+-------------------------------------------------+

Figure 22. Ballistic Coefficient Display 2.

Press "N" to skip printing the table, and return to the Main

Menu. Again note that the information line shows the calculated

muzzle velocity and the inferred ballistic coefficient. You may

now go on to calculate ballistic data using the values obtained

using this routine; QBALL will use the calculated ballistic

coefficient and muzzle velocity until changed.

Now, granted, the calculated muzzle velocity does not equal what

we know it has to be, that is, 2200 FPS, nor does the ballistic

coefficient exactly match what we know to be correct. If, however,

you run a range table using 2201 FPS and C1 = .298, and another

using 2200 FPS and C1 = .300, the difference is well within

QBALL's roundoff error. (Try it using the input data used to

develop the table on page 9 and see.)  You may use the calculated

muzzle velocity and still expect excellent practical accuracy.

[ QBALL                                                       27 ]

Chapter 4

Printed Output

""""""""""""""

4.1 Printer Warnings.

QBALL is internally configured to print to parallel printers,

only, specifically, to LPT1. If your printer is connected to

another parallel port, you will need to "swap" ports for the

duration of the QBALL session, or even reconfigure your printer

setup.

QBALL will detect whether or not your printer is on/on-line; it

is not specifically set up to detect "paper out."  Ensure that

your printer is properly set up before you try to print any QBALL

output!  (The error recovery routine dumps you back into the

previous menu, so you do end up losing some displayed data.)

Some "generic" printer buffers will not respond properly to the

print routines. If you are using a printer buffer, and you keep

getting the error message "Printer does not respond," then try

bypassing your buffer. Buffers which are built into printers do

not seem to cause any problem.

While no extensive testing has been done, QBALL's print routines

have been found to work properly on an ALPS Allegro 24, an Epson

LQ-570, and a Tandy DMP-130.

4.2 Screen Dumps.

It is possible to print a screen's worth of tabular information

using the Shift-PrtScr key combination available in DOS. This

practice is not recommended, however, as you might very well lose

information which has already scrolled past on the screen. If your

computer is properly configured, however, you might want to print

out a graphics screen using Shift-PrtScr; see your DOS documenta-

tion, probably under "Graphics," for details.

4.3 Printer Setup.

You need not set up your printer to skip perforations in fanfold

paper; QBALL keeps track of the number of lines printed and issues

a formfeed (ASCII12) when appropriate. Continuation pages are

sequentially numbered, and each page will have the appropriate

column heading printed on it. (No formfeed is issued at the end of

any table.)

You need not set your printer to indent from the left margin,

either. It is best, in fact, to use a printer setup of "left mar-

gin = 0," as QBALL is set up to indent each table an appropriate

number of spaces.

You may also toggle the printer prompt completely off, replacing

it with a "press any key" prompt. This feature is useful if you do

not have a printer, or you are sure you won't want hard copy.

QBALL defaults to printer "ON," and displays the

[ QBALL                                                       29 ]

Typical printed "Infer Ballistic Coefficient..." output, using

the data from Chapter 3, second run, looks like this:

========================================================

> Inferring Ballistic Coefficient for: Typical Firearm <

--------------------------------------------------------

Given the following meteorological conditions:

Temperature..........: 59 deg F Barometric Pressure....: 29.53"Hg

Relative Humidity....: 78 %     Gun Altitude...........: 0 Feet

Relative Air Density.: 1.000    Relative Sonic Velocity: 1.00

and the number of rounds shown, the ballistic

coefficient (C1) was computed using the veloci-

ties indicated.

----------------------------------------------

Round          V1            V2            C1

-----         ----          ----          ----

1           2068          1939          .298

----------------------------------------------

Average C1 for 1 round = .298

Between Mean Velocities 2068 and 1939

Calculated Muzzle Velocity = 2201

==============================================

Figure 25. Sample Printed Output,

Inferring Ballistic Coefficient.

[ Hint: Don't want to wait while the printer churns through ]

[ a long range table?   Rather than choosing <P>rint in the ]

[ Choose Next Output menu,  choose  <S>ave  instead.  Then, ]

[ when you go off to lunch, or  whatever, issue the follow- ]

[ ing command: COPY RGTABLE.nn PRN and let the machinery do ]

[ the work while you do something else that's more fun..... ]

[ QBALL                                                       31 ]

Chapter 5

Configuring Display and Default Starting Values

"""""""""""""""""""""""""""""""""""""""""""""""

5.1  The Utilities and Configuration Menu.

Should you wish to change some or all of QBALL's start-up data

or its current configuration, access QBALL's associated utilities,

or invoke a DOS shell, press <U> at the Main Menu, to display this

"Utilities Menu:"

+===============================+

|  ***   Utilities  Menu   ***  |

|  ---------------------------  |

|  Toggle <S>witches            |

|  Change Display <C>olors      |

|  <W>rite QBALL.INI Data       |

|  Invoke a <D>OS Shell         |

|  <R>ead a Range Table File    |

|  <T>ranslate a Graphics File  |

|  ---------------------------  |

|  <ESC> to  M A I N   M E N U  |

+-------------------------------+

Figure 26. The Utilities Menu.

Toggle <S>witches will change several of QBALL's opera-

ting settings; ...<C>olors is used to choose the display colors;

<W>rite QBALL.INI Data saves the current configuration; and

...<D>OS shell permits "dropping" to the operating system without

exiting QBALL. <R>ead... and <T>ranslate... "shell out" to the two

associated QBALL utilities, RRTABLE.EXE and GRFXLATE.EXE; if the

utility program is not present in the current directory, the op-

tion is disabled. <R>ead and <T>ranslate are discussed in detail

in Chapter 8, QBALL's Associated Utilities, page 39.

5.2  Toggle Active Switches.

Press <S> to display this "Switch Toggle Menu."  Pressing the

appropriate key toggles that particular item; the current setting

is reflected immediately, and QBALL will act accordingly until it

is again changed.

+===============================+

|  *** Switch Toggle Menu ***   |

|  --------------------------   |

|  Yds/Mets <S>wap (Now Yards)  |

|  Graphics <G>rid (Now ON)     |

|  <P>rinter on/off (Now ON)    |

|  <N>oises on/off (Now ON)     |

|  --------------------------   |

|  <ESC> to Utilities Menu      |

+-------------------------------+

Figure 27. The Switch Toggle Menu.

[ QBALL                                                       33 ]

Figure 28 indicates "Now set -- " as the supplied default

settings. If you started QBALL with the "/b" switch, the values

will be, in order: 7, (white/light gray); 15, (high intensity

white); 7, (white/light gray); 0, (black); 0, (black).

5.4  Using The Values You've Set.

Having set the display colors and toggles as you like them,

<ESC> to the Utilities Menu, where you may:

<W>rite QBALL.INI Data

or, you may:

<ESC> to M A I N   M E N U.

If you're sure of the values you've entered, press <W> to write

the new QBALL initialization data to the end of QBALL.EXE. (And

remember, if you change your mind, there's nothing to stop you

from running Configuration again.)

If you'd like to use the values for this session only, perhaps

to see how they work out, press <ESC>. If you decide later to save

your "temporary" setup, choose <U> from the Main Menu, then choose

<W> immediately from the Configuration Menu. QBALL doesn't care if

there's a delay between choosing configuration values and writing

the configuration  data; it will quite cheerfully write that data

using whatever values are in effect at the time the <W> option is

chosen.

If you choose the <W> option, remember that the QBALL.INI data

includes a good bit more information than just the display colors

and toggle status. It also controls the starting text and values

for the:

a. Cartridge/bullet identifier;

b. Meteorological conditions; and

c. Sight height.

Upon invocation of the <W>rite... option, the new configuration

data will include the text or values of these items which are

currently in effect. This allows you to "tailor" your copy of

QBALL to match your own conditions: your weapon, and the actual

(or anticipated) metro which is encountered at your own firing

range.

Specific times you might very well want to use metro values

other than standard are, for example, if your range is not at sea

level, or if you're shooting during the summer, when the tempera-

ture might very well exceed 59� Fahrenheit. You can configure

QBALL to start with the altitude of your range, or an expected

temperature, without the hassle of changing the values after

start-up.

In similar fashion, you may configure QBALL to start with the

name of your own weapon, say, "Ruger M77/.270" and the actual

measured height of that weapon's sight, say, 1.25 inches.

[ QBALL                                                       35 ]

Chapter 6

Recalling Saved Trajectory Data

"""""""""""""""""""""""""""""""

Assuming that one or more trajectory graphics files exist on/in

the current disk/directory, QBALL's Main menu allows you to access

them without having to work your way to the graphics display

routines. At the Main menu, press <R>ecall Saved Trajectory Data

to access this informational screen and the Recall Trajectory Data

menu:

Recall Saved Graphics Data

--------------------------

EGA_SAV.0                          Typical Firearm

--------------------------------------------------

Muzzle Velocity      2700 FPS   Ballistic Coefficient     .300

Bullet Weight      180 grains   Zero Range           200 Yards

Metro conditions     Standard   Maximum Range        300 Yards

+===================================+

|  *** Recall Trajectory Data ***   |

|  ------------------------------   |

|  <R>ecall <0> thru <1>            |

|  Display Trajectory <G>raphic     |

|  Use Displayed As <D>efault       |

|  ------------------------------   |

|  <ESC> to M A I N   M E N U       |

+-----------------------------------+

Figure 29. Recall Trajectory Data Display and Menu

For purposes of this example, it is assumed that you have copied

the two saved trajectory files from the distribution disk, and

that one trajectory file was saved when you ran the "tutorial" in

Chapter 2.

The information shown above the menu specifies the saved file-

name, the identifier, and sufficient other information to permit

you to decide which one you're looking at. (If you are using a

CGA-equipped computer, the specified filename will be

"CGA_SAV.0.") This routine always displays *GA_SAV.0 first. You

may recall the others by pressing <R>ecall, then the <number>;

e.g., to examine EGA_SAV.1, press <R>, then press <1>.

Option <G> and option <D> will reset the input data. If you

don't want to change the input data, press <ESC> to return to the

Main menu.

[ QBALL                                                       37 ]

Chapter 7

File Handling

"""""""""""""

QBALL does perform disk reads and disk writes. Running QBALL

from a floppy disk is possible, but reads and writes are necess-

arily slow. It is recommended, therefore, that you run QBALL

from a hard disk, or, if at all possible, a RAM disk.

That said, QBALL deals with the following disk files.

QBSHELIT.BAT batch file.

When first started, QBALL writes a special batch file to the

current disk or into the current directory: QBSHELIT.BAT. This

batch file does several things, the most visible of which is

setting the DOS prompt within the shell. That prompt looks like

this:

[ Exit >> QBALL ] [drive:]\[directory] > _

Additionally, QBALL will look for this batch file when being

started. If QBSHELIT.BAT is found, you'll be shown "QBALL is

already loaded!" and get dumped back to the DOS prompt; this

prevents starting a second copy of QBALL while you're in a DOS

shell. (If you're running QBALL from a floppy, ensure that there

is a copy of COMMAND.COM on that floppy! Otherwise, you'll be able

to run only one command in the shell.)

QBSHELIT.BAT is erased upon normal exit from QBALL, i.e., when

you <Q>uit... QBALL from the Main menu. If, however, you break out

of QBALL using, say, Cntrl-Alt-Del to reboot, you will be unable

to start QBALL again until QBSHELIT.BAT is erased, which you'll

have to do manually in this case. Log onto the disk or directory

where QBALL resides, and enter this command:

DEL QBSHELIT.BAT (+ [enter]).

QBALL will now start normally.

Under ordinary circumstances, the operation of QBSHELIT.BAT is

completely transparent; you won't even know it's there.

Graphics Trajectory Data.

Trajectory data saved with QBALL versions 2.09 and preceding are

not compatible with those saved with QBALL 2.20, and QBALL 2.20

will simply ignore them. The only workaround, if you've got some

favorites, is to "start from scratch" with version 2.20 and regen-

erate the graphics files. Please note, too, that CGA-equipped

computers are directed to ignore EGA graphics files; EGA compu-

ters, to ignore CGA graphics files. It is possible to translate

from one format to the other; see Chapter 8, QBALL's Associated

Utilities, page 39.

[ QBALL                                                       39 ]

Chapter 8

QBALL's Associated Utilities

""""""""""""""""""""""""""""

Two custom utilities are included with QBALL 2.20: RRTABLE.EXE

and GRFXLATE.EXE. Both may be accessed from the QBALL Utilities

menu, and either may be started from the command line, as well.

Once loaded, the procedures are exactly the same; the remainder of

this chapter assumes that you are accessing the utilities from

within QBALL.

8.1 Range Table Reader (RRTABLE.EXE).

In order to access the Range Table Reader from within QBALL,

both of the following conditions must be true: first, QBALL.EXE

and RRTABLE.EXE are on/in the same disk/directory; and second, a

saved range table--" RGTABLE.[ext]"-- exists on/in that disk/di-

rectory. If either of these conditions is untrue, QBALL disables

the <R>ead a Range Table File option in the Utilities menu, i.e.,

the option is displayed in low-intensity video.

At the Utilities menu, assuming that the option is activated,

press "<R>." After a short pause, the following screen will

appear:

+==============================================================

===== -       - -   ------  ----------  ---  ------------------

|   | |-+  -+ | |   +-+| -   | |-|+-+|  |-   +-+|- |-|| ||- +-+

|   | | | +-| | |   ||_|_|   | | ||_||__|__  ||_|__| ||_||__||_

|___| |_| |_| | |                                           1.0

-- |_----------------------------------------------------------

---------------------------------------------------------------

1 QBALL-generated Range Table found:

------------------------------------

RGTABLE.01

------------------------------------

Enter file EXTENSION ("0" exits):

Figure 31. QBALL's Range Table Reader

There is one range table available to be read. Enter "1" (or

"01") to examine that range table. Looks a lot like the printed

output from Chapter 4, doesn't it? It should: it's the same file,

stripped of blank lines, form feeds, and page continuation head-

ers. If this saved range table were longer, you would use the up-

and down arrows, PgUp, PgDn, Home, and End keys to navigate. Press

<ESC> to return to the opening screen as shown in Figure 31. If

you had more than one range table available, you could then read

one (or more) additional tables. When you're finished looking at

range tables, enter "0" to return to QBALL.

[ QBALL                                                       41 ]

After a short pause, during which GRFXLATE will report that it

is, indeed, translating, you will be asked if you wish to trans-

late another file. A "no" response returns you to QBALL.

If you invoke GRFXLATE and decide that you don't want to

translate any files at all, press <ESC>, which returns you to

QBALL. The newly-translated EGA_SAV.2 is now fully usable by

QBALL's graphics routines.

----------

Both RGTABLE.EXE and GRFXLATE.EXE may be used as stand-alone

utilities, invoked from the command line, as noted above. Their

use, either accessed from within QBALL or from the command line

requires a series of disk reads, and in the case of GRFXLATE.EXE,

disk writes. These are necessarily slow on floppy disks. Further-

more, you may discover that a floppy doesn't have sufficient space

for QBALL, its associated utilities, and any saved data.

Experimentation is in order.

Note, too, that both RGTABLE.EXE and GRFXLATE.EXE obtain their

start-up colors from QBALL.EXE. If you have temporarily changed

QBALL's display colors, those in the utilities display won't

match.

[ QBALL                                                       43 ]

Appendix A

Glossary

absolute zero - Properly, that temperature at which all molecular

motion ceases. For practical purposes, -459.67d Fahrenheit, or 0d

Rankine.

air density - The mass (or loosely, the weight) of a unit volume

of air.

air resistance - The retardation of a bullet's forward flight due

to the atmosphere.

altitude - The vertical distance above or below sea level.

Altitudes above sea level are taken as positive; those below, as

negative.

ballistics - The study of moving projectiles. Commonly, what is

meant is exterior ballistics: that portion of the bullet's flight

between the muzzle and the point of impact. (Internal ballistics

studies events prior to the bullet's exit from the muzzle; term-

inal ballistics, the interaction between the bullet and the

target.)

ballistic coefficient - The ratio of the sectional density of a

projectile to its coefficient of form. A measure of how well a

bullet retains its velocity. Commonly supplied by reloading

manufacturers, or it may be derived from their ballistic tables.

In QBALL, the ballistic coefficient is always taken at standard

metro. The program corrects for nonstandard conditions, but does

not display the corrected ballistic coefficient(s). See also:

standard metro.

ballistic efficiency - The relative ability of a bullet in flight

to overcome air resistance.

bore - The inside of a firearm's barrel. In rifled firearms, the

bore diameter is the original dimension of the barrel before the

rifling grooves are cut or swaged in it. Bore diameter may also be

taken as the transverse dimension between the lands of the rif-

ling. See also: grooves, lands, rifling.

brass - Alloy of copper and zinc commonly used in the fabrication

of cartridge cases. Also, one or more cartridge cases containing

neither powder, primer, nor bullet.

bullet - The actual projectile in small arms ammunition. Commonly

corrupted to refer to the entire cartridge.

bullet weight - Weight of the actual projectile, usually stated in

grains. See also: grain.

[ QBALL                                                       45 ]

Ft.-Lbs. (also FtLbs and Ft#) - Foot pounds. A unit of energy

equal to the effort required to lift one pound a vertical distance

of one foot.

form factor - A multiplier which relates a bullet's shape to that

of the standard projectile used to prepare a particular ballistic

table.

grain - A unit of weight equal to 1/7000 of a pound, or 1/(437.5)

ounces. May also refer to an individual particle, or kernel, of

powder. In cartridge specifications, always refers to the weight

of the powder charge, never to individual particles.

grooves - Swaged impressions or cuts spiraled through a bore to

impart spin to projectiles. See also: lands.

group - The pattern made at the target by a number of shots fired

at a single point of aim and (usually) a common sight setting.

Usually measured from center-to-center of the two most widely

spaced holes.

gun - In popular usage, refers to firearms, generally. Properly, a

type of artillery or naval ordnance.

handgun - A firearm designed to be held and fired with one hand.

hold off - The distance a shooter must shift his point of aim to

the left or right to compensate for wind deflection.

hold over - The distance a shooter must raise his point of aim to

be on target when his firearm is zeroed at a lesser range.

horizontal fire - That type of fire occurring if the firearm's

barrel is not elevated significantly above or below the horizon-

tal. Generally taken as 1 degree of arc or less.

Ingalls' Tables - A set of ballistic tables first calculated by

Col. J. M. Ingalls in 1918. Probably the most widely used tables

for small arms ballistics calculations.

iron sights - In popular usage, metallic sights.

lands - Those portions of a rifled bore not cut away by rifling

grooves. See also: grooves.

lead - (leed) How far ahead of a moving target a shooter must aim

to be assured of hitting it.

lead - (led) Soft, dense, silvery-gray metal with relatively low

melting point which forms the basis for many bullets.

line of departure - An imaginary line formed by extending the axis

of a firearm's bore in a downrange direction.

[ QBALL                                                       47 ]

point of aim - That downrange point with which a firearm's sights

are aligned.

point of impact - That point which a bullet actually strikes. In

most cases, considered in relation to point of aim.

powder - The propellant material used in firearms. Usually not in

powder form, but in the form of small rods, spheres, or flakes.

primer - Impact-sensitive component of a cartridge which ignites

the powder charge upon being struck by the firing pin or the

striker of a firearm.

projectile - A bullet, shell, golfball, rock, or other object

shot, hit, thrown, or hurled in space; a projectile receives a

single impetus, and continues in motion by its own inertia. A

bullet is not properly a projectile until it is in motion.

relative air density - The density of air at the measured metro

divided by the density of air at standard metro, expressed as a

dimensionless decimal fraction; e.g., 1.000.

relative sonic velocity - The velocity of sound at the measured

metro divided by the velocity of sound at standard metro, ex-

pressed as a dimensionless decimal fraction; e.g., 1.000.

remaining energy - A projectile's energy in foot-pounds at a given

range.

remaining velocity - A projectile's velocity in FPS at a given

range.

rifling - The spirally-cut or swaged grooves in a firearm's bore,

taken collectively with the resulting lands, which impart a stab-

ilizing spin to the bullet. See also: grooves, lands.

round - One complete cartridge. May also refer to the sequential

number of a shot in a number of shots; e.g., "Round #3."

scope - Optical or telescopic, as opposed to metallic, sights.

sight height - The measured vertical distance between the horiz-

ontal centerline of the sights and the axis of the bore.

sighting in - The process of firing to determine point of impact

at a given range and adjusting the sights so that the point of

impact has the desired relationship to the point of aim. See also:

zero.

sight radius - The distance between the rear and the front sight,

on those weapons equipped with iron (non-optical) sights.

small arms (also smallarms) - Rifles, pistols, or revolvers,

generally.

[ QBALL                                                       49 ]

Appendix B

Example of Ballistic Coefficient from Tabular Data

The following tables summarize the determination of a ballistic

coefficient using tabular values. The values for Table B1 are

taken from data published in the 1990 edition of The Gun Digest,

page 229; the values for Table B2 were calculated using QBALL.

+-----------------------------------------------------+

| From Gun Digest Table for 30-06 Spfd, 180 gr bullet:|

|                                                     |

|                          Range in Yards             |

|               ------------------------------------  |

|               Muz     100      200     300     400  |

|               ------------------------------------  |

| Velocity      2700   2469     2250    2042    1846  |

| Energy        2913   2436     2023    1666    1362  |

| Trajectory    -1.5    2.5      0.0    -9.3   -27.0  |

+-----------------------------------------------------+

Table B1. Data Taken From Published Table.

Using the "Infer Ballistic Coefficient" routines in QBALL,

entering 100 and 200 yards for the required ranges, and using the

velocities as published, QBALL reports that C1 (the ballistic

coefficient) is equal to 0.384.

Returning to QBALL's Main Menu, selecting the Calculate

Ballistic Data option, and entering the appropriate values for

muzzle velocity, bullet weight, and zero range, QBALL produces

the data shown in Table B2.

+-----------------------------------------------------+

| QBALL Data Table for 30-06 Spfd, 180 gr bullet:     |

|                                                     |

|                          Range in Yards             |

|               ------------------------------------  |

|               Muz     100      200     300     400  |

|               ------------------------------------  |

| Velocity      2700   2470     2251    2043    1846  |

| Energy        2913   2437     2024    1668    1362  |

| Trajectory    -1.5    2.1      0.0    -9.0   -26.2  |

+-----------------------------------------------------+

Table B2. Data Calculated Using QBALL.

The maximum variations are: Velocity, 1 FPS; Energy, 2 FtLbs;

and Trajectory 0.8 inches. It may be seen, therefore, that QBALL

can produce results in substantial agreement with published data.

[ QBALL                                                       51 ]

Appendix C

Typical Saved Trajectory Data File

The following table represents, in part and in somewhat com-

pressed form, the EGA_SAV.0 file provided in the distribution

package. The header data contains data as detailed in the key,

below. The first data column is the range; the second, the

trajectory height relative to the sight line. Note that the range

increment is 1/100 of the maximum range. This is true if the

computer used is equipped with an EGA card/monitor. CGA_SAV.*

tables use a range increment of 1/50 of the maximum range.

+----------------------------------------------------+

|  Yards                                             |

|  1                                                 |

|  2700      1876.864  .3        200       2.580286  |

| -10.07583  111       300       166.53              |

|  180       Typical Firearm                         |

|  59        0         29.53     78                  |

|              0                           -1.0      |

|              3                           -0.8      |

|              6                           -0.6      |

|              9                           -0.5      |

|  - - - - - - - - - - - - - - - - - - - - - - - -   |

|            (major portion of data omitted)         |

|  - - - - - - - - - - - - - - - - - - - - - - - -   |

|            291                           -8.8      |

|            294                           -9.2      |

|            297                           -9.6      |

|            300                          -10.1      |

|  06-01-1993    12:00:00                            |

|  [end EGA_SAV.0 ]                                  |

+----------------------------------------------------+

Table C1. Saved Trajectory Data File.

Key to values listed in the table:

[Unit of Range Measure]

[Sight Height]

[Muzzle velocity] [Terminal velocity] [Ballistic coefficient]

[Zero range] [Max Height][Min Height] [Range at Max Height]

[Max Range] [Metro Constant]

[Bullet weight] [Identifier]

[Fahrenheit Temperature] [Gun Altitude] [Barometric Pressure]

[Relative humidity]

[Range]          [Bullet position � sight line]

[Date] [Time]

[End of file indicator with Filename and number]

+----------------------------------------------+

| Graphics files generated by QBALL v 2.20 are |

| not compatible with any other QBALL version. |

+----------------------------------------------+

[ QBALL                                                       53 ]

Appendix D

Selected Bibliography

Davis, William C., Jr. "Ballistics on the Home Computer," The

American Rifleman, CXXXI, 6 (June, 1983), p. 36.

Davis, William C., Jr. "Ballistics on A Pocket Calculator," The

American Rifleman, CXXXV, 6 (June, 1987), p. 42.

Davis, William C., Jr. "Calculate Ballistic Coefficients," The

American Rifleman, CXXXVII, 3 (March, 1989), p. 44.

Davis, William C., Jr. "What Is The Ballistic Coefficient?," The

American Rifleman, CXXXVII, 3 (March, 1989), p. 12.

Federal 1992 Ammunition [Catalog and Guide]. Anoka: Federal

Cartridge Company, 1991.

Hodgdon Basic Data Manual, Shawnee Mission: Hodgdon Power Company,

Incorporated, 1987

Ramage, C. Kenneth (ed.). Lyman Reloading Handbook. 46th ed.

Middlefield: Lyman Products Corporation, 1982.

Reiber, Ron L., and Heers, Edward A. (eds.). Hornady Handbook of

Cartridge Reloading, Rifle-Pistol. 3d ed. Grand Island: Hornady

Manufacturing Company, 1980.

Reloaders' Guide for Hercules Smokeless Powders. Wilmington:

Hercules Incorporated, 1988.

Remington Firearms, Ammunition, Clothing and Accessories Catalog.

Wilmington, Remington Arms Company, Inc., 1992.

Speer Reloading Manual #11, Lewiston: Omark Industries,

Incorporated, 1987

Warner, Ken (ed.). Gun Digest, 1990. 44th ed. Northbrook: DBI

Books, 1989.

Warner, Ken (ed.). Gun Digest, 1991. 45th ed. Northbrook: DBI

Books, 1990.

Warner, Ken (ed.). Gun Digest, 1992. 46th ed. Northbrook: DBI

Books, 1991.

Warner, Ken (ed.). Gun Digest, 1993. 47th ed. Northbrook: DBI

Books, 1992.

Winchester Ammunition Product Guide. East Alton: Winchester/Olin

Corporation, 1993.

[ QBALL                                                       55 ]

Appendix E

Error Analysis

Results from several QBALL runs were compared with comparable

data from two sets of published tables: the Speer Reloading Manual

#11, and the Hornady Handbook of Cartridge Reloading, 3d edition.

Two questions were to be answered: first, how well do velocity

figures compare between the three sources?; second, are the

trajectories found by the different sources comparable?

Some difficulty was experienced in the choice of ballistic coef-

ficients and velocities due to the differences between the two

published sources. Speer, for example, rounds all ballistic coef-

ficients to two decimal places, while Hornady carries three. In

addition, when comparable values of ballistic coefficients could

be found, the Hornady tables quite often did not have the spread

of velocities available in the Speer manual.

The following table summarizes as-comparable-as-possible velo-

city data from all three sources. While severely limited, the data

shows that the results obtained from QBALL are in general agree-

ment with the two published sources.

+------------------------------------------------------------+

|          Remaining Velocity, Conditions as Shown           |

|          =======================================           |

|            [ Range = 200 yards, Standard Met ]             |

+------------------------------------------------------------+

|         MuzVel = 2100     MuzVel = 2500     MuzVel = 3300  |

|            C1 = .12          C1 = .26          C1 = .50    |

|        ---------------   ---------------   --------------- |

| QBALL        1112              1879               2899     |

| SPEER        1102              1872               2895     |

| HORNADY      1126              1875               2892     |

+------------------------------------------------------------+

(Key: MuzVel = Muzzle Velocity  C1 = Ballistic Coefficient)

Table D1.  Remaining Velocities With Given Inputs. Note should

be made that the Hornady ballistic coefficients were actually

.122, .259, and .496 for the three sets of data shown. This

accounts for the apparent "high-on-low" spread of values seen

here. QBALL was run with the ballistic coefficients as shown.

An examination of the data in Table D1 indicates that remaining

velocities found using QBALL will be, on average, 0.47% high rel-

ative to the Speer data, and 0.42% low relative to to the Hornady

data. The Hornady data, however, must be considered suspect, as

the actual ballistic coefficients available in the Hornady table

do not exactly match those specified. Even so, it would appear

that QBALL's velocity results are certainly within one or two

percent of published data. In practical terms, there is probably

that much variation in velocity between successive rounds in a

string, if not more. QBALL, it would appear, can produce velocity

figures which are accurate enough for practical purposes.

[ QBALL                                                       57 ]

Appendix F

QBALL user Jim McCausland, out Washington State way, provided sev-

eral sets of data developed with other ballistics programs, all

believed to be "commercial" in nature (i.e, they aren't "share-

ware." The following tables contain the information which Jim sent

along, with as-comparable-as-possible QBALL data.

+--------------------------------------------------------------+

| Program: Sierra Ballistics.                                  |

| Input Data:                                                  |

|     Ballistic Coefficient (C1)....... .299                   |

|     Bullet Weight.................... 130 grains             |

|     Zero Range....................... 200 yards              |

|     Maximum Range.................... 450 yards              |

|     Range Increment..................  50 yards              |

|     Sight Height..................... 0.9 inches             |

|     Crosswind........................ -- no wind --          |

|     Metro............................ Standard               |

|--------------------------------------------------------------|

|Range R e m a i n i n g  Total Path/Sight Line Defl'n Time    |

|Yards Velocity   Energy  Drop  M.O.A.   Inches 0 MPH  Flght   |

|--------------------------------------------------------------|

|Sierra Ballistics Reports:                                    |

|                                                              |

|    0  3000.0     2597   0.00            -0.90 +0.00 0.000000 |

|   50  2840.3     2328   0.46  +-----+    1.07 +0.00 0.051389 |

|  100  2686.3     2083   2.00  | Not |    1.96 +0.00 0.105659 |

|  150  2537.4     1858   4.74  |Given|    1.66 +0.00 0.163151 |

|  200  2393.3     1653   8.83  +-----+       0 +0.00 0.224021 |

|  250  2253.9     1466  14.43            -3.17 +0.00 0.288606 |

|  300  2119.1     1296  21.74            -8.05 +0.00 0.357243 |

|  350  1989.0     1142  30.98           -14.85 +0.00 0.430310 |

|  400  1863.9     1003  42.41           -23.85 +0.00 0.508220 |

|  450  1744.1      878  56.34           -35.35 +0.00 0.591424 |

|                                                              |

|QBALL Reports:                                                |

|                                                              |

|    0    3000     2598    0.0     0.0     -0.9  0.0   0.000   |

|   50    2840     2329    0.5     2.1      1.1  0.0   0.051   |

|  100    2686     2083    2.1     2.0      2.0  0.0   0.106   |

|  150    2537     1858    4.9     1.1      1.6  0.0   0.163   |

|  200    2393     1653    9.0     0.0      0.0  0.0   0.224   |

|  250    2254     1466   14.6    -1.3     -3.2  0.0   0.289   |

|  300    2119     1296   22.0    -2.7     -8.1  0.0   0.357   |

|  350    1989     1142   31.4    -4.3    -15.0  0.0   0.431   |

|  400    1863     1002   42.7    -6.0    -23.8  0.0   0.508   |

|  450    1744      878   56.9    -7.9    -35.5  0.0   0.591   |

+--------------------------------------------------------------+

Table F1. Sierra Ballistics and QBALL Compared

Summary of absolute differences. Remaining Velocity: 0.9 FPS; Re-

maining Energy: 1 FtLb; Total Drop: .56 inches; Path/Sight Line:

0.15 inches; Time of Flight: 0.000424 seconds.

[ QBALL                                                       59 ]

+--------------------------------------------------------------+

| Program: BALTEC.                                             |

| Input Data:                                                  |

|     Ballistic Coefficient (C1)....... .295                   |

|     Bullet Weight.................... 130 grains             |

|     Zero Range....................... 200 yards              |

|     Maximum Range.................... 600 yards              |

|     Range Increment..................  50 yards              |

|     Sight Height..................... 0.9 inches             |

|     Crosswind........................  10 MPH                |

|     Metro............................ Standard               |

|--------------------------------------------------------------|

| Range R e m a i n i n g Total Path/Sight Line Defl'n    Time |

| Yards Velocity   Energy Drop* M.O.A.   Inches 10 MPH*  Flght*|

|--------------------------------------------------------------|

| BALTEC Reports:                                              |

|                                                              |

|     0    3000     2598    0.0    0.0    -0.9  -0.00    -.000 |

|    50    2838     2325    0.5   -2.2     1.1   0.24    0.051 |

|   100    2682     2076    2.1   -2.0     2.0   1.04    0.106 |

|   150    2531     1850    4.9   -1.1     1.7   2.36    0.163 |

|   200    2386     1643    9.0    0.0     0.0   4.26    0.224 |

|   250    2245     1454   14.7    1.3    -3.2   6.91    0.289 |

|   300    2109     1283   22.1    2.7    -8.2  10.28    0.358 |

|   350    1977     1128   31.5    4.3   -15.0  14.38    0.432 |

|   400    1850      988   42.9    6.0   -24.0  19.27    0.509 |

|   450    1730      864   57.2    8.0   -35.8  25.34    0.594 |

|   500    1616      754   74.4   10.1   -50.6  32.43    0.684 |

|   550    1507      656   94.7   12.4   -68.4  40.40    0.780 |

|   600    1407      572  119.2   15.1   -90.4  49.73    0.883 |

|                                                              |

| QBALL Reports:                                               |

|                                                              |

|     0    3000     2598    0.0    0.0    -0.9    0.0    0.000 |

|    50    2838     2325    0.5    2.2     1.1    0.2    0.051 |

|   100    2682     2076    2.1    2.0     2.0    1.0    0.106 |

|   150    2531     1850    4.9    1.1     1.7    2.4    0.163 |

|   200    2386     1643    9.0    0.0     0.0    4.3    0.224 |

|   250    2245     1454   14.7   -1.3    -3.2    6.9    0.289 |

|   300    2109     1283   22.1   -2.7    -8.2   10.3    0.358 |

|   350    1977     1128   31.5   -4.3   -15.0   14.4    0.432 |

|   400    1850      988   42.9   -6.0   -24.0   19.3    0.509 |

|   450    1730      864   57.2   -8.0   -35.8   25.3    0.594 |

|   500    1616      754   74.4  -10.1   -50.6   32.4    0.684 |

|   550    1507      656   94.7  -12.4   -68.4   40.4    0.780 |

|   600    1407      572  119.2  -15.1   -90.4   49.7    0.883 |

|--------------------------------------------------------------|

| *BALTEC requires two  tabular displays. Combined data shown. |

+--------------------------------------------------------------+

Table F3. BALTEC and QBALL Compared

Summary of absolute differences. Deflection for 10 MPH crosswind:

0.04 inches.

Available with BALTEC, unavailable with QBALL. Bullet Momentum in

Lb-Seconds; Maximum Ordinate, Maximum Height, both in inches.

[ QBALL                                                       61 ]

Appendix G

QBALL error messages, with a description of the circumstances

under which they will be encountered.

----------

AMMO.DAT Not Found: You've called for bullet lookup and the required

bullet data file is not on/in the current disk/directory.

Cannot sort by [key]: The required bullet lookup index file is not

on/in the current disk/directory, or the listing is already sorted

on the requested sort [key].

Disk Full: Insufficient space remains on/in the default disk/dir-

ectory to write the desired file. This most commonly occurs when

running QBALL from a floppy disk which has other files on it. Copy

QBALL.EXE to a fresh disk, and start over.

Enter  0.000 < Ballistic Coefficient < 1.000: You're trying to

enter a value outside the acceptable range of between 0.000 and

1.000. Ballistic coefficients are, by definition, greater than

zero and less than unity.

Enter  25 < Barometric Pressure < 35: You're attempting to enter a

barometric pressure value which is out of range. (These are rea-

sonable "real world" values.)

Enter  Bullet Diameter > 0: In determining sectional density, the

bullet must have a diameter.

Enter  0 < Bullet Weight < 1000: You're attempting to enter a zero

bullet weight. (1000 gr = 2.3 ounces.)

Enter  -1312 < Gun Altitude < 29028: You're attempting to enter a

gun altitude value which is out of range. (-1312 is the altitude

of The Dead Sea; 29028, of Mount Everest.)

Enter  0 < Gun Weight <= 30 lbs: In determining gun free recoil,

you're attempting to enter an out-of-range gun weight.

Enter Lbs + [space] + 0 <= Ounces < 16: In determining gun free

recoil, you're entering an incorrect format for pounds/ounces

data. Enter whole pounds, then a space, then ounces.

Enter  0 < Powder Weight < 200 gr: In determining gun free recoil,

you're attempting to enter a zero propellant weight.

Enter  Range greater than zero: You're really trying to enter a

range of "zero." Range = 0 is valid only when inferring ballistic

coefficients.

[ QBALL                                                       63 ]

Maximum Deviation Reset to Sight Height: Not an error, really, but

a warning. In determining point-blank range, the sight height is a

built-in deviation.

Not Enough Velocity Remains - Input Greater MV: QBALL is unable to

complete a computation because the remaining velocity is less than

300 FPS.

Printer does not respond: Your printer is off or off-line.

QBALL is already loaded!: You're attempting to load a second copy

of QBALL from within the DOS Shell. Alternatively, QBSHELIT.BAT

wasn't deleted from the current disk/directory. See page 34.

[ QBALL                                                       65 ]

Appendix H

Default Starting Values

QBALL stores its initialization data in binary format at the

"tail end" of QBALL.EXE. That data is used to set the display col-

ors and several of the values with which QBALL starts. As supplied

that data is as follows:

"Typical Firearm" 15 10 7 1 7 "Yards " "Meters" 1 -1 59 0 29.53 _

78 1 -1 -1

In order, the individual entries translate as follows:

"Typical

Firearm"      Cartridge/bullet identifier

15      Foreground (text) color ( = HI white    )

10      Highlighted text color  ( = HI green    )

7      Title/Info line color   ( = white       )

1      Background color        ( = blue        )

7      Help display background ( = white       )

"Yards "      Unit of range measure (trailing space necessary)

"Meters"      Alternate unit of range measure

1      Sight height in inches

-1      Noises on or off        ( = on  )

59      Air temperature, degrees Fahrenheit

0      Gun altitude in feet

29.53      Barometric pressure in inches of mercury

78      Relative humidity, in percent

1      Relative air density at given metro

-1      Graphics grid on or off ( = on  )

-1      Print prompt on or off  ( = on  ).

Invoking the "<W>rite .INI Data" option from QBALL's Utilities

Menu will replace the existing intialization data with whatever

values are in effect when the option is invoked. This will cause

DOS to redate the QBALL.EXE file, and may (or may not) change

QBALL.EXE's size on disk. If you regularly use a virus-scanning

utility, it may react to these changes. If your virus detector can

be directed to ignore certain files, you may wish to instruct it

to ignore QBALL.EXE.

[ QBALL                                                       67 ]

Index

-A-                             ballistic coefficient 7

Already Loaded! 34, 37               bullet [L]ookup 7

AMMO.DAT 21                          bullet diameter 20

Automatic 3                          bullet weight 7

file renaming 38                   default metro 7

graphics detect 3                  gun weight, decimal pounds 20

printer detect 3                   gun weight, pounds/ounces 20

maximum deviation 17

-B-                             maximum range for table 9

Ballistic Coefficients 23            muzzle velocity 7

[L]ookup 7                         powder (propellant) weight 20

entry 7                            range, slant angle of fire 19

example from tabular data 49       range increment, slant angle

inferring (calculating) 23-26        of fire 19

valid limits 16                    range increment for table 9

Bibliography 53                      target above/below gun 19

Bullet [L]ookup 7                    wind direction 18

AMMO.DAT 21                        wind velocity 18

changing input data 16             zero range 8

index (*.NDX) files 21           Default values 4

initial data entry 7               changing/saving 33

using and display 21-22            metro 4

Bullet Weight                        range unit 4

[L]ookup 7                         sight height 4

valid limits 16                    supplied 65

DOS Shell 34

-C-

Cartridge/Bullet Identifier 15

Changing Input Data 15-16               -E-

ballistic coefficient 16         Error Analysis 55-56

Bullet [L]ookup 15               Error Messages 61-63

bullet weight 16                 Exiting QBALL 5

Cartridge/Bullet

Identifier 15                       -F-

metro 16                         Files 1

muzzle velocity 16                 automatic renaming 38

sight height 16                    copying 1

swap range units 16                distribution 1

Chronograph 23                       QBSHELIT.BAT 37

using single 26                    required 1

Configuring... 31                    trajectory data 51

alert noises 32                    trajectory incompatible 37

graphics grid 32                 Free Recoil Velocity, Energy 20

printer on/off 32                  data entry 20

range units 32                     examples of... 21

saving configuration data 33       gun weight 20

screen colors 32                   powder (propellant) weight 20

valid data limits 20

-D-

Data Entry 7