COM: port support functions for Microsoft C

#include <com.h>

char com_getc( port );

int port;

int com_init( port, baud, parity, stop, length );

int port, baud, parity, stop, length;

unsigned int com_putc( port, c );

int port; char c;

int com_ready( port );

int port;

int com_stat( port );

int port;

o Description

The com functions provide control over the serial ports of your PC.

The function com_getc waits until a character is present at the specified

serial port, and it then inputs the character and returns it.

The function com_init sets the baud rate, parity, stop bits and word length

of the specified serial port and returns the status word, as described

below.

The function com_putc outputs a character to the specified serial port

and returns the status word after attempting to send the character.

The function com_ready checks the serial port specified and returns a 1 if a

character is waiting or a 0 if there is no input waiting.

The function com_stat returns the status word for the specified port.

NOTES

-----

Status word bit assignments:

bit	MASK	Meaning

---	----	-------

0	0x0001	Clear to Send has changed state

1	0x0002	Data Set Ready has changed state

2	0x0004	End of ringing pulse detector

3	0x0008	Carrier Detect signal has changed state

4	0x0010	Clear to Send signal

5	0x0020	Data Set Ready signal

6	0x0040	Ringing indicator

7	0x0080	Carrier Detect signal

8	0x0100	Received data is ready

9	0x0200	Overrun error (previous character not read & a new

character has arrived)

10	0x0400	Parity error (in incoming data)

11	0x0800	Framing error (an incorrect start/stop bit was received)

12	0x1000	Break detect

13	0x2000	Transmitter holding register empty (ready for another character

to transmit)

14	0x4000	Transmitter shift register empty (not currently transmitting)

15	0x8000	Time-out error

com_init values for baud, parity, stop, and length

baud: 0 = 110 baud;  1 = 150 baud;  2 = 300 baud;  3 = 600 baud;

4 = 1200 baud; 5 = 2400 baud; 6 = 4800 baud; 7 = 9600 baud.

parity: 0 = no parity; 1 = odd parity; 2 = no parity; 3 = even parity.

stop: 0 = 1 stop bit; 1 = 2 stop bits.

length: 0 = 5-bit words; 1 = 6-bit words; 2 = 7-bit words; 3 = 8-bit words.