MAIN.DOC Usage

The  hex  record  that  is  contained  in  the  file

MAIN.S19 has been configured so that it will run corectly

within the  M6800 emulator  enviroment. In  fact all that

was needed  to be  done  was  the  standard  output/input

routines be  replaced with  those that would communicated

with  the   emulator.  This   is  done  using  the  extra

instructions of  the emulator called 'chout a' and 'chint

a'.

To begin  execution  of  the  program  MAIN.S19

enter the  following  beginning  from  the  DOS

prompt:

C:68emu> 68em main.s19

Read 7672 bytes from file main.s19

Lowest  address loaded to was $E000

Highest address loaded to was $FFFF

M6800 Emulator

1 - May - 1991  V1.0

Kevin Bertram   1991

Written on:  17-April-1990       (Version 0.1)

Last update: 1-May-1991          (Version 1.0)

6800emu> _

The  program  MAIN.S19  has  been  loaded  into

memory and  the emulators  prompt is  displayed

waiting for  your next   command.  To  get  the

context switching to work within this operating

system the  NMI interupt  delay time  has to be

enabled and  set to a preset time. This is done

by the following:

6800emu> I N 800

IRQ = off            NMI = 2048

The NMI  interupt timer  has now  been  set  to

cause  a   NMI  interupt   after   2048   M6800

instructions  have   been  executed.  All  that

remains now is be begin the program:

6800emu> G

And the hello prompt will appear at the top of

the screen along with other status information

concerning the environment. The program

MAIN.S19 requires that a vt100 type terminal is

connected to its standard output therefore make

sure that ANSI.SYS has been installed into your

machine at boot-up time for a proper display.

ABOUT MAIN.S19

==============

MAIN.S19 was  written as  the house  keeping program

for  a  M6800  based  computer  that  has  been  remotely

situated with  its  communication  to  the  base  station

computer being a radio transmitter/receiver. The data you

see displayed  after  the  program  start  up  shows  the

current status  of the machine as it is seen at the time.

The information  is not  true when run under the emulator

because the  I/O hardware  is not  present that feeds the

program its data.

The commands  are  similar  to  that  of  the  M6800

emulator so  the document  68em.txt will  describe  those

commands that can be found there.

The system  had only  6K of  ram beginning from $800

through to  $1FFF with  $1C00 - $1FFF reserved for system

use. Everything else is free for transient programs.

There are  also software  hooks to  the main program

but I have not included a description of how to use them.

If you are interested then give me a call.

-Kevin Bertram