File:   IBMTOKEN.DOC

Auth:   Brian Fisher

Date:   October 3 1989

Purp:   Notes on IBMTOKEN.COM, the 'Ethernet' packet driver

for TCP/IP and NCSA Telnet.

========================================================================

IBMTOKEN is an Ethernet packet driver for the IBM Token Ring

Adapter card.  The IBM Lan support program must be installed before

you use IBMTOKEN.

CONFIG.SYS - to install the LAN Support program and increase the

size of the stacks.  The default stack size is too small and leads

to crashes.

device=C:\PCLAN\DXMA0MOD.SYS

device=C:\PCLAN\DXMC0MOD.SYS

device=C:\PCLAN\DXMT0MOD.SYS

stacks=9,512

AUTOEXEC.BAT - to install IBMTOKEN and define NCSA config file:

IBMTOKEN 0x61   0

SET CONFIGTEL=C:\CONFIG.TEL

CONFIG.TEL

myip=xx.yy.zz.qq                # put your IP address here

netmask=255.255.255.0           # use your network mask here

hardware=packet                 # tell NCSA to use the packet driver

# interface

address=0                       # class for packet driver

ioaddr=0x61                     # vector given to IBMTOKEN

mtu=1500                        # max IBMTOKEN can transmit

maxseg=1500                     # max IBMTOKEN can receive

rwin=536                        # max if NCSA can't do IP fragments

IBMTOKEN:

IBMTOKEN <vector> <adapter>

where    vector is the interrupt assigned to the packet driver

interface

adapter is 0 for the first adapter card, and 1 if the

second adapter card is to be used.

When IBMTOKEN installs, the following occurs:

1)   DIR.INITIALIZE is called to initialize the adapter.

2)   DIR.OPEN.ADAPTER is called to set up buffers for

IBMTOKEN.

3)   RECEIVE is called to set up the receive service

routines.

4)   DIR.STATUS is called to determine the address of the

adapter.

If any of these calls fails, the installation is aborted.

IBMTOKEN may not install properly if the token ring adapter

has been in use by another program, and to closed properly.

If this is the case, reboot and run IBMTOKEN.

OPERATION:

IBMTOKEN translates 'Ethernet' packet driver calls to 802.2 LLC

format packets for transmission.  Currently it supports IP and

ARP transmissions.  RARP and BOOTP probably won't work.

IP packets are sent 'as is'.  ARP packets require a minor modification

before they are sent.  The hwr address space value must be changed from

1h to 6h.  On receive, the driver changes 6h back to 1h.  The size (60

is min for Ether) must be set to 28 or some token hosts won't accept

the transmission.

Receive packets smaller than 60 bytes are rounded up so the upper

level will accept them.

Token Ring source routing.  When the upper level call back routine

accepts a packet, its RIF is recorded in a table interal to IBMTOKEN.

When IBMTOKEN sees the destination address requires a RIF, the

info is pulled from the table and added to the MAC header for transmit.

IBMTOKEN can handle a maximum of 32 source route entries.  One

default entry is present for the 'broadcast' address.  The first

source route received for an address is assumed to be the shortest.

Subsequent source routes from the same host are ignored.