TITLE:  Extra Half Bright mode

There are lots of fun things you can do in 6 bit planes on the Amiga,

including Hold and Modify color displays and dual-playfield mode with two

separately scroll-able, completely independent, 8 color playfields with

overlay and transparency.

But tonight I'm going to discuss something you don't already know about

your Amiga -- even if you've read all the tech manuals cover to cover.

The specs for the Amiga say that in a completely static, low-resolution

display, with nothing fancy going on (like Hold and Modify), you can

display up to 32 colors simultaneously out of a pallette of 4096. Well not

quite.  You can actually display up to 64 distinct colors out of that

pallette of 4096! Have I got your attention?  Welcome to Extra-Half-Brite

mode.

The Amiga display hardware automatically enters Extra-Half-Brite mode

whenever you select a 6 plane display (which must be low res due to the

hardware) and have NOT selected either Hold and Modify or Dual Playfield

mode.

The graphics kernel software further enforces the rule that you must set

the EXTRA_HALFBRITE flag in the ViewModes for the ViewPort (Screen in

Intution lingo) or it will automatically trim your screen down to 5 planes.

This is done both for backward compatability and to insure support in

future Amiga architectures.

Consider the playfield data bits for a given pixel.  The bits from the

first five planes form a color register selector for that pixel, allowing

you to choose among the 32 color registers in the Amiga.  The bit from the

sixth plane is interpreted as follows:

0 -- Use the color in the selected color register just as specified.

1 -- Take the color in the selected color register, shift each of its R,

G, and B components right one bit, and use the new color value thus formed.

The net result is as if you had 64 color registers where the colors in the

top 32 were "half-intensity" counterparts of the colors in the bottom 32!

Of course, that means there is a dependency between the choice of colors in

the 32 real registers and the resulting colors in the 32 psuedo-registers.

Nevertheless, I assert we have as much right to claim 64 colors on screen

as IBM has to claim 16 colors from a monitor that is physically capable of

producing only 8 colors at 2 intensities!  At least we can select our 32

colors out of a pallatte of 4096!

Note also that, since fractional color components have no meaning in the

hardware, there are several distinct real colors that produce the same

extra color.  For instance (in hex):

888 --> 444,    988 --> 444,   898 --> 444,   999 --> 444,  etc.

Despite all this quibbling, with a little thought it's easy to see how you

can choose a set of 32 real colors to make sure all 64 real plus extra

colors are distinct.  And they are every-pixel-addressable!

Why have we kept this little jewel a secret?  No, it's not that we were

planning to lull the competition into complacency and then spring an

instant double of the Amiga's color capacity on them.

Not all Amiga 1000s have Extra Half Bright mode.  All Amiga 500s and 2000s

have it.

This revision of the Amiga chipset occurred after the Amiga Hardware manual

was written, so it is not discussed there.  Note that you can't hurt

anything by running such a program on an older machine -- the values in the

6th bit plane will simply be ignored.  Color printing of Extra Half Bright

mode should work on AmigaDOS 1.1 and 1.2.

This sample program shows Extra-Half-Brite mode in operation.

*  Dale Luck    -- October, 1985

*  Bob Pariseau -- November 8, 1985  (Editorial changes)

*  John Foust   -- June 29, 1987 (more edits and updates)