PKARC AND PKXARC ARE GREAT PROGRAMS.  HOWEVER, HAVE YOU NOTICE THAT ON

SOME SYSTEMS YOU GET AN INSUFFICIENT MEMORY ERROR WHEN RUNNING

PKARC.COM.  THIS IS BECAUSE PKARC.COM ONLY ALLOWS ENOUGH "ROOM"

FOR A 110 BYTE ENVIRONMENT.  THE FOLLOWING IS A PROPOSED FIXED FOR

RUNNING PKARC.COM ON A COMPUTER WITH A ENVIRONMENT SIZE OF GREATER

THAN 512 BYTES.  THIS SHOULD BE SUFFICIENT FOR MOST APPLICATIONS.  I

HAVE TRACED THROUGH PKARC.COM ENOUGH TO FIGURE OUT ITS INITIALIZATION

PROCEDURES. I HAVE TESTED THE PATCH ON MY COMPUTER AND IT SEEMS TO

WORK. TO THOROUGHLY TEST IT HOWEVER, I WOULD HAVE TO TRACED THROUGH

ALL OF PKARC.COM. UNFORTUNATELY, I DON'T HAVE ENOUGH TIME FOR THAT.

REMEMBER, I DO NOT WARRANT THIS FIXED, NOR AM I LIABLE FOR ANY DAMAGES

CAUSED BY THIS FIX.  BASICALLY, USE THIS PATCH AT YOUR OWN

DISCRETION. I WOULD ALSO LIKE TO KNOW ANY FEEDBACK SINCE I AM RUNNING

THE PATCHED VERSION OF PKARC.COM AND WOULD LIKE TO KNOW OF ANY

ANOMALIES.

PATCH

A>DEBUG ARC.COM

-E DS:417D

XXXX:417D 08.<07>

-W

-Q

THIS PATCH EFFECTIVELY DECREASES STACK SIZE BY 100H BYTES AND

INCREASES ENVIROMENT SIZE BY 100H BYTES.  THE WORSE THAT SHOULD

HAPPEN IS THAT THE STACK WILL GROW OVER THE "COPY" OF THE ENVIROMENT.

WHICH IS NO GREAT LOSS.

PLEASE LEAVE COMMENTS ON MIKE'S C BOARD 1-619-722-8724

HAPPY COMPUTING!

GARLAND WONG