BUG.DOC

We will be interested in receiving bug reports.  We will do everything possible

to eliminate bugs, but our resources are limited and we will place first

priority on most common bugs from licensed users.

We have built into the program a number of bug finding procedures which we are

leaving in the early release.  These are initiated from the number pad keys.

In order to activate the bug keys, both the caps lock and the num lock key must

be on.

KP9     Pushing this key will clear most loops and transfer you back to the

main menu.  It also produces a monster listing which con be aborted by F9.

This is the best way to escape from a program hang.  If this doesn't work, the

situation is probably hopeless.  While we close files with this key, DOS does

not always respond.  There is a problem in that the system does not always

recover from a filled up disk file.  So be sure to have enough room when you do

a Save.

KP3     If you can identify a sequence which will cause a problem, this key

will give us the information we usually need to unscramble it.  To use this

key, start well before the problem.  Each time you press a key, working toward

the problem, hit KP3 (with caps lock and num lock on).  In answer to the

prompt, enter what you have just done.  It is best to continue the procedure

backing out to the main menu after the bug.  This key produces about a dozen

lines of printer output each time pressed.  Send us the printer output, and the

job and we will try to decode it.

KP1     If the job seems to be really hung in a loop, pressing this key will

produce helpful print out.  Stop it, the key is a toggle, after a page or so of

print out, or after you see clear repetition.

KP7     Produces a very large listing, so this is best done with a very minimum

job.  Not very useful unless we also have a symbol list.  This listing will

tell us everything, more than we will usually want to know.  Send this only if

we request it.