-------------------------------

VTOT: VIDEO TOOLS ON TAP

1988 by Mike Berro - FreeWare

-------------------------------

Version 1.00  11-Jul-88

VTOT V1.00 is FreeWare, and may be distributed at no charge. This text

file must be included with the program.

VTOT is a collection of tools and effects useful when recording Amiga

output onto video tape. It can run in the background, and the commands are

available at any time.

VTOT commands are initiated by holding down the LEFT-SHIFT and the

LEFT- AMIGA keys, and then pressing the appropriate letter key. The VTOT

window does not have to be "active". That means that the commands will

function even when running another program. However, see notes, below.

There are three types of commands: Display, Effects, and Miscellaneous.

The "Display" commands are grouped near the top of the window. B, C, F,

G, L, and P each displays the appropriate image in front of whatever else is

being displayed. X deletes that image. Each image is full overscan

(368x484). Your screen should be exactly centered according to preferences

(not your monitor!) or fringing on the right edge of the image will result.

The "Effects" commands are grouped in the middle of the window. These

commands only have an effect on the topmost (i.e. visible) screen. These

are potentially dangerous commands. It is not likely they will crash the

software, but they can make it very confusing for you. The effects are all

reversible, but garbage can result if both VTOT and the original program are

changing the screen display. For example, if you flip the Workbench screen,

the screen title will reappear at the top, because it is constantly being

updated (use Redraw to fix it). For this reason, use these commands only on

static images.

The "Miscellaneous" commands are grouped at the bottom of the window

and are described below.

DISPLAY COMMANDS

----------------

B Black Screen           Displays a black screen with proper setup level.

C SMPTE Color Bars       Displays SMPTE color bars.

F Full-field Color Bars  Displays full-field color bars.

G Grey Scale             Displays 16 level grey scale pattern.

L Line Pattern           Displays horizontal and vertical lines.

P Show s:VTOT.pic        Displays IFF picture stored in file "s:VTOT.pic".

EFFECTS COMMANDS

----------------

N Negate Colors          Negates the colors.

V Vertical Flip          Does a mirror flip on a center horizontal axis.

V Horizontal Flip        Does a mirror flip on a center vertical axis.

D Fade Down              Fades to black.

U Fade Up                Fades up from from black (true fadeup).

0-9 Fade Speed           Changes the fade speed (9 being slowest).

MISCELLANEOUS COMMANDS

----------------------

I Toggle Interlace       This command does not affect screen resolution.

S Shuffle Screens        This sends the front screen to the back.

W Window Size            Toggles between the large and small VTOT window.

Q Quit VTOT              Exits.  Fade will be restored, and image removed.

NOTES

-----

This program should not cause any system problems but could easily

cause visual confusion if used indiscriminately. It is designed to help the

recording process only.

Most of the functions will not work properly with animation programs.

Only one VTOT window is allowed at one time.

If you have any questions, suggestions or comments (especially nice

ones), let me know. I will continue to support the Amiga for video as long

as there is interest in my doing so. Reach me on CompuServe at [73267,3361],

on the "1939" BBS at (818) 368-4248, or write me: Mike Berro, BCS Software,

13432 Lochrin Lane, Sylmar, CA  91342.

END OF TEXT