Trademarks

----------

Amiga is a registered trademark of Commodore-Amiga, Inc.

Kickstart is a registered trademark of Commodore-Amiga, Inc.

Workbench is a registered trademark of Commodore-Amiga, Inc.

PostScript is a trademark of Adobe Systems Incorporated.

PageStream is a registered trademark of Soft-Logik Publishing Co.

Professional Page is a trademark of Gold Disk Inc.

AGFA, Compugraphic and Intellifont is a registered trademark of AGFA

All  other  trademarked  product and company names used herein are

used   for  identification  purposes  only.   No  infringement  of

trademarks   is  intended.   Trademarks  remain  property  of  the

trademark holder.

Warranty, Registration, Support and Upgrades

--------------------------------------------

Copyright      1992   Wolf  Faust.   All  rights  reserved.   All

distribution-  and  marketing  rights to the software (driver) are

vested in Canon Europa N.V.

The  driver  has  been  provided  to you strictly 'as is', without

warranty  of  any  kind.   In no event shall Canon and/or Faust be

held  liable  for  any  loss or damages arising out of any action,

from  whatever  source, concerning the use or inability to use the

driver.

For  obtaining a copy of the driver, or for more information about

Amiga driver, please contact your local Canon representative.

Following driver revisions have been made:

25.07.91  Version 35 Rev 1

25.10.91  Version 35 Rev 1.1 test version / not released to public

shading/reverse bug removed

28.10.91  Version 35 Rev 1.2 lf/cr bug removed

08.12.91  Version 35 Rev 1.3 On request: HQ/DRAFT setting doesn't affect

graphic dumps

20.04.92  Version 35 Rev 3 major update:

+ arXon support added       + new WB paper sizes added

+ I' accent fixed           + CanonBeep option added

+ CanonTime option added    + CanonResX option added

+ CanonConv option added    + CanonTab  option added

+ CanonPerf option added    + CanonTop  option added

+ CanonForm option added    + CanonPref program added

+ CanonType option added    + CanonFeed option added

+ Report program added      + aTBCALL command fixed

+ Check for MemCpy() fixed  + MemCpy()/MemClr() improved

25.05.92  Version 35 Rev 3.01 HQ/DRAFT setting doesn't affect

graphic dumps (now works... sorryyy)

01.06.92  Version 35 Rev 4

- changes in the documentation

- a small bug in MemCpy() fixed, causing chars to appear in

dark graphics

- misaligned slider in CanonPref fixed

03.06.92  Version 35 Rev 4.1

- bug in margin command fixed

Thanks to

---------

Several  people were involved during the writing of the driver and

documentation.   To  all of them, thank you!  Also I would like to

thank  all  the people who registered the previous driver release.

Without  their  support  and  letters, this driver surely wouldn't

exist.  Keep on supporting the Amiga community!

Contents

--------

This  disk  contains a printer driver for use with the Canon BJ300

and BJ330 printers and Commodore Amiga computers.  Note, there are

special  driver  version  available  for  the BJ10, 10ex, 20, 130,

130e, LBP and BJC printers!

The following files are included on the disk:

"CanonBJ300" -  The printer driver revision 4.1

"CanonPref"  -  A KS 2.0 (or higher) driver preferences program

"Install"    -  Workbench install program for the CanonBJ300 driver

"Report"     -  A report program telling your preferences/driver settings

"Readme"     -  The documentation file (and/or printed manual)

Highlights of the CanonBJ300 driver

-----------------------------------

- builtin support for the arXon electronic centronic switch box

- very fast graphic dumps

- automaticly uses improved code for 68020/30 processors if available

- fully supports WB 1.3, 2.0 and 2.1(!)

- all margins are definable

- paper size is freely definable

- comes with an extra KS 2.0 preferences program for controlling the driver

- install program supplied

- full documentation (should be normal, but it's not!)

- free definable tab stops

- free definable greyscale conversion

- job end signal

- default typeface selectable

- paper feed selectable

System requirements

-------------------

The  CanonBJ300  driver  is  compatible  with the entire family of

Amiga  computers.   These  include  the  A500, A500+, A600, A1000,

A2000,  A2500,  A2500/30  and  A3000  computers.   The  driver was

designed  to  run  under Workbench versions 1.3, 2.0 and 2.1.  The

CanonPref program was designed to run under Workbench 2.0 and 2.1.

The  CanonBJ300  driver was designed for the Canon BJ300 and BJ330

printers.

Installation

------------

1. Setup and connect your BJ-300/330 as described in the

BJ-300/330 Users Manual.

2. Set the printers DIP Switches SW1-1 and SW1-2 to ON position

all other switches may be left at default or customised

appropriatly.

3. Boot your Amiga with your usual System Disk.

4. Insert the Canon-Driver-Disk and select the appropriate

"Drawer" for the printer.

5. Double Click on the "Install-BJ300" program. The "Install" program

will copy the driver on to the system disk.

If there is not enough free disk space to install the printer

driver a requester will pop up saying so. In this case free up

some disk space on your system disk by deleting unused files.

If you are a CLI user, simply copy the driver into your

"devs:printers" directory.

6. Start Preferences on your system disk (if you own Kickstart

2.0 use the printer preference program). Select the CanonBJ300

driver and adjust all other preference parameters to your

environment. See your system manual for more information

about the preference program.

7. In case you use Workbench 2.0 (or higher) use the mouse to drag

the CanonPref icon into your Workbench prefs

drawer. If your Workbench disk does not allow this because

no space is left on disk, you can simply drag the programs

to any other wanted drawer.

8. Start the CanonPref program if you are using Workbench 2.0 (or

higher). Adjust all parameters to your wanted environment.

Note:   the  install  program  will erase any previously installed

CanonBJ300  drivers  without  asking  you  so.  Also note that the

Commodore  CanonBJ10 driver is a subset of this driver and because

of  this,  you  get  all the functions of the Commodore driver and

even more.

Workbench Preferences

---------------------

While  the Workbench preferences program is described in detail in

your  system  manual,  there are three settings which belong to the

driver documentation:

Density:

The driver supported graphic densities:

Density   X-DPI    Y-DPI   X*Y-DPI

1    360       360        129600

2    180       180         32400

3    120       360         43200

4    180       360         64800

5    240       360         86400

6    360       180         64800

7    360       360        129600

The  desired density can be selected in Preferences or in case you

use  Workbench  2  "PrinterGfx".   Most  application programs also

allow you to select the density before doing a graphic dump.  This

setting overrides temporarily the preferences density setting.

Paper Size:

Under  Workbench 2 and 2.1 (or later) this driver supports the new

introduced papersizes.  Anyway, DIN A4 users may select the Letter

paper  size  instead of DIN A4 to print a DIN A4 letter (2804 dots

wide) in the highest resolution (2880 dots wide).

PaperType:

A  papertype  of  SINGLE will eject a sheet with a Form Feed after

every  print.  Form Feed after a graphic dump is sent according to

the  SPECIAL_NOFORMFEED  flag.   You can use the FANFOLD papertype

setting  to force several dumps to appear on the same page.  Also,

if you need to run InitPrinter often, you may choose the papertype

FANFOLD to avoid printing empty pages.

The Driver in detail

--------------------

Now  that  you have installed and adjusted the driver, printer and

preference  setting,  there  are some details you should know when

using the driver.

This   driver  automatically  checks  for  the  processor  in  use

(68000,68010   or   68020,68030,68040)  and  optimizes  the  print

routines  accordingly.   Because  of  this, there is no need for a

special .020 version of the driver in order to get the most out of

your turbo amiga.

Aborting  printing  may  often  leave  the  printer  in an unknown

status.  This causes erronious printing on the next dump.  In case

you  abort printing, also switch your printer off and on. Also run

InitPrinter in order to bring back the printer in a known status.

The driver supports the unique Amiga commands shown below:

aRIS    reset

aRIN    initialize

aNEL    CRLF

aSGR0   normal char set

aSGR3   italics on

aSGR23  italics off

aSGR4   underline on

aSGR24  underline off

aSGR1   boldface on

aSGR22  boldface off

aSHORP0 normal pitch

aSHORP2 elite on

aSHORP1 elite off

aSHORP4 condensed fine on

aSHORP3 condensed fine off

aSHORP6 enlarge on

aSHORP5 enlarge off

aDEN6   shadow print on

aDEN5   shadow print off

aDEN4   double strike on

aDEN3   double strike off

aDEN2   NLQ on

aDEN1   NLQ off

aSUS2   superscript on

aSUS1   superscript off

aSUS4   subscript on

aSUS3   subscript off

aSUS0   normalize the line

aPLU    partial line up

aPLD    partial line down

aFNT0   Courier 10 or selected typeface

aFNT1   Letter Gothic 10

aFNT2   Prestige 10

aFNT3   Font Modul BJFC-301 General: Script 10

aFNT4   Font Modul BJFC-301 General: Press Roman PS

aFNT5   Font Modul BJFC-302 OCR: OCR-A

aFNT6   Font Modul BJFC-302 OCR: Olde World 10

aFNT7   Font Modul BJFC-302 OCR: Prestige Symbol

aFNT8   Font Modul BJFC-303 Modern: Presentator 10

aFNT9   Font Modul BJFC-303 Modern: Orator 10

aFNT10  Font Modul BJFC-303 Modern: Gothic Symbol 10

aPROP2  proportional on

aPROP1  proportional off

aVERP0  1/8" line spacing

aVERP1  1/6" line spacing

aSLPP   set form length

aPERF   perf skip n (n > 0)

aPERF0  perf skip off

aSLRM   set L&R margins

aCAM    clear margins

aTBC3   clear all horiz tabs

aTBC4   clear all vert tabs

aTBCALL clear all h & v tabs

aTBSALL set default tabs

aRAW    next 'n' chars are raw

The CanonPref Program

---------------------

Beside  all  the functions mentioned in the chapter "The Driver in

detail",  there  are  some  more  options unique to the CanonBJ300

driver.   These  options are controlled using commodore's standard

environment  variables.   Those  variables can be set by using the

CLI setenv command, or by using the CanonPref preferences program.

CanonPref  provides  a nice and intuitive user interface and needs

at  least  Workbench  2 to run.  In case you don't use Workbench 2

(V37  and  above), see the chapter "Environment options" on how to

control  the  special  option with a Kickstart 1.3 system.  If you

don't  use these options, default values will be used.  The driver

will  read  the  arguments  set by CanonPref on opening.

Note:   changes  to the options may not take place immeadiatly!!!!

The  options  take  place with the next send aRIN or aRIS command.

You  can send an aRIN command by using Initprinter or flushing the

driver  out  of  memory  (WShell  users may use "Flush"; Commodore

users  may  use the "Stack 40000000" command in order to do this).

CanonPref  automatically flushes the driver out of memory for you.

So  this  note  is  specialy  importent  for  CLI  users not using

CanonPref.

CanonPref  can  be  controlled by the mouse and/or keyboard.  Each

gadget  of the main program corresponds to the underscored letter.

Pressing  that letter is equivalent to clicking on the gadget.  In

the  case  of  a cycle gadget, you can use the upper or lower case

letter  to  cycle  back  and  forth between them.  See your system

manual  on  how to control the file- and fontrequester by keyboard

or mouse.

All  windows  are opened directly under your mouse pointer, so you

don't  have  to  move or scroll the screen to control the program.

Also,  all  windows  are opened on the default public screen.  You

may  specify  a  special  public  screen  by setting the Tool Type

"PUBSCREEN=<screenname>"   (name   is  case  sensitive!)  and  the

CanonPref  program  will  open  it's  windows  on your application

programs  screen.   You  can  specify  a  screen from CLI with the

PUBSCREEN option:

CanonPref PUBSCREEN <screenname>

For  instance,  if you use the ASDG text editor "CygnusEd" you may

specify  "CanonPref  PUBSCREEN CygnusEdScreen1" and CanonPref will

open  up  its  windows on the CEDPro screen.  Make sure the CEDPro

screen is public before running the example.

After  starting,  the  main  window  will contain several gadgets,

described  in  detail.  All those gadgets represent an environment

variable  which  can  also  be  controlled  using  the  CLI setenv

command.   I strongly recommend using CanonPref because it is user

friendly and more secure.

Job End:              No Signal (default)

Beep

Flash

Beep&Flash

If  the  printer device gets closed after a dump, you can tell the

CanonBJ300 driver to signal you the end of the print job.  This is

especialy  usefull  on  huge graphic dumps wich might require some

time.   There  are  two  kinds  of  signals.  You can cause a beep

(CTRL-G)  on  the  printer,  a Flash (DisplayBeep) on the Amiga or

both.

Note:   some  applications  print  graphics  in  stripes.   If the

application  program closes the printer.device after every stripe,

you  will  get a signal after each stripe.  So far, I havn't found

an application program with such a bad behaviour.

Port:                 Disabled (default)

Port 1

Port 2

Port 3

The CanonBJ300 driver supports the arXon parallel switch box.  The

arXon  switch  box  is  an  external  1  to  3  centronics  switch

controlable  by  software.   You  can ask the CanonBJ300 driver to

automatically switch to a wanted port for printing and switch back

to  the  previously used port after printing.  This enables you to

use  a scanner, digitizer and printer from your Amiga's centronics

port without switching between the devices "by hand".  Specify the

wanted  printer port in "sbox_printer" and the default port (which

should be set up after printing) in "sbox_active".  If you specify

0 (default) as the port, no switching will take place.

Note:   "sbox_active" MUST be specified before using the driver in

order  to  enable  the  "sbox_printer"  command.   This is because

"sbox_active"  is  used  to  determine  whether  a  switch  box is

installed  or  not.   Information  about  the  switch  box  can be

obtained from:

arXon GmbH

Assenheimer Str. 17

D-W-6000 Frankfurt

Germany

Tel:  (++49)-69-7896891

Fax:  (++49)-69-7896878

Custom Res-X:         0-65535 dots (default 0)

If   the   selected   papersize  in  preferences  (Workbench  2.0:

"Printer")  is  CUSTOM,  this  option  allows  you  to specify the

horizontal  resolution of the printed paper.  For instance, if you

use  Workbench  1.3,  you can specify a X-resolution of 2804 dots,

which  is equal to the DIN A4 size.  If you specify 0 (default) as

X-resolution,  the US-LETTER resolution of 2880 dots will be used.

The allowed range is 0 to 65535 dots.

Top Margin:           1 - <page length-1>  (default 1)

If  "Enable  Form"  is  set  to  ON  (checkmark  on), "Top Margin"

specifies  the  top  margin  in lines.  The top margin defines the

vertical distance between the top of the page and the base line of

the first line on the page.  When the cut sheet paper loading mode

is  selected, this command is ignored.  Using this option together

with  the  preference  settings  for  left,  right margin and page

length, you are totaly free in defining the margins of a page.

Enable Form:          not checked (default)

checked

If  "Enable Form" is set to ON (checkmark on), page length (as set

in preferences), top margin (defined by CanonTop), and perforation

skip  (defined  by CanonPerf) will be defined by the driver.  Some

applications  require  a  well  defined paper size and margins for

printing   forms.    The   CanonBJ300  driver  provides  you  such

possibilities!

Perforation Skip:     0 - (form length-1)  (Default: 0 lines)

If  "Enable  Form" is set to ON (checkmark on), "Perforation Skip"

specifies the number of lines that the printer skips at the end of

each  page.   This  causes  the  printer  to  skip the perforation

between  pages  of  continuous  forms.   If 0 lines are specified,

perforation  skip  will  be  disabled.  If the specified CanonPerf

value  is  greater than or equal to the form length (as defined in

the WB preferences program), the skip perforation is cancelled and

the top and bottom margins become inactive.

Paper Length:

This  is a read only gadget showing the paper length as defined in

Workbench  preferences.   You  can  use  the  value  to  set  your

perforation  skip and top margin value.  Changes made to the paper

length  preferences  are  automatically  detected by the CanonPref

program  and  the  gadget  is updated immeadiately without exiting

CanonPref.

Timeout:               1 - 999 (default 30)

In  case  huge  amounts  of  data  are  transfered to the printer,

rendering a picture by the device is much faster than the transfer

of  the  picture to the printer.  If this happens, a requester may

appear  on the screen, saying "Printer trouble...".  The requester

appears  because  the printer device thinks no data got transfered

because  of  a hard- or software failure.  In reality your printer

was busy with feeding a new paper.  With CanonTime you can specify

a timeout before the printer device brings up the error requester.

The  CanonTime  var  is  especially usefull when printing multiple

pages  or  downloading huge fonts using the drivers automatic font

download abilty.  A value of 1 to 999 seconds is allowed.  Setting

a  high  timeout  value  also  has it's drawbacks:  in case a real

error  appears,  you  also  have to wait the given time before you

really can cancel the print job.

Instead  of  defining  a  huge  timeout, there is another solution

available:   use  the  CMD  program  (see  you  Workbench disk) to

redirect  the  parallel/serial  output  to  a file on disk.  After

this,  use  the copy command or a printer spooler to copy the file

to  "par:".   Using this technique, you can safely specify a small

amount of seconds before a timeout.

Feed:                     Disabled   (printer default)

Manual

Top Bin 1

Top Bin 2

Envelope

The  CanonBJ300  driver allows you to select a special paper feed.

The  setting takes effect on the installed optional auto cut sheet

feeder.  The setting is ignored when the FORM mode is selected for

using fan fold paper.  Note:  if the Envelope Bin is selected, the

BJ-300/330 will select Bin 1 to load from.

Type:                    Courier (default)

Letter Gothic

Prestige Pica

Script

Press Roman

Boldface

Thesis

OCR-A

OCR-B

Olde World

Prestige Symbol

Presentor

Orator

Essay PS

Gothic Symbol

This  cycle  gadget specifies the typeface to be selected on aRIS,

aRIN  and aFNT0 (i.e.  your normal typeface).  If an external font

cartridge  is  selected  without  font cartridge installation, the

printer will ignore the typeface setting.

Customize Tabs...:

The  ellipsis  (three  dots)  in  this gadget means, that an extra

window will be opened when you select this gadget.  The gadgets of

the  upcoming  window  let  you customize the horizontal tab stops

settings  of  your  printer.  This is usefull for printing program

listings  and  tables.   For instance:  you can change the default

tab stops settings:

1         2         3         4         5

12345678901234567890123456789012345678901234567890...

----+---T+----+-T--+----T----+--T-+----+T---+---T+

to:

1         2         3         4         5

12345678901234567890123456789012345678901234567890...

---T+-T--T--T-+T--T+-T--T--T-+T--T+-T--T--T-+T--T+

Up  to 28 tab stops may be defined.  All tab stops above the limit

of  28  are  ignored and a requester will tell you so.  Horizontal

tabs  are  selected and stored by the printer in character spaces,

not  inches.  Thus, any change in the character spacing can change

the  physical  locations  of  horizontal tabs on the line, but the

logical  positions  remain  unchanged.  With PS fonts, a column is

defined  as  the  width of the current space character at ASCII 32

($20).

You can set/unset a tab stop by simply clicking into the gadget at

the  wanted  position.  A "T" is used for a tab stop.  The "-" and

"+"  chars  are  used  for  helping  you  in  finding  the correct

position.   Using  the  slider below the gadget you can scroll the

"tab  stops"  gadgets to a wanted position.  With "Clear Tabs" you

can  clear  all  tab stops.  With the "n:" and "Set every n chars"

gadgets,  you  can  easily  set  tab stops with the same distance.

Simply  enter  the  distance  into the "n:" gadget and select "Set

every n chars".  Note:  the printer driver uses a default distance

of  8.   If you want to reset you customized tabs stop settings to

the  printer  default  values, simply call "Customize Tabs..." and

the  window  will come up with a default "n:" value of "8".  Press

"Set every n chars" to get the default values.

You  can  leave  the  window  with  the  "Use" or "Cancel" gadget.

Changes  made  to  the tab stops settings are accepted with "Use".

"Cancel"  leaves the tab stops unchanged and any changes made will

be  lost.   Note:   "Use"  does  not save the changes made to your

environment.   This is done by using the "Use" or "Save" gadget in

the main window.

Greyscale Conv...:

The  ellipsis  (three  dots)  in  this gadget means, that an extra

window will be opened when you select this gadget.  The gadgets of

the  upcoming  window  let  you customize the greyscale conversion

function of the printer driver.

You  can define various greyscale conversions for the graphic dump

of  the driver.  This allows you to make a picture brighter (which

is  specially  usefull in high density modes), darker, increase or

decrease   contrast  level.   Now  using  this  feature  is  easy,

understanding  it might not.  So let's try to describe some things

and  after  reading  it twice, simply try it!  You'll be extremely

satisfied with the results....

For more information see the "greyscale conversion" chapter first!

The  "Greyscale  Conversion"  window  contains  16  sliders.  Each

slider represents a value between 0 and 15, were 0 means black and

15  white.   The  order  of  the  16 sliders goes from black (left

bottom)  to  white  (right  upper  side).   For example, following

function (which is the default linear conversion of the driver) is

shown by the sliders:

bright|                             15

|                           14

13| <- <- <- <- <- <- <- <- 13

O            |                       12 ^

|                     11   |

U            |                   10

|                  9       ^

T            |                8         |

|              7

P            |            6             ^

|          5               |

U            |        4

|      3                   ^

T            |    2                     |

|  1

dark|0                         ^

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                      13  bright

I N P U T

As  you  can easily see, a greyscale value of 13 is printed by the

driver  as  greyscale  value of 13.  So no change.  To define this

linear  greyscale  conversion, you can also click on the "default"

gadget  in  the  lower right side of the window.  "Invert" inverts

every  slider  setting  resulting  in  a printed "negativ" of your

picture.  "gamma +" sets the 16 slider to a value defining a gamma

corretion  function.   The  "gamma" conversion function provides a

way to significantly brighten an image without losing much detail.

"Gamma  -"  is  the  opposite of "gamma +" and results in a darker

printout.  By repeatidly clicking on the "Bright" gadgets, you can

make your printout darker ("Bright -") or brighter ("bright +").

You  can  leave  the  window  with  the  "Use" or "Cancel" gadget.

Changes  made  to the conversion function are accepted with "Use".

"Cancel"  leaves the conversion function unchanged and any changes

made will be lost.  Note:  "Use" does not save the changes made to

your  environment.   This  is  done  by  using the "Use" or "Save"

gadget in the main window.

Cancel:

You  may  leave  the CanonPref program by clicking on the "Cancel"

gadget.  Changes made to the settings will not be saved.

Use:

You  may  leave  the  CanonPref  program  by clicking on the "Use"

gadget.   Changes  made  to the settings will be saved to the ENV:

directory.   Note:   on normal Workbench 2.0/2.1 environments, all

vars  in  the ENV:  directory are temporary and will be erased the

next  time  you  boot  the  machine.  In case you want to make the

settings permanent, use the "Save" gadget.

Save:

You  may  leave  the  CanonPref  program by clicking on the "Save"

gadget.   Changes  made  to  the  settings  will  be  saved to the

ENVARC: and ENV: directory for permanent and temporary use.

Environment Options

-------------------

You can set options for the CanonBJ300 driver by using the "setenv"

CLI  command instead of the CanonPref program.  This command saves

your option in the "ENV:" directory using a file.  See your system

manual  for  a  complete  description  of  the setenv and "getenv"

command.   The  driver  will  read in the arguments on opening the

printer  device.  In case you don't have enough memory, the driver

refuses to open.

Note:   reading  the  options does not always mean they take place

immeadiately.  So if you make changes by using "setenv", flush the

driver  out  of  memory  or  run "Initprinter" in order to let the

driver  know  about  your changes.  CanonPref will do this for you

automatically.

Remember  that  environment  vars  (they  are stored in the "ENV:"

directory)  are  normaly  erased by booting.  If you don't want to

set  the  environment vars every time you boot, you can place them

in  "ENVARC:"  (Workbench  2.0/2.1)  or  assign  "ENV:"  to  a non

volatile  disk (KS 1.3).  Make sure you use the commodore "setenv"

command  to set the environment vars!  Under KS 2 (or higher), the

driver supports local and global environment vars!  Here is a list

of  the  possible options and their meaning.  As usual, everything

is case insensitve:

SetEnv sbox_printer <0|1|2|3>  (Default: 0)

The CanonBJ300 driver supports the arXon parallel switch box.  The

arXon  switch  box  is  an  external  1  to  3  centronics  switch

controlable  by  software.   You  can  ask  the  CanonBJ driver to

automaticly  switch  to a wanted port for printing and switch back

to  the  previously used port after printing.  This enables you to

use  a  scanner, digitizer and printer from your Amigas centronics

port without switching between the devices "by hand".  Specify the

wanted  printer port in "sbox_printer" and the default port (which

should be set up after printing) in "sbox_active".  If you specify

0 (default) as the port, no switching will take place.

Note:   "sbox_active" MUST be specified before using the driver in

order  to  enable  the  "sbox_printer"  command.   This is because

"sbox_active"  is  used  to  determine  weather  a  switch  box is

installed or not.

SetEnv CanonBeep  0|1|2|3  (Default: 0)

If  the  printer device gets closed after a dump, you can tell the

CanonBJ300 driver to signal you the end of the print job.  This is

especially  usefull  on huge graphic dumps wich might require some

time.   There  are  two  kinds  of  signals.  You can cause a beep

(CTRL-G) on the printer, a DisplayBeep on the Amiga or both.

0 = No Action (default)

1 = Printer beep (CTRL-G)

2 = DisplayBeep

3 = Printer beep + DisplayBeep

Note:    some   application   print   graphics  striped.   If  the

application  program closes the printer.device after every stripe,

you  will get a signal for each stripe.  So far, I havn't found an

application program with such bad behaviour.

SetEnv CanonResX <num>  (Default: 0 dots)

If  the papersize is CUSTOM, this option allows you to specify the

horizontal resolution for graphic dumps.  For instance, if you use

Workbench 1.3, you can specify a resolution of 2804 dots, which is

equal  to  the  DIN  A4  size.   If  you  specify  0  (default) as

resolution,  the  US-LETTER  resolution of 2880 dots will be used.

The allowed range for <num> is 0 to 65535 dots.

SetEnv CanonTime <num>  (Default: 0 seconds)

In  case  huge  amounts  of  data  are  transfered to the printer,

rendering a picture by the device is much faster than the transfer

of  the  picture  to  the  printer.  A requester may appear on the

screen,   saying  "Printer  trouble...".   The  requester  appears

because  the  printer device thinks no data got transfered because

of  a hard- or software failure.  With "CanonTime" you can specify

a  timeout until the printer device brings up the error requester.

The  "CanonTime"  var is especially usefull when printing multiple

pages  or downloading huge fonts, using the drivers automatic font

download abilty.  If you specify 0 (default) as timeout, a timeout

of  30  seconds  will  be  used.  A value from 1 to 999 seconds is

allowed.   Instead  of  defining  a huge timeout, there is another

solution  available:   use  the  "CMD"  program (see you Workbench

disk)  to  redirect  the parallel/serial output to a file on disk.

After  this, use the copy command or a printer spooler to copy the

file  to  "par:".   Using this technique, you can safely specify a

small number of seconds before a timeout.

SetEnv CANONFEED  <num>   (Default: 0)

With  this  var  you can set the paper feed mode.  <num> can range

from  0  to 4.  The setting takes effect on the installed optional

auto  cut sheet feeder.  The setting is ignored when the FORM mode

is  selected for using fan fold paper.  Note:  if the Envelope Bin

is selected, the BJ-300/330 will select Bin 1 to load from.

<Num>                     Meaning

0                Disabled   (printer default)

1                        Manual

2                       Top Bin 1

3                       Top Bin 2

4                       Envelope

SetEnv CanonType <num>  (Default: 0)

Specifies the typeface to be selected on aRIS, aRIN and aFNT0.  If

you  select 0 (default) as typeface, the internal Courier typeface

will  be  selected.   If  an  external  font cartridge is selected

without  font  cartridge installation, the printer will ignore the

typeface setting.

CanonType <num>             Meaning

0                    Courier (default)

1                     Letter Gothic

2                     Prestige Pica

3                         Script

4                      Press Roman

5                       Boldface

6                        Thesis

7                         OCR-A

8                         OCR-B

9                      Olde World

10                    Prestige Symbol

11                      Presentor

12                        Orator

13                       Essay PS

14                     Gothic Symbol

SetEnv CanonConv <num0-15>  (Default: 0,1,2,3,4,5,6,7,8,9,10,

11,12,13,14,15)

You  can define various greyscale conversions for the graphic dump

of  the driver.  This allows you to make a picture brighter (which

is  specialy  usefull  in high density modes), darker, increase or

decrease  contrast level, and do gamma correction.  Now using this

feature  is  easy,  understanding  might not.  Simply try it after

reading  the documentation!  You'll be extremly satisfied with the

results....

For general information see the "greyscale conversion" chapter!!!

You  must  define exactly 16 greyscale values for CanonConv.  Each

value  must  be  in  range from 0 to 15, were 0 means black and 15

white.   The order of the 16 values goes from black to white.  For

example,  you  want  to  define  following  function (which is the

default linear conversion of the driver):

bright|                             15

|                           14

13| <- <- <- <- <- <- <- <- 13

O            |                       12 ^

|                     11   |

U            |                   10

|                  9       ^

T            |                8         |

|              7

P            |            6             ^

|          5               |

U            |        4

|      3                   ^

T            |    2                     |

|  1

dark|0                         ^

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                      13  bright

I N P U T

As you can easily see, a greyscale value of 13 is printed by the

driver as 13. So no change. To define this linear greyscale

conversion, you must enter in CLI:

SetEnv CanonConv "0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15"

Next time you print graphics, the new conversion function is used.

Defining  and  editing  a  greyscale conversion function using the

setenv  command  is  uncomfortable.  You can use the CanonPref for

editing and defining a function.  When finished with defining, you

can  get  the  values  set  in  CanonPref  by  using  the  "getenv

CanonPref" CLI command.

SetEnv CanonTab <num1-28>  (Default: 9, 17, 25,...)

This  environment  let  you  customize  the  horizontal  tab stops

settings  of  your  printer.  This is usefull for printing program

listings  and  tables.   For instance:  you can change the default

tab stops settings:

1         2         3         4         5

12345678901234567890123456789012345678901234567890

T       T       T       T       T       T

with 'Setenv CanonTab "3 6 9 12 15 18" '  to:

1         2         3         4         5

12345678901234567890123456789012345678901234567890

T  T  T  T  T  T

Up  to 28 tab stops may be defined.  All tab stops above the limit

of   28   are  ignored.   All  previous  tabulators  are  cleared.

Horizontal  tabs  are  selected and stored internally in character

spaces, not inches.  Thus, any change in the character spacing can

change  the physical locations of horizontal tabs on the line, but

the  logical  positions remain unchanged.  With PS fonts, a column

is defined as the width of the current space character at ASCII 32

($20).

SetEnv CanonPerf <0-form length-1>  (Default: 0 lines)

If CanonForm is set to ON, CanonPerf specifies the number of lines

that  the  printer skips at the end of each page.  This causes the

printer to skip the perforation between pages of continuous forms.

If  0  lines are specified, perforation skip will be disabled.  If

the specified CanonPerf value is greater than or equal to the form

length  (as  defined  in  the  WB  preferences  program), the skip

perforation  is  cancelled  and  the top and bottom margins become

inactive.

SetEnv CanonTop <0-form length-1>  (Default: 0 lines)

If CanonForm is set to ON, CanonTop specifies the top margin.  The

top  margin  defines  the vertical distance between the top of the

page  and  the  base line of the first line on the page.  When the

cut  sheet paper loading mode (MAN, SF1, or SF2) is selected, this

command is ignored.

SetEnv CanonForm  ON|OFF  (Default: OFF)

If  CanonForm  is  set to ON, page length (as set in preferences),

top margin (defined by CanonTop), and perforation skip (defined by

CanonPerf) will be defined by the driver.

Greyscale Conversion

--------------------

While your Amiga is able to show color and greyscale pictures on a

monitor, there is no way to reproduce those picture on a black and

white   printer   like   the  Canon  BJ300.   In  order  to  print

color/greyscale  pictures  the  printer driver converts colors and

greyscales  to  an  array  of simple b/w dots.  The greyscales and

colors  are  hereby simulated by using a 4 by 4 dither array.  The

main  process  of  this  function  is  done  by  a function called

"greyscale conversion".  This function takes a greyscale value and

translate  it to a value wich is finaly used to print the original

greyscale value.  The greyscale conversion makes sure, the printed

greyscale matches the greyscale shown on your Amiga monitor most.

Because every printer and picture is different, you need a special

conversion  function  for  every  printer and picture.  Most Amiga

drivers simply use a linear conversion function for every picture.

This  means the amount of printed dots on your printer is directly

proportional   to  the  greyscale  level  of  your  picture.   The

brightness  of printed picture using a linear greyscale conversion

is often not satisfying.  There are two main reasons this:

-  the greyscale levels shown on your monitor are not linear.  For

instance,  the  difference  between two neighboring dark greyscale

levels is not as big as between to brighter levels.

- the Canon BJ printers have a precise printerhead movement.  This

gives  you  the possibility to print 360 dots per inch (dpi).  But

there  is  one  drawback,  which most printers have:  a 1/360 inch

huge  dot  can  not  be  printed.  The dot is a bit greater, which

causes the dots to overlap each other.  This causes a dark printed

picture and a loss of contrast.

To  compensate  these affects, the CanonBJ300 driver allows you to

customize  and  adjust the greyscale conversion function.  You can

also  do  some art work (for instance pseudo colors) by using this

function.

The Amiga printer.device supports up to 16 greyscale levels. These

levels are represented by values from 0 (black) to 15 (white). Now

let's have a look at the default linear conversion function:

bright|                             15

|                           14

13| <- <- <- <- <- <- <- <- 13

O            |                       12 ^

|                     11   |

U            |                   10

|                  9       ^

T            |                8         |

|              7

P            |            6             ^

|          5               |

U            |        4

|      3                   ^

T            |    2                     |

|  1

dark|0                         ^

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                      13  bright

I N P U T

As  you  can easily see, every greyscale level is unchanged by the

greyscale  conversion  function.   For  instance, a value of 13 is

unchanged.   Now  we  could  simply increase the brightness of the

output, by simply adding 1 to each value:

bright|

|                           1415

14| <- <- <- <- <- <- <- <- 13

O            |                       12 ^

|                     11   |

U            |                   10

|                  9       ^

T            |                8         |

|              7

P            |            6             ^

|          5               |

U            |        4

|      3                   ^

T            |    2                     |

|  1

|0                         ^

dark|                          |

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                      13  bright

I N P U T

Now  the greyscale value of 13 would be printed as 14.  A drawback

for  brighten  up a picture this way is the greyscale value 14 and

15.   Both levels would be printed with the same value of 15 which

means  white.  A better method to brighten up a picture is kind of

gamma   correction  function.   The  "gamma"  conversion  function

provides  a  way to significantly brighten an image without losing

much detail:

bright|

|                             15

|                           14

|                         13

O            |                       12

|                   1011

U            |                  9

|                8

T            |              7

|          5 6

P            |        4

|      3

U            |    2

|

T            |  1

|

dark|0

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                         bright

I N P U T

As you can see, there is no detail lost because several levels got

totaly  black or white.  Though, some levels are printed using the

same  value.   You  can alter is this function to brighten up your

picture even more.  Just try...

There  is  a  gadget for the above shown function in the CanonPref

program  called  "gamma +".  Simply click on the gadget to get the

function above and if you like, alter the function afterwoods.

You  can  make  a  darker  picture  the same way you brighten up a

picture.  Simply lower the levels instead of increasing them.

Now  there  is  even  more you can control by using the conversion

function.   For instance you can increase or decrease the contrast

of the printed picture. For increasing the contrast, you must

increase the steepness of the conversion function for instance

like this:

bright|

|                     15

|                     14

|                   13

O            |                   12

|                 11

U            |                 10

|                9

T            |                8

|              7

P            |              6

|            5

U            |            4

|          3

T            |          2

|        1

dark|        0

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                         bright

I N P U T

To decrease the contrast, simply make the function more flat:

bright|

|

O            |

|

U            |

|                           1415

T            |                       1213

|                   1011

P            |                8 9

|            6 7

U            |        4 5

|    2 3

T            |0 1

|

|

|

dark|

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

dark                         bright

I N P U T

Using the greyscale conversion function, you are also able to

filter out parts of a picture with a special color. Just try and

I'm sure you'll be satisfied.

Hints & Trouble Shooting

------------------------

*  In  case  of stripes and empty lines check your printer margins

(DIP  switches,  preferences  setting, settings within the program

you  use,...).   Also  make  sure  your  computer does have enough

memory.   Specialy Art Department (TM) Professional 2 users should

use  the  "MAXMEM"  option to make sure the driver does get enough

memory  for  printing  or  ADPro  will simply print trash or empty

lines.   About  100  kb  memory  should  be available before start

printing!

*  Data  should  be  sent  to the printer only via parallel.device

(par:)  or printer.device (prt:) and not both together as this may

confuse the printer.

*  In  case  you  found  a  problem with the driver, please send a

description  which makes it possible to reproduce the problem.  For

this purpose, you should provide following information:

-  The disk contains a Report program. Allways include the

output of this program.

-  Which application program are you using and which version?

-  Which Workbench revision are you using?

-  Include a dump of the program as file (use the CMD program from

the Workbench disk for this) and in printed form.

-  and if possible: provide a copy of your used Workbench

disk/application ready set up for printing

*  If  you  are  interested  in  buying  AmigaTeX from Radical Eye

Software,  you  should  buy  the  dvipref  or  dvips+Post AmigaTeX

driver.   This driver also needs the 360*360 fonts which come with

the dvip6 driver.

*  Unable  to  start  CanonPref?   Make  sure  you  have  at least

Kickstart 2.04 and Workbench 2.04 installed right.

*  Unable  to print?  Check you printer cable!  Also make sure the

CanonBJ300 driver is selected in the Workbench preferences program!

*  Printer  doesn't  work  under  Kickstart  2.04  while Kickstart

1.2/1.3  works  fine?  Check your printer cables "online" and "out

of  paper"  lines!  Make sure they are corrected right.  Also if a

device is connected to your serial port:  try after removing it!

*  Paper sheets are ejected and you don't want them to be ejected?

Check  your  Workbench  "paper type" preferences.  Note:  the aRIN

and  aRIS  command  eject a "used" sheet of paper!  So it might be

totaly normal!

*  "Printer trouble..." requester?  Have a look at the "CanonTime"

environment  description  on  how  to  avoid  such requesters from

appearing!    Anyway,  clicking  on  "Resume"  will  continue  the

printings without losses.

*  After  sending  data  to the printer, the last page doesn't get

ejected?   Check  your  Workbench  preferences paper type setting.

Select "Single" sheet!

*  The  driver  doesn't print graphics?  There are several reasons

why this might happen.  First check you Workbench disk.  Make sure

it  is  as  least  Workbench  1.3  and  no  prerelease  version of

Workbench 1.3.  Also make sure your computer has enough memory for

printing graphics (100KB should be enough)

*  Your  graphic  dumps are not really black?  Try a different ink

cartridge  and  paper!   Believe  me:   you can get a really black

dumps!

* After switching from Workbench 1.3 to 2.0/2.1 your dumps doesn't

have  the  same size and resolution?  This most likely will happen

to  users  using  the  DIN  A4  paper  size.   For  instance under

Workbench  1.3  you  used  the  Letter size and after switching to

Workbench  2.0/2.1  the  DIN  A4  size.   Because  the  CanonBJ300

supports  the  new  paper sizes (DIN A4,A5...) you get a different

printouts  under  Workbench  2.0/2.1.   Simply  switch back to the

Letter size to get the same result.

_\/_

this is it....   END       /~  ~\

\| o  o |/

--------  -------  <- the author ;-)

\/