_Using .DO files to store 8-bit binary data_

by Tracy Allen, 5/8/89

released through Club 100 to the public domain, except for for-profit

publication.

.

I often need to store 8-bit data in .DO files on the M100/102/200.  This

data comes from sensors (temperature, etc.) and takes on byte values in

the range of 0 to 255.  If it is stored in its ascii form it takes a lot

more room in memory than it does when it is stored in binary.  For

example, a temperature of 212 degrees Celsius stored as ascii takes three

bytes "2"  "1"  "2" plus a space or a tab to separate it from the next

datum.  The same datum stored as binary takes only one byte, and no space

or tab is needed, because _all_ data take only one byte with a binary

range of 0 to 255.  It is efficient data compression.

.

The problem is that 5 out of the 256 possible byte values have special

meaning to the M100/102/200.  These special ascii byte values are:   0

(zero, null), 10 (line-feed), 13 (carriage return), 26 (end-of-file) and

127 (delete).  Of those, the the 0 or the 127 cannot be entered in a file

using normal PRINT# statements.  The 26 means end-of-file and you will get

an error message if you try to read past it using normal INPUT#

statements, and the operating system sooner or later will zap anything

that follows the 26.  The 10 and the 13 can be stored and retreived from

the files, but carriage return and linefeed  have special significance to

the LINEINPUT# statement.  I like to use LINEINPUT# to retrieve groups of

readings.  I store groups of hourly readings on lines terminated by a

carriage return, and doing so helps to maintain the integrity of the data

should a reading happen to be lost.  The line-feed caharacter is special

only because of a peculiarity of the LINEINPUT# command.  If line-feed

precedes carriage-return, the LINEINPUT# command does not recognize the

carriage-return as the end-of-line character and will instead read it in

as data up to the next carriage return that is not preceded by line-feed.

Line-feeds that immediately follow a carriage return are treated as part

of the end-of-line.  So you canUt have LF as part of data either directly

before or directly after a CR.

.

An approach that gets around this is to map the five special characters up

to characters 251->255.  I can discard those high values, because the

temperatures and other data never get so high:

.               If D%>250 then D%=250

.where D% is the datum to be stored.  Map the characters as follows:

.               If D%=0 then D%=255

.               If D%=10 then D%=254

.               If D%=13 then D%=253

.               If D%=26 then D%=252

.               If D%=127 then D%=251

At this point the data contain none of the offending characters.  Write it:

.               PRINT#1,D%;

And when it's time to end a line of data, put in a CR:

.               PRINT#1,CHR$(13);

That's it for storing the data.

Reading in the data is exactly the opposite process.  Let's say you want

to send the data (TAB-delimitd) over the modem to another computer:

.              MAXFILES=2

.              OPEN "DATA.DO" FOR INPUT AS 1

.              OPEN "MDM:8N1E" FOR OUTPUT AS 2

.LOOP:         IF EOF(1) THEN PRINT#2,CHR$(26):CLOSE:END

.              LINEINPUT#1,A$

.              FOR I=1 TO LEN(A$)

.              D%=VAL(MID$(A$,I,1))          'get the I'th ascii chr in

line

.              IF D%=255 THEN D%=0           'reverse mapping

.              IF D%=254 THEN D%=10

.              IF D%=253 THEN D%=13

.              IF D%=252 THEN D%=26

.              IF D%=251 THEN D%=127

.              PRINT#2,D%;

.              IF I<LEN(A%) THEN PRINT#2,CHR$(9);      'TAB-delimit

.              NEXT I

.              GOTO LOOP

.

That's it!  -- Tracy