SLIP8250 -- SLIP-CLASS (CLASS 6) PACKET DRIVER FOR DOS

Command-Line format:

SLIP8250 [-n] [-d] [-w] packet_int_no [-h] [driver_class] [int_no]

[io_addr] [baud_rate] [recv_buf_size]

Note: All parameters are given on one line, in the order shown.  Trailing

arguments (after packet_int_no) can be omitted.  The defaults are:

driver_class  = SLIP (Class 6)

int_no        = 4 (for IRQ 4)

io_addr       = 03f8h (for COM1)

baud_rate     = Whatever the port is presently set to

recv_buf_size = 3000

The driver_class should be SLIP, KISS, AX.25, or a number.  A 3000-byte

receive buffer is about twice as large as most programs need, but

experimenting is ok.

-h enables hardware handshaking (RTS/CTS) (recommended!).  Software flow

control (Xon/Xoff) is not available.

Run SLIP8250 from the DOS prompt, or from your AUTOEXEC.BAT file.  Example:

slip8250 0x60 SLIP 4 0x3f8 19200 3000

(End of SLIP8250.DOC)