K E R M I T - 1 1   f o r   R T - 1 1 / T S X - P L U S

I N S T A L L A T I O N   a n d   R E L E A S E   N O T E S

V  0  3 . 6  3        2 7 - S E P - 1 9 9 7

KRTTSX is  a  special  version  of  Kermit-11 (PDP-11 Kermit written by Brian

Nelson of the University of Toledo) enhanced for use with TSX-Plus.  This was

begun primarily to support its use on a public bulletin board system and then

was expanded to produce a  reliable,  general purpose communications program.

Numerous bugs have been fixed and several  new  routines  added.  All changes

are documented in the edit history in KRTCMD.MAC,  or may be read from within

Kermit by HELP Release-Notes.  Please note not only has KRTTSX's on-line HELP

text been greatly enhanced and expanded, lookups in it now execute at maximum

speed, thus this document is rather short.  Use HELP for operating info.  You

may use HPRINT to generate hard copy of anything needed for handy reference.

For an in-depth discussion of Kermit fundamentals there's no substitute for:

"Kermit, A File Transfer Protocol" by Frank da Cruz, 1987

Digital Press, order number: EY-6705E-DP  (in USA phone: 800 343-8321)

DP ISBN 0-932376-88-6, PH ISBN 0-13-514753-0

This Kermit does not yet offer character set translation, labeled file types,

sliding windows, a script programming language or command macros, but it does

provide everything else one would expect after reading the above.

This program is for use with RT-11 V4.0 or TSX-Plus V5.0 and up, though a few

new features use EMTs added in slightly later releases.   KRTTSX runs what it

can with earlier releases of either RT-11 or TSX.   Debugging was done on TSX

V6.5 and RT-11 V5.5, under which its operation has been thoroughly tested and

is extremely reliable.  All RT-11 V5.6 and above monitors are also supported,

including separate I&D space.

Thanks for some of the ideas, additions, fixes and above all, bugs eliminated

in this program are due to Tim Clarke, Tom McCoy, Bill Walker, Roger Wallace,

Jim Whitlock and Chester Wilson.  Additionally,  appreciation is owed all who

contributed  to  VMS-Kermit  and C-Kermit, from which many other new features

were derived, and to Brian Nelson, author of the original Kermit-11.  Some of

the most  obscure  yet  serious  bugs  were  discovered by Bill Walker, Roger

Wallace and Jim Whitlock, without  whose  help this Kermit would hardly be as

bullet-proof as it now is.

Billy Youdelman				billy@MIX.COM

PO Box 1207

Culver City, California  90232-1207

USA

Installation and Release Notes	KRT V03.63			Page Two

T A B L E   O F   C O N T E N T S

---------   ---   ---------------

I N S T A L L A T I O N				Three

K R T . I N I   E x a m p l e s			Five

R T - 1 1   S p e c i f i c   I n f o		Six

A   "K e r m i t   M o d e m"   H a n d l e r	Eight

T S X - P l u s   S p e c i f i c   I n f o	Ten

M O D E M S					Thirteen

Anchor Automation MK24			Fourteen

Telebit T2500/T3000			Fifteen

US Robotics "Courier" 2400/2400e	Seventeen

USER-DEFINED MODEM			Eighteen

SET DIAL Commands		TwentyOne

R E S T R I C T I O N S				TwentyNine

O b t a i n i n g   U p g r a d e s		TwentyNine

F i l e s   R e q u i r e d			Thirty

Installation and Release Notes	KRT V03.63			Page Three

I N S T A L L A T I O N

-----------------------

This program will run under RT-11 or TSX-Plus.  The system clock must be

running.  As  distributed  KRT  is built to run on a non-EIS CPU, KRTTSX

is built for EIS processors.  These monitors and handlers are supported:

RT-11SJ   V5.0 + up   TT  KM  XL	; SYSGEN for TIMER support

RT-11FB   V4.0 + up   TT  KM  XL	; SYSGEN for..

RT-11XM   V5.1 + up   TT  KM  XC  XL	;  ..SPCPS support

RT-11 all V5.6 + up   TT  KM  XC  XL	; including separate I&D space

TSX-Plus  V5.0 + up   TT  CL  KM  XL	; see HELP INSTALL TSX

NOTE:  XL first appeared in RT-11 V5.1.  Prior releases must use either

TT (the console terminal port) or KM (a "Kermit Modem" handler supplied

with this program) for communicating with the outside world.

KRTTSX.SAV uses extended memory overlays for XB,XM,ZB,ZM,TSX and KRT.SAV

low memory overlays for  FB,SB,SJ,TSX  to accommodate small systems.  FB

is the ONLY RT-11 V4 monitor which can run this program.  VBGEXE or VRUN

may be used under mapped RT-11 monitors.  It's best to load any handlers

required first, even for a background task.  KRT's maximum packet-length

is 600., KRTTSX's is 1920., this is determined by the NONEIS conditional

code for MAXLNG in KRTMAC.MAC.

KRTTSX requires 166kb of memory to load, KRT only needs 41kb.  Under TSX

it's 172kb and 47kb for V6.5 or 171kb and 46kb with V6.4.  KRT increases

disk I/O as its overlays are not kept resident.  When none of these will

load KRTMIN (sometimes distributed separately) is a special version that

uses less memory by only communicating through the console terminal port

(see KRTMIN.DOC for details).

If you already have some form of Kermit on your system, you may try this

one out by copying to a disk containing no other Kermit files, assigning

that disk DK, deassigning HLP, INI and KRT, and running the new program.

Assuming LD0 is a scratch disk and you have an 8" floppy:

.COP DY0:*.(SAV,HLP,INI) LD0:

.ASS LD0 DK

.DEA HLP

.DEA INI

.DEA KRT

.SET USR SWAP	! if running RT-11

.RU KRTTSX	! use KRT if KRTTSX won't load

To dial out, edit the copy  of  KRT.INI  on LD0 as needed for your site.

It is not necessary to use a command file for such things as attaching a

CL unit to a TSX line before running this Kermit - everything may now be

easily handled during its initialization.  At this point it is suggested

you use the running Kermit to read through its own HELP,  especially the

SET LINE and SET CL topics.  If you opt to use any sort of indirect .COM

file to actually run  KRT/KRTTSX,  please  note that under TSX using the

^>R KRT.. option to force command file input to the running program will

cause the test for TT input during the strike any key abort when dialing

to hang and you won't be able to dial out from the command file.  Kermit

does process indirect command (take) files as well as arguments from the

command line used to run it, either of which may be used instead of ^>R:

Installation and Release Notes	KRT V03.63			Page Four

.KRTTSX DIAL TOMMY  ! dials a phone number defined as "tommy"

.KRTTSX TAKE TOMMY  ! runs the indirect command file tommy.com

You may also have to force arguments for the program to it when run from

a command file under TSX:

R KRTTSX ^1 ^2 ^3   ! as needed, up to 6 args

There is an additional problem when forcing command line input to Kermit

in that comment delimiters "!" and ";" are processed as normal chars and

will cause errors.  This allows them to be passed to a remote system for

RSX/VMS version numbers and the like.  They are treated as delimiters in

Kermit's internal command file processing, thus the best procedure is to

dump the "^>R KERMIT" line from such files and treat them as Kermit take

files.  If typing in arguments when starting Kermit is a problem one may

use a separate command file run from the monitor containing only:

R KRTTSX TAKE TOMMY

This allows tommy.com to contain comments without crashing when run.

If it's decided to permanently install this program all old Kermit files

(K*.HLP, K*.INI, K*.SAV, etc) should be backed up then replaced by those

from this package.  Dialing from a command file (if in use) will need to

be treated per the above.

After ensuring RT-11 or TSX requirements are met patch KRT.INI (examples

follow) for your site, then copy these to SY:

KRTTSX.SAV  -  or KRT.SAV, the executable image, rename as desired

KRTHLP.HLP  -  HELP text file, may also be on HLP, KRT or DK

and this to the logical INI, KRT or a default disk of your choice:

KRT.INI  -  initialization command file

When  started  KRT/KRTTSX  will  check  INI,  KRT, DK and SY for KRT.INI.

Keeping it off SY allows others to use a different init file, and permits

the program to be easily run in the remote mode by callers to the system.

Or, one may SET LINE TT to accomplish the same thing.  For multiple users

sharing a group of lines check HELP SET CL before editing KRT.INI.

The HELP text file search (for KRTHLP.HLP) path is HLP, KRT, SY and DK.

KRTTSX.SAV/KRT.SAV  may  be  renamed as is convenient (K11 or KERMIT) but

KRT.INI  and  KRTHLP.HLP  are called (by these names) by Kermit, and thus

must not be renamed.  If the HELP text (KRTHLP.HLP) is altered in any way

Kermit must be rebuilt.  See KRTHLP.PRE for details.

Kermit is then run by its name, and an initial command line may be passed

to it as argument(s) to the KMON command:

.K11		! run Kermit

.K11 DIAL DEC	! run Kermit, dialing number set in init file

Also check the specific info for your operating system if Kermit will not

be the only task running.

Installation and Release Notes	KRT V03.63			Page Five

K R T . I N I   E x a m p l e s

- - - - - - -   - - - - - - - -

! Multiple users under TSX-Plus with possibly assorted modems

! This seeks the first available line/port from those specified

! May be installed on SY to be shared by all users, see HELP SET CL

SET CL PORTS 9/19200/T25/3 11/2400/H/6 8/2400/H/4

SET CL UNITS 2 1 0

SET CL PRIORITY 71	! don't drop chars at high speeds

SET CL LINE *	! search for first available combination of the above

! For efficiency's sake, the above may be reduced to:

!SET CL POR 9/19200/T25/3 11/2400/H/6 8/2400/H/4,UNI 2 1 0,PRI 71,LIN *

SET VLSWCH REMOTE	! emulate SET HOST/PORT if needed

... other items as required ...

! Single user under TSX-Plus

SET CL0 LINE 9		! includes "SET LINE CL0," unneeded by PRO-series

SET SPE 2400		! CL or KM only, others must do from monitor

SET PHO XMODE 4		! enable CONNECT/speed messages for fall back

SET MOD HAYES

...

! Generic RT-11 ..

SET LINE KM		! to call out..

SET FLOW-CONTROL RTS	! if preferred, must be enabled in hardware too

SET MOD HAYES

...

! System-independent items

SET LONG ON		! enable long packets

SET SL ON,KED		! turn on SL editor, enable its KED mode

SET PHONE TONE

SET PROMPT "Kermit-11> "

SET PHO NUM DECUS 1 800 521-8950	! Username: INFORMATION for info..

By default init file commands and informational responses to them are not

echoed to the screen.  After the init completes the default terminal mode

becomes NOQUIET, unless set in said file, which overrides these defaults.

If you wish to display the entire init process, make SET TT NOQ the first

line in KRT.INI.  A file may be tested by running it as a take file:

Kermit-11>@dev:KRT.INI

SHOW FILE to check the name of the init file Kermit-11 used at start-up.

Installation and Release Notes	KRT V03.63			Page Six

R T - 1 1

---------

The RT-11 FB,XB,XM,ZB,ZM monitors must include .SPCPS support if you want

^C to abort anything in the program without actually stopping the program

itself.  If KRT is not the only job running it should be FRUNed to ensure

it will be able to process incoming data from the remote system.  RT-11SB

and SJ must be sysgenned for .mrkt and .twait (TIMER) support.

If you are using RT-11 V4, the ONLY monitor which can run KRT is RT-11FB.

SET USR SWAP if it is not already there, this program needs the space.

RT-11 multi-terminal support has been pulled to keep the size reasonable.

Only a handler, or the controlling terminal line, may be used to exchange

files with the outside world.  Use XL with RT-11, XC with PRO/RT.  KM may

be used with either, it adds speed fallback and support for TSX functions

that make life with Kermit more enjoyable.

As distributed KRTTSX has just enough buffer to load the KM handler built

for a DLV11-E in a PDP-11 system.  This may be altered by the extend size

(octal number of bytes) specified in KRTLNK.COM:

krttsx,krttsx=krtxm/t/e:3010//	! APR0, the root   extending HNBUFF

WARNING:   Because it's impossible to bomb an outstanding read completion

routine once KM/XC/XL has been assigned the link the only way to deassign

it is to exit Kermit.

WARNING:   Any SET from KMON on KM/XC/XL while its interrupts are enabled

will OVERWRITE THE QUEUE and hose any data then in it.

Note XC/XL is NOT an eight-bit device thus Kermit's parity will be forced

to SPACE under RT-11.  If you have hacked it into handling 8-bits you may

SET PARITY NONE either in the init file or from the keyboard after Kermit

is running and AFTER you have SET LINE XL or XC.

HANGUP (DTR control) isn't available unless XL is from RT-11 V5.2 and up,

or the KM handler is used.

To use a handler, you must have an DL-11/DLV-11 interface (in addition to

the console interface).  The XL handler supports two DCL commands:

.SET XL CSR=n

.SET XL VECTOR=m

Where "n" is the address of the CSR (control status register)  and "m" is

the  interrupt  vector address.  The defaults are 176500 for the CSR, and

300 for the interrupt vector.  The DLV-11 speed is set with wire wrapping

and it's not settable on the XL handler.  If you have a DLV-11E XL may be

built with XL$DVE=1 and the speed set from KMON as with XC.

Installation and Release Notes	KRT V03.63			Page Seven

The XC handler is a special version of XL for the PRO/300 series, its CSR

is 173300 and vector is 210(8).  You must SET XC SPEED=nn from KMON prior

to running  Kermit  to  change XC's 1200 baud default.  You may also need

to UNLOAD XC before Kermit can be FRUN:

.UNLOAD XC

.FRUN KRTTSX.SAV/PAUSE

.LOAD XC=KRTTSX

.RESUME

Also, when running in the foreground

Kermit-11>SET TT QUIET

allows one to work in the background during file transfers.

To operate properly at high speeds (2400 and up) and with Kermits

not supporting XOFF flow control the XL or XC device handler MUST

be modified to increase the internal buffer size and repress XOFF

transmission within a packet.  This is very easy to do.

First copy XM.MAC, XL.MAC, XM.ANS, SYSGEN.COM and for the PRO/300

series XC.MAC from the RT-11  distribution  to DK, ASSIGN DK SRC,

and then in the copy of the file XL.MAC change the line:

from:	BUFSIZ	= 64.

to:	BUFSIZ	= 256.

Then IND SYSGEN and tell it to use the saved answer file XM.ANS.

When SYSGEN exits, it  will  have created, among other things, a

file called XM.CND which is needed for the driver assembly.  For

the PRO/300 series substitute XC for XL in the following command

procedure to build the handler:

.COPY SY:XLX.SYS SY:XLX.OLD		! save a copy

.REN/NOPRO SY:XLX.SYS SY:XLX.SYS	! unprotect file

.UNLOAD XL				! unload handler

.REMOVE XL				! and remove it

.MACRO/OBJ:XLX (XM,XM.CND,XL)		! assemble new

.LINK/EXE:SY:XLX.SYS XLX		! RT V4, handler on SY

.LINK/NOBIT/EXE:SY:XLX.SYS XLX		! if RT V5, do this

.INSTALL XL				! install it and

.LOAD XL				! load into memory

Last, but not least, if there is no way to get an  additional  interface

into your system (perhaps you have a four slot Q-bus backplane), you can

force Kermit to use the console.  This implies, of course, that it won't

be possible to dial out from the RT-11 system - it may only be used with

the remote Kermit connected via the console port.  Kermit-11 defaults to

TT as its link device and sets the packet length to 60 bytes.  Otherwise

the command

Kermit-11>SET LINE TT

does the same thing.  You may increase the packet size if larger packets

will work, or modify RT-11's default SPACE parity, AFTER the SET LINE is

done.

Installation and Release Notes	KRT V03.63			Page Eight

The KM Handler

KM is a "Kermit Modem" communications handler provided to support

TSX-Plus features not otherwise available under RT-11, as well as

older releases of TSX that preceded its CL handler.   KM combines

the best of both worlds, including the following under RT-11:

o  Works with DL(V), Falcon and PRO serial interfaces

o  Speed may be SET and SHOWn from within Kermit

o  Automatic fallback to the connected speed

o  An eight-bit data path

o  Selectable hardware (RTS/CTS) flow control

o  Emulation of TSX's CLCLR (flush pending I/O)

o  DTR toggling to support Kermit's HANGUP command

o  Usable with RT-11 V4.0 and up, and TSX-Plus V5.0 and up

To use KM you must first edit the appropriate conditional assembly

files for the serial line interface in your system:

KRTSJ .CND  -  for RT-11FB, RT-11SB, and RT-11SJ

KRTXM .CND  -  for RT-11XB, RT-11XM, RT-11ZB and RT-11ZM

KRTTSX.CND  -  for TSX-Plus

Then assemble and link using KRTSYS.COM if being done under RT-11

V5.0 or above, or KRTSV4.COM if using any RT-11 V4 release.  This

will create KM.SYS, KMX.SYS and KM.TSX, which should be copied to

your system device (SY) where they may be INSTALLed and LOADed as

needed.  Note that KRTKM.MAC (the handler source file) must be on

the default disk for this assembly.  Under TSX you'll have to use

TSXMOD to add a definition for KM as shown below, or define it in

TSGEN.MAC and rebuild the system.  In either case you must reboot

as it only loads handlers when started.  KM may be mapped to save

low memory:

DEVDEF	<KM>,MAPH

To build KM under RT-11 V5 (KRTSYS.COM):

MAC/OBJ:KM KRTSJ.CND+KRTKM

LINK/NOBIT/EXE:KM.SYS KM

MAC/OBJ:KMX KRTXM.CND+KRTKM

LINK/NOBIT/EXE:KMX.SYS KMX

MAC/OBJ:KMTSX KRTTSX.CND+KRTKM

LINK/NOBIT/EXE:KM.TSX KMTSX

To build KM under RT-11 V4 (KRTSV4.COM):

MAC/OBJ:KM KRTSJ.CND+KRTKM	! KRT will only run under RT-11FB if V4

LINK/EXE:KM.SYS KM

MAC/OBJ:KMTSX KMTSX.CND+KRTKM

LINK/EXE:KM.TSX KMTSX

NOTE:  Comments must be stripped to run these command files under RT-11.

Installation and Release Notes	KRT V03.63			Page Nine

KM supports the following SET commands from the monitor (KMON):

SET KM CSR=octal-address	! except on the PRO for

SET KM VECTOR=octal-address	! which these are fixed

Hardware flow control REQUIRES pins 4 and 5 be carried through in

the modem cable, 4 to 4 (RTS) and 5 to 5 (CTS), DTE (the port) to

DCE (the modem), in addition to those otherwise normally present.

The port should be set up for 8 data bits and NO parity as Kermit

does parity in software.

WARNING:  If the interface provides selectable interrupt priority

and your max speed will be greater than 4800 you will likely have

to use BIRQ 5 to avoid dropping chars.

NOTE:  This handler does NOT support VTCOM, nor is it intended to

be used with anything other than KRT V03.62 or future releases.

WARNING:  Because it's  impossible  to  bomb  an outstanding read

completion routine once KM has been assigned the  link  the  only

way to deassign KM is to exit Kermit.

See the top of KRTKM.MAC for detailed installation instructions.

Installation and Release Notes	KRT V03.63			Page Ten

T S X - P l u s

---------------

One may communicate via TT when calling in,  and  use CL0 to CL7, KM or XL

when calling out.  The CL handler is suggested as it's more efficient with

TSX-Plus.  See HELP SET CL for details.   On PRO/300 series systems Kermit

will try to assign CL0 or CL1 to TSX Line #3 at startup.

If there is any chance of swapping on your system use TSX's MEMLOCK option

when running the program.  This is easily done via the INSTALL facility.

WARNING:  If you build XL with XL$DVE (set speed) enabled be advised under

TSX-Plus (as of V6.5) the KMON command SET XL SPEED=nnnn will modify the

hardware port which XL is serving instead of the handler itself and thus

fail to produce the expected result.  And regardless, if you must use XL

see HELP INSTALL RT-11 regarding this handler's limitations.  Mapping it

is recommended to save low memory.

KRTTSX uses  208.  blocks  in the  TSX  PLAS  swap file.  The TSGEN/TSXMOD

parameter  determining  its size is SEGBLK.  If modified, TSX must then be

restarted to create the new swap file.

In the CONNECT mode 11.  activation  characters  are declared so the TSGEN

parameter MXSPAC must be => 11.

Under TSX one must SET IO ABORT so errors won't cause waits for the I/O to

rundown.  In  TSGEN  this may be accomplished by making IOABT = 1.  It may

also be done as a keyboard command.

XOFF operation without dropping characters in the CONNECT mode at 2400 and

up may require enlarging TSX's silo and input buffer on lines to which the

CL units are attached.

BUFSIZ	1024.,1024.

SILO	255.,64.,16.

in the LINDEF does the job though this is a bit big if 2400 is the fastest

speed used.  Kermit also can raise its priority when actually talking to a

CL unit so as not to drop chars at high speeds, see HELP SET CL.

The above also applies to file transfers via the controlling terminal line

(TT).   When sending to a TSX system you may also need to set its priority

below any other interactive job and use TSX's /HIGHEFF switch when running

Kermit-11 to avoid excessive degradation of response time for other users.

This is due to the overhead involved in processing input by TSX's terminal

handler, and is not a problem when sending output from a TSX system.  Note

this only concerns I/O via TT - comm handlers (CL, XL, etc) are no problem

in either direction.

Binary file transfers  via  TT  might require you to SET TT 8BIT.  You may

also need to SET TT BITS=8 but this is a hardware function and thus should

ONLY be done from a primary line first, before any subprocess that may run

Kermit is spawned.  Setting this via a subprocess may write garbage in the

line-parameters word.

TERMINAL privilege is required to assign a CL unit to a time-sharing line.

Modem control is an extremely significant system security issue under TSX.

The problem is if a dial-in user's phone connection fails that job must be

Installation and Release Notes	KRT V03.63			Page Eleven

killed before a following call can be answered and connected to it, and to

the still-logged-on session of the previous caller.

Beginning with V6.4 TSX was given improved modem control, with which these

"tailgating" incidents are easily prevented.  The following parameters are

suggested for MNP/V.42 auto-answer operation (modem time-out = 40 secs):

.SH MODEM

PHONE=1    Other modem control parameters (in 0.5 sec):

ONTIM=80 OFFTIM=1 TIMIN=80 TIMLOC=1 TIMOUT=1

When no error correction is in use, ONTIM and TIMIN may be halved, and the

modem's wait time for carrier (S7 register in Hayes-style units) set to 20

seconds.  If the modem's wait time is not settable such that it's restored

when reset (ala ATZ), then ONTIM and TIMIN must be set to match the modem.

OFFTIM, TIMLOC and TIMOUT must = 1 (the minimum, don't use zero) to ensure

the job is logged off immediately.  The $PHONE flag must be defined in the

LINDEF (in TSGEN.MAC) for any line connected to an auto-answer modem.

One then includes this in the start-up command files for "phone" lines:

SET ERROR FATAL

!ACCESS SY:CLEAR.COM/READ		! add if other access limited here

SET LOGOFF FILE=SY:CLEAR.COM		! add this..

SET TT 8BIT,BITS=8			! ..and this for Kermit

...					! other necessary items

R LOGON

and CLEAR.COM contains:

SET ERROR NONE

SET PRIO 70				! be sure this runs before next call

SET TT NOPAGE,NODTR,PHONE,AUTO,XON

...					! anything else needed should follow

This may seem a bit extreme on the SET TT end of things, but it definitely

gets the job done, even after in and out buffers are overflowed, XOFF'd in

both directions (deadlocked) and the caller has to hang up.  The rationale

for the various options used above is:

NOPAGE	ensures possible noise induced XOFF on DTR drop is harmless

NODTR	resets the modem

PHONE	ensures port is not in local mode, just in case..

AUTO	ensures port not stuck at any particular speed

XON	resets any XOFF which may have snuck through..

Note that TERMINAL privilege is required for the above to drop DTR.  As of

TSX V6.5 installing a .COM file with TERM privilege doesn't work, the user

must be given it or DTR will not be dropped and the phone line may not get

hung up.

If your version of TSX-Plus is earlier than 6.4 and you must allow dial-in

operation there is a program named NOTAIL which runs as a detached job and

provides the same protection against tailgating.  See KRTAIL.MAC, included

with this Kermit's distribution.

Installation and Release Notes	KRT V03.63			Page Twelve

It's also a good idea when using a Hayes-style modem to disable its escape

character means of invoking the modem's command mode.  If one phones in to

a TSX system and then uses another line on it to call out or talks to some

one such that the other person's data are going into the modem (the one at

the TSX system, to which your local system is connected) via its DCE port,

there's no other way to prevent someone from using the escape sequence and

taking control of that modem, after which they've got your phone line..

Installation and Release Notes	KRT V03.63			Page Thirteen

M O D E M S

-----------

Generally, connecting a modem requires an appropriate interface such

as a DL-11E, which supports DTR and software speed selection.  It is

possible to get by without such things, however (and particularly on

TSX) this may pose a security risk in that the system has no control

signals with which to log out a disconnected job.

Keeping the above in mind, note that if your modem requires  DTR  to

be present, you must use an interface that asserts it, force it high

internally in the modem, or build a cable to force it high.  Without

an appropriate interface HANGUP will not function and TSX is open to

serious security problems if incoming calls are allowed.

WARNING:  If the interface provides selectable interrupt priority

and your max speed will be greater than 4800 you will likely have

to use BIRQ 5 to avoid dropping chars.  The port should be set up

for 8 data bits and NO parity as Kermit does parity in software.

The modem cable must at a minimum carry through pins 2 and 3 (data),

7 (ground), 8 (DCD) and 20 (DTR).  Under TSX the modem may be set to

answer the line (pin 22, the ring indicator, must also be connected)

then when Kermit is started if a CL unit is attached to the line and

a modem set Kermit retains control of it all until dropping the line

or exiting the program,  when the modem and line will be restored to

their idle (auto-answer) state.

The SET MODEM command is used to establish the type of modem you are

using for the DIAL command.  Enter SET MODEM ? for a current list of

of pre-defined modem types.  See HELP SET DIAL for information about

entering a USER-DEFINED modem structure.

This Kermit is capable of matching the DTE (system) speed to that of

the DCE (modem) in response to a message by the modem (CONNECT nnnn)

only if using the CL or KM handler.  Currently the generic Hayes and

Telebit T2500/T3000 modem definitions support this.  Under XL/XC the

speed can't be changed after Kermit is running, it must be done from

the monitor first otherwise the queue will be lost.  The DCE must be

locked to the same speed, which must be at least the highest desired

carrier speed.  The MNP and V.42 versions of the above support this.

If this is not suitable, a USER-DEFINED modem structure may be used.

Be aware that file transfers FROM this Kermit via a modem who's DTE

speed is greater than the connected speed (with the distant system)

will require XOFF flow control, and servicing the interrupts caused

by it may substantially increase the cpu's loading.  If the maximum

connected speed possible is 2400 or less, dropping the DTE speed to

9600 (when same must be locked for MNP/V.42) is a good idea.  Using

the KM handler's hardware flow control is another option.

NOTE:  This program toggles DTR to reset the modem thus this option

must be enabled as part of the modem's default configuration.

The following examples have been taken from actual daily use with

this program to both dial in and out under TSX-Plus.  Note the S7

register (wait time to connect) is set to 255 when Kermit inits a

modem to dial out so Kermit's SET DIAL TIME-OUT takes precedence.

Installation and Release Notes	KRT V03.63			Page Fourteen

Anchor Automation "Signalman Lightning 24" Model MK24:

CONFIGURATION

S00=002    S01=000    S02=128    S03=013  NOTE: S0=2 here because this

S04=010    S05=008    S06=002    S07=020  modem occasionally glitches

S08=002    S09=006    S10=005    S11=040  DCD when answering the first

S12=050    S13=000    S14=044    S15=000  ring, which TSX will catch &

S16=000    S17=000    S18=000    S19=000  then hang up on when it's not

S20=000    S21=016    S22=118    S23=054  there 0.5 sec later..

S24=000    S25=005    S26=000    S27=000

S28=000    S29=000    S30=000    S31=255

S32=001    S33=000    S34=000    S35=001

B=0  Y=0  &M=0   &D=2   &C=1  &J=0  &T=0

X=4  M=1   F=1    E=0    Q=1   V=1

DIAL=TONE    SPEED=2400     PARITY=NONE

The Anchor modem will save all of the above, including its "S" registers

in RAM (backed up with a lithium battery), via AT&W.  Then one need only

send ATZ to restore its auto-answer operation, which this Kermit will do

when exiting.  SET PHONE XMODE 4 for all result codes, and use the HAYES

modem type (SET MODEM HAYES).  Note that because this modem restores its

time-out (S7) when reset it is set to 20 secs for answering which allows

for a more timely response following a failed call.  If using MNP modems

on the same system, S7 should be set to 40 seconds.  If you have the MNP

model of this modem, stick with the HAYES definition as there is not any

provision for locking the DTE speed.

Configuration switches:

1 - as req'd, ON for RJ-12/13 busy support

2 - ON   DCD/DSR follow detected carrier signal

3 - ON   DTR from the computer controls modem (manual says OFF, in error..)

4 - OFF  remote digital loopback

5 - OFF  enables command recognition by modem

6 - OFF  no CTS support

7 - unused

8 - unused

Installation and Release Notes	KRT V03.63			Page Fifteen

Telebit T2500/T3000 Modems

Three Telebit T2500 definitions are provided.  T2500 is intended for

use without MNP or V.42 LAP-M error correction.  It uses the modem's

connect messages to match the DTE speed to that of the data path and

that requires using either the CL or KM handler.  Set up the T2500's

default profile in the "enhanced" command mode as shown below:

E0 F1 M0 Q3 T V1 W1 X3 Y0 &P0 &T5     Version GF7.00-T2500SA

S00:001 S01=000 S02:128 S03=013 S04=010 S05=008 S06=002 S07=040 S08=002

S09=006

S10=007 S11:050 S12=050 S18=000 S25=005 S26=000 S38=000

S41=000 S45=000 S47=004 S48:001 S49=000

S50=000 S51=255 S52:002 S54:002 S55=000 S56=017 S57=019 S58=003 S59=000

S61=150 S62=003 S63=001 S64=000 S65=000 S66=000 S67=000 S68=255 S69=000

S90=000 S91=000 S92=000 S93=008 S94=001 S95=000 S96=001 S97=000 S98=003

S100=000 S101=000 S102=000 S104=000 S105=001 S106=000 S107=020

S110=255 S111=255 S112=001

S121=000 S130=002 S131:001

S150=000 S151=004 S152=001 S153=001 S154=000 S155=000 S157=000 S158=000

S160=010 S161=020 S162=002 S163=003 S164=007 S169=000 S255=000

Then, when MNP operation is desired, set the DTE port (via SET SPEED

if using TSX's CL handler) to 9600 or 19.2kb if your cpu can keep up

with it, then SET MODEM T2500MNP.  This enables MNP only as attempts

at a V.42 connection first with some MNP modems appear to leave them

in a state where nothing following works.  When an MNP link can't be

established operation falls back to the normal mode.

V.42 and LAP-M facilities are enabled by setting the modem T2500V.42

which operation is just like that of the MNP only above, except V.42

is tried first, then MNP, and then normal operation if an error free

path can not be established.  Negotiation of these protocols can add

considerably to the time needed to connect.

All of this should be done on the same  default  profile with which

the modem will  automatically  answer  incoming calls to ensure the

modem will be properly reset to  its auto-answer state when exiting

Kermit-11 to the monitor.  The "enhanced" command mode is suggested

because the "conventional" mode does not allow XOFF flow control in

the command mode.  While this is not normally a problem with result

messages it does make listing the modem's status impossible at high

speeds.

Installation and Release Notes	KRT V03.63			Page Sixteen

Under RT-11 either the T2500MNP to T2500V.42 definitions may be used

to work around XL/XC's inability to modify its speed as is necessary

for the T2500.  Both will connect normally if error correction isn't

supported by the remote system.  The KM handler may also be used.

SET PHONE XMODE 3 to enable all possible result messages, or use 14

if you prefer to force display of the carrier speed and protocol in

all cases, even when an error correcting path is not established.

If by accident the modem is sent something at an unwanted speed and

becomes stuck there, toggle DTR (HANGUP or SET DTR) to reset it.

If you are using PEP/MNP/V.42 Telebit's Kermit protocol spoofing is

the fastest of all possible options even though it only uses normal

94 byte packets between modems.  It is set in the above to run when

requested by the other end.  To force it on first SHO LINE to check

the DTE speed (if a fallback speed is shown then one must first SET

SPEED 9600 or 19200 if using a mode where the DTE speed is locked),

then CONNECT and issue the command ATS111=n, when "n" is one of:

0    No protocol supported

10   Kermit protocol with no parity

11   Kermit and odd parity

12   Kermit and even parity

13   Kermit and mark parity

14   Kermit and space parity

20   XMODEM/YMODEM protocol

255  Use protocol specified by other end

Then, disable long packets (SET NOLONG will suffice), so the modems

run most efficiently, and proceed with the transaction.

Telebit T3000

Three Telebit T3000 definitions are provided.  T3000 is for disabling

MNP/V.42 error correction.  It checks the modem's connect messages to

match the DTE speed to that of the carrier.  T3000V.42 enables V.42 &

MNP modes and locks the speed.  Set up the T3000's defaults as listed

below.  This definition  (T3000V.42)  is for use with hardware  (RTS)

flow control.  This is ONLY available under the KM handler, where you

have an option to SET FLOW-CONTROL RTS before setting the modem-type.

WorldBlazer - SA - Version LA5.00W- Configuration A

B1  E0  L1  M0  Q1  T   V1  X12  Y0

&C1 &D2 &G0 &J0 &L0 &Q0 &R3 &S0 &T5 &X0

S000=0   S001=0   S002:0   S003=13  S004=10  S005=8   S006=2   S007:255

S008=2   S009=6   S010=14  S011=70  S012=50  S018=0   S025=5   S026=1

S038=0   S041=0   S045=0   S046=0   S047=4   S048:1   S050=0   S051=255

S056=17  S057=19  S058:2   S059:15  S060=0   S061:0   S062=15  S063:1

S064=0   S068=255 S069=0   S090=0   S092=0   S093=8   S094=1   S100=0

S104=0   S105=1   S111=255 S112=1   S151=4   S155=0   S180=2   S181=1

S183=25  S190=1   S191=7   S253=10

Installation and Release Notes	KRT V03.63			Page Seventeen

The T3000 does NOT do XOFF flow control in the command mode even when

it's enabled and functioning for data between modems unless S48=0 and

this of course kills an 8-bit data path.  A third modem definition is

provided (T3000X.42) which enables XOFF rtesraint - Kermit quotes the

8-bit counterparts of XOFF and XON so this will only be a problem for

binary data transfers using the Xmodem protocol.  This defintion will

force S48 to 0 and enable XOFF restraint when initializing the modem.

US Robotics "Courier" 2400/2400e

C1  E0  F1  M1  Q1  V1  X6

BAUD=02400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK  TIMER

&A1  &B0  &H0  &I0  &M0

&N0  &R1  &S0  &Y1

S00=001  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=032  S14=252  S15=000

S16=000  S17=000  S18=000  S19=000

S20=000

Configuration switches:

1 - UP    DTR controls modem

2 - UP    verbal result messages

3 - UP    quiet mode, for auto-answer

4 - DOWN  don't echo local commands, for auto-answer

5 - UP    enable auto-answer, modem answers on the first ring

6 - UP    DCD on pin 8 follows action of received carrier

7 - as req'd, DOWN for RJ-12/13 busy support

8 - UP    to recognize "AT" command set

9 - UP    drop phone connection when escape code received

10 - UP    load power-on defaults from NVRAM

QUAD - as req'd for your system and modem cable..

Use the Hayes modem type (SET MODEM HAYES).  Set the "&" registers per

the above and save them with AT&W.  The "S" registers aren't preserved

so ONTIM and TIMIN must = 60. (TSGEN.MAC)  if incoming calls are being

accepted (auto-answer enabled) under TSX-Plus.  If wish to use the MNP

version, lock the DTE speed as desired to at least 4800 (AT&B5&W) then

SET MODEM Hayes-MNP.

US Robotics modems such as the Courier 2400 or 2400e will occasionally

glitch when answering the phone and start a TSX port at 19.2kb instead

of 2400.  This is more likely to happen when the modem is not properly

cooled.  Replacing its rubber feet with some that hold it higher above

the surface on which it resides may help.  Setting the modem to answer

after the first ring (that is, on the second) may also help.  Starting

with TSX V6.5, 2400 is tried first, before 19200, which also may help.

SET PHONE XMODE 6 enables all possible result messages from the modem.

Installation and Release Notes	KRT V03.63			Page Eighteen

U S E R - D E F I N E D   M O D E M

-----------------------   ---------

Kermit-11 contains operating parameters for several of the more common

auto-dial modems.  To find out if your modem is directly supported use

the SET MODEM ? command.  If it is not, you'll need to use SET DIAL to

load the data used to control the modem and implement the various DIAL

routines.  These involve formatting commands to the modem, and what to

expect in the way of modem responses.  As an example the Vadic VA212PA

modem is awakened from an idle state by the character sequence (octal)

005 015

which is Ctrl-E followed by carriage return.  The modem responds with

HELLO: I'M READY

*

Thus Kermit needs to know when it sends the wakeup sequence it should

wait for the asterisk to be returned by the modem, after which it may

presumed the modem is awaiting further commands, such as that to dial

a phone number.

Suppose we had to tell Kermit about the Racal Vadic VA212PA (though in

reality Kermit already knows this modem).  Checking the owner's manual

for it, we find that:

To wake the modem up, we type a Ctrl-E followed by a return.

To dial a number, we type the letter D  followed by a return.  At

this point, the modem prints a NUMBER? prompt and we then type in

the desired number.  The number  is  then reprinted and the modem

waits for a return from us to confirm it's correct.

When it completes dialing it will print "ON LINE" or "ONLINE" for

a successful call, otherwise it may return "BUSY", "FAILED CALL",

"NO DIAL",  "VOICE"  or  "TIME OUT".  While it is waiting for its

call to be answered it may print the line "RINGING" several times

to tell you it is working on the call.

The Kermit commands required would be:

Kermit-11>SET DIAL WAKE-STRING \005\015

Kermit-11>SET DIAL ECHO

Kermit-11>SET DIAL WAKE-ACK *

Kermit-11>SET DIAL INITIATE D\015

Kermit-11>SET DIAL FORMAT %P%S\015

Kermit-11>SET DIAL DIAL-PAUSE 9K

Kermit-11>SET DIAL CONFIRM \015

Kermit-11>SET DIAL SUCCESS ONLINE

Kermit-11>SET DIAL SUCCESS ON LINE

Kermit-11>SET DIAL RINGING RINGING

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE FAILED CALL

Kermit-11>SET DIAL FAILURE NO DIAL

Kermit-11>SET DIAL FAILURE TIME OUT

Kermit-11>SET DIAL FAILURE VOICE

Installation and Release Notes	KRT V03.63			Page Nineteen

The notation "\005\015" indicates a Ctrl-E followed by a return.  5 is

octal for Ctrl-E,  15 is octal for return.   Octal numbers may also be

specified by placing the value inside of  inequality  characters as in

SET DIAL WAKE <005><015> though the former is preferred.  Octal values

may be obtained from HELP ASCII.  If for any reason you need to pass a

"\" or "<" to your modem, simply prefix the character with another "\"

or "<" as in "\\" or "<<".

"%P%S\015" indicates the phone number from the  DIAL  command is to be

followed by a return.  "%S" is  a format effector for the phone number

string itself,  which causes it to be inserted in the dial string sent

to the modem.   "%P" inserts the dial pause string, as in this case we

need to dial 9 and wait for a second dial tone.  The "K" is the  Racal

Vadic  code to get the modem to pause.  If you are dialing on a direct

line or your modem supports pausing via something embedded in the dial

string itself, the DIAL-PAUSE command is unneeded.

Many modems require only WAKEUP, WAKE-ACK, FORMAT and result strings.

The Digital DF112 is a good example.  Its definition would look like:

Kermit-11>SET DIAL WAKEUP \002

Kermit-11>SET DIAL WAKE-ACK READY

Kermit-11>SET DIAL FORMAT %S#

Kermit-11>SET DIAL SUCCESS ATTACHED

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE DISCONNECTED

Kermit-11>SET DIAL FAILURE ERROR

Kermit-11>SET DIAL FAILURE NO ANSWER

Any undefined result string is considered informational only thus this

class of response does not need to be expressly defined.

If you are using a "Hayes" style modem, its extended response mode may

be set by using the %X format effector in the WAKE-STRING.  The number

desired is then specified by SET PHONE XMODE n, where n is 0 to 14, or

"OFF" to disable it.  %X may also be used in the FORMAT (dial) string,

allowing modification without re-initializing the modem.

Here's a sample Hayes definition:

Kermit-11>SET PHONE TONE,XMODE 4

Kermit-11>SET DIAL COMMENT Generic Hayes Command Set

Kermit-11>SET DIAL TIME-OUT 45

Kermit-11>SET DIAL WAKE-ACK OK

Kermit-11>SET DIAL WAKE-STRING AT E1 Q0 V1 S0=0 S7=255 %X \015

Kermit-11>SET DIAL ECHO

Kermit-11>SET DIAL INIT-ONCE

Kermit-11>SET DIAL FORMAT AT %X %A D%M %S \015

Kermit-11>SET DIAL ABORT \015

Kermit-11>SET DIAL CON19200 CONNECT 19200

Kermit-11>SET DIAL CON19200 CONNECT 14400

Kermit-11>SET DIAL CON9600 CONNECT 9600    ; fallback only works with

Kermit-11>SET DIAL CON4800 CONNECT 4800    ; the CL or KM handlers..

Kermit-11>SET DIAL CON2400 CONNECT 2400

Kermit-11>SET DIAL CON1200 CONNECT 1200

Kermit-11>SET DIAL CON9600 CONNECT FAST

Kermit-11>SET DIAL CON300 CONNECT	   ; embedded, must be last!

Installation and Release Notes	KRT V03.63			Page Twenty

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE ERROR

Kermit-11>SET DIAL FAILURE NO ANSWER

Kermit-11>SET DIAL FAILURE NO CARRIER

Kermit-11>SET DIAL FAILURE NO DIALTONE

Kermit-11>SET DIAL FAILURE NO DIAL TONE

Kermit-11>SET DIAL FAILURE VOICE

Kermit-11>SET DIAL RINGING RING		; includes RINGING

Kermit-11>SET DIAL RINGING RRING	; Telebit style..

Kermit-11>SET DIAL PULSE P

Kermit-11>SET DIAL TONE T

Kermit-11>SET DIAL ANSWER S0=1

Kermit-11>SET DIAL NOANSWER S0=0

Kermit-11>SET DIAL IDLE AT Z \015

Some modems are unable to accept data at the line speed, in which case

one must employ SET DIAL WAKE-RATE and SET DIAL DIAL-RATE.   These two

functions accept a delay time in clock ticks,  which  are 1/50 or 1/60

second units depending on where in the world this program is running.

If your modem has a settable timer it should be either disabled or set

to the maximum possible (example: the Hayes wakeup string contains S7=

255) to allow Kermit-11's internal TIME-OUT to prevail.

If your modem hangs, see HELP SET DIAL SETTLE-TIME.  This and TIME-OUT

are two special parameters here in that they are global, and used even

with Kermit's internal modem definitions.  However, default values are

loaded by SET MODEM, thus these must be altered AFTERWARDS, except for

the USER-DEFINED modem where what is set BECOMES the default.  This is

also the case for SET PHONE XMODE.

NOTE:  This program toggles DTR to reset the modem thus the modem must

have this option enabled in its default configuration.

If the modem echoes command strings sent to it SET DIAL ECHO to enable

display of anything output by it.

Once all pertinent data have been entered, it is then necessary to SET

Kermit's modem-type:

Kermit-11>SET MODEM USER-DEFINED

If a line (and speed if necessary) has been previously set this causes

Kermit to try initializing the modem, the result of which is reported.

If successful, you may then use the DIAL command to call out.

SET DIAL -- Additional information is available on:

ABORT             ANSWER            BINARY-RESPONSE   BLIND

COMMENT           CON300            CON1200           CON2400

CON4800           CON9600           CON19200          CON38400

CONFIRM           CONFIRM-ACKNOWLEDGE                 DIAL-ACKNOWLEDGE

DIAL-PAUSE        DIAL-RATE         ECHO              FAILURE

FORMAT            Format-effectors  IDLE              Important-Notes!

INITIATE          INITIATE-PROMPT   INIT-ONCE         NOANSWER

NOBINARY-RESPONSE NOECHO            NOINIT-ONCE       PULSE

RINGING           SETTLE-TIME       SUCCESS           TIME-OUT

TONE              WAKE-ACKNOWLEDGE  WAKE-RATE         WAKE-STRING

Installation and Release Notes	KRT V03.63			Page TwentyOne

SET DIAL ABORT

Defines a string to force the modem to stop dialing.  This is passed

to the modem by the strike-any-key abort function of Kermit-11 while

waiting for a dialed  call  to complete.  Non-printing chars must be

specified in octal notation.

Kermit-11>SET DIAL ABORT \003	; use ^C to stop modem

SET DIAL ANSWER

Defines a string the %A format effector will insert in the dial FORMAT

string when it contains said %A effector and you have SET PHONE ANSWER

to allow answering a call back from a security modem requiring it.

Kermit-11>SET DIAL ANSWER S0=1	; Hayes does it thusly

SHOW MODEM to display its current setting and status.

SET DIAL BINARY-RESPONSE

If your modem responds with single character result messages (sans any

terminator ala a return) set this so Kermit will as well.  The default

may be restored with SET DIAL NOBINARY-RESPONSE, which enables testing

of strings of more than one character.

SET DIAL BLIND

Defines a string to force the modem to dial regardless of its detecting

a dial tone or recognizing tone dialing capability on the line.  Actual

use of this string in dialing is enabled by SET PHONE BLIND.

Kermit-11>SET DIAL BLIND string

SET DIAL COMMENT

Specifies a brief description of the modem definition which is included

in the SHOW MODEM display.  Embedded blanks are ok, but multiple spaces

will be reduced to a single blank.

Kermit-11>SET DIAL COMMENT Hayes-MNP with DTE speed locked

SET DIAL CON300

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 300 baud and then CONNECT to it:

Kermit-11>SET DIAL CON300 CONNECT  ; Hayes defaults to 300

SET DIAL CON1200

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 1200 baud and then CONNECT to it:

Kermit-11>SET DIAL CON1200 CONNECT 1200

Installation and Release Notes	KRT V03.63			Page TwentyTwo

SET DIAL CON2400

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 2400 baud and then CONNECT to it:

Kermit-11>SET DIAL CON2400 CONNECT 2400

SET DIAL CON4800

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 4800 baud and then CONNECT to it:

Kermit-11>SET DIAL CON4800 CONNECT 4800

SET DIAL CON9600

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 9600 baud and then CONNECT to it:

Kermit-11>SET DIAL CON9600 CONNECT 9600	; Hayes

Kermit-11>SET DIAL CON9600 CONNECT FAST	; Telebit

SET DIAL CON19200

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 19,200 baud and then CONNECT to it:

Kermit-11>SET DIAL CON19200 CONNECT 19200

Kermit-11>SET DIAL CON19200 CONNECT 14400

SET DIAL CON38400

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 38,400 baud and then CONNECT to it:

Kermit-11>SET DIAL CON38400 CONNECT 38400

SET DIAL CONFIRM

Some modems use a command syntax wherein the number to be dialed is

echoed back to the user, after which the user must confirm it is ok

by returning an appropriate string:

Kermit-11>SET DIAL CONFIRM \015   ; usually just a return

SET DIAL CONFIRM-ACKNOWLEDGE

If your modem issues some sort of confirmation to the DIAL CONFIRM

sent to it, and there is  some  reason  to  test that in fact said

confirmation has actually been received by Kermit, you may specify

what to expect thusly:

Kermit-11>SET DIAL CONFIRM-ACK string

Installation and Release Notes	KRT V03.63		      Page TwentyThree

For example, older Vadic modems would say "DIALING" here.

SET DIAL DIAL-ACKNOWLEDGE

Specifies a string issued by the modem to acknowledge it is dialing.

Use of this may be  required  for  error  detection  with some older

modems, but it's normally not necessary.

Kermit-11>SET DIAL DIAL-ACK \015   ; usually just a return

SET DIAL DIAL-PAUSE

Defines the string for dial tone pause, which string is passed to the

modem by placing the %P format effector in the actual dialing string,

which in turn is defined with the SET DIAL FORMAT command.

Kermit-11>SET DIAL DIAL-PAUSE string

If you are dialing on a direct line or your modem supports pausing via

something embedded in the dial string itself the DIAL-PAUSE command is

unnecessary.  As long as the first char is numeric, commas and similar

tokens will be passed to the modem as part of the number from the DIAL

command.

SET DIAL DIAL-RATE

Allows pacing of chars in the dial string sent to the modem.  There is

an inherent wait of one clock tick (1/50 or 1/60 second, per your LTC)

to ensure an echo is not missed.  If this remains too fast for a modem

additional delay between each char may be specified in ticks:

Kermit-11>SET DIAL DIAL-RATE 3

SET DIAL ECHO

If the modem echoes command strings, SET DIAL ECHO causes the display

during DIALing to be driven directly from the modem's output.  If the

modem doesn't support this, SET DIAL NOECHO forces display of what is

sent, regardless of what may be received from the modem.

SET DIAL FAILURE

Define a message from the modem which, as a result of a dialed call,

indicates failure to complete the connection.  More than one message

may be defined if necessary to include all possibilities.

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE NO ANSWER

SET DIAL FORMAT

This defines the actual DIAL string sent to the modem.  In  addition

to specifying the entire thing verbatim several format effectors are

provided to make life easier.  Most notable of these is %S, which is

the number to dial taken from the argument to the DIAL command.  See

HELP SET DIAL Format-effectors for detailed info.

Kermit-11>SET DIAL FORMAT %P%S\015

Kermit-11>SET DIAL FORMAT AT %X D%M %S \015

Installation and Release Notes	KRT V03.63		       Page TwentyFour

Most modems will require %S be the last item in the list (other than

the return as an input terminator), as otherwise a following command

will be taken as a request to abort dialing.  If blanks are embedded

they will be passed to the modem, although contiguous blanks will be

reduced to a single space.

SET DIAL Format-effectors

FORMAT (dialing) string format effectors supported:

%A - SET PHONE [NO]ANSWER string

%B - Dial blind

%M - Mode, pulse or tone

%P - Insert pause string

%S - Phone number string, passed by DIAL command

%X - Insert Hayes xmode string, unless set off

WAKE-STRING (initialization) string format effector supported:

%X - Insert Hayes xmode string, unless set off

See  HELP SET DIAL ANSWER, NOANSWER PULSE, TONE  and  HELP SET PHONE *

for info concerning exactly what each of these does.

If you are using a "Hayes" style modem, its extended response mode may

be set into the init string via SET PHONE XMODE n, where n is a number

between 0 and 14, or the word "OFF".  The  format  effector  for XMODE

is %X.  It may also be used in the dial string, allowing modifications

without re-initializing the modem.

Strings to be inserted will be displayed in SHOW MODEM when the format

effectors (SET PH BLIND,PULSE,TONE,XMODE) for them are enabled, except

for the phone number string, which is not built until DIALing.

SET DIAL IDLE

Defines the string used to return the modem to idle state, which is

sent to the modem upon exiting Kermit-11 or dropping the connection

(ala SET LINE) to it.  If the modem was preset to auto-answer, this

restores it to that state.

Kermit-11>SET DIAL IDLE ATZ \015

SET DIAL Important-Notes!

The SUCCESS, CONnnnn, RINGING  and  FAILURE fields are built as linked

lists thus you can have as many as necessary.  At least one SUCCESS or

CONnnnn message must be defined or any call is going to fail!

NOTE: Where a modem response string (SUCCESS, CONN, FAILURE) is a sub-

string of another response string the substring MUST FOLLOW the string

containing it as the comparison scheme used here ignores the return at

the end of the result string from the modem.  Thus, when you are using

CONNECT 1200 and CONNECT, "CONNECT 1200" MUST be defined first, BEFORE

"CONNECT" on its own.  All tests are done without respect to case.

Use of CONNECT messages to conform the speed of the DCE port (the one

on the PDP-11) to that of the modem (DTE port) is only possible using

the CL (TSX-Plus) or KM (RT-11/TSX-Plus) handlers.  These are for use

Installation and Release Notes	KRT V03.63		       Page TwentyFive

with modems which alter the speed of their data port to the connected

(telephone port) speed.  The old speed will be restored when the next

call is placed, unless SET otherwise first.  See HELP SET DIAL CONnn,

where nn=300 ,1200 ,2400 ,4800 ,9600 ,19200 or 38400.

Once you have  established  a  working set of parameters for your user

defined modem,  you  may  include  them  in  KRT.INI, Kermit's startup

initialization command file.  If memory runs out before everything you

need can be entered, increase the value of ALSIZE in KRTMAC.MAC.  This

same buffer is used to store phone numbers.  Also note malloc as it is

here does not deallocate anything, thus while re-entering data updates

pointers to them the superseded data's memory is NOT recovered.

The following pseudo code was extracted from the dialer source module,

with the hope of clarifying the functions of and relationships between

the seemingly excessive number of parameters available here.  Anything

that can modified by SET DIAL is shown in UPPER CASE.  Items listed as

optional should only be used when really necessary, or performance may

not remain as fast as it could be.

format_dial_string		; build the modem's dial_command_string

using FORMAT string and

%B BLIND dial string	; if set phone blind has also been done

%M PULSE or TONE string	; set phone pulse,tone selects which

%P DIAL-PAUSE string	; used if defined

%S the_phone_number		; as passed by the dial command

%X XMODE			; depending on set phone xmode

call tty_initialize		; open the link device

call fix_speed			; restore set speed (b4speed) if need be

test INIT-ONCE			; need to re-init modem?

if <>, goto do_dial

send WAKE-STRING	; ya, send it an init string

pace WAKE-RATE ticks	; pause between chars if <>

call check_abort	; check strike any key to abort loop

wait_for WAKE-PROMPT	; now wait 2 secs for an ack

on error, loop a maximum of three times then give up

do_dial:			; try to dial a number

; these two are optional and only for modems which need them

send INITIATE		; dump the string to initiate dialing out

wait_for INITIATE-PROMPT; if defined, wait up to 4 secs for a response

send dial_command_string	; this is from FORMAT, et.al. above

pace DIAL-RATE ticks		; pause between chars if not zero

test DIAL-ECHO		; does it echo?  if not, Kermit will do it

call check_abort		; strike any key to abort loop

; these three are optional and only for modems which need them

wait_for DIAL-ACK	; wait up to 4 secs for numb to echo perhaps

send CONFIRM		; stuff a confirm

wait_for CONFIRM-ACK	; if defined, wait up to 5 secs for the string

get_response:			; get the modem's response

wait_for SUCCESS ,RINGING ,FAILED ,CON300..CON19200

test BINARY-RESPONSE		; exit loop on a single character?

call check_abort		; strike any key to abort loop

wait_until TIME-OUT secs

Installation and Release Notes	KRT V03.63			Page TwentySix

call_fails_or_was_aborted:

send ABORT			; get the modem to STOP

wait SETTLE			; let modem recover from abort!

call_succeeds:

; if under TSX-Plus with CL handler and a CONnnnn result message

call b4speed		; save speed before fallback..

call set_speed		; then set it to what modem wants it to be

jump c$connect		; and connect automatically..

SET DIAL INITIATE

Some modems use a command syntax wherein the number to be dialed must be

preceded by a command to first initiate the dial mode itself.   Normally

these modems will also require using a DIAL CONFIRM string after sending

the number string.

Kermit-11>SET DIAL INITIATE D\015

SET DIAL INITIATE-PROMPT

If after use of the DIAL INITIATE option for some obscure reason you

find it necessary to check for proper response to it from your modem

the desired string may be specified thusly:

Kermit-11>SET DIAL INITIATE-PROMPT string

SET DIAL INIT-ONCE

If once initialized your modem remains in that state (some such as the

Concord 224 will time out and return to an idle mode) you may skip any

further unnecessary and time consuming inits by calling this function.

This is tested when dialing, and will speed up the redial looping time

if your modem allows its use.

Kermit-11>SET DIAL INIT-ONCE

To restore Kermit-11's default operation where the modem is re-initted

before trying anything and everything else SET DIAL NOINIT-ONCE.

SET DIAL NOANSWER

Defines a string the %A format effector will insert in the dial FORMAT

string when it contains said %A effector and you've SET PHONE NOANSWER

to disable the modem from answering an incomming call.

Kermit-11>SET DIAL NOANSWER S0=0  ; Hayes does it thusly

SHOW MODEM to display its current setting and status.

SET DIAL NOBINARY-RESPONSE

Enables testing modem result messages consisting of strings greater than

one character.  If your modem responds via single character result codes

SET DIAL BINARY-RESPONSE so Kermit will as well.

Installation and Release Notes	KRT V03.63		      Page TwentySeven

SET DIAL NOECHO

If the modem echoes command strings, SET DIAL ECHO causes the display

during DIALing to be driven directly from the modem's output.  If the

modem doesn't echo, SET DIAL NOECHO will force Kermit to display what

is sent to the modem in place of what would be echoed back.

SET DIAL NOINIT-ONCE

If your modem, once initialized for dialing out, doesn't remain in such

a state, particularly after a dial out call fails, SET DIAL NOINIT-ONCE

will force Kermit-11 to re-initialize the modem before each dial/redial

attempt.  This is the default operation.

SET DIAL PULSE

Defines a string to force the modem to use pulse dialing regardless of its

recognizing tone dialing capability on the line.  This string is passed to

the modem by the %M format effector (which must in the DIAL FORMAT string)

when one has SET PHONE PULSE to enable it.

Kermit-11>SET DIAL PULSE P	; "Hayes" uses this

SET DIAL RINGING

Defines a ring response, as returned by the modem to report the status of

a call in progress.  Kermit-11 maintains a count of received RINGS, after

seven of which the call will be considered failed due to no reply.

Kermit-11>SET DIAL RINGING RING   ; "RING" includes "RINGING"

Multiple messages may be defined as required to cover all possibilities.

SET DIAL SETTLE-TIME

Allows modifying the delay between sending a reset string to a modem and

dumping the handler connected to it, which also drops DTR.  If the modem

doesn't recover in time to resume its auto-answer mode, increase this to

something beyond the 20 tick default by setting a new value as a decimal

number of ticks.  One tick is 1/50 or 1/60 second (hardware dependent).

Kermit-11>SET DIAL SETTLE 30

IMPORTANT NOTE:  This is a global function effective for ANY modem type,

not just USER-DEFINED, but for KRT's built-in modem types this will only

prevail when this is done AFTER setting the MODEM-TYPE.

SET DIAL SUCCESS

Define a message from the modem which, as a result of a dialed call,

indicates successful completion and a data path has been established

between modems, and which will force Kermit to CONNECT your terminal

to it.  More than one message may be defined if necessary to include

all possible responses.

Kermit-11>SET DIAL SUCCESS CONNECT

NOTE:  This particular function neither checks nor changes speed.

If this is something you need, see HELP SET DIAL Important-Notes.

Installation and Release Notes	KRT V03.63		      Page TwentyEight

SET DIAL TIME-OUT

Defines the period in seconds after which if there is no conclusive

response (excluding informational messages) from the modem the call

will be considered to have failed.

Kermit-11>SET DIAL TIME-OUT 60	; for overseas calling..

IMPORTANT NOTE:  This is a global function effective for ANY modem type,

not just USER-DEFINED, but for KRT's built-in modem types this will only

prevail when done AFTER setting the MODEM-TYPE, and the modem's internal

time out is set to something greater than the desired time period here.

SET DIAL TONE

Defines a string to enable the modem to use tone dialing regardless of its

recognizing tone dialing capability on the line.  This string is passed to

the modem by the %M format effector (which must in the DIAL FORMAT string)

when one has SET PHONE TONE to enable it.

Kermit-11>SET DIAL TONE T	; "Hayes" uses this

SET DIAL WAKE-ACKNOWLEDGE

Defines the expected response to the WAKE-STRING (initialization to dial

out) sent to the modem.  This is tested three times over a period of six

seconds, after which the init will be considered to have failed.

Kermit-11>SET DIAL WAKE-ACK OK

SET DIAL WAKE-RATE

Allows pacing of chars in the init string sent to the modem.  There is

an inherent wait of one clock tick (1/50 or 1/60 second, per your LTC)

to ensure an echo is not missed.  If this remains too fast for a modem

additional delay between each char may be specified in ticks:

Kermit-11>SET DIAL WAKE-RATE 5

SET DIAL WAKE-STRING

Defines the wakeup (initialization) string sent to the modem to place

it in the ready-to-dial-out state:

Kermit-11>SET DIAL WAKE-STRING \005\015

Kermit-11>SET DIAL WAKE-STRING AT E1 Q0 S0=0 S7=255 V1 %X \015

Installation and Release Notes	KRT V03.63		       Page TwentyNine

R E S T R I C T I O N S

-----------------------

A problem was found with versions of Kermit-11 prior to  3.49  regarding

the  sending and processing of attribute packets.  Basically, two of the

attribute types were improperly implemented.  The support for  attribute

packets  was  added  back  in April 1984, at a time when testing against

other implementations was not possible.  At this writing, in March 1986,

some problems have been found.

The corrected version of Kermit-11 is V3.49.  This version will  NOT  be

able  to exchange attribute packets with previous versions.  In order to

make an initial download of Kermit-11 V3.49 to a host running  an  older

version, you must explicitly disable attribute support, as in:

Kermit-11>SET NOATTR

and then force both ends to binary mode, as in:

Kermit-11>SET FILE TYPE BINARY

The best way to accomplish the updating of remote sites is to use a pre-

3.49 version of Kermit-11 at both sides,  transfer the image appropriate

to your system, then replace the old executable image on the remote side

and your own side and transfer the help text and other needed files.

U p g r a d e s

---------------

This special version of Kermit-11 (for RT-11 and TSX-Plus) may be from

time to time updated/upgraded, or various bugs may be found and fixed.

New versions may be obtained from kermit.columbia.edu by anonymous ftp

(kermit/b/krt*.*) or by calling the system on which the work is done:

Host:             Billy's Place, Culver City, California, USA

Available:        24 hours, 3 lines, immediate access

Phone:            +1.310.837.0892

Logon (username): 1000

Password:         moving_target

Modem speeds:     9600 (V.32) and down, or PEP (modems are T-2500s)

Protocols:        Kermit, VTCOM/TRANSF, Xmodem

Access is free other than whatever it costs to place the call.  Any bug

found will be fixed, send mail to billy@MIX.COM detailing your problem.

Also on-line here are the complete Kermit-11 T3.60 sources along with a

massive amount of RT-11 SIG and other public domain/gratis software.

Installation and Release Notes	KRT V03.63			Page Thrity

The following files comprise everything required for KRT/KRTTSX:

KRT   .BWR	"Beware" file, known bugs and possible things to come..

KRT   .COM	Links low memory overlays image, KRT.SAV

KRT   .DOC	Installation and Release Notes

KRT   .IND	Bob Schor's Kermit-11 init file manipulator, an IND process

KRT   .INI	Sample initialization file

KRT   .MAP	Link map of low memory overlays image, KRT.SAV

KRT   .SAV	Low memory overlays image, for RT-11FB,SB,SJ and TSX+

KRTAIL.MAC	Tailgating protection for TSX+ V6.31 and below

KRTASM.COM	Assembles all source modules for an EIS CPU

KRTASM.NON	Assemble all sources for a non-EIS CPU using macros in KRTNHD

KRTATR.MAC	Attribute packet processing

KRTCDF.MAC	Command macro definition .INCLUDE file

KRTCMD.MAC	Main program loop, commands calling APR4

KRTCM1.MAC	APR4 overlay for the rest of the commands

KRTCOM.MAC	Main loop commands data table

KRTCON.MAC	CONNECT terminal emulator

KRTCVT.MAC	File name conversion routines

KRTDAT.MAC	Global read/write data

KRTDEB.MAC	EXAMINE command, display memory by symbol or address

KRTDEF.MAC	Packet definition .INCLUDE file

KRTDIA.MAC	DIAL command, also SET MODEM and SHOW DIAL

KRTDIR.MAC	Directory operations

KRTDSP.MAC	I/O dispatching to handler or terminal

KRTEDI.MAC	SL command line editor

KRTERM.MAC	I/O support for the controlling terminal line (TT)

KRTERR.MAC	Error messages

KRTHIS.DOC	Narrative in-depth edit history

KRTHLP.HLP	HELP text file

KRTHLP.MAC	HELP command processor

KRTIDX.BAS	Builds KRTIDX.MAC from KRTHLP.HLP

KRTIDX.MAC	HELP text file index for high speed lookups

KRTINI.MAC	Initialization, packet logging

KRTKM .MAC	"Kermit Modem" handler

KRTLNK.COM	Links extended memory overlays image, KRTTSX.SAV

KRTMAC.MAC	Macro definitions .INCLUDE file

KRTMDM.MAC	Pre-defined modems database

KRTNHD.MAC	MUL, DIV, SOB, ASH macros for non-EIS CPU

KRTOID.COM	Link for RT-11 V5.6 separate I&D space, low memory overlays

KRTOSI.MAC	One-shot initialization at program start

KRTPAK.MAC	Low level packet processing

KRTREC.MAC	Incoming file protocol processing

KRTRMS.MAC	File access, RMS emulation

KRTSEN.MAC	Outgoing file protocol processing

KRTSER.MAC	Server support

KRTSHO.MAC	SHOW, COPY, DELETE, PRINT, RENAME commands

KRTSJ .CND	RT-11FB,SB,SJ conditional assembly file for KM handler

KRTSJ .MAC	Root module for low memory overlays image

KRTSTD.MAC	SET command data table

KRTST0.MAC	SET command overlay 0

KRTST1.MAC	SET command overlay 1

Installation and Release Notes	KRT V03.63			Page ThrityOne

KRTSUB.MAC	Misc common utility routines

KRTSV4.COM	Build KM handler under RT-11 V4

KRTSYS.COM	Build KM handler under RT-11 V5

KRTTRA.MAC	TRANSMIT command

KRTTSX.CND	TSX conditional assembly file for KM handler

KRTTSX.MAP	Link map for extended memory overlays image, KRTTSX.SAV

KRTTSX.SAV	Extended memory overlays image, for RT-11XB,XM,ZB,ZM and TSX+

KRTUTL.MAC	Misc utility routines, mount for TSX+

KRTVID.COM	Link for RT-11 V5.6 sep I&D space, extended memory overlays

KRTXL .MAC	I/O support for CL, KM, XC and XL

KRTXM .CND	XM conditional assembly file for KM handler

KRTXM .MAC	Root module for extended memory overlays image

KRTXMO.MAC	XMODEM, also SEND command processor

Billy Y..

27-Sep-1997

-eof-