12/15/83 - Revised to reflect operation with initial packet timeout.

KERMIT ON MTS

Written in Pascal/VS by

William S. Hall

Mathematical Reviews

611 Church Street

Ann Arbor, MI

313-763-6831

Kermit is a file transfer program allowing movement of files

among a variety of computer systems including micro-mainframe and

mainframe-mainframe.  The following note describes how to use the

program SJ1K:Kermit.exe on the Michigan Terminal System (MTS).

It is assumed that the user already has a version of Kermit

running on his or her computer and is familiar with its operation.

To obtain copies of Kermit for various micros, UNIX, TOPS-10,

and TOPS-20 operating systems as well as detailed documentation,

contact the author or

Kermit Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY 10025

The source code for Kermit on MTS is in the file

SJ1K:Kermit.pas.  It is permitted to all to read.  It is known to

work against DEC-20's running TOPS-20 (the definitive test),

Zenith Z100's running CP/M-85, and Heath H8/89's running CP/M.	At

present it will not work the Z100 under ZDOS and thus its

performance with the IBM PC is doubtful.  It is most likely due

to a bug in PCKERMIT, which should be fixed soon.

Running Kermit

To transfer text files between MTS and your system proceed as

follows:

(1)  Run Kermit on your controlling micro or mainframe.

(2)  Set the IBM flag ON and then set local-echo OFF.

(3)  Log onto MTS as usual.

(4)  To initiate a file transfer run SJ1K:KERMIT.EXE.

(5)  Respond to command prompts by entering 'R' (or 'r') to

have MTS receive a file.  Answer 'Y' if you want

MTS to reserve the first character in each line of

your file for carriage control (see below).  Exit to

controlling system and send file.

(6)  Respond with 'S' (or 's') to send a list of files.

Answer 'Y' if ALL files to be sent from MTS use

column 1 for carriage control (see below).  Enter

file names one at a time when prompted.  Terminate

list with a carriage return.  Exit to controlling

system and prepare to receive a file.  In about 5

seconds, MTS Kermit will attempt to handshake with

your machine.  If you miss the first packet and

your version of Kermit does not have time-outs

(most micro versions do not), trigger MTS with two

carriage returns from your machine.

Current Limitations:

(1)  Only text files can be transferred.

(2)  File names should be less than 40 characters and

have no device names when sent from MTS.  TEST.PAS

is OK, but SJ1K:TEST.PAS is not.

(3)  MTS Kermit has no timeout, hence deadlocks can

occur.  If the process seems to stop, try a carriage

return or two to break the deadlock.  Use several of

them to exit the file transfer mode, and connect

back to MTS.  If the MTS Kermit is still running, you

will have to kill the program with "ATTN" (control-E).

To restore normal terminal operation enter the command

"%reset".

(4)  Pascal/VS does not at present allow a run-time error

to be intercepted by the program.	Such errors can

occur if files to be sent are not present or if disk

space is exceeded.  Hence the transfer will stop,

and the resulting deadlock must be broken as

described above.

(5)  If a horizontal tab is encountered when a file is

being received, it is expanded to spaces for the

usual tab stops of 1, 9, 17, 25, 33, etc. before

being written to an MTS file.  Spaces are not

compressed to tabs during transmission from MTS.

(6)  The carriage control option allows the user to

reserve column 1 in an MTS file for printer carriage

control.  Otherwise, column 1 in the file is regarded

as data.  See also the next item.

(7)  If an ASCII form feed is encountered in a file begin

received, the Pascal/VS "page" call is made.  This

has the effect of placing a '1' in column 1 of the

MTS file.	Thus, files with form feeds should choose

the carriage control option 'Y' described above.

Conversely, if the MTS file has a '1' in column 1 it

is converted to a form feed when being sent if a 'Y'

is given to the carriage control prompt.  Best

results are obtained if the form feed is on its own

line in the file to be sent to MTS.

(8)  Other control characters are handled by simply

substituting the corresponding EBCDIC control

character.