QuietSCSI

a program to prevent SCSI drives#from making noise

release 1.1

by Franz-Josef Reichert, 10/20/1995

UNLESS OTHERWISE NOTED, ALL FILES ARE Copyright � 1994-95 by F.J. Reichert.

All  Rights  Reserved.  WARNING:  THIS IS *NO* PUBLIC DOMAIN PROGRAM, BUT A

COPYRIGHTED PIECE OF SOFTWARE.

Franz-Josef Reichert

Kuchlingerstra�e 13

D-66271 Kleinblittersdorf, GERMANY

FAX:   +49-6805-218101

eMail: fjrei@kbsaar.saar.de

*** WHAT'S NEW FOR 1.1? ***

This is release 1.1 of QuietSCSI. A release which shouldn't have

happened at all, though, since only two (2) people actually sent me a

donation for my work. Would you care about to invest further development,

time, money to a project only two people are interested in? Well, I'm a

hopelessly amiga addicted fool, so *I* did!

Please, make me believe that it's worth even more effort! ;-)

MANY more people asked for more features. I had to answer several

dozends of questions, but nearly all people obviously using my program all

days never sent a donation. Anyway, the most popular enhancement requests

made their way into this release:

� Recognizes "special" drivers "ALF.device" and "oktagon.device" and

handles them properly. In earlier versions, QuietSCSI refused to work on

those hostadapters if there was more than one SCSI target installed. On the

other hand, Oliver Kastl (the programmer of the original ALF-software) told

me that there's some QuietSCSI-like feature already built in, so there

should be no need to use QuietSCSI with this software, anyway. Don't ask

me...

� Additional code to make removable media user happy (well, almost), such

as SyQuest or some other magnetooptical toys. It was introduced in

QuietSCSI 1.1. Removable media support *is* tricky. Shortly after the drive

is turned off, a requester will appear on workbench screen to ask for

restart (you need at least 2.0 ROMs (V37)). The choice is to leave the

requester hanging around as you like or satisfy it, which will result in a

restart. As long as the requester is up, all driver related commands will

be queued until the request is satisfied. This will of course suspend any

command waiting for queued driver requests to complete. For example, issue

the command "C:Info", "C:Dir" etc. and it will not return until a pending

restart requester is satisfied. Warning: Workbench might even be blocked!

On the other hand, note that the drive will never be stopped when there is

no medium inserted. This is usually no problem since most changeable media

drives stop their motors in this case, anyway.

+-------------------------------+

| QuietSCSI			|

+-------------------------------+

|				|

| Restart requested on		|

|				|

| removable media		|

|				|

| "scsi.device" unit 0		|

|				|

+-------------------------------+

|          +---------+		|

|          | Restart |		|

|          +---------+		|

+-------------------------------+

If you consider the requesters and the necessary (manual) restart a

problem, please don't use QuietSCSI on these drives at all! Other than some

people might think, this topic needs *no* further discussion. It's a matter

of fact that the filesystem will always try to re-read a volume directory

tree once it received a diskchange event (caused by the STOP command

QuietSCSI issues), and there's *no* way to prevent it from doing that. All

QuietSCSI can do about this is to queue all commands, and thus suspending

the filesystem from further investigations. Other than with

non-removable-media drives, there is no chance for an automatic restart,

sorry.

� LUN numbers are correctly handled now. At least for device drivers being

able to handle them properly.

� -h Option brings up the usage help text.

*** FAQ, COMMOM PROBLEMS SOLVED ***

� Please don't ask me whether there might be a chance of QuietSCSI to work

on some AT/IDE-style devices. I simply don't know. QuietSCSI MAY work on

those drives, IF the driver is designed to emulate some related SCSI

commands AND if the drive used has some suitable command equivalents the

driver software might want to call. If not, it probably _won't_ work. Try

it. I tried it on my A1200 with a Seagate IDE drive. No go, ha! Don't blame

me if it doesn't work. This software is intentionally named Quiet-S-C-S-I,

not QuietIDE, nor QuietAT, nor QuietATAPI.

� To make QuietSCSI work on GVP hostadapter cards equipped with the new

Guru-ROM (omniscsi.device) by Ralph Babel, put the following command in

your startup sequence:

OmniScsiCtrl <unit> NOSCSICMDINT

_before_ you start QuietSCSI. This topic is discussed in the Guru-ROM's

user's manual FAQ chapter, too.

*******************

� Disclaimer

The author disclaims all warranties, expressed or implied, including, but

not limited to, any implied warranties of merchantability and/or fitness

for a particular purpose. The author does not warrant, guarantee or make

any representations regarding the use of, or the output from, the program.

You rely on the program and output solely at your own risk. In no event

will the author be liable for any action, including lost profits,

incidental or consequential damages arising out of the use or inability to

use this software, exceed the possible shareware fee you paid for this

software, even if the author or an authorized distributor of this software

has been advised of the possibility of such damages.

� Distributing this program

This piece of software, including all accompanying files in the archive, is

what some people tend to call �Freeware�. This implies:

This program may be freely DISTRIBUTED, by any means, as long as there are

no charges specifically for the program. Charges for downloading from

electronic BBS systems or network services, or charges for copying and

media for freely redistributable software libraries are certainly

permissible. The entire archive has to be left intact and unchanged. You

must not distribute any changed or imcomplete set of the original archive.

� Using this program

This program may be USED for any purposes, commercial, personal, public,

and private, as long as ALL of this text, all files, copyright, mailing

address, and this notice are retained in their entirety. It is provided

without warranty of any kind, so use it at your own risk!

This piece of software, which includes all accompanying files in the

archive, is what some people tend to call �Giftware�. This implies:

� You may use it as often as you like

� You may use it as long as you like

� You are *NOT* obliged to pay anything for using it

� You *can* send a donation to the author, if you feel

so or if you like to do so

� If you like to send a donation to the author, you can pay any

amount you think that this piece of software is worth it

� Thank you for supporting a poor programmer, who still insists in

writing quality amiga software ;-)

� What this program actually does

� The problem

Do you remember your first Amiga 1000 computer? Everybody asked you 'Is it

really on' when they saw it first, since it was so q-u-i-e-t. I'll never

forget that perfect feeling having a nice, quiet computer on my desk. Nine

years later! Computers make noise. Computers which contains a harddisk make

even more noise. Computers containing several harddisks make even much more

noise. And they burn energy! Today, people spend a lot of $$$ to buy

themselves screen blankers. And cooling fans which make less noise than the

el-cheapo thingies all power supply manufacturers use. Oh well, everybody

has a low power monitor and computer. But nobody realized, that a harddisk

spins most time of the day without reading or writing a single byte! It

just spins, burns a lot of energy, and makes noise.

Can you sleep in the same room when your computer is on? I can not! The

harddisks make too much noise. Well, the power supply fan makes noise, too.

But since I got a temperature controlled model, I can stand it much better.

And there's another problem. The power led. Yes, it's so bright that I can

read books!

� The solution

Speaking of SCSI harddisks, there is a mechanism to switch the motor off.

It will just stop to spin as if you switched your computer off. But the

drive is not dead. It's just q-u-i-e-t. It won't allow you to read or write

data, though. But the controller electronic on the harddisk is still alive,

and it might get a command to restart the drive at any time. Then it will

behave just as if you switched your computer on. The motor will spin up

again, and the drive will be ready to read or write data.

This little program does exactly this. It monitors all read and write

accesses to the drive. It also monitors all input events. If there is

inactivity for a certain period of time, it will spin down the drive motor

and dim the power led. If the next command arrives, it will spin up the

motor and switch to led back to bright.

In other words: Leave your amiga alone for some time, and it will switch

the harddisk's motor and the power led OFF. Come back to your amiga after a

while, continue your work. You won't realize any difference, the harddisk

is still quiet. But the next time you try to read or write data from the

drive, it will switch the power led and the motor ON, and your harddisk

will spin up again. That's the whole magic. It save energy, and it saves

noise.

Maybe you run a BBS and only 2-3 users will log in during night? Or your

computer is alive to catch FAXes all nights, which will only happen once a

week or so? So this is for you!

� Starting the program

This one is really easy to use. Just put it to your startup-sequence (or

even better: user-startup):

RUN <NIL: >NIL: quietscsi [-h] <device> <unit> <seconds>

where

<device> is the name of your SCSI driver, e.g. scsi.device,

or gvpscsi.device. Default is "scsi.device".

<unit> is the unit number where you harddisk is connected to. You

can figure this out by starting HDToolBox, for example. Default is 6.

<seconds> is the number of seconds for the inactivity timeout.

Default is 900 for approx. 1/4 hour, maybe you'd like another value. You

shouldn't choose the value too low, though. Switching the motor off and on

very often burns more energy and shortens your harddisk's lifetime (Well,

don't go nuts about that ;-)). In the current release, the timer is quite

inaccurate, since it doesn't compares with the system clock, just counts

timer requests of one second. So it may vary, maybe 900 means 16, 17 or

even 18 minutes. It is guaranteed to count always some seconds more, but

never less. This isn't a big problem in general. I was just too lazy to fix

it, maybe in the next release ;-).

[-h] is optional and will bring up the default help text.

� Quitting the program

You might want to try to send a <Ctrl>-<C> signal to the process. Either by

typing that to the shell you started it from (but that's not the way you

should do it, since it blocks a window), or by using the shell command

�break�. This *may* remove the patch. It is not guaranteed that it will

succeed, since there might be some requests in the queue which need to be

handled first. Just try a while later. Anyway, there shouldn't be a need to

remove the patch while your system is alive. I merely added that option for

debugging purposes. Well, maybe I shouldn't have told you that at all ;-).

Don't bother me if you encounter problems related to this.

� Things to do

You can send me a donation if you like. That's what *you* can do ;-).

Now the things that *I* plan to do when I notice that some people like my

program:

� More accuracy for the timer

� Add TSR startup code

� Add some more command line options

� Find a new girlfriend

� Implement a commodity interface, perhaps a GUI which lets

the user switch the drives off and on, select an new

timeout and all those useful and marvellous things

� (...)

Maybe you have some more ideas! Let me know. Have fun using QuietSCSI, I

use it all the day. It's small, but it's probably one of the most useful

tools I ever wrote.

<EOT>