Here are the messages printed via TapeMon by the BTN handler version 3.0,

in no particular order, with reasons for each message.

*** Sense: %s, other= %02X,%02X

Printed when the tape drive returns a SCSI "check condition"

in response to a SCSI command that BTN has sent to the drive.

Sometimes this may indicate an error or problem, but not always.

The reason for the condition is encoded into a "sense key"

which is spelled out for you in this message.  The SCSI spec

defines these keys:

NO SENSE - nothing unusual to report

RECOVERED ERROR - something went wrong, but the drive fixed it up

NOT READY - human intervention is required (insert tape)

MEDIUM ERROR - an unrecoverable flaw in the tape material

HARDWARE ERROR - the drive is broken (still have your receipt?)

ILLEGAL REQUEST - the SCSI command/function is not supported by

this drive, or there are bits in the command that the drive

doesn't like, or the drive is being asked to do something

it doesn't feel like doing right now.

UNIT ATTENTION - a transient condition generated whenever a

new tape is inserted.

DATA PROTECT - the tape medium is write protected

BLANK CHECK - the drive has attempted to read past the blank spot,

the end of the last-written file, or an erased tape.

FILEMARK - the drive has attempted to read a filemark, which

indicates the end of the file has been reached.

END-OF-MEDIUM - the drive has reached physical end of the tape

VOLUME OVERFLOW - some drives indicate this at end of tape

INCORRECT LENGTH - BTN asked to read a block smaller than

what was written on the tape.  BS is probably incorrect.

The "other" bytes printed are bytes 12 and 13 of the sense data.

For some drives, such as the MCD-40 and SCSI-2 drives, this is

a subcode indicating more information about the sense key.

*** SCSI %s ERROR

The scsi-direct driver/adapter returned an error in response to BTN's

attempt to send a SCSI command to the drive.  These errors are

defined by Commodore:

SELF-UNIT - the UN number is the same as that of the SCSI adapter

DMA - I don't know what this means.  Maybe an adapter problem.

PHASE - the drive and the SCSI adapter are not in-sync with

their SCSI phases.  There may be a mismatch in data transfer

caused by incorrect block size (BS value), or the SCSI bus

may be improperly terminated.

PARITY - a parity error on the SCSI bus

SELECT-TIMEOUT - the adapter timed out waiting for a response

from the drive.

*** Can't get %u bytes for tape buffers

There is insufficient memory to allocate two buffers of NB*BS

bytes (or maybe the memory is too fragmented).  Use a smaller

value for NB, or free up some memory, or reboot.

*** Invalid open name/combo %s

* The name used to access the tape file is unrecognized.

The name must be one of the following:  TAPE:  TAPE:R  TAPE:END

TAPE:APP  TAPE:NR  TAPE:*  TAPE:number  TAPE:RAWCMD  TAPE:MODESEL

* The function is incompatible with the open mode or drive model.

Examples: you cannot read from TAPE:APP, and TAPE:APP is always

invalid for direct access drives.

*** Current block size = %u

*** Mode Select, new block size %d

Informative messages printed when the BS value is different than

the block size of the drive.  BTN will attempt to command the

drive to switch to the new size.

*** Capacity: %u blocks

Informative message printed only for direct access drives describing

the number of blocks available on this particular cartridge.

*** %s Opened file %d / at block %d

Informative message printed when Open() is successful.

*** Closed at block %u, next file= %d

Informative message printed when Close() is finished.

The block number is the next available block.  The tape head

is positioned ready to access this block for the next operation.

The number of the file or block that will be written next is shown.

*** Function/mode error

The application is attempting to mix Read()s and Write()s.

It can't read while opened for writes, and vice versa.

*** %s is read-only

The application is attempting to write to the device when

the handler is operating in read only mode (RO-1).

*** %s is append-only

The application is attempting to write over an existing file

when the handler is operating in append-only mode (AO-1).

*** Disallowed seek type=%d off=%d curr=%d

A Seek() request was rejected because the type of seek is not

supported or it cannot be done at this point.

*** Error during read

*** Error during write

Indicates that a medium error was encountered during a read or write.

*** Unsupported_Pkt=%d

Informative message indicating that an AmigaDOS packet was

rejected as "unknown", because the requested function does

not apply to BTN.  Not usually a problem.

*** Skipping to end of data

*** (doing it the hard way)

Informative message printed when Open()ing TAPE:APP.  If the

drive doesn't directly support this function, the "hard way"

message is printed, and BTN begins skipping individual files.

*** Skipping to file %d

Informative message printed when reading from TAPE:number

on a sequential drive.

*** Unable to position tape

Printed when some kind of error occurs when BTN tries to get

the tape into the necessary position for Open().  You will

get a "replace volume" requester.

*** Time to insert tape# %d

Informative message printed at end of tape.  Also a requester.

*** Writing block %u x %d

*** Reading block %u x %d

Informative messages printed just before a SCSI read or write

command is sent to the drive.  The block number is relative

to the beginning of the file for sequential drives, and is

the actual block number for direct access drives.  The length

is the number of bytes (NB*BS).

*** Rewinding

You guess this one...

*** Writing filemark

Informative message printed when Close()ing a file being written.

*** Retensioning

Informative message printed when Unit Attention status occurs and

RT-1 was specified in the mountlist.  The retensioning operation

may take several minutes to complete.

*** Raw command %02X

Informative message printed when writing to TAPE:RAWCMD.

*** User Mode-Select

Informative message printed when writing to TAPE:MODESEL.