NWDOSIPX.DOC           07/10/91

Copyright 1991, Compaq Computer Corporation.

NOVELL NETWARE DOS IPX WORKSTATION INSTALLATION INSTRUCTIONS

1. INTRODUCTION

2. FEATURES

3. RELEVANT FILES

4. SYSTEM REQUIREMENTS

5. INSTALLATION

6. SOURCE ROUTING SUPPORT

7. HIGH MEMORY SUPPORT

8. NETWIRE

1. INTRODUCTION

This file details the installation of the NetWare DOS IPX

workstation driver for the COMPAQ 32-Bit DualSpeed Token Ring

Controller.

2. FEATURES

1. Provides improved performance over previous releases of

the driver.

2. Supports Source Routing.

3. May be loaded into high memory.

4. Supports Microsoft Windows 3.0.

3. RELEVANT FILES

NWDOSIPX.DOC - This file

SCPQT416.OBJ - NetWare DOS IPX workstation device driver object

SCPQT416.LAN - NetWare DOS IPX workstation configuration file

MAC380.BIN   - Controller download program image

4. SYSTEM REQUIREMENTS

This device driver requires:

*   Any COMPAQ EISA computer with the COMPAQ 32-bit DualSpeed

Token Ring controller installed. The system must have

been configured using the COMPAQ EISA Configuration

Utility.

*   NetWare workstation generation software. Either shgen.exe

or wsgen.exe can be used to generate the workstation

software with this driver. These programs are located on

the workstation services diskette included with the

NetWare system software.

5. INSTALLATION

In the following procedures, refer to your NetWare documentation

for information on installing DOS workstation drivers.  To install

the device driver for the token ring controller, follow the

procedure that best serves your needs:

Floppy Diskette Drive Installation Procedure:

1.   If you are installing NetWare Device Drivers from

diskettes, format a blank diskette with the volume label

"LAN_DRV_428".  To this diskette, copy the following

files:

scpqt416.obj

scpqt416.lan

Note:  For more information about the MS-DOS FORMAT,

LABEL, and COPY commands, refer to your MS�DOS

documentation.

2.   Follow the instructions in the NetWare documentation to

configure and install a network IPX workstation shell

onto your workstation(s). At some point in the

installation you will be prompted to insert the diskette

labeled "LAN_DRV_???". Insert the diskette you prepared

earlier.

3.   The NetWare workstation generation procedure creates an

IPX.COM file which contains the driver for the 32-Bit

DualSpeed Token Ring Controller.  This file and the

NETx.COM file must be copied to your workstation fixed

disk or diskette.  Refer to your NetWare documentation

on which files are required for the version of MS-DOS you

are using.

4.   COPY THE MAC380.BIN FILE TO THE SAME DIRECTORY ON THE

WORKSTATION WHERE YOU COPIED THE IPX.COM FILE.

If you are unable to correctly load the device driver,

verify your Token Ring Controller(s) configuration by

running CHKTOK.EXE, found on the OPTION CONFIGURATION

diskette.  To test your system hardware, run DIAGNOSTICS

Version 7.02 or later.  Refer to the 32-Bit DualSpeed

Token Ring Controller Installation Card for more

information on using these utilities, or contact you

Authorized COMPAQ Computer Dealer.

Fixed Disk Drive Installation Procedure:

1.   If you are installing the workstation drivers from a

fixed disk or network drive, create a subdirectory named

LAN_DRV_.428 on the same directory where the installation

program is stored. Copy to that subdirectory the

following files:

scpqt416.obj

scpqt416.lan

2.   The NetWare workstation generation procedure creates an

IPX.COM file which contains the driver for the 32-Bit

DualSpeed Token Ring Controller.  This file and the

NETx.COM file must be copied to your workstation fixed

disk.  Refer to your NetWare documentation on which files

are required for the version of MS-DOS you are using.

Note:  If you are installing NetWare from a fixed disk

or network drive, the drivers will be automatically

loaded from the LAN_DRV_.428 subdirectory.

3.   COPY THE MAC380.BIN FILE TO THE SAME DIRECTORY ON THE

WORKSTATION WHERE YOU COPIED THE IPX.COM FILE.

If you are unable to correctly load the device driver,

verify your Token Ring Controller(s) configuration by

running CHKTOK.EXE, found on the OPTION CONFIGURATION

diskette.  To test your system hardware, run DIAGNOSTICS

Version 7.02 or later.  Refer to the 32-Bit DualSpeed

Token Ring Controller Installation Card for more

information on using these utilities, or contact your

Authorized COMPAQ Computer Dealer.

6. SOURCE ROUTING SUPPORT

Source routing is a method of routing data across multiple

networks through bridges. To enable the workstation to pass

data through a source routing bridge, the workstation must be

running the source routing program, route.com from Novell.

This program is necessary only if external source routing

bridges are present on the network. The source routing

program, file route.com, is available on the Novell NetWare

DOS Workstation Generation diskette. The Novell source routing

program and installation instructions are also available on

Compuserve NetWire under NOVA forum in library 8 (Novell

Patch/Drv'r) in the TRN050.ZIP file.

See the Novell NetWare documentation for more information on

source routing.

The following is an example of how to load source routing. The

following lines should be placed in the workstation

autoexec.bat file.

ipx

route

net3

7. HIGH MEMORY SUPPORT

The driver may be loaded into what is sometimes referred to

as "high memory". High memory is the memory located between

640K and 1Meg. The NetWare DOS IPX workstation driver is

compatible with programs such as COMPAQ runhi, QUALITAS

386max, or QUARTERDECK QEMM. All of these programs allow the

driver to be loaded into high memory. The primary advantage

of loading the driver into high memory is that it will free

up memory in the base 640K, thus leaving more memory available

to applications. The following is an example of how to load

the driver with COMPAQ runhi.

config.sys:

device = c:\dos\himem.sys

device = c:\dos\cemm.exe NOEMS

autoexec.bat:

runhi ipx

runhi net3

Notice the NOEMS option is being used on the command line for

cemm.exe. The NOEMS option will disable expanded memory. The

NOEMS option frees up more high memory, but its use may not

be advisable if you are using applications that require

expanded memory.

8. NETWIRE

The NetWare DOS IPX driver files for the COMPAQ 32-Bit

DualSpeed Token Ring Controller reside on Compuserve NetWire

under NOVA forum in library 11 (or library 17 for new uploads)

in the CPQTR2.ZIP file.