TITLE:    Drive Deactivation Troubleshooting Tips

DOCUMENT ID#:    FYI.P.11081

DATE:    12JAN93

PRODUCT:    NetWare

PRODUCT VERSION:    v3.11

SUPERSEDES:    NA

SYMPTOM

NA

ISSUE/PROBLEM

Drive Deactivation Tips and Techniques for NetWare v3.11.

The purpose of this document is to establish what drive deactivation is

and why it occurs.  Drive deactivation is when one or more devices

(drives) in the server become unable to communicate with the operating

system (OS).  Drive deactivation is caused by an inability of NetWare and

the disk driver to communicate with the device.  When the driver cannot

communicate with the drive, it will normally retry several times.  If,

after a predetermined number of retries, the driver still cannot

establish a communication link with the drive, the driver tells the OS

that the drive is not responding and the OS then deactivates the device.

Drive deactivation can occur across any of the disk interfaces, such as

IDE, ESDI, or SCSI.  Drive deactivation is also not confined to just one

manufacturer.  The following list is a "checklist" that Novell suggests

you go through in troubleshooting a drive deactivation problem.  In many

cases, you must be very specific with hardware, firmware, and software

revision levels as you work with your third-party Host Bus Adapters

(HBAs) and drive manufacturers in solving your problems.  The priority of

the checklist is based on solutions that will cost the least and still

receive the highest priority.  For checklist purposes, this discussion

will be restricted to the SCSI interface.  All suggestions should be

easily translated for any interface.

CHECKLIST

1.   Verify that termination along the SCSI bus is correct.  The first

basic rule to remember is that both ends of the bus must be

terminated.  You must be careful that if you have drives in an

internal or external cabinet that you remove the termination from

the individual drives and then just terminate the cabinet

externally.  This avoids future headaches; because, as you add more

drives, you do not have to worry about which drive was terminated in

which cabinet.

Another problem arises when you use both the internal and external

ports on an HBA.  When you do this, you must remove the termination

from the HBA and terminate both ends of the SCSI bus, which will

usually be at the drives themselves unless an external cabinet is

used.

2.   Verify that the SCSI IDs are set properly.  In all situations, the

HBA will have a SCSI ID of 7.  This may be set either through a

hardware setting (jumpers) or through software utilities (such as

EISA CONFIGURATION or REFERENCE).

In PS/2 situations and possibly with HPs, the HBA takes a SCSI ID of

7 and the attached devices start at 6 and work down to 0.  In all

other situations, the HBA takes an ID of 7, and the attached devices

start at 0 and work up to 6.

In the PS/2 environment, there are specific things that need to be

addressed.

Using the OPTION diskette v1.0 from IBM, the settings for

Fairness are reversed.  Normally, fairness is turned ON.

However, if you are using the OPTIONS diskette v1.0, you must

set "Fairness" to OFF.  On all other versions of the OPTIONS

diskette, simply set fairness to the normal setting of ON.

3.   Verify the disk driver.  Verify with your third-party HBA

manufacturer that you are running the latest Novell certified driver

and that it is compatible with the firmware revision level on your

HBA.

4.   Verify the HBA firmware.  Verify with your third-party HBA

manufacturer that the version of firmware on your HBA is the correct

version applicable to the version of the disk driver that you are

using.

5.   Verify the firmware revision level on the drive.  Verify with the

third-party drive manufacturer that you have the proper or latest

version of firmware on your drive.

6.   Check and if necessary replace the cabling on the SCSI bus.  Reseat

all connectors, then reseat the HBA into the bus.  Look for pinched

or broken leads on all connectors and cables.

7.   Verify proper wattage of the power supply.  Novell had seen many

drive deactivation problems solved by upgrading or replacing a weak

or flaky power supply.  Drive deactivation problems seem to be an

ever increasing problem as drives grow larger in capacity yet shrink

in physical size.  This smaller size allows more drives to be placed

into the server box itself thus placing a larger demand on the

server's power supply.

8.   Examine other devices attached to the SCSI bus.  If you have a tape

unit, CD-ROM, or other devices attached to the SCSI bus, ensure you

enable the "disconnect" feature on those devices.  This allows the

HBA to issue a command or series of commands to that device and then

disconnect and return to servicing disk requests.  If the

"disconnect" feature is not enabled, the SCSI bus must wait for the

device to return with a completion code and by so doing may not be

able to service the drivers requests to "talk" to the drive.

9.   Examine other bus mastering boards.  If your server has bus

mastering boards, you may need to examine these closely.  If a bus

mastering board holds the bus too long, this may prevent the disk

driver from establishing a communication link with the drive.

Sometimes it may be an issue of getting a newer driver for the bus

mastering device or taking drastic steps of replacing or upgrading

the suspected bus mastering device.  Experience has shown that this

can happen, but it is rare.

10.  Replace the HBA.  If the above steps have failed to solve the drive

deactivation problem, then it is time to take drastic action.

Replace the HBA with preferably the same make and model number.  If

a different brand or model number is chosen, a reformatting and

repartitioning of the drive may be necessary.

11.  If you are still unsuccessful in solving the drive deactivation, it

is time to seriously look at replacing the drive.  Experience has

shown that replacing the drive with the same make and model number

may solve the problem; however, in some situations a switch to a

different manufacturer may be necessary.  Drive deactivation can

sometimes be very finicky and may require drastic steps to stabilize

the server and eliminate the drive deactivations.

SOLUTION

NA