ENET P-SERIES ETHERNET NETWORK ADAPTERS       Jan 11, 1993

(930111)

���������������������������������������������������Ŀ

�                                                   �

�               DIRECTORY STRUCTURE                 �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

This diskette provides different network device drivers in

order to let our network adapters work in different networking

environment. The configuration and diagnostic utilities included

are for configuring and testing the hardware of the adapters. All

files are arranged in the following directories.

a) <Root>

It contains NetWare IPX workstation and server

drivers. Two configuration information files (.LAN)

are found here for generating IPX server and

workstation drivers. The files used for Windows for

Workgroups can also be found here.

b) <Root>\DOSODI

NetWare ODI workstation driver can be found.

NET.CFG is a sample configuration file for your quick

reference.

c) <Root>\NETWARE.386

Users can find NetWare ODI server driver.

d) <Root>\MSLANMAN.DOS

This directory is used for installing DOS NDIS

driver in Microsoft LAN Manager. Please refer to the

section, LAN MANAGER, in this document for more

details.

e) <Root>\MSLANMAN.OS2

The driver for installation of OS/2 workstation

or OS/2 server can be found here. Please refer to the

section, LAN MANAGER, in the document for more

details.

f) <Root>\UTILITY

There are configuration and diagnostic utilities.

The document file, HELP.DOC provides some guidelines

for both utilities.

g) <Root>\PKT

There is a Packet Driver for our network adapters.

���������������������������������������������������Ŀ

�                                                   �

�       HISTORY OF DRIVERS AND UTILITIES            �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

This section records all modifications made on different

versions of device drivers and utilities.

1.1) ODI Server Driver

A) ENET386.LAN v2.10

a) This copy was modified from v2.02 and was

enhanced to support our P-series network adapters. It

can also support both M and U series network adapters.

1.2) ODI Workstation Driver

A) ENETCPSH.COM v1.30

a) This copy was modified from v1.22 and was

enhanced to support our P-series network adapters. It

can also support both M and U series network adapters.

1.3) IPX Server Drivers

A) ?ENETCP.OBJ v2.30

a) This copy was modified from v2.22 and was

enhanced to support our P-series network adapters. It

can also support both M and U series network adapters.

1.4) IPX Workstation Driver

A) ENETCPSH.OBJ v2.30

a) This copy was modified from v2.22 and was

enhanced to support our P-series network adapters. It

can also support both M and U series network adapters.

1.5) DOS NDIS Driver

A) ENET.DOS v2.30

a) This copy was modified from v2.00 and was

enhanced to support our P-series and ENET16-MC/P

network adapters. It can also support both M and U

series network adapters.

b) The driver was modified to support the full range

of I/O base address from 0x200 to 0x3e0.

c) After compiling for x88 processor, DOS NDIS driver

can run in XT machine.

d) After adding the codes to support more primitives

specified in the NDI specification v2.0.1, the driver can

work in DEC Pathworks for DOS.

1.6) OS/2 NDIS Driver

A) ENET.OS2 v2.30

a) This copy was modified from v2.00 and was

enhanced to support our P-series and ENET16-MC/P

network adapters. It can also support both M and U

series network adapters.

b) The driver was modified to support the full range

of I/O base address from 0x200 to 0x3e0.

c) Extra codes were added to support more primitives

in the NDI specification.

1.7) Packet Driver

A) ENETPKT.COM v1.00

a) This is our first release of Packet driver. It can

support our M, U and P series network adapters.

1.8) Configuration and Diagnostic Utilities

A) ENETUTIL.EXE v3.00

a) This copy was modified from ENETDIAG.EXE v2.10. We

did a lot of modifications in order to support P-series

network adapters. Please refer to the document, HELP.DOC

(which can be found in <Root>\UTILITY) for more

information.

���������������������������������������������������Ŀ

�                                                   �

�      LAN DRIVER CONFIGURATION PARAMETERS          �

�                                                   �

�����������������������������������������������������

IMPORTANT:

:- Please read this section before reading the driver

configuration parameters in other sections.

1) INTRODUCTION

All drivers included in this driver diskette can support all

ENET M, U and P series network adapters (for ISA). In order to

make use the software programmable capability in our P-series

network adapters, several new configuration parameters have been

added in the drivers. This section is intended to give users some

guidelines to make use some general configuration parameters to

configure the drivers. Users can refer to further information on

these parameters for different drivers in next sections.

2) CONFIGURATION PARAMETERS

Normally, the configuration parameters can be divided into

two different groups.

2.1) Basic Configuration Parameters

This group of parameters can be used for our M, U and P

series network adapters. They include I/O base address, IRQ

number and RAM base address. Users can follow the procedures

listed below to configure the drivers using these parameters.

a) Use jumpers to set the ENET M or U series network

adapter to correct I/O base address and IRQ number. For P-

series network adapter, users can use our configuration

utility to software configure it.

b) Configure the driver. Select correct I/O base address,

IRQ number and correct RAM base address in the driver. They

will be used to initialize the network adapter.

NOTE:

:- If one of the parameters is entered incorrectly, the

driver MAY not work properly.

2.2) Advanced Configuration Parameters

These parameters are VALID for P-series network adapters

ONLY. The drivers will prompt you errors if you use them with M

or U series network adapters. There are two advanced

configuration parameters: Mode and Physical Media.

a) Physical Media

Users can use this parameter to change the physical

media to whatever they want. The drivers will program the

storage device (EEPROM) on the P-series adapter and change

the physical media to that specified by this parameter.

Normally, if this parameters is NOT defined, the drivers

will use the preset physical media stored in the storage

device (EEPROM) on the network adapter.

NOTE:

:- Different types of network adapters can support

different types of physical media. Please refer to your

manual for further information.

:- The drivers DO NOT change the physical media

PERMANENTLY. Once users hardware-reset the workstation, the

physical media will be changed to the original setting that

is stored in the storage device (EEPROM).

b) Mode

For a P-series adapter, a driver can support two

different configuration modes: AUTO and PROGRAM.

If a driver is configured to AUTO (Automatic

Configuration Mode), it will ignore THREE basic

configuration parameters: I/O base address, IRQ number and

RAM base address. It will search all valid I/O base address

and initializes the FIRST P-series network adapter in your

workstation. Then it READS the hardware settings, IRQ

number and RAM base address, from the storage device

(EEPROM) on the network adapter.

If a driver is configured to PROGRAM (Program

Configuration Mode), it will interpret the meaning of THREE

basic configuration parameters differently. The driver will

search all valid I/O base address and initialize the FIRST

P-series network adapter. And then it will change the I/O

base address, IRQ number and RAM base address to that

specified by THREE basic configuration parameters. So the

network adapter will use the hardware settings specified by

the parameters, I/O base address, IRQ number and RAM base

address.

NOTE:

:- In "Program Configuration Mode", the driver DO NOT

change the I/O base address, IRQ number and RAM base

address PERMANENTLY. Once users hardware-reset the

workstation, hardware settings will be changed to the

original settings that are stored in the storage device

(EEPROM).

���������������������������������������������������Ŀ

�                                                   �

�                 NOVELL NETWARE                    �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

The diskette contains drivers and utilities to enable ENET

M, U and P (ISA) series Ethernet network adapters, to operate

with Novell NetWare Network Operating Systems. These include:

a) NetWare Lite v1.x

b) NetWare      v2.1x and v2.2     (NetWare 286)

c) NetWare      v3.11              (NetWare 386)

The main features and functions of the drivers are listed

below.

a) NetWare v3.11 (ODI) Server Driver

(ENET386.LAN) v2.10

This Ethernet driver is for NetWare v3.11 File

Servers and complies to Novell ODI (Open Data-link

Interface) Specifications.

b) NetWare DOS (ODI) Workstation Driver

(ENETCPSH.COM) v1.30

This ODI driver is for DOS workstations when

using Novell ODI driver stack, mandatory for NetWare

Lite, and optional for NetWare v2.1x, v2.2 and v3.1x.

c) NetWare v2.1x IPX Server Driver

(?ENETCP.OBJ) v2.30

This set of Ethernet drivers are used in NetWare

v2.1x and v2.2 File Servers. All of them have high

throughput with their double buffering technique.

d) NetWare DOS IPX Workstation Driver

(ENETCPSH.OBJ) v2.30

This DOS IPX workstation driver is for use in DOS

workstations with either NetWare v3.1x or v2.x.

The following lists the functions of the utilities.

a) ENETCP.LAN

This is a configuration information file for

generating NetWare v2.1x and v2.2 server drivers.

b) ENETCPSH.LAN

This is a configuration information file for

generating NetWare IPX workstation driver.

c) NET.CFG

This is a sample configuration file for DOS ODI

driver.

2) NETWARE SERVER AND SHELL GENERATION

2.1) For NetWare Lite v1.x

a) Follow the standard installation procedure.

b) When prompted to select the "Network Interface Card

Name" please select the option "*OTHER CARDS".

c) Insert the driver diskette, and press  <ESCAPE>.

d) Select ENETCPSH.COM from the menu.

e) Re-insert the NetWare Lite Install diskette and

complete the installation.

NOTE:

:- Should any problems be encountered, the ENETCPSH.COM driver

for ENET network adapters can be manually added to the NetWare

Lite startup procedure. Simply select any other Interface card

type (eg. the Western Digital Ether Plus Elite) when prompted

during the NetWare Lite installation. After this a file called

STARTNET.BAT is created by the installation process in the

C:\NWLITE sub-directory. Complete the following steps:

a) Copy the ENETCPSH.COM file from the driver diskette

to the C:\NWLITE sub-directory.

COPY A:\DOSODI\ENETCPSH.COM C:\NWLITE

b) Edit the STARTNET.BAT file so that the second line

in the file states ENETCPSH.COM and not the name of

the other Interface card type (eg. WDPLUS.COM) that

you selected in the installation process.

c) Start NetWare Lite by typing STARTNET.BAT.

2.2) For NetWare v2.2

a) Copy the ENETCP.LAN file onto the SYSTEM-1 and the

ENETCPSH.LAN file onto the WSGEN directories.

b) Copy the ?ENETCP.OBJ and ENETCPSH.OBJ files onto

the LAN_DRV_.203 directory.

Refer to the NetWare manual to generate the files for the

server and workstation.

2.3) For NetWare v3.1x

As NetWare v3.1x is a different concept from NetWare v2.1x

and v2.2, there is no need to generate server operating system

files. The server driver for NetWare (i.e. the ENET386.LAN) is

loaded through the system console. Refer to the NetWare 386

Installation manual for details on loading a LAN server driver.

To generate the IPX.COM in NetWare v3.1x, copy the

ENETCPSH.LAN file onto the WSGEN directory and copy the

ENETCPSH.OBJ file onto the LAN_DRV_.203 directory. Then, you can

perform WSGEN.

3) DESCRIPTIONS OF CONFIGURATION PARAMETERS FOR NETWARE DRIVERS

3.1) NetWare v2.1x or v2.2 IPX Server Drivers and IPX Workstation

Driver

When generating NetWare 2.1x or NetWare 2.2 server files or

IPX workstation drivers, the utilities, NETGEN/INSTALL or

WSGEN/SHGEN, will prompt you the "LAN Driver Configuration"

options that are listed below in Table 1 and Table 2. Comparing

the two tables, four more options are provided in the table (1)

in order to support multiple network adapters in the servers.

Option (0) is VALID for P-series network adapters ONLY. The

driver is configured to "Automatic Configuration Mode" and reads

all hardware settings from the storage device (EEPROM). If there

is no P-series network adapter, the driver will prompt errors.

The options (1) to (3) can be used for M, U or P series

network adapters. I/O base address and IRQ on the network adapter

SHOULD be set to the settings specified by the corresponding

options. The driver will map the shared memory to the system

address space specified by "RAM address".

The options (4) to (7) are used for the file server with

more than one P-series network adapters. The driver is configured

in such a way that it searches all valid I/O locations and finds

out ALL P-series network adapters in the machine. And then, it

will initialize that one it is configured to. For example, if it

is configured to option (5), it will initialize the SECOND P-

series network adapter among all P-series network adapters it

found.

The last option, "Driver configurable by Jumpers Utility",

sets the driver in such a way that all configuration parameters

can be changed by Jumpers utility later.

������������������������������������������������������������ͻ

�         �              �               �                   �

� Option  � IRQ          � I/O Address   � RAM Address       �

������������������������������������������������������������Ķ

�         �                                                  �

� 0       � Auto configuration                               �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 1       � 3            � 280h          � D000:0h           �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 2       � 5            � 300h          � CA00:0h           �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 3       � 7            � 240h          � D800:0            �

������������������������������������������������������������Ķ

�         �                                                  �

� 4       � Autoconfigure FIRST ENET P-series adapter        �

������������������������������������������������������������Ķ

�         �                                                  �

� 5       � Autoconfigure SECOND ENET P-series adapter       �

������������������������������������������������������������Ķ

�         �                                                  �

� 6       � Autoconfigure THIRD ENET P-series adapter        �

������������������������������������������������������������Ķ

�         �                                                  �

� 7       � Autoconfigure FOURTH ENET P-series adapter       �

������������������������������������������������������������Ķ

�         �                                                  �

� 8       � Driver configurable by Jumpers Utility           �

������������������������������������������������������������ͼ

Table 1: LAN Driver Configuration Options for NetWare v2.1x and

v2.2 IPX server driver

������������������������������������������������������������ͻ

�         �              �               �                   �

� Option  � IRQ          � I/O Address   � RAM Address       �

������������������������������������������������������������Ķ

�         �                                                  �

� 0       � Auto configuration                               �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 1       � 3            � 280h          � D000:0h           �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 2       � 5            � 300h          � CA00:0h           �

������������������������������������������������������������Ķ

�         �              �               �                   �

� 3       � 7            � 240h          � D800:0            �

������������������������������������������������������������Ķ

�         �                                                  �

� 4       � Driver configurable by Jumpers Utility           �

������������������������������������������������������������ͼ

Table 2: LAN Driver Configuration Options for IPX workstation

driver

For all "Jumpers" configurable drivers, users can execute

Jumpers utility to change the configuration parameters in the

drivers. After executing the utility, it prompts you to select

the type of network adapters. It is straightforward to configure

the driver if you are using M or U series network adapters.

If you select P-series network adapter, there are five

configuration parameters.

a) I/O PORT #1

It refers to the I/O base address.

b) IRQ #1

It is the IRQ number.

c) MEMORY #1

It specifies the RAM base address.

d) MODE

It support two choices, MANUAL and PROGRAM. If MANUAL

is selected, the user must ensure that the parameters (a)

and (b) MATCH the hardware settings of the network

adapters. If it is PROGRAM, the driver is configured to

"Program Configuration Mode". It will CHANGE the hardware

settings of a P-series network adapter to that specified by

the parameters (a) to (c).

e) PHYSICAL MEDIA

It defines the physical media. If it is set to

"DEFAULT", the driver will use preset physical media stored

in storage device (EEPROM) on the network adapter. If it is

one of the following selections.

:- TPI (Twisted Pair)

:- AUI (Thick Ethernet)

:- BNC (Thin Ethernet)

The driver will change it to the physical media

specified.

3.2) ODI Workstation Driver

Users can easily configure the drivers through the file,

NET.CFG. Our ODI workstation driver can support the following

configuration parameters.

a) PORT   (I/O base address)

b) INT    (IRQ number)

c) MEM    (RAM base address)

d) MODE   (drive configuration mode)

It is VALID for P-series network adapters ONLY. It

defines the configuration mode of the driver. The supported

values are:

:- AUTO

:- PROGRAM

If this parameter is NOT defined, the hardware

settings of the network adapters MUST match to the values

specified by the parameters PORT and INT. If it is AUTO,

the driver is configured to "Automatic Configuration Mode";

it reads all hardware settings from the storage device

(EEPROM) on the network adapter and the values for PORT,

INT and MEM are IGNORED. If it is PROGRAM, the driver is in

"Program Configuration Mode"; it changes the hardware

settings of the P-series network adapters to the values

specified by PORT, INT and MEM.

e) MEDIA  (physical media)

It is VALID for P-series network adapters ONLY. It

defines the physical media of the network adapter. If it is

NOT defined, the driver uses the preset physical media

stored in the storage device (EEPROM) on the network

adapter. It can be one of the following values.

:- TPI (Twisted Pair)

:- AUI (Thick Ethernet)

:- BNC (Thin Ethernet)

If it is one of the above values, a driver will change

the physical media of the network adapter to that specified

by this field.

3.3) ODI Server Driver

In order to support our P-series network adapters, there are

three additional command line options to configure the drivers.

Users can enter the options when loading the server driver.

LOAD ENET386.LAN [<Command Line Option>][<=>][<Parameters>]

There are three command line options supported by our server

driver. They are:

a) OMODE

It is same as the parameter, MODE, described in above

section, (3.2). Please refer to above section for more

details. It supports the following two parameters.

:- AUTO

:- PROGRAM

NOTE:

:- If OMODE=AUTO, the driver WILL NOT prompt users for I/O

base address, IRQ number and RAM address.

b) TMEDIA

It is same as the parameter, MEDIA, described in above

section, (3.2). Please refer to above section for more

details. It supports the following three parameters.

:- TPI    (Twisted Pair)

:- AUI    (Thick Ethernet)

:- BNC    (Thin Ethernet)

c) ADAPTERNUM

The parameter is added to support more than one P-

series network adapters working in a server. The supported

parameters are:

:- FIRST

:- SECOND

:- THIRD

:- FOURTH

If this command line option is defined to one of above

value (eg. SECOND), the driver will search all valid I/O

base addresses and locate all P-series network adapters.

Then, it will initialize the particular P-series network

adapter SPECIFIED by this parameter (In this case, it will

program the second P-series network adapter). ALL hardware

settings from the storage device (EEPROM) on this

particular P-series network adapter will be read.

NOTE:

:- Above command line options are VALID for P-series

network adapters ONLY.

NOTE:

:- User COULD refer to the section, LAN DRIVER CONFIGURATION

PARAMETERS, for more information.

���������������������������������������������������Ŀ

�                                                   �

�             MICROSOFT LAN MANAGER                 �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

The NDIS drivers v2.30 (930104) for OS/2 and DOS were

written to and tested with the Network Driver Interface

Specification (NDIS) Version 2.0.1 (Oct 8, 1990). Both of them

are modified from ENET NDIS drivers v2.0 which have received a

"Certificate of Conformance" from Microsoft Corporation. The

drivers can support M, U and P (both ISA and MCA) network

adapters.

2) INSTALLATION

The following installation procedures are provided as

general guidelines only.  Your exact installation requirements

may vary.

2.1) Creating an OEM Supplemental driver diskette

The driver diskette was prepared to be a OEM Supplemental

driver diskette. Users can use it to install NDIS drivers in both

LAN Manager workstation and server. Any OEM Supplemental driver

diskette should contain the following directory structure.

����������� Root ���������Ŀ

�                          �

MSLANMAN.OS2               MSLANMAN.DOS

�                          �

�� DRIVERS                 �� DRIVERS

�                          �

�� NIF                     �� NIF

�� ETHERNET                �� ETHERNET

�                          �

� ENET                     � ENET

There are three important files that are necessary for

proper driver installation.

a) ENET.NIF

This file is found under the subdirectory, "NIF" and

is a Network Information File.  It contains information

used by the LAN MANAGER SETUP program to install the NDIS

driver.

DO NOT CHANGE THE CONTENTS OF THIS FILE UNDER ANY

CIRCUMSTANCES!

b) PROTOCOL.INI

This file is located under the subdirectory, "ENET"

and is a sample initialization file. It holds the

hardware settings of the network adapter and all

necessary information for the driver to configure itself

to work in different environments.

c) ENET.DOS (or ENET.OS2)

This is the NDIS driver and is found in the

subdirectory, "ENET".

2.2) Installing the driver

2.2.1) LAN Manager 2.0

a) Start the SETUP program.

b) Choose "Actions" from the top manual bar.

c) Choose "Import network drivers" to load the driver.

Insert the OEM Supplemental driver diskette into a drive

and enter correct source drive letter. Then SETUP will

create the appropriate directory and transfer the driver

from the diskette.

After this step, the driver is loaded.  If you want to

change the existing driver, then you should go through the

following steps:

d) Choose "View/Modify" from the top manual bar.

e) Press [ENTER] until you find the "Network Drivers"

manual. In this manual, you will find the existing

network driver configuration and all network adapters and

protocols which it supports.

f) Remove the existing configuration by choosing

"Remove".

g) Select "Compex ENET Ethernet Adapter" in the "Network

adapter card" box and choose the appropriate protocol

layer in the "Protocol" box.

h) Choose "Add Driver" to add a hardware driver and "Add

Protocol" to add a protocol driver to the "Network

Configuration" box.

i) Then choose "Done" to exit to the main manual.

j) Before exiting the SETUP program, choose "Save" from the

"Actions" box to update the changes you have made.

2.2.2) LAN Manager 2.1

The setup procedures for LAN Manager 2.1 is similar to LAN

Manager 2.0.  Follow the procedure listed below to perform your

installation.

a) Start the SETUP program.

b) Choose "Configuration" from the top manual bar.

c) Choose "Network Drivers" to enter to the "Workstation

Configuration" box.

d) Choose "Add New Config" to check the existing network

adapter drivers.

e) Select "Other Driver" to import the driver from the

diskette.

Insert the OEM Supplemental driver diskette into a drive and

enter correct source drive letter. Then SETUP will create the

appropriate directory and transfer the driver from the diskette.

2.2.3) Windows for Workgroups Ver 3.1

a) You can use our driver diskette, or copy both

<root>\OEMSETUP.INF and DOS NDIS driver, which can be found

in <root>\MSLANMAN.DOS\DRIVERS\ ETHERNET\ENET, to the a

separate diskette for installation.

b) Start Windows for Workgroups

c) Choose "Control Panel" in the "Program Manager-[Main]"

window.

d) Select the "Network" icon.

e) A "Network Settings" window is displayed. Under the

option,"Options", please choose the button, "Adapters".

f) There is a window to show all network adapters in use.

Select the button, "Add", to add other adapters.

g) A list of available adapters is shown. If you can not

choose the adapter you want, please select the option,

"Unlisted or Updated Network Adapter", to add a new

adapter.

h) Insert the diskette with files, OEMSETUP.INF and DOS

NDIS drivers and then choose "OK".

i) The information of the adapter is displayed and choose

it if correct.

j) You are allowed to configure your driver when

configuration window for your adapter is displayed.

k) Select the button, "OK", if all configuration settings

are correct.

l) A new adapter is added to the list of available network

adapters.

m) Please refer to the section (3) for the definitions of

each input parameter in PROTOCOL.INI, which is found in the

directory of Windows for Workgroups.

2.2.4) Other installations

To install the NDIS driver for use with non-LAN MANAGER

based products, please follow the directions supplied with that

product.

3) DRIVER INITIALIZATION PARAMETERS

These parameters are passed to the driver through a file

named PROTOCOL.INI.  The PROTOCOL.INI file contains information

used to initialize and configure NDIS Drivers and protocol

stacks. The PROTOCOL.INI file provided on the OEM Driver Disk is

a sample file.  It is used by LAN MANAGER during driver setup to

add the appropriate NDIS configuration options to the "Master"

PROTOCOL.INI file on your system.  Under no circumstances should

you directly copy the sample PROTOCOL.INI file over your "Master"

PROTOCOL.INI file.

The PROTOCOL.INI for our drivers can support the following

parameters.

For ISA network adapters:

a) DRIVERNAME

b) IOBASE

c) INTERRUPT

d) RAMADDRESS

e) MODE

f) MEDIA

g) MAXTRANSMITS

For MCA network adapters:

a) DRIVERNAME

b) MEDIA

c) SLOT

d) MAXTRANSMITS

a) The DRIVERNAME field MUST be set to ENET$.  This field

is used by the Protocol Manager to bind the driver to a

protocol stack.  This entry is mandatory.

b) IOBASE is the I/O base address of network adapter. This

value is specified in hex only.  The default value is

0x280. Valid range of values is 0x260-0x340 in increments

of 0x20.

c) INTERRUPT defines the hardware interrupt vector used

by the ENET network adapter.  The default value is 3.

This option may also be specified as IRQ.  The value may

be specified in either decimal or hexadecimal. If

specified in hexadecimal, then it must be preceded by 0x,

for example:  0x03.  Valid selections for an 8-bit board

are: 2, 3, 4, 5, 7.  Valid selections for a 16-bit board

are: 2, 3, 4, 5, 7, 10, 11, 15.

d) RAMADDRESS is the base address of system memory space

where the driver will map its shared memory.  Only four

significant digits are specified. The default hexadecimal

value is 0xD000.  This value is specified in hex only.

Valid range of values is 0xB000:0-0xEE00:0 in 8K (0x200:0)

increments.

e) MODE is VALID for P-series network adapters ONLY. It

defines the configuration mode of the device driver. If

this field is NOT defined, the hardware settings of the

network adapters MUST match the values specified by IOBASE

and INTERRUPT.

If MODE is AUTO, the driver will be configured to the

"Automatic Configuration Mode" and will read all hardware

settings from the storage device (EEPROM) on the network

adapter. Moreover, the values for IOBASE, INTERRUPT and

RAMADDRESS are IGNORED.

If MODE is PROGRAM, the driver will be in the "Program

Configuration Mode" and will change the hardware settings

of the P-series network adapters to the values specified by

IOBASE, INTERRUPT and RAMADDRESS.

f) MEDIA is VALID for P-series network adapters ONLY. It

defines the physical media that will be used by the network

adapter. If it is NOT defined, the driver use the preset

physical media stored in the storage device (EEPROM) on the

network adapter. It can be one of the following values.

:- TPAIR (Twisted Pair)

:- THICK (Thick Ethernet)

:- COAXIAL (Thin Ethernet)

If it is one of above values, the driver will change

the physical media of the network adapter to that specified

by this field.

g) MAXTRANSMITS option can be used to performance tune

the driver.  The value can be specified in either decimal

or hexadecimal.  Valid range is 1 to 50.  Default value

= 10. The value specified directly affects the number of

transmit buffers allocated when the driver is initially

loaded. It specifies the number of transmission requests

from protocol stacks which may be queued by the driver.

If the value is too small, the network performance will

decrease.  Too large a value will waste memory but not

hinder performance.  A value that is larger than

necessary may not increase performance.

h) SLOT is VALID for Microchannel network adapters ONLY. It

defines the MCA slot in the workstation. If it is NOT

defined, the driver searches all MCA slots in the

workstation and initializes the FIRST Microchannel network

adapter.

If it is defined, the driver will initialize the

network adapter in that particular slot ONLY. The valid

range is from 1 to 8.

NOTE:

:- If any option is omitted from the PROTOCOL.INI file, the

default value will automatically be used.  The only

exception to this is the DRIVERNAME field which MUST ALWAYS

be specified.

������������������������������������������������Ŀ

�                                                �

�               FTP SOFTWARE                     �

�                                                �

��������������������������������������������������

1) INTRODUCTION

Packet driver v1.00 (930104) for DOS environment was written

to conform to the "Packet Driver Specification v1.09" developed

by FTP Software Inc. The driver can support our M, U and P (both

ISA and MCA) network adapters.

2) INSTALLATION

You should choose the correct version of FTP Software

package (eg. PC/TCP Version 2.05 for DOS), that supports

"generic" Ethernet version of PC/TCP. Then you can go through the

following steps to install the driver.

a) Follows the PC/TCP's installation guide to install

PC/TCP onto your system.

b) Copy the Packet driver onto the directory

containing PC/TCP files.

c) Configure both Interface Software Driver and

Protocol Driver

You should use utilities to configure PC/TCP

network parameters. IPCONFIG.EXE is designed to

configure the Internet Protocol device driver,

IPCUST.SYS. Similarly, you can configure IFCUST.SYS to

customize the Interface device driver by IFCONFIG.EXE.

Please refer to the manual for further information.

d) Alter the file, CONFIG.SYS, which resides in the

root directory of your startup disk, to install two

device drivers, IPCUST.SYS and IFCUST.SYS. Insert the

following two lines.

DEVICE=[<DRIVE>]\[<PATH>]\IPCUST.SYS

DEVICE=[<DRIVE>]\[<PATH>]\IFCUST.SYS

NOTE: [<DRIVE>]\[<PATH>] refers to the full pathname

of the directory containing the two device drivers.

e) Re-boot your system to load these configuration

files.

f) Load the hardware-dependant device driver included

in the driver diskette for the network adapter. Please

refer to the section (3) to configure the network

adapter device driver.

g) Before running PC/TCP applications, please load the

Generic Kernel (ETHDRV.EXE), which will exchange

information with the network adapter device driver.

h) Now, you can start any PC/TCP applications.

3) DRIVER INSTALLATION PARAMETERS

You can pass the configuration parameters to our Packet

Driver by the command line options.

ENETPKT.COM [</>][<Command line option>][<Parameters>]

For ISA network adapters, the driver supports the following

command line options.

a) ?

The driver displays the help screen.

b) I[<Packet Int Number>]

This specifies the packet driver software

interrupt number. The default value is 0x60. Valid

range is from 0x60 to 0x80.

NOTE:

:- Please consult the PC/TCP manual before using software

interrupt number.

c) B[<I/O Base Address>]

It specifies the I/O base address of ETHER PLUS

network adapter. The default value is 0x280. Valid

range of values is 0x200-0x3E0 in increments of 0x20.

d) Q[<IRQ Number>]

You can specify the hardware interrupt level used

by a network adapter. The default value is 0x3. For a

8-bit network adapter, the valid options are: 2, 3, 4,

5, 7. For a 16-bit network adapter, the valid options

are: 2, 3, 4, 5, 7, 10, 11, 15.

e) R[<RAM Base Address>]

This specifies the base address of system memory

space where the driver will map its shared memory. The

default is 0xD000. Valid range of values is 0xB000:0 -

0xEE00:0 in 8K (0x200:0) increments.

f) D[<Configuration Mode>]

It is VALID for P-series network adapters ONLY. It

defines the configuration mode of the device driver. If

this field is NOT defined, the hardware settings of a

network adapter MUST match the values specified by <I/O

Base Address> and <IRQ Number>.

If it is "A", the driver will be configured to the

"Automatic Configuration Mode" and will read all hardware

settings from the storage device (EEPROM) on the network

adapter. Moreover, the values for <I/O Base Address>, <IRQ

Number> and <RAM Base Address> are IGNORED.

If it is "P", the driver will be in the "Program

Configuration Mode" and will change the hardware settings

of the P-series network adapters to the values specified by

<I/O Base Address>, <IRQ Number> and <RAM Base Address>. It

supports two parameters:

:- A (Automatic Configuration Mode)

:- P (Program Configuration Mode)

g) M[<Physical Media>]

It is VALID for P-series network adapters ONLY. The

driver will change the adapter to use the physical media

specified by this parameter. It supports three different

parameters.

:- P (Twisted Pair)

:- T (Thick Ethernet)

:- C (Thin Ethernet)

If it is NOT defined, the driver use the preset

physical media stored in storage device (EEPROM).

For MCA network adapters, the driver only provides the

following command line options.

a) ?

The driver displays the help screen.

b) I[<Packet Int Number>]

This specifies the packet driver software

interrupt number. The default value is 0x60. Valid

range is from 0x60 to 0x80.

c) M[<Physical Media>]

Same as the option (g) described above.

d) S[<MCA Slot Number>]

Normally, the driver scans all MCA slots and controls

the first P-series network adapter. If this option is

defined, the driver will initialize the adapter at the slot

specified by this option. It's valid range is from 1 to 8.

NOTE:

:- All command line options are case-insensitive. That means the

device driver accepts both uppercase and lowercase characters.

:- All values can be either decimal (eg. 12) or hexadecimal (eg.

0xC). All hexadecimal values must be preceded by 0x.

���������������������������������������������������Ŀ

�                                                   �

�                     UTILITY                       �

�                                                   �

�����������������������������������������������������

ENETUTIL.EXE is our new release configuration and diagnostic

utility. By using different command options, the utility can be

started either in diagnostic menu or configuration menu. With the

modifications we made, the diagnostics can test more operating

modes for M, U and P (ISA ONLY) series network adapters. The

configuration utility can be used to software configure our P-

series network adapters. Please refer to the file, HELP.DOC, for

the detailed descriptions of the utility.

Provided by:  Compex Inc

4055, La Palma

Unit C, Anaheim

CA 92807, USA