RELEASE NOTE

NetWare(R) MHS(tm) Network Edition

----------------------------------

Software v1.5N Revision B

-------------------------

The NetWare MHS Network edition software uses the Standard Message

Format (SMF) to provide DOS-based messaging services on NetWare

networks.  NetWare MHS transfers information containing addresses, text,

and other data from an MHS-compatible application or gateway to

applications and gateways at other locations.  MHS uses a

store-and-forward communications method, which transports messages

across a broad range of data-communications networks.  The NetWare MHS

Network edition stores messages and forwards them to other locations or

applications.  Other applications provide services - such as electronic

mail - that allow users to create and process messages.

NetWare MHS v1.5 incorporates new features and an improved user

interface for installation and administration, and it uses a revised

version of the Standard Message Format (SMF) for exchanging electronic

messages.

If you need connectivity between NetWare MHS and CompuServe, either to

use the CompuServe hub service, or to send mail from MHS to CompuServe

subscribers, obtain the MHS 1.5 upgrade option from CompuServe.  If you

have a CompuServe subscription you can obtain it by signing on to

CompuServe and entering "GO MHS" at the CompuServe Main Menu screen.

For more information, call CompuServe at (614) 457-8600.

Please read this entire release note and Chapter 2 of the NetWare MHS

Installation and Operations Guide before installing the NetWare MHS

software.

If you have any questions or comments, please contact:

Customer Support Response Center

Novell, Inc.

2180 Fortune Drive

San Jose, CA 95131

If you need information about the Novell mail hub (NHUB) or about

registering your workgroup, call the Messaging Hotline at the following

number:

(408) 473-8989

To register your workgroup, you can send an MHS mail message to

hubadmin@NHUB.	In the message, include your workgroup name, your

contact phone number, and the name of the contact person in your

organization.

If you need technical support, call one of the following numbers:

(800) NETWARE

(801)429-5588

For other information, send a fax to the attention of the MHS Product

Manager at the following number:

(408) 433-0775

Components

----------

The NetWare v1.5N Rev. B upgrade consists of these components:

o   UPGRADE.EXE, the program you execute to perform the upgrade.

o   EXE.EXE and SYS.EXE, which are invoked by the UPGRADE program (do

not execute them).

o   README.DOC, this release note.

System Requirements

-------------------

NetWare MHS Rev.  B (Network edition) requires the following hardware

and software:

o   NetWare MHS v1.5N Rev. A

o   Minimum of 200 free directory entries

NetWare MHS v1.5 Rev.  B has the same hardware requirements as NetWare

MHS v1.5 Rev.  A.

***********************************************************************

*				Warning 			      *

*								      *

*   A small number of third-party utility programs change internal    *

*   MHS configuration information and queue headers contained in      *

*   NETMGR.QUE.  NetWare MHS does not support these changes; they     *

*   will cause an unrecoverable error when you run NetWare MHS v1.5   *

*   Rev.  B.  Contact the manufacturer of your MHS utility, to find   *

*   out if your utility is supported.  If it is not, please remove    *

*   it from your system before you upgrade to NetWare MHS v1.5 Rev.   *

*   B.								      *

***********************************************************************

NetWare MHS v1.5 Rev. B Compatibility

-------------------------------------

NetWare MHS v1.5 Rev.  B is fully compatible with NetWare v1.5 Rev.  A,

v1.5C, and v1.1.  It is not compatible with third-party MHS utilities

that make unsupported changes to NETMGR.QUE.

Upgrading from NetWare MHS V1.5

-------------------------------

To upgrade your current version of NetWare MHS to NetWare MHS v1.5N Rev.

B, follow these steps:

1.  Make sure that MHS has delivered all pending mail.

2.  At the MHS host, halt all gateways, MHS applications, and the

Connectivity Manager, if they are currently running. To halt the

Connectivity Manager, press <Esc>.

3.  Back up the existing SYS, MAIL and EXE subdirectories.

4.  Log in as Supervisor or Supervisor equivalent on the NetWare server

on which MHS is installed.

5.  Set the MV environment parameter to point to the path containing

the MHS subdirectory. For example, if NetWare MHS is installed in

SERVER1/SYS:MAIL, enter the command "SET MV=SERVER1/SYS:MAIL".

6.  Change to the directory containing the UPGRADE.EXE program, and

run the Upgrade program by typing:

UPGRADE <Enter>

The Upgrade program prompts, 'Would you like to view the README file

(Y/N)?'

7.  To display the README file, type "Y". If you have already read the

README file, type "N".

The Upgrade program prompts, 'Confirm that the path to the MHS

files is the one you wish to upgrade (path). Is it OK to continue

(Y/N)?'

8.  Type "Y" if the path displayed in the prompt is the path containing

the MHS files and programs. The Upgrade program upgrades your

NetWare MHS installation to Revision B.

Type "N" if the path is incorrect.  The Upgrade program terminates.

Set the MV environment parameter, and begin again at step 6.

Documentation Corrections

-------------------------

This section provides updates and corrections to the NetWare MHS

Installation and Operation Guide.

Passwords for Asynchronously-Connected Hosts

--------------------------------------------

The NetWare Installation and Operation Guide incorrectly describes how

to define passwords for hosts with which you communicate asynchronously.

You can define a Password for access to hubs in the MHS HOST SETUP form.

This is the password that the local host transmits when it communicates

with a remote host.  (It transmits the same password to all the remote

hosts with which it communicates.)

For each remote host the local host calls, and for each remote host that

calls the local host, you can define a Password (in the "A HOST THAT

THIS HOST PHONES" and the "A HOST THAT PHONES THIS HOST" forms,

respectively).	This Password must match the password defined in the

Password for access to hubs field in the remote host's HOST SETUP form.

If you do not define the remote host's password at your local host, MHS

will take the password transmitted by the remote host in the next

communication session, and write it in the Password field.

Once a password has been assigned to a remote host, the remote host

administrator must notify you of changes to the password.  When the

remote administrator changes the password in the Password for access to

hubs field, the local administrator must change the Password field in

the "A HOST THAT THIS HOST PHONES" or the "A HOST THAT PHONES THIS HOST"

form for the remote host.  Otherwise, if Require Password is set to

"Yes," a password mismatch will occur, and the hosts will be unable to

establish a communication session.

Similarly, when you change the password in the Password for access to

hubs field, you must ask the administrator of the remote host to update

the Password in the form for your host.

To understand how passwords work, consider "Eng," a host that calls

another host named "Sales." Sales has chosen "salpas" as its Password

for access to hubs, and Eng has chosen "engpas" as its Password for

access to hubs.  These passwords are defined in the HOST SETUP form.

The administrators at Sales and Eng can get together and coordinate

passwords.  Sales' administrator enters "engpas" in the Password field

of the "A HOST THAT THIS HOST PHONES" form that defines the route to

Eng.  And Eng's administrator enters "salpas" in the Password field of

the "A HOST THAT PHONES THIS HOST" form that defines the route to Sales.

Both administrators can also set Require Password to Yes.

One or both administrators can choose not to define the password for the

other host.  If they leave the Password for the remote host blank, MHS

will insert the password it receives during the next communication

session.  During this session, Eng's MHS transmits its password.  Sales'

MHS enters the password received from Eng in its routing table, and send

its password to Eng's MHS.  Eng's MHS enters the Sales password in its

routing table, and continues the transmission.

From now on, if either administrator changes its Password for access to

hubs, they must ask the other administrator to change the Password field

in the "A HOST THAT THIS HOST PHONES" or "A HOST THAT PHONES THIS HOST"

form for the remote host.

96V32 and 96V32HUB Modem Driver

-------------------------------

The 96V32 and 96V32HUB are not drivers for the Hayes

Smartmodem, as stated in the NetWare MHS Installation and

Operation Guide. They are drivers for the Hayes V-series ULTRA

modem.

Features of NetWare MHS v1.5 Rev. B

-----------------------------------

NetWare MHS v1.5N Rev.	B incorporates the following new and revised

features:

o   Improved Monochrome Support. Several new command-line switches

have been added for use with monochrome monitors.  They are the

following:

-M	  Disables color on IBM PS/2s and monochrome VGA displays.

-R	  Displays the currently selected item in reverse video on

laptops that do not display bold text.

-U	  Displays the currently selected item as underlined text on

laptops that do not display bold text.

The -M, -R and -U switches are mutually exclusive.  To display

selected attributes in reverse video, use the command MHS -R.

o   Faster "Copy a List" Processing. The "Copy a list to a file" process

is faster.

o   Recovery Counter Reset. The Abort/Retry counter is reset after a

successful connection.

o   Retry Counter Reset.  The Retry Counter resets after a successful

connection to another host.

o   Support for Modems That Use One Stop Bit. To support new modems

with software UARTs that use only one stop bit for communication,

NetWare MHS uses only one stop bit. It also supports modem

definitions with large command sets.

o   Support for Additional Modem Drivers. In addition to the modem

drivers listed in Appendix C of the NetWare MHS Installation and

Operation Guide, NetWare MHS now supports the following modem

drivers.

2400B	 For Hayes Smartmodem 2400 and Hayes-compatible modems

operating at 2400 baud.  Use for calling hosts.

2400B	 For Hayes Smartmodem 2400 and Hayes-compatible modems

operating at 2400 baud.  Use for both hubs and calling

hosts.

96V32	 For Hayes V-Series ULTRA 9600 modems.	Use for calling

hosts.

96V32HUB	 For Hayes V-Series ULTRA 9600 modems.	Use for both

hubs and calling hosts.

BULLET96	 For E-Tech BulletModem E9696M modems without MNP.

BULLETEC	 For E-Tech BulletModem E9696M modems with MNP.

DISABLED	 To disable asynchronous activity.  For hosts that do

not exchange messages with remote hosts through

telephone calls.

FAXMDM	 For generic SendFax-compatible modems.

HAYESMAN	 For Hayes-compatible modems that require telephone

numbers to be dialed manually.

HAYES-UN	 For Hayes-compatible modems operating at speeds up to

2400 baud. Use for both hubs and calling hosts.

HAYESX24	 For generic Hayes-compatible 2400 baud modems.

HAYESSM	 For Hayes Smartmodem 2400 modems.

HSTD-HUB	 For US Robotics COURIER HST modems with MNP. Use for

both hubs and calling hosts.

HSTDNMNP	 For US Robotics COURIER HST modems without MNP.

INTELFAX	 For the Intel SatisFAXtion board.

INTELV32	 For Intel 9600EX modems.

MICROCOM	 For Microcom QX4232HS modems.

NULLMDM	 For hosts with a direct cable connection between their

communication ports.  You can use NULLMDM as a platform

for developing modem-control files.

PP96-HUB	 For Practical Peripherals PM9600SA modems. Use for

both hubs and calling hosts.

This list supersedes the one in Appendix C of the NetWare MHS

Installation and Operation Guide.

Note: If you want to use one of these modem drivers, refer to the

MODEM.DOC file in the SYS directory for installation instructions.

o   SMFUTIL.DOC Updated. The online documentation for SMFSEND is more

thorough.

o   SEND-TO Support. When an SMF version 70 gateway delivers a

message to MHS, MHS will not regenerate the SEND-TO field if a

valid SEND-TO field exists.

o   Support for Extended Addresses. While MHS v1.5 Rev. B does not

use extended address to route messages, it now examines extended

address elements in braces ({ }) when checking for duplicate

addresses.

For example, a message contains the following addresses:

mail@fax {FAX: 1-408-555-1212}

mail@fax {FAX: 1-408-555-1211}

Previously, MHS would compare only the base address (mail@fax).

Since the base address is the same in both instances, MHS v1.5 would

only deliver one message, containing the first address, to the fax

gateway.  MHS 1.5 Rev.  B recognizes the two addresses as distinct,

and delivers two copies to the fax gateway.  Note that MHS does not

interpret the contents of the braces; it only knows they are

different.	Thus, if in the above example, the addresses were:

mail@fax {FAX: 1-(408)-555-1212}

mail@fax {FAX: 1-408-555-1212}

then MHS would send two copies.

This change supersedes the information in the SMF Programmer's

Reference.

o   Support for NetWare 386 Encrypted Passwords.  NetWare MHS supports

password encryption on 386 file servers.  If you turned off password

encryption in your AUTOEXEC.NCF file when you installed a previous

version of NetWare MHS, you now have the option to turn it back on.

General Restrictions

--------------------

The following general restrictions apply to networks running NetWare MHS

v1.5:

o   Monochrome Monitor Problems. The MHS screens may be difficult or

impossible to read on certain VGA monochrome monitors.  If you

experience this problem, use the following MODE command at the DOS

prompt before running MHS:

MODE=bw80

o   Color Monitor Problems. If the colors on an application display

incorrectly after you run MHS, use the following MODE command at the

DOS prompt before running your application:

MODE=co80

o   Default Primary Host. In new installations, the default primary

host for non-routing internet hosts is NHUB.  If you do not change

this, no mail will be delivered to or routed from the non-routing

host.

o   Continuous Internet Retries.  Under some circumstances, the

Connectivity Manager may appear to have stopped working.  This

occurs because it may retry 25 times after waiting for the IPX-retry

interval for your NetWare installation (20 seconds by default) for

each retry.  At the maximum, the Connectivity Manager waits

continuously for 8 minutes and 20 seconds if it has continual and

successive retries.  After this period, MHS recovers and restarts

the Connectivity Manager.

o   Inaccurate Estimated Transfer Time. MHS attempts to estimate the

amount of time an asynchronous transfer will take.	It often

estimates incorrectly, especially for high-speed modems.

o   Memory Allocation Error.  If the NetWare MHS workstation has a

limited amount of available memory, and a large number of files

accumulate in the SND directory, a memory allocation error will

occur.  Delete any *.BAD files in the SND directory and, if

possible, terminate memory-resident programs sharing the MHS

workstation.  Do not run MHS from a shell.

o   The MAIL Variable. While it is possible in the Network Edition

to specify the MV variable for one drive and the MAIL for another,

both must be mapped network drives.  You cannot specify any local

drives.  Appendix A of the MHS 1.5 Installation and Operations Guide

incorrectly implies that you can use a local drive.

o   Gateway Version Number.  MHS sometimes sets the default version

number of a gateway to 61.	Please ensure you set the version number

to either 64 or 70.  Read your gateway's documentation or call your

gateway supplier if you do not know which version number to use.

Trademarks

----------

Novell, the N design,  and NetWare are registered trademarks and NetWare

MHS is a trademark of Novell, Inc.

CompuServe is a registered trademark of CompuServe, Inc.  Da Vinci

Systems is a registered trademark of Da Vinci Systems Corp.  E9696M and

BulletModem are trademarks of E-Tech, Inc.  Hayes and V-series are

registered trademarks and ULTRA and Smartmodem are trademarks of Hayes

Microcomputer Products, Inc.  Intel is a registered trademark and

SatisFAXtion is a trademark of Intel Corporation.  Microcom and MNP are

registered trademarks of Microcom, Inc.  Practical Peripherals is a

registered trademark and PM9600SA is a trademark of Practical

Peripherals, Inc.  USRobotics is a registered trademark and COURIER,

HST, and ASL are trademarks of U.S.  Robotics, Inc.

*********************** END OF RELEASE NOTE ***************************