Lan Server/Lan Manager and Novell Server conectivity for OS/2 1.3 Workstations

------------------------------------------------------------------------------

On a recent project there was a need for connectivity to Netware and Lan

Manager servers at the same time.  After many trial and error attempts success

was achieved, inspite of the lack of help MicroSoft.

I have documented here the setup that will achieve the above.  All files can

be found on CompuServe for Novell, ie. NSD004.ZIP.  As for OS/2 and Lan

Server, IBM suppliers can help.

Our configuration of servers includes Novell and MicroSoft Lan Manager

servers.  The main DLL for both pieces of software is the NETAPI.DLL.

Unfortunately each requester, Novell/Lan Manager, does not work with the other's

NETAPI.DLL, and only one NETAPI.DLL can be loaded on the workstation at any

given time.

IBM Lan Server does not use the NETAPI.DLL and is compatible with Lan

Mananger.

Lan Server mostly solved my problems bar the need for two token-ring cards,

though I believe has since been solved for OS/2 1.3 using ODINSUP.SYS.

Some drivers for Lan Server and Netware do not work together unless the

version of OS/2 is 1.30.1, CSD 5015 or higher.

As I had so much trouble achieving the connectivity I wanted, I thought that

this document might help some other unfortunate people trying connect Netware

to Lan Server/Lan Manager.

Regards

Kevin J McCarthy [70374,3540]

24 January 1993

c/o Dun & Bradstreet Europe

High Wycombe

England

AIM

---

The aim of this excercise is to connect an OS/2 1.3 workstation to the

following types of server at the same time:

IBM Lan Server

Microsoft Lan Manager

Novell Netware

This example is for an IBM Token-Ring card.  For other cards that are

supported by both IBM and Novell the relevant driver SHOULD be OK as the

connectivity is achieved at a different layer of the software.

The requirements are:

OS/2 1.30.1, CSD 5015.

Novell Netware v3.11 (with NSD level 4 for named pipes to work)

IBM Lan Server v2.0.

2 Token-Ring cards.

The file dates and sizes for NSD level 4 Netware are:

LSL.SYS        16944    19-11-91 8:22a

DDAEMON.EXE     8496    19-11-91 8:22a

TOKEN.SYS      25392    13-12-91 4:54p

ROUTE.SYS      33200    25-06-91 9:53p

IPX.SYS        10416    22-11-91 8:34p

NWREQ.SYS      17888     9-04-91 4:09p

NWIFS.IFS      34720    10-04-92 1:15p

NWDAEMON.EXE   25168    10-04-92 1:16p

INSTALLATIONS

-------------

Install IBM Lan Server and ensure that the requester is working OK.

Install Novell Netware, on installation specify drives as:

NETWARE\DRIVERS

NETWARE\NETPROG

NETWARE\DLL

This is to keep similarities between Lan Server and Netware file layout.

Check the config.sys.  The Lan Server paths must appear first in all statements

ie. PATH, LIBPATH, DPATH etc...

NET.CFG modifications

---------------------

By using a reference diskette, find out the address, DMA of the Token-Ring

cards.  On IBM machines the Primary adapter is 0A20 and the Alternate adapter

is 0A24.  These values will be used to set the Netware to use the alternate

adapter.

Edit a NET.CFG file as follows and place it ind the DPATH directory stated in

the CONFIG.SYS:

link driver token

port 0A24

Where TOKEN is the file name of the card driver ie. TOKEN.SYS

If the port address is not known then try:

link driver token

DMA   x

Where x is the DMA number of the alternate token-ring card.

There are problems with the Netware reading the NET.CFG file correctly.  If

on rebooting the Netware requester uses the wrong port, interrupt & DMA etc..

then re-edit the NET.CFG with a different editor.  The Netware software appears

to be tab sensitive.

If the NET.CFG file is read correctly by the Netware software both requesters

should start and work together.

CONFIG.SYS

----------

The ordering of the config,sys for loading of drivers etc. is as follows:

REM ********* HPFS *********

IFS=C:\OS2\HPFS.IFS /CACHE:64 /AUTOCHECK:CD

PROTSHELL=C:\OS2\PMSHELL.EXE C:\OS2\OS2.INI C:\OS2\OS2SYS.INI C:\OS2\CMD.EXE

SET COMSPEC=C:\OS2\CMD.EXE

LIBPATH=.;c:\os2\dll;c:\;c:\muglib\dll;c:\ibmlan\netlib;c:\ibmcom\dll;c:\netware\dll;

set path=c:\os2;c:\os2\system;c:\os2\install;c:\;d:\ibmlan\netprog;c:\muglib;d:\netware\netprog;

SET DPATH=C:\OS2;C:\OS2\SYSTEM;C:\OS2\INSTALL;C:\;c:\IBMLAN\NETPROG;c:\IBMLAN;C:\MUGLIB;c:\NETWARE\NETPROG;

SET PROMPT=$i[$p]

SET HELP=C:\OS2\HELP;D:\C600\HELP;

BUFFERS=30

DISKCACHE=64

MAXWAIT=3

MEMMAN=SWAP,MOVE,SWAPDOS

PROTECTONLY=NO

SWAPPATH=C:\OS2\SYSTEM 512

THREADS=255

COUNTRY=044,C:\OS2\SYSTEM\COUNTRY.SYS

DEVINFO=SCR,VGA,C:\OS2\VIOTBL.DCP

SET VIDEO_DEVICES=VIO_IBMVGA

SET VIO_IBMVGA=DEVICE(BVHVGA)

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\IBMMOU02.SYS

DEVICE=C:\OS2\MOUSE.SYS TYPE=IBMMOU$

DEVICE=C:\OS2\PMDD.SYS

DEVICE=C:\IBMCOM\LANMSGDD.OS2 /I:C:\IBMCOM

DEVICE=C:\IBMCOM\PROTMAN.OS2 /I:C:\IBMCOM

DEVICE=C:\IBMCOM\PROTOCOL\LANDD.OS2

DEVICE=C:\IBMCOM\PROTOCOL\LANDLLDD.OS2

SET KEYS=ON

SHELL=C:\OS2\COMMAND.COM /P

BREAK=OFF

FCBS=16,8

RMSIZE=640

DEVICE=C:\OS2\EGA.SYS

DEVICE=C:\OS2\DOS.SYS

DEVINFO=KBD,UK166,C:\OS2\KEYBOARD.DCP

CODEPAGE=850,437

DEVICE=C:\OS2\COM02.SYS

DEVICE=C:\IBMCOM\PROTOCOL\NETBEUI.OS2

REM --- NetWare Requester statements BEGIN ---

DEVICE=D:\NETWARE\DRIVERS\LSL.SYS

RUN=D:\NETWARE\NETPROG\DDAEMON.EXE

DEVICE=D:\NETWARE\DRIVERS\TOKEN.SYS

DEVICE=D:\NETWARE\DRIVERS\ROUTE.SYS

DEVICE=D:\NETWARE\DRIVERS\IPX.SYS

DEVICE=D:\NETWARE\DRIVERS\NWREQ.SYS

IFS=D:\NETWARE\DRIVERS\NWIFS.IFS

RUN=D:\NETWARE\NETPROG\NWDAEMON.EXE

REM --- NetWare Requester statements END ---

DEVICE=D:\IBMLAN\NETPROG\RDRHELP.SYS

IFS=D:\IBMLAN\NETPROG\NETWKSTA.SYS /I:D:\IBMLAN /N

DEVICE=C:\IBMCOM\PROTOCOL\NETBIOS.OS2

DEVICE=C:\IBMCOM\MACS\IBMTOK.OS2

RUN=C:\IBMCOM\PROTOCOL\LANDLL.EXE

RUN=C:\IBMCOM\PROTOCOL\NETBIND.EXE

RUN=C:\IBMCOM\LANMSGEX.EXE

IOPL=YES

SET BOOKSHELF=D:\IBMLAN\BOOK;

If you have any queries then please contact me via CompuServe.

Regards

Kevin J McCarthy [70374,3540]

24 January 1993

c/o Dun & Bradstreet Europe

High Wycombe

England