Memory.DOC

1. You can load part of WSLAN into Upper Memory Blocks (UMB) by entering

the appropriate DEVICE statements into the CONFIG.SYS file.

NOTE: Providing the capability to move device drivers into Upper Memory by

specifying DOS=UMB in CONFIG.SYS excludes WSLAN from Upper Memory.  For

this reason, DOS=UMB is remarked out with REM in the examples below.

UMB, are segments of memory from 640KB to 1024KB.

> For Ethernet Networks and QRAM on 286 machines running MS-DOS 3.31

Enter in CONFIG.SYS:

DEVICE=C:\QRAM\QRAM.SYS R:1

DEVICE=EMM.SYS AT 248

> For Ethernet Networks and EMM386 on MS-DOS 5.0.

Enter in CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE RAM

REM DOS=UMB

DOS=HIGH

> For Ethernet Networks and CEMM on COMPAQ's DOS 5.0.

Enter in CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.EXE

DEVICE=C:\DOS\CEMM.EXE RAM

REM DOS=UMB

DOS=HIGH

> For Ethernet Networks and QEMM386 on MS-DOS 5.0. Enter the following

information in CONFIG.SYS:

DEVICE=C:\QEMM\QEMM386.SYS RAM

REM DOS=UMB

DOS=HIGH

> For Ethernet Networks and 386MAX on IBM's DOS 5.0. Enter the following

information in CONFIG.SYS:

DEVICE=C:\386MAX\386MAX.SYS RAM PRO=C:\386MAX\386MAX.PRO

REM DOS=UMB

DOS=HIGH

> For Ethernet Networks and EMM386 on DR DOS 6.0.

Enter in CONFIG.SYS:

DEVICE=C:\DRDOS\EMM386.SYS /BDOS=NONE

> For Token Ring Networks and EMM386 on MS-DOS 5.0.

Enter in CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE RAM X=CC00-CDFF X=D800-DBFF

REM DOS=UMB

The Token Ring IPX driver version 2.63 uses upper memory from CC000 to

CDFFF and D8000 to DBFFF.  Exclude this area of memory with the X=

parameter when using EMM386.EXE or CEMM.EXE.  For other token ring drivers,

consult your memory management manual for information on how to exclude

areas of memory.

2.  DISABLING THE UPPER MEMORY LOAD FEATURE

You can load WSLAN into conventional memory (0 to 640 KB), without having

to change CONFIG.SYS, by disabling the Upper Memory Load feature.

Add /UN to the WSLAN command, for example:

WSLAN config_filename /UN

config_filename is the name of the configuration file you want to use.

The configuration file is either WSLAN.CFG or a file that was created

previously.  (You need not include the .CFG filename extension on the

command line.)  This command instructs the workstation to load WSLAN into

conventional memory.