Thomas-Conrad NetWare Lite Drivers

----------------------------------

MAKE BACKUP COPIES OF ALL THE DISKETTES SUPPLIED WITH NETWARE LITE.

At the completion of this batch file, It will be possible to select

Thomas-Conrad adapter drivers from within the INSTALL utility supplied with

NetWare Lite.

NetWare Lite is supplied on either 3.5 inch or 5.25 inch diskettes.  The 3.5

inch version of NetWare Lite occupies one diskette and the 5.25 inch version

occupies two diskettes.  This modification is applied against the 3.5 inch

diskette labeled PROGRAM or the 5.25 inch diskette labeled DRIVER.

Insert a BACKUP COPY of the diskette to be modified and enter:

TCCLITE DRIVE:

Where DRIVE is the diskette drive containing the appropriate diskette.

Example:

TCCLITE A: