Program: Epson Stylus Colour 600/800 driver

Author:  Peter Hutchison

Date:    19/06/98

Version: 1.3

Requirements: Any Amiga, WB 1.3 or better

Introduction

------------

This is my fifth driver to be released . This is a colour printer driver for

the 600 series of the Epson Stylus Colour printers. It should also work with

the Stylus 800 which also supports 1440 dpi.

Features

--------

Colour: CMYK (Colour AND Black)

Resolution: 180 to 1440 dpi (using Microdot).

Supports Microweave in Letter Quality mode.

Aspect:  Portrait and Landscape

Output:  B&W and Colour Text,

B&W, greyscale and colour graphics

Optimised for Speed

Compiled with SAS C 6.58

Densities

---------

0  180 x 180

1  180 x 180

2  360 x 360

3  360 x 360

4  720 x 720

5  720 x 720

6 1440 x 720

7 1440 x 720

Paper: 10in for most sizes, 14in for US LEGAL

Recommend densities 0-3 for Word Processing output.

Supported Printers

------------------

Epson Stylus 600, 800

(will support other Stylus' as well but not using higher resolutions).

Installation

------------

Drag the driver to Devs/Printers and select it with the Printer preferences

program. Single for Paper Type and DIN A4 for Paper Format and set

lines to 66. Select Save.

For PrinterGfx, select Floyd-Steinberg dithering, Integer scaling, Positive

Image, Horz. aspect and Colour. To save on ink use Threshold of around 5.

If you are experiencing problems with this driver send me email (see below).

History

-------

1.0 First release.

Tested okay on PPaint, WordWorth 6 (Normal mode) and Blacks Editor.

1.1 Added Close, ConvFunc functions for form feeds and fixed a compilation

warning (probably cause of Gurus on some machines).

NB: Crashes when using a graphics based Word Processor.

1.2 Fixed crashes in Word Processing output using b/w graphics - using

wrong pointer! Added form length command in DoSpecial. Seems much better!

1.3 Added Page Length command (missing in 1.2), output speeded up esp. WP output.

Other minor stuff.

Giftware

--------

This driver is Giftware, you are not obliged to send anything but if you

want to contribute to further driver development for this printer and others

then send me either money (5UKP) or old software (Send me a list first and they

must be originals):

Email: pjhutch@blizzard.u-net.com

Web:   http://www.blizzard.u-net.com

or

Peter Hutchison

75 Lower Skircoat Green

Copley Lane

Halifax

W Yorks

England

HX3 0TG