$VER: SerMouse 2.21, serial mouse package (+driver) for PC mice.

Welcome to:

\                |              /

-   The ultimate PC-Mouse package   -

/                |              \

A lot of people don't like the standard Amiga mouse (and I don't blame 'm).

And if they do like him (I like to 'personalize' my mouse) he (she?) dies

within a short period of time. The market for specific Amiga mice is very

small. Most people like the PC mice and trackballs. I 'm using a PC mouse

of Q-tec, after my Amiga mouse died some years ago. To make it a bit easier

for people to use PC mice and trackballs on the Amiga, I decided to write

this packadge for the use of PC-mice on the Amiga. This package and the

included software is free-ware. You may use it and copy it freely, but you

may not modify it in any way! I'm not responsible for any damage that may

occuur while using the software or using the descriptions in here to modify

hardware, or anything else that might be caused by the existance of this

package. Also, I don't guaranty any mouse or trackball to work with this

package. There are still mice and trackballs with other protocols or

hardware-connections.

This package contains the following things (in this doc or the archive):

* A serial mouse driver for both Microsoft mice and Mouse System mice.

* A description for converting 9-pin Sub-D serial to 25-pin serial.

* A description for converting 9-pin Sub-D BUS-mouse to 9-pin Amiga

mouse.

* A description for converting 9-pin Mini-Din BUS-mouse to 9-pin Amiga

mouse.

The SerialMouse-driver

----------------------

The SerialMouse-driver is called "SerMouse" and should be included in this

package. It can only be used with Kickstart version 2.0 or above. The

current version is 2.21. You can start it from the command-line by typing

"SerMouse" or from the WorkBench by double-clicking on it's icon. To

deactivate it, just restart the program. If you want to activate it every

time your computer is switched on, add the command in your s:user-startup

or drag the icon to the WBStartup drawer (do not remove the DONOTWAIT

tooltype!).

You can specify the following things either as command-line argument or as

icon tooltype:

3BYTES or 5BYTES

This indicates the mouse-type you're using. 5BYTES is the default and most

used type. If you've got a 3-button mouse, it's deffenetely a 5BYTES type.

If the mouse doesn't work, just try the other type... I named the types

by the number of bytes they sent for one event.

DEVICE

The device to be used for reading serial data. Default is the

serial.device. SerMouse has been tested with the VECTOR Connection multi

I/O card and it worked without any problems.

UNIT

The device-unit to be used for reading serial data. Default is unit 0.

Also tested with the VECTOR board.

PRIORITY

This is the task-priority of the mouse-driver. The default is 20. This

should be OK. It's the same priority as the input.device eg. the standard

mouse.

HARDBUTTON

There are still a very few programs which hit the hardware directly in

order to get the state of the mouse-buttons (Protracker is one of them).

SerMouse interacts with the OS only. The mentioned bad programmed programs

won't notice the button-press, because they don't use the OS (for that).

When you specify the option HARDBUTTON, SerMouse will use a 'reversed'

hardware-hit to 'emulate' the Left mouse-button. This is very bad

programming, but there is no alternative! Use this option only if strictly

necsecairy! Avoid pressing buttons on devices atached to game-port 0 when

this option is active! I'm not responsible for any damage!

The program SerMouse is (c) by me (K.P. van Beem) and is distributed as

FreeWare. This means, you may freely use and distribute it, as long as the

programm and this documentation is left unchanged. The source of the

programm is available as GiftWare. Contact me if you're interested in it.

It's partly C and partly assembler.

Be sure pin 22 of the serial-port is NOT connected to any line of the

mouse when you use a serial mouse. This will interfere with your parallel

port.

Converting 9-pin Sub-D serial to 25-pin serial

----------------------------------------------

Most serial mice do have a 9-pin Sub-D serial connector (that's the two-row

connector, NOT a din!). The Amiga has a 25-pin serial connector. In most

stores, you can by adapters from 9 to 25 pins. For those who want to build

them in stead of by them, or just want to connect a 25-pin connector

directly to the wire, this is how you should connect it (the pin-numbers

are normally written on the connectors):

9-pin           25-pin

D-Sub           D-Sub

1       <==>     8          Carrier Detect

2       <==>     3          Recieve

3       <==>     2          Transmit

4       <==>    20          Data Terminal Ready

5       <==>     7          Signal Ground

6       <==>     6          Data Set Ready

7       <==>     4          Request To Send

8       <==>     5          Clear To Send

Do NOT connect pin 22 on the Amiga side (with pin 9 on the mouse side).

This will interfere with the parallel port of your Amiga. This line is the

Ring Indicator which is only used by modems (and most of the time not by

Amiga software).

And don't forget the shield-ground.

Converting 9-pin Sub-D BUS-mouse to 9-pin Amiga mouse

-----------------------------------------------------

Besides serial mice, there's an other type of mouse that is used for the

PC. Although not often these days. It's called the BUS-mouse. It's basicly

the same mouse as the Amiga (and the Atari) is using, but differently

wired. You can easily rewire the mouse (or make an adapter) and connect the

mouse on the mouse-port of the Amiga, just like you do with your standard

Amiga mouse. This way, you can use this type of mouse without a separate

driver. This is the way I use my PC mouse on the Amiga. You should rewire

the mouse as described below:

Amiga side            Mouse side

D-Sub                  D-Sub

5 4 3 2 1             5 4 3 2 1

9 8 7 6               9 8 7 6

1         <==>          4       Vertical pulse

2         <==>          1       Horizontal pulse

3         <==>          3       Vertical pulse Qualifier

4         <==>          2       Horizontal pulse Qualifier

5         <==>          5       Midle button

6         <==>          6       Left button

7         <==>          7       +5V

8         <==>          8       Ground

9         <==>          9       Right button

Converting 9-pin Mini-Din BUS-mouse to 9-pin Amiga mouse

--------------------------------------------------------

For a description of the BUS mouse, see above. Besides the Sub-D plug, most

modern computers do have a mini-Din to connect the bus-mouse to the

computer. Of cause, these can also be rewired. Belose is described how. I

haven't tested this, but I took it from a German magazin.

Mouse          Amiga

Mini-DIN        D-SUB

1     <==>     7      +5V

2     <==>     2      Horizontal pulse

3     <==>     4      Horizontal pulse Qualifier

4     <==>     1      Vertical pulse

5     <==>     3      Vertical pulse Qualifier

6     <==>     6      Left button

7     <==>     5      Middle button

8     <==>     9      Right button

9     <==>     8      GND

I'm not sure the pin-nubers are written on the Mini-Din, besides, if you

don't make an adapter but just rewire it, you won't have the bus. So I'l

give the numbers here. You are looking at the soldier side of the bus, so

at the pin-side of the plug!

|__|

7   8  9

3  4  5  6

__ 1   2  __

|      |

Changes since 2.00 (so new for 2.20)

------------------

* Added the warning in the docs about pin 22 (ring indicator). This pin

should NOT be connected. With some mice, it interacts with the printer

in such a way that you won't be able to print while the mouse in

connected.

* Some mouse-actions involving clicking with the mouse-buttons didn't work

as they should. This included console-clipping, drag-selecting in some

programs, double-click and other combinations in YAK and using PowerSnap.

This was partly coused by the input.device. It doesn't update the current

state of the buttons the way I tell it to do. In order to work around

this, I had to use some undocumented 'features' of the input.device. But

don't fear. It's proven to work safe on all OS versions released up till

now. Now the serialmouse-driver realy works as the 'normal' mouse!

Changes since 2.20 (so new for 2.21)

------------------

* Changed the 5BYTES routine in order to get smoother mouse-movements

while using 3-button (5BYTES) mice.

* Added the option HARDBUTTON, to emulate the left mousebutton for programs

which use direct hardware-hits. Use this option only if strictly

necsecairy! Also, see the explanation above under HARDBUTTON.

Credits

-------

I want to thank the following people (in alphabetical order) who helped me

with this mice stuff:

Marcel Duyn, who lent me a serial mouse for testing the driver (correction

as for version 2.20: I now bought it :-)))

Carsten Gerlach for finding out the couse of the serialmouse driver not

working with commodities like Yak and PowerSnap. (Myself for finding the

optimum solution :-)) )

Stefan Johannesdal, who tested the 5BYTES part of the driver.

Patrick Smith for letting me know some serial-mice do have pin 22 connected.

Christian Stieber, who provided me with some technical info about the

5BYTES serial mouse protocol.

For questions, suggestions or bug-reports, you can always reach me at the

adresses below.

Have fun! (Easy, 'cause you own an Amiga!)

_            _              _

/_)          /_)          __// Hobby programmer...  Fido: 2:280/464.2

/ atrick van /_)eem        \X/  e-mail: patrick.van.beem@aobh.xs4all.nl

Postal address: Parlevinker 5

2152 LC  Nieuw-Vennep

The Netherlands

Nice detail: The latest version of the Microsof mouse driver for windows I

installed on my PC consumed 1.6MB of harddisk space! I think I'll stick to

my Amiga :-))