vbcc - C compiler (c) in 1995-99 by Volker Barthelmann

INTRODUCTION

vbcc is a free portable and retargetable ANSI C compiler.

It is clearly split into a target independant and a target dependant

part and supports emulating datatypes of the target machine on any

other machine so that it is possible to e.g. make a crosscompiler for

a 64bit machine on a 32bit machine.

This document only deals with the target dependant parts of the

SAB c16x version.

This is a pre-alpha version!

LEGAL

vbcc is (c) in 1995-99 by Volker Barthelmann. All code is written by me

and may be freely redistributed as long as no modifications are made

and nothing is charged for it.

Non-commercial usage of vbcc is allowed without any restrictions.

Commercial usage needs my written consent.

Sending me money, gifts, postcards etc. would of course be very nice

and may encourage further development of vbcc, but is not legally or

morally necessary to use vbcc.

ADDITIONAL OPTIONS FOR THIS VERSION

-merge-constants

Place identical floating point constants at the same

memory location. This can reduce program size and increase

compilation time.

-const-in-data

By default constant data will be placed in the code

section (and therefore is accessable with faster pc-relative

addressing modes). Using this option it will be placed in the

data section.

Note that on operating systems with memory protection this

option will disable write-protection of constant data.

SOME INTERNALS

The current version generates assembly output for use with the Tasking

assembler using the small memory model. The generated code should work on

systems with c161, c163, c164, c165 and c167 microcontrollers. Old

versions like the c166 are not supported

The register names are:

r0 through r15 for the general purpose registers and

The registers r1-r5 and r10-r15 are used as scratch registers (i.e. they

can be destroyed in function calls), all other registers are preserved.

r0 is used as user stack pointer. Automatic variables and temporaries

are put on the user stack. Return addresses are pushed on the system

stack.

The first 4 function arguments which have integer or pointer types

are passed in registers r12 through r15.

Integers and pointers are returned in r4.

All other types are returned by passing the function the address

of the result as a hidden argument - so when you call such a function

without a proper declaration in scope you can expect a crash.

The elementary data types are represented like:

type        size in bits        alignment in bytes

char                8                       1

short              16                       2

int                16                       2

long                 currently not supported

all pointers       16                       2

float                currently not supported

double               currently not supported

STDARG

To be written...

KNOWN PROBLEMS

- lots...

Volker Barthelmann                                      volker@vb.franken.de