OS/2 VIDEO PAGES

This program runs under OS/2 and DOS, and demonstrate the use

of video paging under both operating systems.

The OS/2 programming API did not go overboard in its support of

advanced VGA features, such as video paging, and the VIO subsystem for

IBM ver. 1.0 does not support paging at all.

Also, OS/2 is much more aggressive in its management and structure

of memory than was DOS, not to mention the fact that you cannot directly

address the VGA card's registers at all from ring 3 applications level.

There are two basic barriers to low level video programming under

OS/2:  (1) you must get access to the video buffer ram, and OS/2 has

various requirements that you must meet, but ultimately you can do it

from ring 3, and (2) you must get access to the VGA registers, and you

cannot do this from ring 3, but must write a DLL - like a companion prog-

ram to your main .exe file - which runs in ring 2 and which contains

routines that your program can call.

The next thing you have to consider is how aggressive you want

to be in dealing with the OS.  Under DOS, or more correctly, under the IBM

BIOS ROM, you get 8 video pages supported with the VGA.  I was inspired to

do this program by an article by Abrash, Vol 6.5 Programmer's Journal pg 22

(1988) where he presented a program for DOS that got up to 32 pages with the

VGA !  I wondered how many pages OS/2 would allow, so here it is.  Under OS/2

you get none explicitly, but if you are clever, you can get at least 8, and

as this program shows, you can get 15 with a little work.

For various reasons, I was unable to get nearly as many pages out

of OS/2 as Abrash got from DOS.  It seems like OS/2 simply reserves parts of

the video buffer to itself, it is not just the tabla rasa that it was under

DOS.  It appears that you get 4 usable pages, then 4 unusable, then 4 usable,

etc.

I used the same techniques that he used, relocating the text buffer

to 0A000H from 0B800H, getting 64 K ram instead of 32 K, and flipping the

page bit in the miscellaneous register.  I got all of these techniques to

work but not as freely as they did under DOS.  Thus, some of the program

is devoted to avoiding unusable memory and making this technique look

mor or less practical.  I actually don't like writing directly to the

video buffer myself.

This program is presented as a demo of possibilities for page

flipping under OS/2, and not as a full fleged utility.

I left the Codeview material in it, so you can run it thru CVP (if nothing

else, it shows the way CVP handles direct screen writes).   I originally had

the code for locking the screen in the program, but it works better if you

do not lock the screen.

One thing I had hoped was that by relocating the video buffer, any

direct screen writes by the program would not 'bleed' over into other screen

groups, a bug that presently affects OS/2 (notably the M editor).

I also replaced the stub loader with Abrash's program (I added an

ESC key exit to his), so you can run this under DOS and see what it looks

like to get 32 pages.

To run this program, copy xpage.dll to the directory that contains

your dll files ( c:\os2 by default ) before running the program, and put the

statement "iopl=yes" into your config.sys. To see the code, have xpage.dll

and pages.exe in the same directory, and load them into CVP with this command

line: "cvp /L xpage.dll pages".

If you have any questions or comments, I can be reached at:

Compuserve 72261,347.

=Harve=