DOS Auto Configurator V2.0 (AUTODRV.SYS)/

OS/2 Auto Configurator V2.0 (AUTODRV2.SYS)

The Auto Configurator V2.0 is a common enabler which makes many kinds of

PC cards usable.

Explanations not stated for (DOS) or (OS/2) apply to both DOS and OS/2.

1. Functions

The Auto Configurator V2.0

- Is a device driver which uses interfaces with the Card Services

Release 2.0/2.1.

- Makes a PC card usable automatically when the card is inserted.

- Releases the resources such as I/O port addresses, an interrupt

level, or system memory when the card is removed.

- Supports hot insertion/removal so that the card can be inserted/

removed anytime. (*1)

- Makes the card usable based on a file (script file) in which

resource/sequence information is described.

- Supports up to 8 PC card slots.

- Utilities for the Auto Configurator V2.0 are provided.

*1: As for the function of the Auto Configurator, a PC card can be

inserted/removed anytime. These operations may not be supported

by the driver/application of the card.

Refer to the documents for the card/driver/application.

2. Changes from the Auto Configurator V1.0

The Auto Configurator V2.0 provides the following improvements over V1.0:

1. Reduction of the resident size (DOS)

The resident size is reduced from 13KB to 7KB.

2. Script file to enable many kinds of PC cards

To enable PC cards which are not registerd for the Auto Configurator,

a script file in which assigned resource and sequence are described

is used.

3. Setup of beep sound for 4 states

In addition to the time when a PC card is inserted/removed, the

Auto Configurator beeps when the card is enabled/not enabled.

A user can change the beep sound.

Note: An INI file for V1.0 can't be used with the Auto Configurator

V2.0.

3. Enabled PC Cards

Refer to PCMCIA.CRD for the list of PC cards that can be enabled by

the Auto Configurator V2.0.

4. Installation of the Auto Configurator V2.0

The installer installs the Auto Configurator V2.0 and related modules,

and updates CONFIG.SYS.

This section describes how CONFIG.SYS is updated.

Example of DOS CONFIG.SYS

DEVICEHIGH=C:\EZPLAY\IBMDSS01.SYS

DEVICEHIGH=C:\EZPLAY\IBMDOSCS.SYS

DEVICEHIGH=C:\EZPLAY\DICRMU01.SYS /MA=C000-CFFF

DEVICEHIGH=C:\EZPLAY\AUTODRV.SYS  <- Auto Configurator

Example of OS/2 CONFIG.SYS

BASEDEV=PCMCIA.SYS

BASEDEV=IBM2SS01.SYS

BASEDEV=AUTODRV2.SYS          <- Auto Configurator

DEVICE=C:\EZPLAY\VPCMCIA.SYS

DEVICE=C:\OS2\BOOT\COM.SYS

DEVICE=C:\OS2\MDOS\VCOM.SYS

5. Script File

Information for recognition of PC card, resources (I/O ports, an

interrupt level, system memory, and so on) to be assigned, and a sequence

to enable/disable the card are described in a script file.

An utility for the Auto Configurator V2.0 analyzes the contents of the

script file, and registers it into the Auto Configurator.

The Auto Configurator enables each PC card according to the registered

information.

As a default, the Auto Configurator enables almost all modem cards

and the IBM SCSI card. As for DOS, it enables the IBM 3270 card too.

If you have other PC cards you want to enable, read "5-1. Create Your

Script File".

5-1. Create Your Script File

There are script files for some kinds of PC cards in the directory

where the Auto Configurator is installed. When you select the script

files for PC cards you want to enable from the utility, the

utility creates a script file AUTODRV.SCR (DOS), AUTODRV2.SCR

(OS/2). If you can't find a script file for a PC card you want to

enable, read "5-2. Add Script File for New PC Card".

As the next step, the utility analyzes the script file, and registers

it into the Auto Configurator.

After the system is rebooted, you can use the PC cards described in

the script file.

5-2. Add Script File for New PC Card

According to "7. How to Write Script File", write the required

information for the new PC card. After that, do the same as in 5-1.

Note: Do NOT forget "End" at the last line in the

script file.

6. Auto Configurator Utility

The Auto Configurator Utility provides the following functions for the

Auto Configurator. (For more details about Windows or OS/2, refer to

the manual for the Auto Configurator Utility.)

The Auto Configurator Utility provides functions to:

- select script files for PC cards, and create a script file from them.

- analyze the script file, and registers it into the Auto Configurator.

- GUI utilities are provided for Windows and OS/2.

- An utility is provided to register contents of the script file into

the Auto Configurator for DOS.

6-1. DOS Auto Configurator Utility

Do the following to register contents of the script file into the

Auto Configurator for DOS.

1. As explained in "5. Script File", create a script file AUTODRV.SCR

by gathering script files for PC cards you want to enable.

2. Type the following from the DOS prompt.

AUTOUTL /D

Hit the Enter key.

Note that the utility (AUTOUTL.EXE), the script file (AUTODRV.SCR),

and the Auto Configurator (AUTODRV.SYS) must be in the same

directory.

3. When the script file has an error, a message is displayed.

Correct it, and try the same operation again.

7. How to Write Script Files

If you can't find a script file for a PC card you want to enable from

the provided script files for some kinds of PC cards, or you want to

change the parameters in the script files, read this section.

Note: Keywords (CardID, FunctionID, and so on) are described from

column 1 to 15, and "=" (equal) is described in column 16.

Commands, parameters, and so on should be described from column

17.

7-1. First Line

; IBM PC Card Script File for IBM Auto-Configurator V2.0

Note:   - This line is used to make sure that a script file is for the Auto

Configurator V2.0. This line should NOT be deleted nor changed.

7-2. Comment Line

; Modem Card (Common)

Note:   - A line containing ";" (semi-colon) at the first column is a comment

line.

7-3. Beep

Beep           =1000,2,2000,2,800,2,20000,10

Note:   - The frequency(Hz) and duration for a beep are specified.

- The Auto Configurator beeps in the following state:

Card was removed

Card was inserted

Card was enabled

Card wasn't enabled

- "Card wasn't enabled" means "An error occurred".

When the inserted card isn't supported, the Auto Configurator

doesn't beep.

- Explanation of the Example

1000,2,2000,2,800,2,20000,10

------ ------ ----- --------

|      |      |     |

|      |      |     +- Card wasn't enabled

|      |      +------- Card was enabled

|      +-------------- Card was removed

+--------------------- Card was inserted

- The "Frequency,Duration" for each state should be described.

If you don't want a beep for any state, specify 0 (zero) as the

Frequency.

- The "Beep" statement is necessary, and it should NOT be moved or

removed.

7-4. Card ID

CardID         =MODEM,MD24XC,116E2,118C2,FC2400,2460MC,FC24F,MF0B

Note:   - The Auto Configurator uses a string in Tuple(*2) 0x15 (Level 1

Version/Product Information Tuple) to identify which card is

inserted. The string(s) is(are) described as card ID (CardID).

- This line is case sensitive. "MODEM" and "Modem" are different.

- "," (comma) is used as a separator when multiple card IDs are

described.

- " " (blank) is treated as a part of a card ID. Unnecessary

blank should NOT be inserted. (Blank at the end of a line isn't

treated as a part of the Card ID.)

- The total length for a CardID is under 64 characters, containing

commas.

- Explanation of the Example

MODEM     General modem cards

MD24XC    OMRON Fax/Data Modem card

116E2     OKI Modem card 2400

118C2     OKI Modem card Fax/Data

FC2400    Dr.Neuhaus Mikroelektronik GmbH FURY CARD 2400

2460MC    ELSA GmbH MicroLink 2460MC

FC24F     RATOC REX-5565

MF0B      MITSUBISHI Voice/Fax/Data/Modem Card

*2: A PC card has infomation, such as name, type, or functions

in itself. The area of a card information contains links

of data blocks called "Tuple". Each Tuple has different

kinds of information.

7-5. Function ID

FunctionID     =0x02

Note:   - If there is no string described as the card ID in Tuple 0x15 of the

inserted card, the Auto Configurator uses a value in Tuple

0x21 (Function ID Tuple) to identify which card is inserted. The

value is described as the function ID (FunctionID).

- If you don't want to use a function ID to identify a card,

specify "0xFF" for the FunctionID.

- "0x" represents the value in hexa-decimal. For example, "0x10"

is hexa-decimal 10 (decimal 16), "10" is decimal 10.

- Explanation of the Example:

0x02      Serial Port

7-6. Manufacturer ID

ManufacturerID =0x00A4

Note:   - If the Auto Configurator finds a string described as the card ID in

Tuple 0x15 of the inserted card, or a value described as the Function

ID in Tuple 0x21 of the card, it checks a value in Tuple 0x20

(Manufacturer ID Tuple) to identify which card is inserted. The

value is described as the manufacturer ID (ManufacturerID).

- If you don't want to use a manufacturer ID to identify a card,

specify "0xFFFF" for the ManufacturerID.

- Explanation of the Example:

0x00A4    IBM

7-7. Card ID not to enable PC card

IgnoreCardID   =SDLC,Serial IR,Sound Card

Note:   - When a string specified for IgnoreCardID is in Tuple 0x15, the

Auto Configurator doesn't enable the PC card.

- This line is case sensitive. "SDLC" and "Sdlc" are different.

- "," (comma) is used as a separator when multiple card IDs are

described.

- " " (blank) is treated as a part of a card ID. Unnecessary

blanks should NOT be inserted. ( A blank at the end of a line isn't

treated as a part of the Card ID.)

- When IgnoreCardID isn't needed, write nothing.

- The total length for a CardID is under 64 characters, containing

commas.

- Explanation of the Example:

SDLC        IBM PCMCIA SDLC Modem

Serial IR   IBM PCMCIA Serial IR Adapter Card

Sound Card  Roland PCMCIA Sound Card

7-8. Enable Sequence

EnableSequence =ReqIO(0x2F8,0xFF,0,0,0,0,0xFF)

Note:   - A sequence to enable a PC card is described after the parameter

"EnableSequence =".

- Commands for the enable sequence should start from column 17.

7-9. Disable Sequence

DisableSequence=RestoreBDA

Note:   - A sequence to disable a PC card when the PC card is removed is

described after the parameter "DisableSequence =".

- Commands for the disable sequence should start from column 17.

- Resources (I/O port, an interrupt level, or system memory) reserved

by the ReqIO, ReqIRQ, ReqConfig, and ReqWindow commands in the

enable sequence are released when the PC card is removed.

(*3) Since the Auto Configurator does this automatically, you don't

need to describe the operation in the disable sequence.

*3: Since the used resources are released when the PC card is removed,

another PC card can use these resources when it is inserted.

7-10. End of Script File

End

Note:   - "End" means the end of the script file and is necessary.

- "End" is described as the end of a sequence too (explained later),

but "End" for the end of a script file starts from column 1.

7-11. Commands for Enable/Disable Sequence

The following commands are used for the enable/disable sequence.

7-11-1. Get I/O Port Address

ReqIO(0x2F8,0xFF,0,0,0,0,0xFF)

Note:   - The Auto Configurator requests the Card Services to assign an I/O

port address for the PC card.

- Explanation of the Example:

ReqIO(0x2F8,0xFF,0,0,0,0,0xFF)

|     |    | | | | |

|     |    | | | | +- Number of I/O Address Lines

|     |    | | | +--- Attributes 2

|     |    | | +----- Number of Contiguous Ports 2

|     |    | +------- Base Port Address for Range 2

|     |    +--------- Attributes 1

|     +-------------- Number of Contiguous Ports 1

+-------------------- Base Port Address for Range 1

Attributes 1/Attributes 2

bit 7 6 5 4 3 2 1 0

0 0 0 0 x x x x

| | | |

| | | +- Shared (1=true)

| | +--- First Shared (1=true)

| +----- Force Alias Accessibility

+------- Data Path Width for I/O Range

(1=16 bit, 0=8 bit)

"0" should be 0 (zero).

"x" is 1 or 0 (zero).

- If "0xFF" is specified for the number of the I/O address lines,

the contiguous ports 1 or the contiguous ports 2, the value

set for the card is assigned from the tuples in the card.

7-11-2. Get Interrupt Level

ReqIRQ(0,3)

Note:   - The Auto Configurator requests the Card Services to get an interrupt

level (IRQ) for the PC card.

- Explanation of the Example:

ReqIRQ(0,3)

| |

| +- Interrupt Level

+--- Attributes

Attributes

bit151413121110 9 8 7 6 5 4 3 2 1 0

0 0 0 0 0 0 0 0 0 0 0 0 x x x x

| | | |

| | +-+- IRQ Type

| |      0: Exclusive

| |      1: Time-Multiplexed Sharing

| |      2: Dynamic Sharing

| |      3: Reserved

| +----- Force Pulse (1=true)

+------- First Shared (1=true)

- If "0xFF" is specified for the interrupt level, the value set for

the card is assigned from the tuples in the card.

7-11-3. Configure PC Card

ReqConfig(0xFF,0xFF,0xFF,0x48,0,0,0x40)

Note:   - The Auto Configurator requests the Card Services to configure the

PC card.

- Explanation of the Example:

ReqConfig(0xFF,0xFF,0xFF,0x48,0,0,0x40)

|    |    |    |    | | |

|    |    |    |    | | +- CCR: Option Register Setting

|    |    |    |    | +--- CCR: Socket/Copy Register Setting

|    |    |    |    +----- CCR: Pin Replacement Register Setting

|    |    |    +---------- CCR: Status Register Setting

|    |    +--------------- Vpp2

|    +-------------------- Vpp1

+------------------------- Vcc

- If "0xFF" is specified for Vcc, Vpp1, or Vpp2, the value set for

the card is assigned from the tuples in the card.

- If the value for the lower 6 bits in the Option register is 0 (zero),

the index information in Tuple 0x1B (Configuration Entry Tuple) is

set automatically. If it isn't 0, the value specified is set, and

the index information in the tuple isn't set.

7-11-4. Get System Memory Window

ReqWindow(1,0x000C,0xCA000,0x2000,0xFF)

Note:   - The Auto Configurator requests the Card Services to get a system

memory window for the PC card.

- Explanation of the Example:

ReqWindow(1,0x000C,0xCA000,0x2000,0xFF)

| |      |       |      |

| |      |       |      +- Window Speed Field

| |      |       +-------- Memory Window Size

| |      +---------------- System Base Address

| +----------------------- Attributes

+------------------------- Window Number

Attributes

bit151413121110 9 8 7 6 5 4 3 2 1 0

0 0 0 0 0 0 0 0 0 x x 0 x x x 0

| |   | | |

| |   | | +- Memory Type

| |   | |    (1=attribute)

| |   | +--- Enabled (1=true, 0=disabled)

| |   +----- Data Path Width

| |          (1=16 bit, 0=8 bit)

| +--------- Shared (1=true)

+----------- First Shared (1=true)

- The window Number is used to get system memory windows.

It starts from 1.

- If "0xFF" is specified in the window speed field, the value set

for the card is assigned from the tuples in the card.

7-11-5. Map to Memory on PC Card

MapMemPage(1,0xCC000)

Note:   - The Auto Configurator requests the Card Services to map a system

memory window to the memory on the PC card.

- Explanation of the Example:

MapMemPage(1,0xCC000)

| |

| +- Card Offset Address

+--- Window Number

- The value specified by the "ReqWindow" command is used.

7-11-6. Release System Memory Window

RelWindow(1)

Note:   - The Auto Configurator requests the Card Services to release

the system memory window assigned by ReqWindow.

- Explanation of the Example:

RelWindow(1)

|

+- Window Number

- The window number used by ReqWindow should be specified.

7-11-7. Access Configuration Register

AccessConfigReg(0,0,#1,0)

Note:   - The Auto Configurator requests to read or write a PC Card

Configuration Register.

- The offset is a value from the base of the configuration registers.

- Explanation of the Example:

AccessConfigReg(0,0,#1,0)

| | |  |

| | |  +- Reserved

| | +---- Data Store Area

| +------ Offset

+-------- Action (0=Read, 1=Write)

Read data from a PC Card configuration register (offset 0),

and write it in #1.

AccessConfigReg(1,0,#1,0)

Read data from #1 and write it in a PC Card configuration

register (offset 0),

7-11-8. Setup COM Number

SetupCOM(0xFF)

Note:   - Updates BIOS Data Area for a COM number used by, for example,

modem applications. (DOS)

- Registers a COM number to the COM driver. (OS/2)

- Explanation of the Example:

SetupCOM(0xFF)

|

+- COM Number

- When "1", "2", "3", or "4" is specified, the value is treated as

a COM number, and the BIOS Data Area for it is updated (DOS) , or

it is registered to the COM driver (OS/2). If the COM number has

been used already, this operation isn't executed.

- If "0xFF" is specified, the unused area in BIOS Data Area is updated

(DOS), or the unused COM number is registered to the COM driver

(OS/2). If all COM numbers have been used already, this operation

isn't executed.

- If the COM driver isn't installed, this operation isn't executed.

- Even if this operation isn't executed, the PC card is enabled.

7-11-9. Delete COM Number

DeleteCOM

Note:   - Deletes the COM number set by SetupCOM.

- BIOS Data Area is restored (DOS), or the registration to the

COM driver is deleted (OS/2).

- If the COM driver isn't installed, this operation isn't executed.

7-11-10. IBM SCSI Card

IBM_SCSI

Note:   - Unique operation for the IBM SCSI card is executed.

- This command can be used for the Future Domain SCSI Card (SCSI2GO).

7-11-11. IBM Token Ring Card

IBM_TokenRing(4)

Note:   - Unique operation for the IBM Token Ring card is executed.

- Explanation of the Example:

IBM_TokenRing(4)

|

+- Ring Speed (MBPS)

- "4" or "16" is specified as the ring speed.

7-11-12. IBM Ethernet Card

IBM_Ethernet1

IBM_Ethernet2

Note:   - Unique operation for the IBM Ethernet card is executed.

7-11-13. Set Data

Move(#1,0xF8)

Note:   - Sets data in a data store area (*4).

- Explanation of the Example:

Move(#1,0xF8)

|  |

|  +- Data

+---- Data Store Area

Sets 0xF8 in #1.

*4: Data Store Area is an area, where 1 byte of data used by

memory read/write, port in/out, or others is stored.

"#1" to "#9" can be specified for a Data Store Area.

7-11-14. Write Data to System Memory

Write(0xCC000,#1)

Note:   - Writes the data, in a data store area, to the system memory.

- Explanation of the Example:

Write(0xCC000,#1)

|       |

|       +- Data Store Area

+--------- System Memory Address

Writes data in #1 to 0xCC000.

7-11-15. Read Data from System Memory

Read(0xCC000,#1)

Note:   - Reads data from the system memory, and writes it in a data store

area.

- Explanation of the Example:

Read(0xCC000,#1)

|       |

|       +- Data Store Area

+--------- System Memory Address

Reads data from 0xCC000, and writes it in #1.

7-11-16. Output Data to I/O Port

Out(0xA20,#1)

Note:   - Outputs the data in a data store area to the I/O port.

- Explanation of the Example:

Out(0xA20,#1)

|     |

|     +- Data Store Area

+------- I/O Port Address

Outputs data in #1 to 0xA20.

7-11-17. Input Data from I/O Port

In(0xA20,#1)

Note:   - Inputs data from the I/O port, and writes it in a data store area.

- Explanation of the Example:

In(0xA20,#1)

|     |

|     +- Data Store Area

+------- I/O Port Address

Inputs data from 0xA20, and writes it in #1.

7-11-18. Logical And

And(#1,#2)

Note:   - Gets the logical "and" from the data in 2 data store areas, and

writes it in the data store area.

- Explanation of the Example:

And(#1,#2)

|  |

|  +- Data Store Area 2

+---- Data Store Area 1

Gets the logical "and" from the data in #1 and #2, and writes

it in #1. (#1 <- #1 & #2)

7-11-19. Logical Or

Or(#1,#2)

Note:   - Gets the logical "or" from the data in 2 data store areas, and

writes it in the data store area.

- Explanation of the Example:

Or(#1,#2)

|  |

|  +- Data Store Area 2

+---- Data Store Area 1

Gets the logical "or" from the data in #1 and #2, and writes

it in #1. (#1 <- #1 | #2)

7-11-20. Conditional Jump 1

JumpE(#1,#2,$1)

Note:   - If data in Data Store Area 1 equals to the data in Data Store Area

2, the sequence goes to specified label (*5).

- Explanation of the Example:

JumpE(#1,#2,$1)

|  |  |

|  |  +- Label

|  +---- Data Store Area 2

+------- Data Store Area 1

If data in #1 equals to the data in #2, jump to label $1.

*5: In the enable/disable sequence, if you want to jump or go back to

some operations, the sequence can jump to the label you specify.

"$1" to "$9" can be used as the label.

7-11-21. Conditional Jump 2

JumpNE(#1,#2,$1)

Note:   - If data in Data Store Area 1 doesn't equal to the data in Data Store

Area 2, the sequence goes to the specified label.

- Explanation of the Example:

JumpNE(#1,#2,$1)

|  |  |

|  |  +- Label

|  +---- Data Store Area 2

+------- Data Store Area 1

If data in #1 doesn't equal to the data in #2, jump to label $1.

7-11-22. Jump

Jump($1)

Note:   - The sequence goes to the specified label.

- Explanation of the Example:

Jump($1)

|

+- Label

Jump to label $1.

7-11-23. Label

Label($1)

Note:   - Specifies the labels specified in the JumpE, JumpNE, or Jump command.

- Explanation of the Example:

Label($1)

|

+- Label

Specify label $1.

7-11-24. End of Sequence

End

Note:   - "End" means the end of the sequence and is necessary.

8. Sample Script File

---- Start of File ------------------------------------------------------------

; IBM PC Card Script File for IBM Auto-Configurator V2.0

Beep           =1000,2,2000,2,800,2,20000,10

; ---- Modem Card (Common) ----------------------------------------------------

CardID         =MODEM,MD24XC,116E2,118C2,FC2400,2460MC

FunctionID     =0x02

ManufacturerID =0xFFFF

IgnoreCardID   =SDLC

EnableSequence =ReqIO(0x3F8,0xFF,0,0,0,0,0xFF)

ReqIRQ(1,4)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

SetupCOM(0xFF)

End

DisableSequence=DeleteCOM

End

CardID         =MODEM,MD24XC,116E2,118C2,FC2400,2460MC

FunctionID     =0x02

ManufacturerID =0xFFFF

IgnoreCardID   =SDLC

EnableSequence =ReqIO(0x2F8,0xFF,0,0,0,0,0xFF)

ReqIRQ(1,3)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

SetupCOM(0xFF)

End

DisableSequence=DeleteCOM

End

CardID         =MODEM,MD24XC,116E2,118C2,FC2400,2460MC

FunctionID     =0x02

ManufacturerID =0xFFFF

IgnoreCardID   =SDLC

EnableSequence =ReqIO(0x3E8,0x08,0,0,0,0,0x10)

ReqIRQ(1,4)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

SetupCOM(0xFF)

End

DisableSequence=DeleteCOM

End

CardID         =MODEM,MD24XC,116E2,118C2,FC2400,2460MC

FunctionID     =0x02

ManufacturerID =0xFFFF

IgnoreCardID   =SDLC

EnableSequence =ReqIO(0x2E8,0x08,0,0,0,0,0x10)

ReqIRQ(1,3)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

SetupCOM(0xFF)

End

DisableSequence=DeleteCOM

End

; ---- IBM SCSI Card ----------------------------------------------------------

CardID         =SCSI

FunctionID     =0xFF

ManufacturerID =0x00A4

IgnoreCardID   =

EnableSequence =ReqIO(0x140,0xFF,0,0,0,0,0xFF)

ReqIRQ(0,5)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

IBM_SCSI

End

DisableSequence=End

; ---- Future Domain SCSI Card (SCSI2GO) --------------------------------------

CardID         =SCSI

FunctionID     =0xFF

ManufacturerID =0x0130

IgnoreCardID   =

EnableSequence =ReqIO(0x140,0xFF,0,0,0,0,0xFF)

ReqIRQ(0,5)

ReqConfig(0xFF,0xFF,0xFF,0x08,0,0,0x40)

IBM_SCSI

End

DisableSequence=End

; ---- IBM 3270 Card ----------------------------------------------------------

CardID         =3270

FunctionID     =0xFF

ManufacturerID =0x00A4

IgnoreCardID   =

EnableSequence =ReqIO(0x2D0,0x10,0,0,0,0,0xFF)

ReqIRQ(0,9)

ReqConfig(0xFF,0xFF,0xFF,0,0,0,0x40)

ReqWindow(1,0x000C,0xCE000,0x2000,0xFF)

MapMemPage(1,0)

End

DisableSequence=End

; ---- IBM Token Ring Card ----------------------------------------------------

CardID         =TOKEN RING

FunctionID     =0xFF

ManufacturerID =0xFFFF

IgnoreCardID   =

EnableSequence =ReqIO(0xA20,0xFF,0,0,0,0,0xFF)

ReqIRQ(0,9)

ReqConfig(0xFF,0xFF,0xFF,0x20,0,0,0x40)

ReqWindow(1,0x000C,0xCA000,0x2000,0xFF)

MapMemPage(1,0xCA000)

ReqWindow(2,0x000C,0xCC000,0x4000,0xFF)

MapMemPage(2,0xCC000)

IBM_TokenRing(4)

End

DisableSequence=End

; ---- IBM Ethernet Card ------------------------------------------------------

CardID         =Ethernet

FunctionID     =0xFF

ManufacturerID =0x00A4

IgnoreCardID   =

EnableSequence =ReqIO(0x300,0x20,0,0,0,0,0x10)

ReqIRQ(0,5)

ReqConfig(0xFF,0xFF,0xFF,0x20,0,0,0x40)

ReqWindow(1,0x000E,0xCC000,0x4000,0x04)

MapMemPage(1,0x20000)

Read(0xCC000,#1)

Move(#2,0x80)

Or(#1,#2)

Write(0xCC000,#1)

Move(#1,0x41)

Write(0xCC000,#1)

MapMemPage(1,0)

IBM_Ethernet1

RelWindow(1)

ReqWindow(1,0x000C,0xCC000,0x4000,0x04)

MapMemPage(1,0x4000)

IBM_Ethernet2

End

DisableSequence=End

End

---- End of File --------------------------------------------------------------