.G.HPLOGO.TIF;1.35";0.383";TIFF

▀▀▀▀▀

________________________________________

HP─PB 16─Channel

Asynchronous Multiplexers

Technical Data

Product Numbers

HP J2092A, J2093A, J2094A

The 16─channel Asynchronous Multiplexers provide a means to connect up to sixteen asynchronous serial devices to PA─RISC HP9000 Series 800 Computers which use the HP─PB (Hewlett─Packard Precision Bus) as their primary system input/output bus (8x2 and 8x7 Series ) and have HP─UX operating system version 8.0 or later installed.

Based on on─board microprocessor and memory, the Multiplexer handles data transmission and control, inbound and outbound traffic flow control, and modem signals control.  It is thus an intelligent and powerful IO card freeing the system CPU of traffic overhead and guaranteeing data integrity and no character loss.

The 16─channel multiplexers provide one of the following solutions:

J2092A: 16 RS─232─C compatible ports for direct connection

J2093A: 16 RS─423─A compatible ports for direct connection requiring long distance cabling

J2094A: 16 RS─232─C compatible ports for direct and remote connection with support of full duplex modem control

A wide range of configurable transmission modes and formats permits hardwired and remote connection of various CRT terminals, printers, plotters, and other asynchronous devices.

Features

Feature set common to the three products:

┤	Programmable data─rate for each channel

┤	Programmable character size: 7 or 8 data bits

┤	Programmable number of stop bits: 1 or 2

┤	Parity, overrun, framing error check detects transmission faults

┤	Break detection and generation

┤	Automatic and programmable device XON/XOFF handshaking to pace MUX input and output data transmission

┤	Programmable parity: odd, even, or none

┤	On─board buffering to send/receive data to/from host for multiple ports

┤	Firmware based self tests executed at ░power─on▒

┤	A customer diagnostic utility is available to verify hardware/software and data path integrity for each port with the use of loopback hood supplied with the product

┤	Online diagnostic tools are also available to support engineers to troubleshoot the card

Features specific to the J2092A product:

┤	EIA RS─232─C and V24/V28 compatibility for data only connections (no modem control) on default DB─25 female standard connectors provided with the product, or on customized connection scheme

┤	Line speed up to 38.4 kb/s

┤	Remote support of the card is possible through the use of the diagnostic tools.

Features specific to the J2093A product:

┤	EIA RS─423─A and V10 compatibility for data only connections (no modem control) on the default DB─25 female standard connectors provided with the product, or on a customized connection scheme.  This allow the operation of RS─423 & RS─422 devices such as HP terminals (HP700/32, HP700/92, HP700/94, HP2392A) and HP printers (HP LaserJets) at distances up to 1200m.

┤	Line speed up to 38.4 kb/s

Features specific to the J2094A product:

┤	EIA RS─232─C and V24/V28 compatibility for connection of terminals, modems, or serial printers on DB─25 standard connectors.

┤	All lines support full─duplex modem control (8 signals per channel) which permit point─to─point dialup or leased line operation.  .

┤	Three types of modem protocol are supported:

─	Bell protocol, 2 signals: DTR and DCD

─	CCITT protocol,6 signals: DTR,DCD,DSR,RTS,CTS,RI

─	No protocol, user can handle 8 signals (4 input/4 output)

┤	Line speed up to 19.2 kb/s.

┤	Hardware flow─control capability through the use of CTS/RTS signals.

┤	Remote support of the card and connection panels is possible through the use of the diagnostic tools.

Functional Description

The 16─channel Asynchronous Multiplexers comprise three major functional assemblies:

┤	The MUX card: A single width HP─Precision Bus card that interfaces with the system backplane

┤	Two ░distribution panels▒ (8 ports each) offering standard DB─25 connectors for peripheral attachment.  For the J2092A and J2093A products, they are known as ░Direct Distribution Panel ─ DDP.▒ For the J2094A product, they are known as ░Active Distribution Panel ─ ADP II.▒ These distribution panels can be rack mounted in standard 19" bays.  (Refer to the section ░related products▒ of this data sheet for more details)

Optionally, these distribution panels can be removed for the direct connect multiplexers ─ J2092A, J2093A ─ thus allowing the MUX to be linked to any type of connectors or cabling scheme (i.e.  RJ11, RJ45, punch panels, data switches...).  See ░functional specification▒ section of this data sheet for connector pin─out.

┤	The link cable between the MUX card and the distribution panels (4 meters).

At the heart of the HP 16─channel multiplexers is a circuit card that contains a 68000 microprocessor and associated PROM and RAM memory.  The 128 Kbyte PROM contains the power─up self─test and system support code.  The code downloaded from the host is stored in the 256 Kbyte RAM.  Download operation and card diagnostics are performed through a special supervisor on─card buffer.

Functional Specifications

Buffering: On─card buffering to send/receive data for multiple ports to increase throughput and reduce host CPU load.

Transmit Buffer Size: 255 bytes per port

Receive Buffer Size: 511 bytes per port

Software: HP 9000 Series 800 HP─UX revision 8.0 or later

Communications

Signals available on the DB─25 connectors of either the DDP or ADP II for the RS─232─C products.

________________

Input/	Connector	Connector

RS─	V.24	Abbrev.	Description	Output	Pin Nos.	Pin Nos.

232─C					on ADP II	on DDP

______________________________________________________

AA	101		Protective Ground	N/A	1	1

AB	102	SG	Signal Ground	N/A	7	7

BA	103	SD	Transmit Data	O	3	3

BB	104	RD	Receive Data	I	2	2

CA	105	RS	Request to Send	O	8	N/A

CB	106	CS	Clear to Send	I	22	N/A

CC	107	DM	Data Set Ready	I	20	N/A

CD	108	TR	Data Terminal Ready	O	6	N/A

CF	109	RR	Data Carrier Detect	I	4	N/A

CH	111	SR	Signal Rate Selector	O	23	N/A

CE	125	IC	Ring Indicator	O	9	N/A

¬¬	¬¬	GPO	Gen.  Purpose Output	O	5	N/A

________________

Signals available on the DB─25 connectors of the DDP for the RS─423A product for connection of RS─423 or RS─422 devices.

________________

Description	Input/	Connector

Output	Pin Nos on DDP

______________________________________________________

Protective Ground	¬¬	1

Transmit Data (+)	O	15 & 16

Transmit Data (─)	O	17 & 3

Receive Data (+)	I	19 & 14

Receive Data (─)	I	25 & 2

________________

Note:	The pins 15, 17, 19, 25 provide compatibility with the RS─423 interface.  The signals are also provided on pins 16, 3, 14, 2 to offer an interface compatible with the standard mechanical interface: RS─530.

To allow flexible and customized cabling scheme using connectors other than the default DB─25 connectors distribution panels, the direct connect products ─ J2092A, J2093A ─ offer on the MUX card a single sub─D high density 78 pins connector (female) for the 16 ports.  To facilitate cable realization, a 78 pins connector (male, to crimp) is supplied with the product when the default distribution panels are deleted through the product option (see ordering information below)

Pin─out of the 78 Pins Connector:

Naming convention for the pin name in the table:

┤	Each of the 16 ports use up to 4 wires

─	NtxP = Transmit Data (─) output signal

─	txP = Transmit Data (+) output signal (GROUND for RS─232─C)

─	NrxP = Receive Data (─) input signal

─	rxP = Receive Data (+) input signal (not used for RS─232─C)

─	Where P is the port number (from 0 to 15).  The pins unused in the connector are noted: NA

_______________

Name	Pin No	Name	Pin No

__________________________________________________

Ntx8	A1	Ntx9	C1

Nrx8	A2	Nrx9	C2

Ntx10	A3	Ntx11	C3

Nrx10	A4	Nrx11	C4

Ntx12	A5	Ntx13	C5

Nrx12	A6	Nrx13	C6

Ntx14	A7	Ntx15	C7

Nrx14	A8	Nrx15	C8

NA	A9	NA	C9

NA	A10	NA	C10

NA	A11	NA	C11

NA	A12	NA	C12

Ntx0	A13	Ntx1	C13

Nrx0	A14	Nrx1	C14

Ntx2	A15	Ntx3	C15

Nrx2	A16	Nrx3	C16

Ntx4	A17	Ntx5	C17

Nrx4	A18	Nrx5	C18

Ntx6	A19	Ntx7	C19

Nrx6	A20	Nrx7	C20

tx8	B1	tx9	D1

rx8	B2	rx9	D2

tx10	B3	tx11	D3

rx10	B4	rx11	D4

tx12	B5	tx13	D5

rx12	B6	rx13	D6

tx14	B7	tx15	D7

rx14	B8	rx15	D8

NA	B9	NA	D9

NA	B10	NA	D10

NA	B11	NA	D11

tx0	B12	tx1	D12

rx0	B13	rx1	D13

tx3	D14	tx2	B14

rx3	D15	rx2	B15

tx4	B16	tx5	D16

rx4	B17	rx5	D17

tx6	B18	tx7	D18

rx6	B19	rx7	D19

_______________

Data Rates: Baud rate defaults to 9600 and is software programmable to any of the following rates: 75, 150, 300, 600, 1200, 2400, 4800, 9600, 19200.  In addition, 38400 bits/s is supported with J2092A and J2093A products.

__________________________________________________

cross talk between pairs	bits/sec	meters/(feet)

1 volt (RS 423 recommendation)	0 to 38400	300/(1200)

2 volts (with HP peripherals)	0 to 19200	1200/(4000)

38400	300/(1200)

__________________________________________________

Note regarding the J2093A product: The maximum baud rates/distances supported are the following:

Throughput: The 16─channel Multiplexer hardware and firmware is able to support an inbound data throughput of 19.2 K baud simultaneously on each channel (or 38.4 K baud simultaneously on 8 channels), without character loss when flow control is activated, or an outbound data throughput of 19.2 K baud simultaneously on each channel.  However, the global system performance of the system is dependent on the customer application and on the HP 9000 HP─PB Series 800 computer load and performance.

Communication Mode: Bit serial, asynchronous.

Break Detection and Transmission: Break condition is recognized by the interface, and message is sent to the host.  Break condition can also be generated by the MUX at the request of the host.

Device Handshakes: The MUX is capable of pacing both inbound an outbound data via an ░XON/XOFF▒ type of protocol.  The ░XON/XOFF▒ characters are programmable.  The host may program the MUX card to automatically manage ░XON/XOFF▒ flow control, thus avoiding any character loss.  The ░XON/XOFF▒ may also be under application control, but no data loss is guaranteed only if the MUX is responsible for flow─control.

In the J2094A product, in addition to the XON/XOFF flow control mode, inbound and outbound data flow can be managed by the MUX card through the use of CTS/RTS modem signals.

Edit Functions: Edit functions such as backspace, character delete, and line delete are passed to the host and managed by the host operating system.

Physical Specifications

_______________

Height	Width	Depth	Weight

__________________________________________________

MUX card	29.5 mm	100 mm	220 mm	0.25kg

(P/N J209x─60001)	(1.15 in)	(3.90 in.)	(8.58 in.)

ADP II box	257 mm	110 mm	32 mm	0.4kg

(P/N 5062─3054)	(10.12 in.)	(4.29 in.)	(1.25 in.)

DDP box 	215 mm	100 mm	30 mm	0.7kg

(P/N 5062─3066)	( 8.47 in.)	(3.94 in.)	(1.18 in.)

_______________

Electrical Specifications

Direct Current Requirements:

_______________

Voltage	J2092A	J2093A	J2094A

MAX current	MAX current	MAX current

__________________________________________________

+5	1.10 A	1.10 A	1.70 A

+12	0.08 A	0.20 A	0.30 A

─12	0.08 A	0.15 A	0.15 A

_______________

Note for the J2093A: max current on ─12V is 0.25 A if a multireceiver configuration is used.

Installation and Support

The J2092A product is usually bundled with the system when purchased with the system option 60x, however, it may also be purchased, as an add─on product as can the J2093A and J2094A products.  All three products are customer installable, however, customers can choose to purchase installation services from Hewlett─Packard if they wish.

Diagnostics: A self─test is performed at power─on by the system on all cards.  Visual status indicators exist on the MUX panel.  Two LED indicators monitor the ADP II status (power─on and link) of J2094A.

A customer diagnostic utility is available to perform verification of each port data path through the use of a loopback hood provided with the product.

Additional online diagnostics and reset capabilities are provided through the system diagnostic environment (DUI) for use by HP Customer Engineers.

Support Services: Hardware/Software support services available for this product are included in the System Support services.

Ordering Information

The 16─channel RS─232─C direct connect multiplexer (J2092A) with DB─25 connectors is bundled with the HP 9000 Series 800 systems when ordered with the system option 60x (refer to the HP9000 configuration guide).  For add─on cards or other configurations, use the following:

J2092A HP─PB 16 channel RS─232─C Direct Connect Asynchronous Multiplexer

J2093A HP─PB 16 channel RS─423─A Direct Connect Asynchronous Multiplexer

J2094A HP─PB 16 channel RS─232─C MODEM Connect Asynchronous Multiplexer

Software media options (MUST add one of them for use in a system running HP─UX 8.0):

AA0	software on ╟" cartridge tape

AA1	software on ╚" magnetic tape

AAH	software on DDS cartridge

Interface option (can order with J2092A, J2093A products):

1BZ	delete the DB─25 port connection kit▒ (cable and DDPs)

The J2092A includes the following assemblies:

─	1 * MUX card: J2092─60001

─	1 * link cable: 5062─3052

─	2 * DDP: 5062─3066

─	1 * loopback hood: 5181─2030

─	1 * installation manual: 5959─4972

The J2093A includes the following assemblies:

─	1 * MUX card: J2093─60001

─	1 * link cable: 5062─3052

─	2 * DDP: 5062─3066

─	1 * loopback hood: 5181─2030

─	1 * installation manual: 5959─4972

The J2094A includes the following assemblies:

─	1 * MUX card: J2094─60001

─	1 * link cable: J2094─60004

─	2 * ADP: 5062─3054

─	1 * installation manual: 5959─4972

Related Products

The DDP and ADP II are standalone boxes but may be rack mounted in standard cabinets such as the 46298B/C (EIA 19 inch x 1m or x 1.6 m) especially when used in large configurations.

A DDP/ADP Rack Installation Kit has been designed specially to install the DDP/ADP into the above cabinets.  Refer to the D2350A product.

Recommended Peripheral Cables

DDP (RS─423) connected to an RS─423 or RS─422 device: The cable length and the terminal─end connector type are highly dependent upon the layout of system and peripherals and upon the terminal used.  Therefore, customized cables should be made using the pin─out given in the functional specifications above.

_______________

Recommended	Also supported

__________________________________________________

ADP II connected to a modem	40233A	92219Q

ADP II connected to a terminal	40234A	40242Y, 13242Y

(without hardware flow control)		40242M, 13242M

13242N

DDP (RS─232) connected to a terminal	40234A

40242Y, 13242Y

40242M, 13242M,

13242N

_______________

Note:	A DDP or ADP II port (RS─232─C) will be considered as: ─ an HP standard DTE at the termination of a 40233A cable ─ an HP standard DCE at the termination of a 40234A cable

ADP II connected to a device using the ░hardware flow control▒: A cable should be made using the ADP II pin─out in the functional specification above.  In addition to the data signals, the RTS/CTS signals (pins 8/22 of the ADP II) must be connected to the CTS/DTR (pins 5/20) of the device.

Description of the cables:

40234A 5m (16ft), 25─pin M to 25─pin M connector

40233A 5m (16ft), 25─pin M to 25─pin M connector

13242M 5m (16ft), 25─pin M to 25─pin M connector

13242N 5m (16ft), 25─pin M to 25─pin M connector

13242Y 5m (16ft), 25─pin M to 25─pin M connector

40242Y 5m (16ft), 25─pin M to 25─pin M connector, RFI filtered

40242M 5m (16ft), 25─pin M to 25─pin M connector, RFI filtered

92219Q 1m (3.3ft) 25─pin M to 25─pin M connector

Note:	Cables like the 13242M/N, 40242M and 92219Q have the line 23 connected which is used for internal loopback selftest.  If the device attached to the cable monitors this line during the selftest, it could result by a selftest error message.

Note that RS─232─C Standard is limited by cable capacitance.  HP will support the use of non─HP RS─232─C cables provided that the cables used are compliant with RS─232─C recommendations.