DEC Appendix

Alpha AXP Systems Comparison Chart

DEC 3000 Model	DEC 3000 Model

400 AXP	500 AXP

Workstation	Workstation	DEC 4000 AXP

and Model 400S	and Mels 500S	Distributed/	DEC 7000 AXP	DEC 10000 AXP

AXP Desktop	AXP Deskside	Departmental	Data Center	Mainframe

# of Processers	1	1	610: 1, 620: 2	610: 1, 620: 2	610: 1, 620: 2

630: 3, 640: 4	630: 3, 640: 4	630: 3, 640: 4

650: 5, 660: 6	650: 5, 660: 6	650: 5, 660: 6

CPU Clock	DECchip	DECchip	DECchip	DECchip	DECchip

Speed	21064	21064	21064	21064	21064

133 MHz	150 MHz	160 MHz	182 MHz	200 MHz

Cache Size	8 KB I-cache,	8 KB I-cache,	8 KB I-cache,	8 KB I-cache,	8 KB I-cache,

(on chip/on	8 KB D-cache/	8 KB D-cache/	8 KB D-cache/	8 KB D-cache/	8 KB D-cache/

board)	512 KB	512 KB	1 MB per proc.	4 MB per proc.	4 MB per proc.

SPECfp92	111.0	125.1	140.9	178.1	196.5

SPECrate_fp92	N/A	N/A	610: 3317.0	610: 4126.0	In progress

620: 6214.5	640: 15739.4

SPECint92	65.3	74.3	83.5	96.6	106.9

SPECrate_int92	N/A	N/A	610: 1985.8	610: 2188.6	In progress

620: 3816.1	640: 8366.8

SPECmark89	107.5	121.5	610: 135.5	610: 167.4	610: 184.1

SPECthruput89	--	--	620: 247.0	640: 604.4	640: 654.6

LINPACK 	26.4	30.1	36.3	38.6	42.5

100x100

(Mflops)

Dhrystone V1.1	129.9	146.7	158.8	177.3	194.5

(MIPS)

Max. Memory	128 MB/	256 MB/	512 MB/	2 GB/	2 GB/

(4/16-Mbit	512 MB*	1 GB*	2 GB*	14 GB*	14 GB*

Chip)

Max. Disk	2.1 GB/9.5 GB	4.2 GB/11.6 GB	16 GB/56 GB	28 GB/284 GB	56 GB/200 GB

(over 10 TB*)	(over 10 TB*)

Max. I/O	90 MB/sec.	100 MB/sec.	160 MB/sec.	400 MB/sec.	400 MB/sec.

I/O Support	Both systems: 2	Both systems: 2	4 SCSI-2, Fast	4 12-slot XMI, 3	4 12-slot XMI, 3

SCSI-2, 3-slot	SCSI-2, 6-slot	SCSI-2*, 4 DSSI	9-slot Futurebus+*	9-slot

TURBOchannel,	TURBOchannel,	6-slot Futurebus+,	10 CI*, 24 DSSI*,	Futurebus+*

Ethernet, FDDI,	Ethernet, FDDI,	Ethernet, FDDI*,	24 SCSI-2**, 16	10 CI*, 24 DSSI*,

ISDN*,	ISDN*,	Prestoserve*,	Ethernet, 8 FDDI,	24 SCSI-2**, 16

Prestoserve*,	Prestoserve*,	HiPPI*, IPI, VME*,	SDI, Prestoserve*,	Ethernet, 8 FDDI,

DECram	DECram,	DECram	HiPPI*, IPI*, VME*,	SDI, Prestoserve*,

Workstation:	Workstation:		DECram	HiPPI*, IPI*,

Voice-Quality Audio	Voice-Quality Audio			VME*, DECram

OpenVMS	Ethernet*, FDDI*	Ethernet*, FDDI*	Ethernet*, DSSI*,	Ethernet*, DSSI*,	Ethernet, DSSI*,

Clusters			FDDI*	CI*, FDDI*	CI*, FDDI*

Workstation	HX, TX*, PXG+*,	HX, PXG+*,	N/A	N/A	N/A

Graphcis	PXGT+*	PXGT+*

Entry System	$14,995 (W/S)	$38,995 (W/S)	$77,000	$168,000	$316,000

Price-U.S. List	$18,995 (Server)	$41,195 (Server)

ADVANTAGE-	$20,720 (Server)	$44,395 (Server)	$102,500	$187,000	$344,000

SERVER Price

Availability Date	Now	Now	Now	Now	Q1 Calendar 93

*	Available with upcoming operating system release.

**	Eight (8) SCSI-2 controllers supported initially.

N/A=Not applicable.

Features may differ between OpenVMS AXP and DEC OSF/1 AXP systems.

VAX Systems Comparison Chart

VAXstation 4000	MicroVAX 3100		VAX 4000

VLC, Models 60	Models 30, 40,	VAX 4000	Models 400, 500

System	and 90	80, and 90	Model 100	and 600

Performance	VLC: 6.2 SPECmark	30 & 40: 22 TPS	51 TPS	400: 50 TPS

60: 12 SPECmark	80: 28 TPS		500: 68 TPS

90: 32.8 SPECmark	90: 34 TPS		600: 102 TPS

Relative Proc.	N/A	30 & 40: 5	24	400: 16

Perforamnce x		80: 10		500: 24

VAX-11/7801		90: 24		600: 32

# of Processors	1	1	1	1

CPU Clock Speed	VLC: 25 MHz	30 & 40: 25 MHz	72 MHz	400: 63 MHz

60: 55 MHz	80: 50 MHz		500: 72 MHz

90: 72 MHz	90: 72 MHz		600: 83 MHz

Cache Size	VLC: 8 KB on	30 & 40: 6 KB 	8 KB on chip,	400: 8 KB on

chip;	on chip;	128 KB on board	chip,

60 & 90: 2 KB 	80: 2 KB		500 & 600: 10 KB

on chip, 256 KB	on chip, 256 KB		on chip;

on board	on board;		400 & 500: 128 KB

90: 10 KB on		on board,

chip, 128 KB		600: 512 KB on

on board		on board

In-Cabinet	60: upgrades	40 upgrades to	N/A	Each VAX 4000

90	80 or 90;		system upgrades

80 upgrades to		to any higher VAX

90		4000 system

Alpha-Ready	60 & 90: system	80 & 90: system	System upgrade	System upgrade to

System 	upgrade to 	system upgrade	to Alpha desktop	Alpha distributed/

Upgrade	Alpha desktop	to Alpha desktop	system	departmental

and deskside	system		system

workstations

I/O Features:

Max. Memory	VLC: 24 MB	30 & 40: 32 MB	128 MB	512 MB

Capacity	60: 104 MB	80: 72 MB

90: 128 MB	90: 128 MB

Max. Disk	VLC: 8.4 GB	8.7 GB	28 GB	56 GB

Capacity	60 & 90: 8.7 GB

Max. I/O	VLC: 5.0 MB/s;	4 MB/s	8.5 MB/s	12.5 MB/s

Throughput	60 & 90: 5 MB/s

(SCSI), 50.0 MB/s

(TURBOchannel)

I/O Support	VLC: Synchronous	1 SCSI, 1 Ethernet	3 DSSI (1 embedded,	4 DSSI (2 embedded,

SCSI, Ethernet;		2 Q-bus), 1 Q-bus,	2 Q-bus), 2 Q-bus,

60 & 90: Synchronous		1 SCSI, 3 Ethernet	3 Ethernet

SCSI, Ethernet,

TURBOchannel

High Availability Features:

VAXcluster	Ethernet	Ethernet	Ethernet, DSSI	Ethernet, DSSI

System Support

High Availability	Disk shadowing	Disk shadowing	Disk shadowing,	Disk shadowing,

Features Supported			online hardware/	online hardware/

software service	software service

upgrade	upgrade

Continued on next page.

VAX Systems Comparison Chart (continued)

VAXstation 4000	MicroVAX 3100		VAX 4000

VLC, Models 60	Models 30, 40,	VAX 4000	Models 400, 500

System	and 90	80, and 90	Model 100	and 600

Software Features:

System Software	VLC & 60: OpenVMS	OpenVMS	OpenVMS	OpenVMS

and VAXeln;

90: OpenVMS

Network Appl.	NAS 250	NAS 200, 300,	NAS 200, 300,	NAS 400

Support SW		400	400

1	VAX-11/780=1

N/A = not applicable.

Performance is highly dependent on configuration, application, and operating environment.  Individual workloads should be carefully evaluated before making performance estimates for specific applications.  In this chart no warranty of system performance is expressed or implied.

VAX Systems Comparison Chart

VAX 6000	VAX 7000	VAX 10000

Models 510 and	Models 610 to	Models 610 to

System	610	640	640

Performance	510: 50 TPS	610: 123 TPS	610: 123 TPS

610: 101 TPS	Others: N/A	Others: N/A

Relative Proc.	510: 13	610: 35	610: 35

Performance x	610: 32	620: up to 65	620: up to 65

VAX-11/7801		630: up to 95	630: up to 95

640: up to 125	640: up to 125

# of Proc.	1	610: 1	610: 1

620: 2	620: 2

630: 3	630: 3

640: 4	640: 4

CPU Clock	510: 63 MHz	91 MHz	91 MHz

Speed	610: 83 MHz

Cache Size	510: 2 KB	10 KB on chip/	10 KB on chip/

on chip,	processor	processor

512 KB on	4 MB on board/	4 MB on board/

board;	processor	processor

610: 10 KB

on chip,

2 MB on

board

In-Cabinet CPU	Each VAX 6000	Each VAX 7000	Each VAX 10000

Upgrade	system upgrades	system upgrades	system upgrades

to any higher	to any higher	to any higher

VAX 6000 system	VAX 7000 system	VAX 10000

system

Alpha-Ready	System upgrade	In-cabinet CPU	In-cabinet CPU

System Upgrade	to Alpha data	upgrade to Alpha	upgrade to Alpha

center system	data center system	mainframe-class

system

I/O Features:

Max. Memory	510: 512 MB	3.5 GB2	3.5 GB2

Capacity	610: 1 GB2

Max. Disk	Over 8 TB	Over 10 TB	Over 10 TB

Capacity

Max. I/O	80 MB/s	400MB/s	400 MB/s

Throughput

I/O Support	1 SMI, 4CI,	4 XMI, 10 CI,	4 XMI, 10 CI,

12 DSSI, 2 FDDI,	24 DSSI, 8 FDDI,	25 DSSI, 8 FDDI,

6 Ethernet,	16 Ethernet,	16 Ethernet,

5 VAXBI, 2 VME	1 VAXBI3, 2 VME	1 BAXBI, 2 VME

High Availability Features:

VAXcluster	Ethernet, DSSI,	Ethernet, DSSI,	Ethernet, DSSI,

System Support	CI, FDDI	CI, FDDI	CI, FDDI

High Availability	Disk shadowing,	Disk shadowing,	Disk shadowing,

Features 	online hardware/	N+1 redundant	N+1 redundant

Supported	software service	power system,	power system,

upgrade	uninterruptible	uninterruptible

power system,	power system,

battery backup,	battery backup,

online hardware/	mainframe-class

software service	service, online

upgrade	hardware/software

service upgrade

Continued on next page.

VAX Systems Comparison Chart (continued)

VAX 6000	VAX 7000	VAX 10000

Models 510 and	Models 610 to	Models 610 to

System	610	640	640

Software Features:

System Software	510: OpenVMS,	OpenVMS	OpenVMS

ULTRIX

610: OpenVMS

Network Appl.	NAS 200, 300,	NAS 200, 300,	NAS 400

Support SW	400	400

1	VAX-11/780 = 1

2	512 MB available now, higher capacities available with upcoming release of OpenVMS and higher density memory modules.

3	Supported by upcoming release of OpenVMS.

Performance is highly dependent on configuration, application, and operating environment.  Individual workloads should be carefully evaluated before making performance estimates for specific applications.  In this chart no warranty of system performance is expressed or implied.

DECsystems Comparison Chart

DECsystem 5000	DECsystem 5000	DECsystem 5000

System	Model 25	Model 133	Model 240	DECsystem 5900

CPU1/FPU	R3000A/	R3000A/	R3400B	R3000A/

R3010	R3010A		R3010A

SPECmark2	19.1	25.5	32.4	32.8

MIPS3	26.7	34.4	42.9	42.9

Clock Speed	25 MHz	33 MHz	40 MHz	40 MHz

Cache Size	64 KB (inst.)	64 KB (inst.)	64 KB (inst.)	64 KB (inst.)

64 KB (data)	128 KB (data)	64 KB (data)	64 KB (data)

Memory Capacity	8 MB-40 MB	8 MB-128 MB	16 MB-480 MB	64 MB-448 MB

(type)	(parity)	(parity)	(ECC)	(Ecc)

Enclosure	Desktop	Desktop	Desktop	Cabinet

Storage Capacity

Internal	Up to 426 MB	Up to 852 MB	None	Up to 37.2 GB

Total	Up to 25.3 GB	Up to 33.5 GB	Up to 33.1 GB	Up to 37.2 GB

I/O Bus Type	TURBOchannel4	TURBOchannel4	TURBOchannel4	TURBOchannel4

VME (opt.)	VME (opt.)	VME (opt.)	VME (opt.)

Peripheral	SCSI	SCSI	SCSI, VME	SCSI

Support	VME	VME	CI

Network Support	TCP/IP (std.)	TCP/IP (std.)	TCP/IP (std.)	TCP/IP (std.)

NFS (std.)	NFS (std.)	NFS (std.)	NFS (std.)

DECnet-ULTRIX	DECnet-ULTRIX	DECnet-ULTRIX	DECnet-ULTRIX

(opt.)	(opt.)	(opt.)	(opt.)

FDDI (opt.)	FDDI (opt.)	FDDI (opt.)	FDDI (opt.)

1	From MIPS Computer Systems, Inc.

2	SPECmark is the geometric means of ten compute-intensive, public-domain benchmarks compared to performance on a VAX-11/780.

3	Million instructions per second based on the Dhrystone benchmarks.

4	Digital and third-party TURBOchannel options are available.  Contact a sales representative or Digital reseller for third-party options available through the TRI/ADD program.

Entry-level Grayscale System Comparisons

HP 715/33	DEC 20	DEC 133	DEC 25

SPECint92	24	13.7	20.1	15.8

SPECfp92	45	14.8	23.5	17.5

SPECmark89	46	16.3	25.5	19.1

MIPS	41	21.6	34.4	26.7

MFLOPS	8.6	2.4	5.9	2.8

(Linpack DP)

Processor	PA-RISC	R3000A	R3000A	R3000A

Clock Speed	33 MHz	20 MHz	33 MHz	25 MHz

Cache Size	64 KB/64 KB	64 KB/64 KB	64 KB/128 KB	64 KB/64 KB

(Ins/Data)

X11 Perf	7633	---	---	---

2D/3D vec/sec (k)	610	153	298	285

Memory Capacity	8/192 MB	8/40 MB	128 MB	8/40 MB

Internal Disk	2 GB	426 MB	852 MB	426 MB

Capacity

Total Disk Capacity	69.7 GB	25.2 GB	33.9 GB	25.2 GB

Internal Bus	EISA	TURBO	TURBO	TURBO

# of Slots	1 (opt.)	2	3	2

RAM Pricing	$100/MB	$220/MB	$220/MB	$220/MB

$175/MB (16 Mbit)

Disk Pricing	$4.19/MB @ 525 MB	$5.28/MB @ 426 MB	$5.28/MB @ 426 MB	$5.28/MB @ 426 MB

$4.00/MB @ 1 GBSE	$5.20/MB @ 1 GB	$5.20/MB @ 1 GB	$5.20/MB @ 1 GB

$4.56/MB @ 1.38 GB	$4.56/MB @ 1.38 GB	$4.56/MB @ 1.38 GB

Warranty	12 mos	12 mos	12 mos	12 mos

List Price for	$5,690	$5,595	$9,995	$7,595

Equivalent Configs	19G/16/Dskls	17G/16/Dskls	19G/16/Dskls	16c/16/Diskls

Graphics	Intr.	Intr.	HX	CX

SPECint92/$1,000	4.2	2.4	2.0	2.1

SPECfp92/$1,000	7.9	2.6	2.4	2.3

SPECmark89/	8.1	2.9	2.6	2.5

$1,000

Mid-range Desktop System Comparisons

HP

715/50	DEC

725/50	240

SPECint92	36	27.3

SPECfp92	72	29.9

SPECmark89	69	32.4

MIPS	62	42.9

MFLOPS	13	6

(Linpack DP)

Processor	PA-RISC	R3000

Clock Rate	50 MHz	40 MHz

Cache Size (Ins/Data)	64 KB/64 KB	64 KB

X11 Perf	11,190	---

2D/3D vec/sec (k)	920	621/-

Memory Capacity	256 MB	16/480 MB

Internal Disk Capacity	2 GB	None

Total Disk Capacity	69.7/239.8 GB	33.1 GB

Internal Bus	EISA	TURBO

Number of Slots	1/4	3

RAM Pricing	$125/MB	$220/MB

$175/MB (16 Mbit)	$315/MB @ 64 MB

Disk Pricing	$4.19/MB @ 525 MB	$5.28/MB @ 426 MB

$4.00/MB @ 1 GB/SE	$5.20/MB @ 1 GB

$4.56/MB @ 1.38 GB

Warranty	12 mos	12 mos

List Price for	$15,590/$20,490	$22,465

Equivalent Configs	19C/32/525	19C/32/426

Graphics	CRX	HX

SPECint92/$1,000	2.3/1.8	1.2

SPECfp92/$1,000	4.6/3.5	1.3

SPECmark89/	4.4/3.4	1.4

$1,000

High-performance Desktop System Comparisons

HP 730	HP 735	DEC 240

SPECint92	51	80	27.3

SPECfp92	85	150	29.9

SPECmark89	86	147	32.4

MIPS	57.9	124	42.9

MFLOPS	23.7	40	6.0

(Linpack DP)

Processor	PA-RISC	PA-RISC	R3000A

Clock Rate	66 MHz	99 MHz	40 MHz

Cache Size	128 KB/256 KB	256 KB/256 KB	64 KB/64 KB

(Ins/Data)

X11 Perf	10,904	19,920	---

2D/3D	1180	1160	621/-

vec/sec (K)

Memory Capacity	128 MB	400 MB	480 MB

Internal Disk	840 MB	2 GB	---

Capacity

Total Disk 	64 GB	126.4 GB	28 GB

Capacity

Internal Bus	EISA	EISA	TUBROchannel

# of Slots	1	1	3

RAM Pricing	$125/MB	$125/MB	$220 MB

$175/MB (16 Mbit)	$315/MB @

64 MB

Disk Pricing	$4.19/MB @	$4.19/MB @ 	$5.28/MB @

525 MB	525 MB	426 MB

$4.00/MB @	$5.20/MB @

1 GB/SE	1 GB

$4.70/MB @	$4.56/MB @

1 GB/FW	1.38 GB

Warranty	12 mos	12 mos	12 mos

List Price for	#31,400	$37,390	$22,465

Equivlalent 	(19"/32 MB/	19"/32 MB/	19"/32 MB/

Configs	424 MB)	525 MB)	426 MB)

Graphics	CRX	CRX	HX

SPECint92/	1.6	2.0	1.2

$1,000

SPECfp92/	2.7	3.7	1.3

$1,000

SPECmark89/	2.7	3.5	1.4

$1,000

High-performance Expandable System Comparisons

HP 750	HP 755	DEC 5900(2)

SPECint92	51	79	27.3

SPECfp92	85	150	29.9

SPECmark89	86	147	32.8

MIPS	76.7	124	42.9

MFLOPS	23.7	38	6

(Linpack DP)

Processor	PA-RISC	PA-RISC	R3000A

Clock Rate	66 MHz	99 MHz	40 MHz

Cache Size	256 KB/256 KB	256 KB/256 KB	64 KB/64 KB

X11 Perf	10,904	19,120	---

2D/3D	1,180	1,160	Server

vec/sec (K)

Memory Capacity	64/384 MB	64/768 MB	64/448 MB

Internal Disk	2.6 GB	4 GB	35 GB

Capacity

Total Disk	236 GB	297.5 GB	227 GB

Capacity

Internal Bus	EISA	EISA	TURBOchannel

# of Slots	4	4	3

RAM Pricing	$125/MB	$125/MB	$220/MB

$175/MB

(16 Mbit)

Disk Pricing	$3.85/MB @	$3.45/MB @	$5.28 MB @

1.3 GB	2 GB/SE	1 GB

$4.15/MB @	$4.56/MB @

2 GB/FW	1.38 GB

Warranty	12 mos	12 mos	12 mos

List Price for	$55,650	$58,990	$49,950

Equivalent	(19"/64 MB/	(19"/64 MB/	(19"/64 MB/

Configs	1 GB)	2 GB)	1.3 GB)

Graphics	CRX	CRX	Server

SPECint92/	0.9	1.2	0.5

$1,000

SPECfp92/	1.5	2.3	0.6

$1,000

SPECmark89/	1.5	2.2	0.7

$1,000

DECstation Graphics Comparison

MX 	HX

2D KVectors/sec	248	621

2D Area fill,

Mplx/sec	20.3	30.5

3D KVectors/sec	NA	NA

3D KPolygons/sec	NA	NA

Resolution	1280x1024	1280x1024

Graphics Planes	1	8

Pixel Stamp proc	no	no*

1860 accelerator	no	no

Price	$695	$1,995

Z-buffer opt	NA	NA

*	The HX graphics option includes the new SFB ASIC for graphics, as opposed to the PixelStamp.

TX	PXG+	PXGT+

2D KVectors/sec	252	345	445

2D Area fill,

Mplx/sec	7.7	18.5	12.3

3D KVectors/sec	NA	401	436

3D KPolygons/sec	NA	70	106

Resolution	1280x1024	1280x1024	1280x1024

Graphics Planes	24	8/24	96

Pixel Stamp proc	no	yes	yes

l860 accelerator	no	yes	yes

Price	$3,995	$4,000	$15,000

(8 bit)

$8,000

(24 bit)

Z-buffer opt	NA	1,000	Included

for 8 plane,

incl. on 24

plane

.G.DECAPX01.HPG;4.565";3.356";HPGL

.G.DECAPX02.HPG;4.565";3.356";HPGL

.G.DECAPX03.HPG;4.565";3.356";HPGL

.G.DECAPX04.HPG;4.565";3.356";HPGL

PA7100 versus EV4 Alpha chip comparison

The Alpha architecture

Alpha is a true RISC architecture and a radical change from the previous CISC design of the VAX series.  It has 168 instructions, comparable to PA-RISC 1.1, MIPS-III, and IBM's POWER.  The instructions are all 32 bit and, like all RISCs, use a simple load/store model to access memory.  In order to simplify superscalar designs, the architecture does not use branch delay slots (in most RISC designs, the instruction after a branch is executed while the branch target is being fetched) or condition codes (like IBM POWER).

The architecture is simplified in many other ways.  Alpha does not support 8-bit or 16-bit loads or stores, address increment on loads and stores, or decimal arithmetic.  PA-RISC provides all of these features, which are useful for string operations, COBOL programming, and many other applications.  Alpha also has less flexible condition testing and memory addressing, and has no combination instructions such as PA-RISC's "compare-and-branch".  Stripping down the instruction set provides a tradeoff:  the simpler instructions allow Alpha to reach higher clock speeds, but in situations requiring these functions, Alpha will have to do more work to achieve the same result.

Alpha is a full 64-bit architecture, similar to MIPS' R4000.  The 32 general registers and 32 floating-point registers are all 64-bits wide.  PA-RISC has the same number of registers but the integer registers are only 32-bits wide, although the FP registers are also 64 bits.  Alpha's virtual address space is 64-bits, using a flat (unsegmented) design similar to the R4000's.  The architecture allows a physical memory space of up to 48 bits (256,000 gigabytes).  This is in contrast to PA-RISC's 64-bit segmented virtual address space and 32 bit (4 gigabyte) physical address space.

Note:  PA-RISC divides the full virtual space into 4-gigabyte segments.  Multiple objects (such as files or data arrays) of less than 4 GB each can be efficiently handled, but single objects greater than that size must be partitioned across segments, resulting in a performance decrease.  The flat architectures are equivalent to the segmented design for most objects, but have a performance advantage for the very large objects, however, very few customers are using objects larger than 4 GB today.

Instructions for both 32-bit and 64-bit operations are included; there is no mode bit.  Alpha specifies an 8 KB page size, twice that of PA-RISC 1.1.  This larger page size can increase the efficiency of the TLB but can lead to wasted space when dealing with smaller objects.

Alpha includes a few features to ease the transition from VAX systems.  Both architectures are little-endian, allowing data files to be easily exhanged from VAX to Alpha, and also to ACE systems and Intel PCs.   (SPARC, PA-RISC, POWER, and some MIPS systems are big-endian.)  The Alpha chips will also support both standard IEEE floating-point and  VAX floating-point formats.  Despite these features, all VAX software must be recompiled for Alpha to achieve full performance.

To summarize, Alpha is similar to most RISC architectures, but has been simplified to the extreme.  This allows DEC to design very high-clock rate superscalar processors (see next section), but these processors will need to execute up to 30 percent more Alpha instructions to complete the same tasks as a PA-RISC processor.  (For a more detailed analysis, see "Pathlength Reduction Features in the PA-RISC Architecture" by Ruby Lee, et al, COMPCON 2/92.)  As a result, Alpha chips may have higher native MIPS ratings than PA-RISC chips but achieve similar benchmark results.

The Alpha CPU

The Alpha CPU (code-named EV4) is a single chip design including CPU, floating point, and 8 KB each of instruction and data cache.  The chip is two-way superscalar, able to fetch and execute up to two instructions per clock cycle.  It uses a superpipelined design to achieve frequencies up to 200 MHz.  The chip supports an optional external cache at 1/3 to 1/8 of the internal clock rate.  The external cache can be configured from 128 KB up to 8 MB and requires an additional VLSI chip for the control logic.

The superscalar design of the EV4 is more flexible than HP's PA7100  chip.  The DEC design includes four independent units to handle load/store, branch, integer math, and floating point math respectively.  Each cycle, the chip will fetch two instructions and can begin executing both so long as each of the two are sent to different units, with only a few exceptions.  (Integer store cannot be paired with FP math, nor can FP store be paired with integer math.)  The PA7100 uses a similar algorithm but has only two independent units, one for integer and the other for floating point.  The flexibility of the EV4 will provide a significant advantage in integer-only code.  The PA7100 does have a smaller advantage by combining certain integer math and branch operations into a single instruction, as well as in the floating point add-and-multiply combination.  (Alpha does not include any of these combination instructions.)  Neither chip supports speculative or out-of-order execution.

Figure 1:  Pipeline Comparison

EV4	PA7100

Cycle time	5 ns	10 ns

Pipeline depth	7 stages	6 stages

Penalties:

Shift	1 cycle	0 cycles

Load-use	2 cycles	1 cycle

Bad branch	3 cycles	1 cycle

Off-chipcCache	3 cycles*	0 cycles

FP latency	6 cycles	2 cycles

*	At 1/3 clock ratio; up to 8 if lower clock

At first glance, the 200 MHz operation of EV4 appears far superior to the 100 MHz achieved by the PA7100.  However, the superpipelined design results in significantly higher pipeline penalties (see Figure 1).  Like a dragster, the EV4 is fast in the straightways but doesn't corner very well.  The superscalar design compounds these penalties, since up to two instructions are stalled for each penalty cycle.

To help mask the three cycle penalty for bad branch prediction, the EV4 uses a combination of static and dynamic branch prediction.  Some Alpha instructions contain branch hints, and subroutine calls place the return address on a four-entry return stack.  Dynamic branch prediction is done using a 2048-entry branch history table which works on the theory that a branch will tend to go in the same direction as it did the previous time.  The PA7100 does only static branch prediction, achieving a 70 percent--80 percent success rate.  The DEC approach should reach over 90 percent, but the larger branch penalty will produce a net negative comparison.

The small (8 KB), direct-mapped caches on the EV4 chip will produce a high miss rate on many applications (particularly commercial or multiuser), meaning that the CPU will spend much of its time refilling the on-chip cache.  This problem is exacerbated by the superscalar design; at two instructions per cycle, a cache miss could be generated every few cycles.  The PA7100 can access its external cache with no penalty, and typically will have much larger caches than the EV4's on-chip cache, reducing the miss rate to a few percent.  The DEC chip uses "hit under miss" and "critical word first" algorithms to reduce the on-chip cache miss penalty in some cases.  (The PA7100 also uses these algorithms.)  A four-entry write buffer is used to hide delays caused by stores to the external cache.

Another disadvantage of the EV4's on-chip cache is that it increased the chip's die size by 15 percent, resulting in higher cost.  The EV4 is designed to interface to either ECL or CMOS SRAM; since the chip requires 8 ns SRAM for secondary cache at 200 MHz (66 MHz external), expensive ECL SRAM may be required to reach that speed.  Standard CMOS 10--12 ns SRAM are used at lower clock frequencies.  The PA7100 achieves 100 MHz cache accesses using standard 9 ns CMOS SRAM by using a sophisticated circuit design which begins the  next access while the current access is still being completed.  This overlapped access technique allows higher cache access rates with a less expensive CMOS cache.

EV4 implements a 43-bit (8, 192 GB) virtual address space and a 34-bit (16 GB) physical address space as a subset of the full architecture.  The TLB is on-chip and contains 16 instruction entries and 32 data entries.  Four of the ITLB entries map 4 MB blocks.  The DTLB entries are all configurable to map anywhere from 8 KB (one page) to 4 MB (512 pages) each.  This TLB is much smaller than the PA7100's 136-entry unified TLB, but only 16 entries in the HP design are configurable to map large blocks of memory (512 KB to 64 MB).  Unlike the PA7100, the EV4 does not implement a hardware TLB update algorithm.

The Alpha chip is implemented using 1.68 million transistors.  The  on-chip cache consumes about 900,000 of those using 6 transistors per bit.  This includes some redundant rows, which can be used to replace failed cells.  DEC may use this redundancy to mask field failures or to improve the fabrication yield.  The logic functions on the chip consume the remaining 780,000 transistors, slightly less than the 850,000 used in PA7100.

Figure 2:  IC Process Comparison

HP	DEC

CMOS-26B	CMOS-4

Gate (drawn)	0.75 micron	0.75 micron

Gate (effective)	0.61 micron	0.50 micron

Gate thickness	160 Ang	105 Ang

Metal 1	2.6 microns	2.25 microns

Metal 2	2.6 microns	2.65 microns

Metal 3	6.0 microns	7.50 microns

Power supply	5.0 V	3.3 V

The EV4 uses DEC's 0.75 micron CMOS-4 process.  This process is about a half-step ahead of HP's CMOS-26B process (see Figure 2), which is used for the PA7100.  The effective gate length and gate thickness are significantly better than 26B, but the metal layers are about the same.  Since DEC is already using CMOS-4 for the current NVAX CPU (VAX 6600 series), there is little manufacturing risk.

Figure 3:  Chip Cost Comparison

EV4	PA7100

Die width	16.8 mm	14.0 mm

Die length	13.9 mm	14.0 mm

Package type	431 pin	504 pin

Cost ratio	$1.43	$1.00 (estimated)

The chip area is about 20 percent larger than the PA7100; combined with an estimated 10 percent higher wafer cost, this indicates a 43 percent higher chip cost (see Figure 3).  DEC has announced that the EV4 will be sold as the 21064-AA for $1,557 in volume for the 150 MHz version; most RISC CPUs are sold for well under $1,000.  (The PA7100 is not sold on the open market.)  The EV4 also has a high power dissipation of 30 Watts at 200 MHz, despite using reduced (3.3V) power levels; this is about 50 percent more power usage than a 100 MHz PA7100.  DEC will "bin" to reach its 200 MHz goal, since only a small percentage of chips work at that speed.  The slower chips will be used in a lower-performance system at 150 MHz.

Although DEC has not announced the SPECmark performance for the EV4, several articles have reported 140-150 SPECmarks, which would be about 10 percent-15 percent higher than the PA7100 at 100 MHz.  The chip will have better integer SPECmark performance than the PA7100 due to its more flexible superscalar design and branch prediction, although the penalties to access the off-chip cache may hamper EV4 in large applications such as OLTP.  Floating-point SPECmark performance will be less than HP's because of the long FP latencies, but vector performance could be higher if the on-chip cache is used exclusively.

VAXcluster configuration overview

A VAXcluster system is a single, large system made up of several VAX processors running the VMS operating system, and configured with globally shared, mass storage subsystems.  DEC developed VAXcluster systems in order to allow the sharing of data, disks, printers, and the computers themselves among users, across multiple CPUs.

VAXcluster configurations

VAXcluster systems are configured with the following groups of components:

┤	CPUs -- VAX processors, from desktop systems to the VAX 9000 mainframe  system, can be members of a VAXcluster system.

┤	Interconnects -- There are four types of interconnects that are currently used in VAXcluster configurations:

-	Ethernet

-	DSSI

-	CI

-	FDDI

┤	Storage Subsystems

VAXcluster configurations

┤	Ethernet-based VAXcluster systems:  Ethernet can be used to connect VAX systems and VAXstations in an Ethernet-based, or local-area VAXcluster.  An Ethernet-based VAXcluster configuration can be connected to CI or DSSI configurations to create larger mixed-interconnect VAXcluster systems.

.G.DECAPX07.HPG;4.565";3.356";HPGL

┤	DSSI VAXcluster systems:  The Digital Storage Systems Interconnect (DSSI) can be used to connect up to three VAX systems in a DSSI VAXcluster.  This three-system DSSI VAXcluster allows all three systems to share disks across the DSSI.  Data and applications remain available to all users in the VAXcluster, timesharing, or server, even if two of the three systems go down.  Systems in a DSSI VAXcluster can include three VAX 6000 systems, three VAX 4000 Model 300 or 500 systems, any combination of those, or one VAX 4000 Model 300 or 500 and any two Q-bus systems.  Some MicroVAX and VAX 4000 systems can include multiple DSSI buses in VAXcluster configurations.

.G.DECAPX08.HPG;4.565";3.356";HPGL

┤	CI VAXcluster Systems:  The Computer Interconnect (CI) can be used to connect up to 32 multiuser VAX VMS systems and terabytes of data storage in a CI VAXcluster.  CI VAXclusters can also configure up to 96 systems including VAXstations.

.G.DECAPX09.HPG;4.565";3.356";HPGL

Note:  The STAR stands for "Star Coupler". The Star Coupler is the fully redundant common connection point for all VAXcluster nodes connected to the CI.  It connects together all CI cables from the individual nodes into a radial, or star, arrangement that has a maximum radius of 45 m.  The Star Coupler can be configured to support VAXcluster systems of up to 32 CI interfaces (90 meters maximum between nodes).

HSC stands for "Hierarchial Storage Controller".  It is a self-contained, intelligent, mass storage subsystem that connects one or more host processors to a set of mass storage disks and/or tapes.

┤	FDDI VAXcluster System:  The VAXcluster Multi-Datacenter Facility extends VAXcluster systems across multiple geographic locations, and serves as a platform for disaster tolerance.  If a problem occurs at one site, the other site or sites can continue to process critical applications.  FDDI's bandwidth (100 Mbit/second) and allowable distance between systems (up to 40 Km) make it especially important as the interconnect in a disaster-tolerant VAXcluster system.

.G.DECAPX10.HPG;4.565";3.356";HPGL

Note:  The single point of failure for these VAXcluster configurations are the "star couplers" Which are the interface between the VAXs being clustered and the HSCs interfaced with the disks.

Also, keep in mind that the degree of system availability of the cluster is determined by the level of hardware redundancy in the system, that is the HSC redundancy.

VAXcluster benefits common to all these configurations:

┤	Single-point system managment across multiple systems or across multiple geographically dispersed sites.

┤	Expandability of bandwidth, number of systems in the VAXcluster, storage devices, distance between systems or sites with no application rewrites.

┤	Disaster tolerance for business-site protection.

┤	Shared mass storage.  All CPUs in a VAXcluster system share all storage devices whether on an HSC subsystem or a CPU.

┤	Shared batch and print resources with the VMS job controller providing load balancing across all VAXcluster system CPUs.

┤	Shared access to all disks in the cluster, providing access to all applications and utilities in the VAXcluster system.

┤	Cluster system-wide standard VMS system and security features.

Note:  If a CPU in the cluster goes down, users logged onto that system must relog onto another CPU in the cluster which will have access to the shared storage containing the application and the data the users were using at the time of CPU failure.

VAXcluster disadvantages:

VAXclusters are very expensive.  One needs to consider that not only are there extra CPUs to buy, but also other redundant components to consider that will eliminate single points of failure, as the typical configuration examples above illustrate.

In addition to acquiring the additional hardware components, there are several pieces of additional software required to maintain and manage the VAXcluster.  These software pieces include:

┤		A distributed file system that allows all VMS processors in a VAXcluster system to share disk mass storage to the record level.

┤		A distributed lock manager:  The VMS Distributed Lock Manager is a tool for synchronizing access to resources for processes.  The resources can reside on a single CPU or in a VAXcluster system.  The Lock Manager provides a namespace in which processes can lock and unlock resource names.

┤		The MSCP Server:  The MSCP (Mass Storage Control Protocol) is a protocol for logical access to disks and tapes.  The VMS MSCP Server implements the disk-only portion of this protocol.  It permits any VAX processor in the VAXcluster system to access disks that are connected locally to another VAX processor VAXcluster node.

System administators need to be trained to use this software, which several customers complain is very complex to use.  In addition, there is a lot of system overhead required to run these tools to manage the VAXcluster.

If one considers cost, you can easily see how at a very minimum 2 system configuration, the solution price is more than doubled in hardware/software and system administration costs.

Finally, keep in mind that these VAXclustering products are not based on any standards and are proprietary products that DEC uses to lock customers into VAX solutions.

NetBase clustering solutions for the HP 3000

NetBase is a comprehensive software solution consisting of five major services (that can be used alone or together).  Provided by QUEST in Newport Beach, Calif, NetBase offers functionality that can benefit high-end customers with distaster-recovery needs as well as customers with the need for loosely coupled systems.  This provides a unique capability for the HP 3000.  In fact, NetBase is the cornerstone of an ongoing direction that HP will continue to develop that will strengthen HP's ability to provide loosely coupled systems.  We will work with QUEST to tie more of their technology to our technology over time.

The five services that make up NetBase are listed below.  Each service is discussed in more detail later in this document.  In addition, if you have questions about NetBase you may call QUEST directly at (714) 720-1434 (and ask for technical support).

1.		NetBase Network File Access (also known as Central File Directory) - Gives applications transparent access to data and programs on other HP 3000 systems.

2.		NetBase Spooling - A network spooling product for HP 3000 systems that controls and and distributes all spooling activity.

3.		NetBase Shadowing - Automatically maintains copies of data throughout a network.  This feature is key because it helps deliver disaster tolerance.

4.		NetBase Statistics - A performance tool that captures data at a file and user level.

5.		NetBase AutoRPM (Remote Process Management) - Gives users transparent access to programs located on remote machines.

When NetBase can help you close a deal

NetBase is a unique capability for the HP 3000 and you should learn to recognize when it is appropriate to bring QUEST in on your sales cycle.  When you think it may be appropriate, call QUEST at (714) 720-1434.

For Installed-Base Customers:

High availability and disaster tolerance is required:  This is one of NetBase's most significant benefits.  Using shadowed data on geographically dispersed systems, customers are protected in the event of a natural disaster in one location.  If System-A fails, they can relog on to System-B (which resides elsewhere) and have access to the redundant applications and data.  This is a differentiator vs. DEC (see the information on DEC VAXclusters).

Multiple CPUs exist on a network with NS performance problems:  NetBase can actually improve network performance because it is very efficient about packing the data together to minimize the network traffic.

Batch and online jobs are competing for resources:  By moving some applications to other systems, or by letting batch jobs run against shadowed data on another system, you can use NetBase to reduce resource contention.  For example, you can create specialized servers for batch reporting, database engines, or print servers.  This can be an effective way to introduce sales for additional hardware.

Single CPU is overloaded:  NetBase can take part or all of an application and move it to another system, improving performance on the first system.  No application changes are necessary.  This customer scenario is the most difficult to qualify.  In general, it's necessary for the application to be modular or for the customer to have multiple applications on the system in order for NetBase to be an effective answer.  If the system is running just one large application against one large database, NetBase may not be able to help.  When in doubt, give them a call.  There are some situations they can quickly assess, and others they will have to investigate.

New business deals:

Mainframe offloading:  The qualification process here is the same as in the point above (single CPU overload).  If the applications on the mainframe are modular (or are being replaced with a modular application), NetBase can enable the customer to distribute the programs and data where they make the most sense geographically.  Yet to users, it will still look like one large system.

Mainframe replacements:  Same as above.  Disaster tolerance is required:  Same as in the high-availability point above.  The key thing to remember here is that NetBase is a cost-effective disaster-tolerance solution, and is better than what DEC clusters and the IBM AS/400 can offer.

Positioning NetBase with SPU Switchover/iX and Mirrored Disk/iX

SPU Switchover/iX is an effective solution for people who want one system to backup another system on that same site in the event of failure.  After a failure, System-B takes over System-A's HP-FL disks.  When System-A is up again, users simply relog onto System-A.

NetBase, on the other hand, is an effective solution for people who want one system to backup a system on another site, over a WAN.   In the event of a failure, users log on to System-B and use the shadowed data.  Thus it is the shadowed data that is being updated now, not the original data on System-A.

With both SPU Switchover/iX and NetBase, the operator must initiate the switchover process.

Mirrored Disk/iX is similar in that it is suitable for data-availability protection within one site.  However, for data-availability protection across the network, NetBase is a better solution.

How NetBase compares to DEC VAXclusters.  NetBase is very different than a DEC VAXcluster, although it provides some of the same benefits.  Customers who use clusters have typically identified three main benefits they wanted that led them to choose a cluster.

1.		High-end performance growth.  DEC was lacking a high-end multiprocessing system in the '80s, and VAXclusters were their way of providing more performance.  So DEC will try to position a VAXcluster that consists of 8 machines as being one system with the processing power of all those machines combined.  Since DEC came out with its multiprocessing systems, they have not bid clusters as aggressively.

In reality, VAXclusters don't offer linear performance improvements with each system that is added on.  Neither does NetBase.  HP provides performance growth through its cost-effective, high performing PA-RISC systems.  DEC still has to wait for its Alpha project to have RISC commercial systems.

Still, don't position NetBase against a cluster when the customer needs performance growth for a single (nonmodular) application.  Bid HP 3000 PA-RISC systems. We estimate that clusters have an edge over NetBase with regard to overall performance because all the systems logically share the same disk farm, and are connected with high-performing cables.  In fact, the disk controller for the disk farm is actually a PDP-11 computer.  NetBase, while it provides distributed data sharing, must go through an HP 3000 to get to the disk.  It uses standard networking, and is slightly slower as a result.  It is probably more cost effective than a cluster, but probably doesn't have the same performance.

2.		Loosely coupled data sharing and load balancing.  Both clusters and NetBase offer this benefit, with different implementations.  Clusters enable systems to access the same set of disks, through star couplers and redundant disk controllers.  These components have a distant restriction of 5--10 meters.  So true distributed data sharing across a WAN is not possible.

This diagram illustrates the concept of shared disk farms:

.G.DECAPX05.HPG;4.565";3.356";HPGL

NetBase enables users to transparently share the data that is spread across multiple HP 3000 systems on a network.  It enables a customer to fit the systems to the business, rather than fitting the business to the systems.

This diagram illustrates the concept of users being able to transparently access databases anywhere on the network, which is how NetBase works.  Notice how this differs from cluster's shared disk farms:

.G.DECAPX06.HPG;4.565";3.356";HPGL

Clusters will load balance several ways.  When a cluster gets a batch job, it will look for the least-busy system and assign the batch job to it.  Interactive session load balancing is possible if the cluster has a front-end terminal processor or front-end system (a more expensive cluster configuration).

NetBase provides load balancing by enabling customers to predefine where different types of jobs and applications will run.  For example, the local system could be reserved for heavy OLTP use during the day, while daytime batch jobs could be spooled out to run remotely on another system.  During a year-end crunch, the system operator could "borrow" another system's resources for a week or two to offload the first system.  NetBase's load balancing is flexible, because it works through a central directory that does not require the applications themselves to be modified.

Neither NetBase nor clusters provide truly dynamic load balancing.

3.		High availability.  DEC pushes this benefit heavily, stating that clusters have "no single point of failure".  And it's true that clusters can be configured so that every component is redundant.  Sometimes the customer will put a fault-tolerant VAX between the users and the HSC controller.

Clusters do a good job at providing high availability, but they are expensive.  And they CANNOT provide this benefit over a WAN.  Thus, they cannot position the cluster as a true disaster-tolerant solution.  If the cluster in Kansas is hit with a tornado, the customer is at risk.

Gartner Group has written a fact sheet stating that they consider clusters that are linked over an FDDI (high speed) network to be a disaster-tolerant solution.  But be aware that the FDDI has a distance limit of about 50 km.  If those two clusters had been in northern California during the '89 earthquake, they would have BOTH been down.

NetBase, on the other hand, provides redundancy over a WAN through data shadowing.  So when the HP 3000 in Kansas is hit with a tornado, the users can relog on to the system with the shadowed data in San Francisco.  Then if the system in San Francisco is hit by an earthquake, the users can relog on to the system in Los Angeles with ANOTHER shadowed copy of the data.  And if the system in LA goes down due to riots, they can log on to yet ANOTHER system in Florida with yet ANOTHER copy of shadowed data.  There is no distance limit.  NetBase is the solution for GLOBAL disaster tolerance.

Don't underestimate the value of this feature.  One of NetBase's customers, Discount Corp. of New York, is a trading firm in NY.  All of NYC suffered from an electrical failure one day, and all of the trading firms were frozen.  But Discount Corp. of New York had been shadowing data to New Jersey all along.  Since their users were completely without power in NY, they literally put their traders on a bus and rushed them to the NJ location.  Soon, they were trading again and they were the ONLY trading firm that was able to continue working that day.  That ability resulted in a lot of incremental revenue for them.

NetBase Network File Access

NetBase Network File Access (NFA) is also known around HP as Central File Directory.  It is high-performance networking software that gives applications transparent access to data and programs on other HP 3000 systems.  NetBase NFA offers increased networking performance and provides some level of horizontal system growth.  Spreading applications across multiple machines isolates application usage, increases throughput, and offers maximum flexibility in upgrade paths.  If additional processing power is needed, adding another system becomes simple.

Using NetBase NFA, an application designed to run on a single computer can easily be distributed across multiple computers without having to change a single line of application code.  NetBase maintains a centralized directory of all files and databases that are available to network users.  Once entered into the directory, all requests to that file or database are directed to the appropriate machine.

Features of NFA:

Remote file access without

remote session or NS/3000

Dynamic self-tuning for best

performance

No application changes needed

Comprehensive network security

Online network status displays

Supports all MPE file types

Online user application tracing

Optimizes use of the network

Dynamic node failure recovery

Benefits of NFA:

Low use of system resources

Simple to implement

Immediately improves

performance for users of

RFA and RDBA

Minimizes data communications

Comprehensive network security

overhead

Simplifies network management

NetBase spooling

Network spooling is a standard NetBase feature.  This makes it both simple and practical to have your programs output to any spooled printer anywhere in your network.  Simple, by providing you with several transparent methods of defining the printing environment.  Practical, by performing the task with a minimal amount of overhead.

NetBase spooling transports multiple lines of output per network transaction.  Sending the output in this manner drastically reduces networking overhead, yielding over 15 times the performance across the LAN.

Other features include integration with Novell NetWare, bidirectional transfers to HP-UX, and complete automation of any repetitive tasks.

Features of NetBase spooling:

Dynamic device mapping and

direction

Spools directly to remote

systems

Automates report distribution

Easy-to-read and flexible

banners

Online viewing of spool files

Automatic spool file archiving

Fast spool file search for error

recovery

Supports IBM, HP-UX, and NetWare hosts

Benefits of NetBase Spooling:

Increases application performance

Centralized printing

Shared print resources

Simplified operations

Scalable to print volumes

Reduces paper and printer cost

Network and system fault

tolerance

NetBase shadowing

NetBase shadowing replicates data in real time to multiple machines (more than 12).  It maintains redundant copies of files and databases on the network that can be useful for disaster recovery, increasing performance, off-loading busy machines, providing online backup, and providing 24-hour uptime.

By giving users concurrent access to both master and shadow copies of data, NetBase shadowing lets you off-load busy systems.  Batch reports and inquiry access can be executed on the shadow computer providing true load balancing with better throughput.

NetBase shadowing was designed to function through all modes of operation and failure.  This includes backups, network failures, and system failures.   If the shadow computer is unavailable, NetBase will automatically queue all updates on the master computer until the shadow machine becomes available.

Disaster recovery is simplified if multiple copies of data exist on the network.  Should a machine go down, all file access can be redirected to another computer almost instantly, bringing an otherwise unavailable application back online in an incredibly short period of time -- literally in a few minutes.

Features of NetBase Shadowing:

Real-time replication of data

across systems

Provides concurrent backup

capability

Uses low overhead transport

algorithms

Shadows both TurboIMAGE and

ALLBASE/SQL databases

Guarantees synchronization

between the master and the

shadow system

Supports multiple and partial

shadow copies

Works with all supported HP

network links

User exits increase functionality

and flexibility

Benefits of NetBase Shadowing:

Provides 24-hour data center

uptime

Reduces network traffic

Spreads file and database I/O

across nodes

Quick disaster recovery

Offloads batch reporting jobs to

second CPU to reduce resource

contention

Easy to configure

NetBase Statistics

While most performance tools provide a global picture of system health, NetBase statistics capture data at the file and user level.  File access data, process overhead, and response times are captured to quantify an application's use of system resources.  With this level of information available, application monitoring, profiling, and tuning are simple.  In fact, network strategies can be modeled in advance with NetBase Statistics Scenario Generator.

Standard, easy-to-read reports are provided with NetBase Statistics.  All data is kept in a documented log file format, so users can produce their own reports with the 4GL or language of their choice.  Statistics can be enabled and disabled at any time.

Features of NetBase Statistics:

Provides statistics of file access

by program and by user

Profiles application activity by

intrinsic, program, and user

Predicts performance of network

changes.

Generates reports at

varying levels

of detail

Advanced modeling capability

for "what if" scenarios

Very low overhead

Benefit of NetBase Statistics:

Points out poor locking strategies

Simplifies performance analysis

Uncovers application inefficiencies

Reveals all network file access

activity

Can be turned on or off at any

time

Simple to use

NetBase Auto RPM

AutoRPM (Remote Process Management) gives users transparent access to programs located on remote machines.  With one command, users gain instant access to virtually any software on the network.  AutoRPM automatically transports the user to the appropriate computer, reducing the need for duplicating data and applications.

Features of NetBase AutoRPM:

Provides transparent program

access across the network

Instantly redirects users to

remote applications

Handles all remote session

management

Transports file equations and

other session environment

information

No changes to application

Benefits of NetBase AutoRPM:

Simplifies network application

integration

Easy to manage

Reduces software and data

redundancy (which also reduces

software costs)

Transparent to users

RISC architecture overview and competitive comparison

PA-RISC delivers 64-bit functionality

In 1986, HP became the first company to ship a RISC processor architected for 64-bit virtual addressing, a feature which has been included in all PA-RISC processors.  Today's PA-RISC chips include many 64-bit features such as 64-bit registers, 64-bit loads and stores, and 64-bit addressing, which are similar to those used by DEC Alpha and the MIPS R4000, among others.

PA-RISC uses a cost-effective approach to deliver 64-bit functionality to the customer.  In some parts of its chips, 32-bit registers and data paths are used to reduce the cost of  the processor and deliver better price/performance to the end user.  These areas have been carefully selected so that they impact few (if any) customers, while the cost savings are shared by all.  As customer needs change, HP will continue to optimize its products to deliver needed functionality at the best possible price.

Keep in mind that 64-bit features are not a measure of performance.  It is a myth that 64-bit chips are "twice as good" as 32-bit chips. For a few customers with special needs, 64-bit features may enable them to do their work better or faster.  For the vast majority of customers, these 64-bit features will never be used.

64-bit addressing

There are two types of addresses, virtual and physical.  Virtual addressing determines the maximum amount of data that can be used by the system at any given time, and physical addressing determines the maximum amount of memory that can be installed in the system.  We will deal with these issues separately.

When a user needs to access data, whether it is a file, a set of numbers in a spreadsheet, or the result of a single calculation, virtual addressing is used.  For typical applications, a single user may need several megabytes (MB) of data, although a large application may require as much as 100 MB.  Since even a 32-bit processor has up to 4 GB (4,096 MB) of virtual  address space, it is very difficult for a single user to exceed the total virtual space available.  On a multiuser system, however, all data requirements of all active users must fit into the virtual address space.  Thus, a system with 50-100 users may exceed the address limits of a 32-bit processor, depending on the needs of each user.  If this happens, no new users can access the system.

PA-RISC solves this problem by extending the virtual address space to  64-bits using segments.  With this method, each user (or process) is assigned a "segment", or section of memory, of up to 4 GB in size.  Since the system has literally billions of these segments available, even very large systems will not run out of segments.  And the segments are large enough that very few users will need more than one.  IBM's POWER architecture uses a similar method with smaller 256 MB segments.

Both DEC's Alpha and the R4000 solve the 32-bit address limit by using a "flat" (unsegmented) 64-bit address space.  Using this method, users can be allocated more than 4 GB of space if needed.  Today, very few applications need this much space.  Even in situations where more than 4 GB is needed, PA-RISC will assign multiple segments to that process, although some performance overhead is required to switch between the segments.

PA-RISC does not use a flat 64-bit virtual address space because the segmented method is less expensive.  Using segments reduces the size of the register file and the TLB, allowing PA-RISC chips to be smaller (and thus less expensive) than comparable chips using a flat address space.  Allowing user programs to use 32-bit pointers also means that these pointers take up less space (than 64-bit pointers) when stored in memory, improving the efficiency of the data cache.  Although the flat method offers some performance improvement to a very small number of customers with extremely large applications, HP does not want the vast majority of customers to have to pay for a feature that only a few will use.  When more customers begin to need larger segments, HP will deliver a flat 64-bit processor, since the changes  required to do so are fairly minor (as shown by MIPS' evolution to the R4000).

There is also the issue of physical addressing.  The PA-RISC architecture supports up to a maximum of 4 GB of physical memory.  Most systems (including competitors') are limited to a much smaller number due to hardware limitations.  In particular, at today's prices, the cost of the DRAM alone for 4 GB of memory would be over $500,000, which would not be feasible except for a large mainframe or supercomputer.  With the price of memory falling by about 40 percent per year, 4 GB of memory will be feasible for high-end mini-computers by the middle of the decade, and possibly for high-end workstations by the end of the decade.  HP will expand the physical address range of PA-RISC when our customers require this feature.

As a final note, although the PA-RISC, MIPS, and Alpha architectures all support a 64-bit address range, it is common for the actual chips to use a "subset" of the full range.  For example, the first Alpha CPU only implements 43 bits out of 64.  HP is the only company to ever ship a full 64-bit virtual address implementation, which begin shipments in 1989.  Furthermore, in order to actually use more than 32-bit addressing, the MIPS and DEC operating systems must be modified.  Today, neither company is shipping a 64-bit operating system, but HP operating systems today use 64-bit segmented addressing.  The final section shows the address ranges specified and actually implemented by the various vendors.

Other 64-bit issues

There are several other areas of the chip that can be specified as 64-bits.  One critical area is the floating-point unit (used for scientific calculations).  There is a growing trend in technical application  toward highly accurate double-precision (64-bit) calculations.  PA-RISC, like Alpha and others, uses 64-bit floating-point registers  and can perform double-precision operations very quickly.  These registers can store very large numbers (as big as 10 followed by over 300 zeros) and can perform calculations with an accuracy of better than one part in nine quadrillion.

On the integer side, PA-RISC implements 32-bit registers while Alpha and the R4000 use 64-bit registers.  These 32-bit registers can store values as large as 4,200,000,000.  HP has found that customers who use even larger numbers or even more precise calculations use the floating-point unit.  Thus, there is no need to add the extra cost of larger integer registers.  The PA7100 chip can perform 64-bit floating point calculations (add, subtract and multiply) in just two clock cycles, so there is little penalty, and often a significant speedup, for doing precise calculations on the floating-point side.

A final issue is moving data into and out of the CPU.  Since processors can spend 20 percent--30 percent of their time moving data, it is an advantage to be able to move lots of data quickly.  The PA7100 can load 64-bits per clock cycle from either the instruction cache or the data cache.  (This is also true for store operations.)  Thus, Alpha and the R4000 have no advantage in this area.

PA7100 versus the competition

In general, the new PA7100 is very similar to the new DEC Alpha and the MIPS R4000.  The only differences are in the segment size and the integer register file; in each case, the PA7100 implements only 32 bits to reduce costs, since almost no customers will use the extra bits, as explained above.  All other PA-RISC processors implement the same 64-bit features as the PA7100 except for instruction loads, and some implement the full 64-bit virtual space, while the DEC Alpha and R4000 are not the same as previous CPUs from those vendors.

The IBM POWER processors are very similar to the PA-RISC processors, except that the segments are slightly smaller.  The older MIPS (R3000 and previous) and SPARC (SPARC 2 and previous) processors are basically 32-bit chips.  Sun has not released full details on the forthcoming SuperSPARC chip, but it is believed to be similar to SPARC 2 except for a new 64-bit floating-point and instruction loads.

HP	DEC

Feature	PA7100	Alpha

Instruction size	32	32

Architecture:

Max virtual address	64	64

Max segment size	32	62

H/W implementation:

Max. virtualaAddress	48	43

Max. segment size	32	39

Max. physical addr	32	34

Operating System:

Max virtual address	64	N/A

Integer register size	2	64

Floating-point regs	64	64

Instruction load width	64	64

Data load/store width	64	64

The table above shows a comparison of HP and DEC RISC chips.  The left column shows the various features that were discussed in this paper, while the numeric entries show the number of bits supported for each of the features.  For example, the first line shows that all of the processors use 32-bit instructions.  "Max. virtual address" is the total size of the virtual address space implemented.  The "Register size" lines show the width of the integer and floating-point registers.  The final section shows the number of bits of either instruction or data that the processor can load per clock cycle.

Conclusion

The PA7100 (and other PA-RISC chips) use a cost-effective approach to deliver 64-bit functionality to the end user.  In this way, PA-RISC customers do not pay for 64-bit features that they do not need.  In the future, the PA-RISC architecture will continue to evolve to meet the needs of its customers.  the 64-bit features are not a measure of performance, and most applications do not require or use 64-bit features.  The best measure of performance is still a benchmark, or better yet, running the target application itself.

Note:  The information contained here has been verified where possible with the manufacturer's published literature, but Hewlett-Packard assumes no responsibility for any errors.  Trademark names are used in an editorial fashion only with no intent to infringe.

HP 9000 versus DEC DECsystem/VAX Cost of Ownership

16-User configuration

List			List

HP 9000 Model F10	$11,250		DECsystem 5000	$15,715

16 MB RAM, 566 MB Disk,	incl		Model 240	incl

2.0 GB DDS, LAN, Console,	incl		16 MB RAM, Tape drive	incl

Multifunction I/O card,	incl		LAN	incl

8-user OS	incl		665 MB disk	6,700

16-user license	3,150		16-user OS	1,470

________			________

Total H/W and S/W cost	$14,400			$23,885

3-Year support	$8,030			$11,026

3-Year C.O.O.	$22,430			$34,540

128-User configuration

List			List	Support

HP 9000 Model G-30	$20,000		VAX 6000, Model 610	$202,336	$21,240

96 MB RAM, 2.0 GB disk	incl		System with OpenVMS	incl

2.0 GB DDS, Console,	incl		Base OS Lic., LAN	incl

566 MB disk,	incl		128 MB RAM	incl

32 MB RAM, LAN	incl		2.0 GB disk	7,952	2,280

64 MB RAM addition	6,400		128 user lic.	33,792	17,040

2.0 GB disk replacment	4,700		Tape drive	7,084	2,736

128-user license	17,875		Console	655	96

________			_________	________

Total H/W and S/W cost	$48,975			$251,829

3-Year support	$17,047			$43,392

3-Year C.O.O.	$66,022			$295,221

Or:

List

DECsystem 5900, 64 MB,	$59,496

RAM, 1.38 GB disk, LAN,	incl

4-user lic., media/doc.	incl

128-user OS	2,746

Tape drive	2,040

_________

Total H/W and S.W cost	$74,362

3-Year support	$26,712

3-Year C.O.O.	$101,074

256-User configuration

List			List	Support

HP 9000 Model H-50	$72,000		VAX 6000, Model 620 Sys.	$361,768	$38,160

64 MB RAM, 1 GB disk,	incl		with Open VMS Base Lic.	incl

Console, LAN, tape drive,	incl		LAN	incl

Media/doc.	inc		128 MB RAM	incl

192 MB RAM	19,200		4 GB disks	21,520	4,560

4 GM disk	13,800		1 GB disk	5,680	1,560

256-user lic.	22,0750		256-user lic.	33,792	5,400

Tape drive	8,868	2,736

Console	665	96

Media and doc.	incl	6,960

_________			_________	_________

Total H/W and S/W cost	$127,075			$432,293

3-Year support	$38,558			$55,368

3-Year C.O.O.	$165,633			$487,661

HP 3000 versus DEC VAX Cost of Ownership Comparisons

128-User configuration

List	Support		List	Support

HP 3000 Model 947LX	$106,500	$14,832	VAX 6000 Model 610 Sys.	$202,336	$21,240

SPU (FOS only)	incl		with Base OS	incl

Media & doc.	incl	408	128 MB RAM	incl

Standard chassis	incl	3,688	LAN	incl

64 MB memory	incl		2.0 GB disk	7,952	2,280

1.0 GB disk drive	incl	691	128-user OS/media & doc.	33,792	17,040

Console	incl	224	Tape drive	7,0840	2,736

2.0 GD DDS DAT tape drive	incl	1440	Console	665	96

32 MB add. memory	4,000

________	________		_________	________

Total H/W and S/W cost	$110,500			$251,829

3-Year support		$21,283			$43,392

3-Year C.O.O.	$131,783			$295,221

256-User configuration

List	Support		List	Support

HP 3000 987SX	$227,800	$21,000	VAX 6000 Model 620 Sys.	$361,768	$38,160

SPU (FOS only)	incl		with Open VMS Base OS Lic.	incl

Standard chassis	incl	36,040	LAN	incl

Media & doc.	incl	408	128 MB RAM	incl

64 MB memory	incl		4 GB disks	21,520	4,560

128 MB memory	incl		1 GB disk	5,680	1,560

2 GB disk	6,900	691	256-user OS	33,792	5,400

1 GB disk	incl	576	Tape drive	8,868	2,736

2 GB disk	6,900	691	Console	665	96

2.0 DDS DAT tape drive	incl	1,440	Media & doc.	incl	6,960

Console	incl	224

__________	________		__________	________

Total H/W and S/W cost	$260,800			$432,293

3-Year support		$61,070			$55,368

3-Year C.O.O.	$321,870			$487,661

Note:  For additional cost of ownership comparisons between the HP 3000 and DEC VAX, please refer to the CSY Hotline article "Compare".