CUBASE AUDIO FALCON cracked by MCA of ELITE

Consider this a Test version for the simple fact that we dont have a

Digital/Audio Converter to test it with ( Cubase Audio Clock ) !

For as far as we can check it , it seems to work ok . You will need

a D/A Converter and a SCSI Harddisk to use this Awesome program .

At least you know have the chance to take a glimpse of what this program

can do , so you can consider buying the Original ! Ist worth every penny !

The main loader + mainfile may not be packed !

To all these assholes that are selling cracked versions of Cubase for

large amounts of money .. FUCK YOU ! Your the real Pirates ! Hope you

get caught soon !

Please note that this Documentation is from a earlier version , so there

might be considerable changes to whats written below , but hey the real

fanatics figure it out , or buy the Original !

Elite here for the challenge ! Not the fucking money !

--------------------------------------------------------------------

W A R N I N G:

If You are not yet familiar with SCSI, and You want

not to destroy files, READ THE SCSI SECTION before

you start!

FALCON SPECIFIC HARDWARE AND SETUP TOPICS

To start, you will have to

- make sure the computer is switched OFF.

- plug the CAC (Cubase Audio Clock) interface

into the DSP Port.

- connect an SCSI Harddisk to the SCSI port. If

you are not familiar with SCSI yet, you should

most definately examine the following SCSI

section before you proceed!

- connect the Stereo Mini-Jack Audio Output from

the Falcon. This Output provides Headphone

Level, which can be lowered by some     amount

in the software. You can connect this Output

to a mixing desk, or directly into a Monitor

System (or Hi-Fi Amplifier, for instance,

Auxiliary Input).

- connect the Stereo Mini-Jack Audio Input to

your Audio Source. This Input has Microphone

Level Sensitivity, which can be adjusted by

some amount in the software. You can connect

this to a Mixing Desk, plug in a Microphone,

or, for instance, a Guitar Effects Processor.

In the case where you want to feed line level

into this input, you may run into problems, as

the Input is much too sensitive for such signals.

There are two ways around this: Make a cable which

has build-in serial resistors (10K - 50K Ohm).

The other method is to change the Hardware inside

the Falcon (doing so, you might want to install

cinch type connectors as well). See the Atari

recommendation at the bottom of this document

for better Audio Quality.

- Switch on the HD (HardDisk) first. Then turn on

the Falcon.

- Make sure that the colo(u)r resolution is limited

to 2 colo(u)rs. Unfourtunately, due to the design

of the Falcon, the communication between the

CPU and the DSP may loose data if more than 2

colo(u)rs (black+white) are used, and the Audio

will be distorted. 16 colo(u)rs seem to work,

but aren't guaranteed yet.

- For this version, you should most definately NOT use

MultiTos. Also, if you encounter any problems you

can't explain to yourself, try and remove all

Accessories (and re-boot).

- Start CUBAUDIO.PRG .

- Read the Manual!

SCSI

Cubase Audio for the Falcon requires a so called SCSI

(Small Computer Systems Interface) Harddisk for Harddisk

Recording. It is not possible to record or playback directly

from the internal (IDE) disk. However, Samples for the

built-in Sampler Device, and Ram Tracks may be played back

(but not recorded) from any drive (even the Floppy Disk),

as these are loaded into the internal memory.

SCSI is a very sensitive beast if you don't know about

its potential dangers. On the other hand, once you learned

to always check for proper installation, you are equipped

with one of the fastest mass storage systems so far.

There are three most significant items that you should

always check whenever you change your setup:

- SCSI ID: there may be up to 7 devices connected

in parallel on the SCSI bus. To be able to distinguish

between these devices, each device (HD, Laser Printer,

Streamer, CD-Rom etc) has to be assigned a unique

IDentifier. Selecting the SCSI ID is different for

every vendor's device, and you must read the manual

of the device to find out how to assign an SCSI ID

for that specific device. To make things easier, if

you just want to connect one SCSI Harddisk to the

Falcon, you should just set the HD's ID to zero. For

Cubase Audio, it could be any ID, but unfortunately

most versions of the Atari's SHDRIVER.SYS only check

from ID zero upwards until no more device is found,

ignoring devices which have a higher ID. Eventually,

there will be a SHDRIVER.SYS on one of the Cubase Disks,

which you should copy to the boot partition (normally,

named 'C'). This will also allow you to use

certain Harddisks which your SHDRIVER.SYS eventually

can't handle (because it has a bug). If you want to

use more than one SCSI device, ALWAYS make sure

that each device is assigned its own ID before you

switch on anything, otherwise you may loose up to

all of your data.

- SCSI TERMINATION: This topic is not so easy to

understand for us normal human beeings, albeit even

more dangerous if not handled very carefully. The

SCSI bus is current-driven to achieve high speed

data transmission. Whatever this means, it is ex-

tremely important that THE LAST (AND ONLY THE LAST!)

DEVICE IN THE SCSI CHAIN NEEDS A TERMINATOR! Yes,

Arnold Schwarzenegger should always point his gun on

you whenever you change anything. Actually, a Terminator

is a bundle of Resistors which 'pull' the current up

so that there is a closed loop. Otherwise, the current

is freely floating, and proper data transmission is

no more guaranteed (in the worst case, SCSI devices may

even be damaged). What makes this termination buisiness

even more dangerous is that it may seem to work even

without proper Termination; then, from time to time,

although everything looks good, important data such as

the bootsector, or rootsector, or directories may be

destroyed forever.

A Terminator is normally installed in a Harddisk when

you buy it. It represents itself as a couple (mostly,

2 or 4) of little boxes with 8 or 16 pins, located

directly above the SCSI connector and can be easily

plugged and unplugged. You should most definately

exactly know whether your HD is terminated or not. If

it is, as in most cases, and you just want to connect

only this one HD to the Falcon, you may safely connect

the HD to the Falcon and start the fun. If you ever

think about using more than one SCSI device, you should

remove the internal termination resistors from all of the

SCSI devices, and connect an external Terminator to the

last (and only the last!) device's socket in the chain:

----------   ---------   --------   --------   ------------

| Falcon |---| ID 0  |---| ID 1 |---| ID 2 |---|TERMINATOR|

|        |   |           |   |          |   |      |   ------------

----------   ---------   --------   --------

In this example, you don't necessarily have to assign

the ID's in ascending order, but it helps to keep things

more clear. Also, it doesn't matter which of the two

connectors of an SCSI device you use for what

connection, as these are always connected in parallel.

If you are sure that no device is terminated internally,

it is easy to see that ALL SCSI connectors must have

a plug applied.

The third item about SCSI is quite easy to understand,

but can be as easily forgotten about: ALWAYS SWITCH

ON ALL SCSI DEVICES! First switch all SCSI devices

on, then the Computer. If some device is not powered,

this may also cause a loss of data.

Last but not least, you should only use high quality

SCSI cables, which must not exceed a certain length.

The shorter, the better.

To sum it up: If you are about to buy a Harddisk (or

other SCSI device), you should ask your dealer all

the questions that may arise from what was said above.

It is a good idea to let your dealer adjust the SCSI

ID, make sure whether termination resistors are applied

or not, and buy good cables and an external SCSI Termi-

nator.

PROBLEMS THAT YOU MAY RUN INTO:

- If Cubase ever crashes (which it shouldn't), while

you are in a 'ready to record audio' mode, there

will be a huge file on your Harddisk which leaves

almost no more space on that partition. This is so

because the program creates a file which covers

almost all the space on the partition when you

want to record something. This is also why you

should always leave the program by 'QUIT' instead

of re-setting the Computer (otherwise Cubase can

not free the disk space not used anymore). If

this happens, delete this file from the desktop.

On the other hand, you should NEVER delete any

file while running Cubase manually (don't use

'Delete Files' for Audio files!), for instance,

via a 3rd party fileselector or some acessories.

To delete an Audio file you may do so from the Pool

Window, or select an Audio Part, hold the Control

key, and select 'Delete Part' from the menu, or

hold 'Control' and delete a segment in the Audio

Editor.

Related to this topic: If you want to save anything

(e.g. a song, or a file created by Offline Tools such

as 'Time Compression') onto the partition on which a

file is prepared for Audio Recording, a 'Write Error'

or 'Disk Full' error may occur. To avoid this, select

an Audio Track which can not record (no name assigned

in the Monitor Window) before such operations, or use

a different partition or disk.

- from time to time, an 'HD too slow!' alert may

appear, although the song has been played a hundred

times without problems before. This may happen with

some Harddisks having a so called 'Auto Re-Calibration'

feature. Unfortunately, in most cases this can not be

switched off. What happens is that the HD starts some

extensive calibration procedure at a more or less

regular interval, and during this time it can't

proceed data to the Host Computer fast enough.

- if you don't hear anything after you recorded

Audio, make sure the Monitor is switched off!

- if you don't hear anything at all, or only some

channels: make sure that the Mixer Track belonging

to the current Effect (in 'Hardware Setup') is

active and has the right Mixermap. Mixer Tracks

for Effects which are not used should be muted for

better performance.

- if you try to adjust the incoming audio signal,

it may be totally distorted but the monitor

meters just raise to 3/4 of the maximum?? Note

that the total volume of a channel is composed

of the volume in the Monitor Window AND the

volume in the Midi Mixer. The input clip is

shown in the Hardware Setup dialog. Always make

sure that the mixermap belonging to the current

Effect is active.

CHANGES TO THE FALCON HARDWARE SOUND INPUTS AND

OUTPUTS TO INCREASE THE AUDIO QUALITY (as recommended

by ATARI):

- first please note that these changes will

invalidate your warranty.

- to remove the bass boost on the output:

- change C58 and C59 from 100PF to 1000PF

- change R123 and R130 from 10K to 15K

- change R59 and R68 from 100K to 4.7K

- change C95 and C90 from 0.033yF to 1K Resistor

- to reduce the speaker amplifier gain:

- add 10K Resistor between pin 2 of U21

(@R119) and AGND (@R111).

- to reduce the Input Gain by 6dB:

- remove R56,R57,R90,R92,C21, and C81

- change R66 and R87 from 100K to 47K

- add 470pF NPO +/- 5% in parallel with R67

- add 470pF NPO +/- 5% in parallel with R68

- add 100pF NPO +/- 5% in parallel with R66

- add 100pF NPO +/- 5% in parallel with R87

Hardware Requirements

- a Falcon030, if EVER possible with full (14MB) ram,

should have internal hard drive

- Cubase Falcon Audio software and key

- the C.A.C (Cubase Audio Clock) interface connected

to the dsp port (it won't do without!)

- audio cables for audio in/out

- Monitor, cable, adaptor

- min. 1 SCSI drive

- stereo amp + speakers, or headphones (can be

directly connected to audio output)

Set it up

- READ THE SETUP DOCUMENT!!!

- connect the HD

- plug in oscillator

- forget the key

- connect monitor

- connect audio

- start the HD and wait until it's booted

- start the falcon

- start CUBAUDIO.PRG

- start the fun

when new software comes in...

- contains the program and evtl. FFONT.SYS. This file

MUST be present in the CUBASE.DAT folder (important!).

The README will inform about changes. Eventually the

mixermaps will change (and be provided). You should

use the DEF.ALL provided with the program.

How does it work?

- Just like any other Cubase Audio. Only SCSI drives

are supported. Samples and Ram Tracks may be played

from any drive, though.

Only 44.1 kHz sample rate is supported.

- there are 8 audio CHANNELS in the audio engine.

Normally, a channel is an output for an hd audio track.

With fast hard drives, 8 tracks of contigous audio

can be played back at 44.1 kHz simultaniously!

The engine can handle 3 different types of audio

sources:

- hard disk tracks as usual

- ram based tracks

- sampler outputs

- Ram tracks:

- these behave exactly like hd tracks. You can

edit them in the audio editor as usual. The

segments are loaded into ram whenever it becomes

necessary (editing). Then these segments are

played back from ram leaving power for the hd.

Very useful for short loops and repetive stuff.

- Sampler outputs:

- You can drag a segment from the pool into

the configuration page (described later), and

assign it a note, channel, and output. Then

you can trigger these samples from a normal

midi track on the SAMPLER output device. Samples can

not be tuned in realtime, and each voice requires

one output.

- the 8 audio channels are mixed down in the dsp with

additional effects. Currently, reverb, chorus

(2 short delays with sine wave lfo), and filters (3 para-

metric eq's per channel) are available.

- The mixes are on a mixer track, each channel has a volume, pan,

and either reverb send, and a common reverb parameters section,

or 2 delay sends per channel and two common delay parameter

sections, or the filters. Note you need the according

mixermaps.

- VU meters are displayed in the monitor window, where

you can also adjust the overall volume and mutes for

each channel.

- recording can be done as usual. Mono and Stereo recording

is possible without additional harware, but only either

Stereo, Left, or Right may be recorded. On the other hand,

you may record the entire mix. Note that you can record the

mix to an audio track which is set to a sampler channel

(thus it can't be played back, but recorded on).

If you attempt to record onto a track set to 'any' in the

Arrange Window, channel 1+2 will record Stereo if that

Audio Track is selected (and a record name was assigned

to channel 1 in the Monitor Window).

Hardware Setup:

- selected from 'Hardware Setup' in the audio menu. The right

side shows a list with the samples loaded and their note,

channel, and output assignment. Just drag segments from the

pool into the window.

- the left hand column is the most important one. You can

assign each of the 8 channels a hd track, ram track, or a

sampler output.

- above this list the effects can be selected. As mentioned

before, currently reverb, chorus, and filters are available.

You must activate the associated mix track when selecting

a new effect (for better performance, you should mute

all other mixer tracks for dsp effects). Also, it is a very

good idea to have chase events ON for at least the mixer.

- the Audio input select determines the source for Recor-

ding. If a mono Recording is made, the LEFT or RIGHT

assignment becomes valid, for Stereo these are don't care.

The MIX is always exactly what you hear; in this way, you

can layer Recordings and use the effects several times.

Of course, the mix stays in the digital domain, thus there

is no loss of quality.

--------------------------------------------------------------------

known problems:

- Midex doesn't work with the falcon because of a

hardware change on the rom port. It may be used

for key-switching, though. The modified Midex will

also not work with THIS Cubase because of not enough

processing power for midex AND audio.

- The SMP 24/SMP II works with the provided driver.

Also note that you MUST use MROS3_37 or higher.

- the early Falcons have a serious hardware bug. One

effect of this is, that the audio has crackles in it.

Much more serious is the fact that GARBAGE DATA ARE

WRITTEN TO (HARD) DISKS! There is a test program

provided from ATARI, but we can't guarantee that

it will work. We haven't yet tried whether the

hardware modifications recommended by ATARI will

solve the latter problem.