Welcome to a4000_atapi.device by Elaborate Bytes!

Copyright  1994, 1995 Elaborate Bytes, Oliver Kastl

GENERAL

This device allows you to use an ATAPI CD-ROM drive

with the A4000 or A4000T embedded IDE controller.

A4000 users can use the very popular and cheap CD-Roms for PCs

without any major disadvantages against SCSI CD-Rom drives.

The major advantage for A4000 users without a SCSI controller

is, that they don't have to buy one.

The major advantage for A4000 users with a SCSI controller

is, that they have an unused IDE port in their machine anyway

and SCSI CD-Roms are usually more expensive.

The device maps SCSI-II commands to ATAPI packets using the

standard Commodore HD_SCSICMD interface, so any "CD-Rom

specific" programs (Multisession Filsystems, Audio Players and

Photo-CD readers) designed for SCSI-II CD-Roms should work.

This has been tested with CacheCDFS, PlayCD, CD32-Emulator

(all written by me) and PhotoWorX (an excellent PhotoCD Reader

by Corporate Media).

However, I cannot guarantee that every SCSI-II specific program will

work, but I do not expect any major problems.

The device is fully interrupt driven and will use only a small

amount of CPU time for the transfer (exceptions are listed below).

COMPATIBILITY WITH EXISTING IDE HARDDISK

Every (and I mean it!) IDE drive that works with the original

Commodore scsi.device WILL continue to work if the a4000_atapi.device

is installed. The reason is simple: My device will use the code of the

Commodore scsi.device (if present) to access the IDE harddisk.

If you connect an ATAPI CD-Rom and you harddisk stops to work, this

usually is a hardware, not a software problem.

Check cables & jumpers.

Some harddisks must be rejumpered, if a slave drive is connected.

Consult the manual that came with your harddrive before doing so

or ask your dealer for assistance.

Make sure, the CD-ROM is configured as a slave device.

Some CD-Roms (e.g. the Wearnes CDA-120) have an option to enable or

disable interrupts. As the a4000_atapi.device is interrupt driven to

reduce CPU use, it will NOT work if you disable interrupt generation

on the CD-Rom drive.

Please consult the CD-Rom installation manual.

USING MORE THAN TWO UNITS

Starting with version 6.x, a4000_atapi.device is able to support up to

4 units. This may be up to two harddisks or any number of CD-Roms.

For this feature to work, you need a special cable which has one connector

for the Amiga IDE port and four connectors for different IDE units.

This cable will be shortly available from official Elaborate Bytes

distributers.

This cable will in fact add a "second" full operational and independend

IDE port to the system. (WOW! How is this possible? I would really love

to share this secret, but I am not allowed to tell :-)

Connecting the CD-Rom to the "second" IDE port will effectively hide

the CD-Rom from Commodore's scsi.device during boot.

As a side effect this eliminates any problems which could be caused

by the scsi.device which may be confused by the presence of a CD-Rom.

Connecting the CD-Rom to the "second" port will eliminate any possible

master/slave problems with existing harddisks.

A600 & A1200

Support for A600 and A1200 has been added. The device is called

"a1200_atapi.device" and will work with both machines.

You should use the SetAtapiSpeed command to maximize the CD-Rom performance.

Using more than two units is not possible with the A600.

TESTED DRIVES

A couple of drives have been tested with the a4000_atapi.device.

The test configuration was an A4000/40 with and without Oktagon 2008

SCSI controller, with and without an IDE Harddisk (Quantum Maverick 540

or Seagate ST328A). The drives were selected always as the slave device,

even if no IDE Harddisk was installed.

Recommended drives (work 100%):

Mitsumi FX-001DE (2x speed)

(Don't mix this up with the FX-001D! The FX-001, FX-001D and LU005

are NO atapi devices and will NOT work! Connecting a non IDE/ATAPI

device to the Amiga IDE port may damage both your Amiga and your CD-Rom!)

Mitsumi FX-300   (3x speed)

Mitsumi FX 400 (4x speed)

Chinon CDS-545 (4x speed) <-- very good drive!!!!!

Wearnes CDD-220A (4x speed)

Sony CDU-55E (2x speed) <-- Audio play requires CacheCDFS 2.5 or better

NOTE: The CD32-Emulator will only work with the Sony CDU-55E, if no volume

control is enabled in the emulator's preference window.

Acer 6825-P      (2x speed)

Wearnes CDD-120A (2x speed)

Chinon CDS-525I (2x speed)

The following drive cannot play Audio with Amiga CDDA Audio players,

but data read operation is ok. The CD32-Emulator will not work correctly

due to the problems with CDDA-Audio.

Unfortunately the SCSI-II Inquire command does not work with this drive,

if no CD is inserted.

Even if it may be used to read data, I can not really recommend this

drive:

Elitegroup Vertos 300SSD (2x speed)

These drives use a bit more CPU as they have problems with the

implementation of interrupts with the Amiga IDE port, but they work

quite well (Audio & Data) and can still be recommended:

NONE (All drives use interrupts with Release 5.9)

TESTED FILESYSTEMS

The device has been tested with the CacheCDFS (yes, this is written

by me, too) and my PlayCD Audio player.

It works very well with my CD32-Emulator.

However, I don't expect any problems with other filesystems.

INSTALLATION

Installation is simple.

Connect an IDE-ATAPI drive to your IDE port.

You should jumper the drive as a SLAVE device, even if you do

not have an IDE harddrive connected, as the A4000 may not boot

if it encounters an ATAPI CD-Rom as the Master device.

However, IF your Amiga does boot with a CD-Rom as the master

device, you could use two CD-Roms at the same time.

The unit numbers will be 0 (for the master) and 1 (for the slave).

If you have an IDE harddrive, you should buy a standard PC-IDE

harddisk cable, as the original A4000 cable is too short for both an

IDE harddisk and a CD-Rom drive.

The harddisk should be jumpered as the Master device and the

CD-Rom as the slave, otherwise the Amiga will not recognize

the harddrive.

Copy the a4000_atapi.device to your devs: directory.

The supplied installer script will do this for you.

Now you must tell your CD-Rom FileSystem about the device.

This differs from product to product, please consult the manual

of the Filesystem you wish to use.

With CacheCDFS, simply run the CacheCDFS installation program.

It will automatically detect your CD-Rom.

A CD-Rom connected as the slave device will have unit number 1, which

will be the common case.

A CD-Rom connected as the master device will have unit number 0.

The name of the device driver is a4000_atapi.device.

KNOWN PROBLEMS

It is possible, that some drive models cannot be mounted after a reboot,

because they need some time to wake up after a hardware reset.

(On a PC it takes often a long time until the CD-Rom is accessed)

This depends on how "fast" your startup-sequence reaches the mount

command.

If you have this problem, add a delay (e.g. Wait 3) before the mount

command in your startup-sequence.

That's it!

Please send comments, bug reports, etc to:

Elaborate Bytes

Oliver Kastl

Am Westpark 5

81373 Munich

Germany

Compuserve: 100447,3361

Internet: 100447.3361@compuserve.com

To order the CacheCDFS, the CD32 Emulator, the a4000_atapi.device

or all of the above, please contact:

Hirsch & Wolf OHG

Mittelstr. 33

56564 Neuwied

Germany

Tel.: ++49-2631-83990

Fax.: ++49-2631-839931

or

NBG-USA, Inc.

482 Holly Ave.

St. Paul MN55102

USA

Tel.: ++1-612-2909447

Fax.: ++1-612-2909449