-----BEGIN PGP SIGNED MESSAGE-----

NAME

HDOff -- a program for the A600/A1200 and A4000 to stop the HD

motor.

SYNOPSIS

HDOff T=TIME/N,T2=TIME2/N,FORCE/S,OTHER/S,OTHER2/S,NOW/S,NOW2/S,

SET1200/S,SET4000/S,UPDATE/S,QUIET/S,STOP/S,STOP2/S

DESCRIPTION

HDOff  is  a program like NoiseSaver for the A1200 and A4000.  It stops

the HD motor x minutes after the last access.

TIME, TIME2 :

Time in minutes after the 1st/2nd HD will shut down.

FORCE :

Forces  HDOff  to  quit.  Normaly HDOff doesn`t quit if another

programm  (like  cachesoftware  ) patched the scsi.device after

HDOff  did.   But if you are using SetFuncManager (or something

similar), HDOff can nevertheless quit without danger.  ONLY USE

THIS  OPTION  IF  YOU ARE USING SETFUNCMANAGER OR SOMTHING LIKE

IT!

OTHER, OTHER2 :

This  option  makes  HDOff  to  use an alternative command ($94

instead  of  $E0)  to  stop the HD`s motor.  If your HD doesn`t

spin  down,  try  this  option.   Some drives, especially older

ones,  don`t  support  the  `STANDBY`  (all newer drives should

support  it,  at  least  the "green-PC" drives supporting power

management).

NOW, NOW2 :

The  selected HD spins down immediately on startup, the program

just does it`s normal job (usefull for seldom used HD`s).

SET1200, SET4000:

With  some odd HD`s, HDOff isn`t able to detect wich controller

is  present  in  your  Amiga  (this happens, too, if you have a

CD-Rom  connected  but no CD inserted).  If you start HDOff and

get  a  "Incorrect  HD  controller!"  message, but are actually

using  an  A600/A1200/A4000,  use  the  option  SET1200  for an

A600/A1200 or SET4000 for an A4000.

UPDATE:

With this option you can adjust the shutdowntime online.

QUIET:

Using  this  option  HDOff won`t print anything out (similar to

>NIL:).  Usefull e.g.  if you start it in the user-startup.

STOP, STOP2:

HDOff  will  just stop the 1st (STOP) or 2nd (STOP2) harddrive,

but HDOff won`t be installed.

NOTE

Be shure not to set the "time" value too low (maximum are 65535 minutes

:)  as permanently spinning the drive up and down does shorten the life

expectancy of the drive (60 minutes is a good choice).

You don`t need to start HDOff with run.

To quit HDOff, simply restart it.

If   a   requester   with   the   message  "Someone  else  has  patched

scsi.device!..."  pops  up,  someone  else (e.g.  caching software) has

latched  into  the  scsi.device.   For security reasons HDOff cannot be

terminated this time.

This  program is 100% written in assembler (DevPac 3.14) and (as far as

its possible) systemconform.

EXAMPLES

1. HDOff 30

Stops the motor of the 1st HD 30 minutes after the last access.

2. HDOff 30 15 OTHER2

Stops  the  motor  of the 1st HD 30 minutes, the motor of the 2nd HD 15

minutes  after the last access.  For the 2nd HD the alternative command

will be used to stop the motor

3. HDOff T2=20

Stops the motor of the 2nd HD 30 minutes after the last access.

4. HDOff 30 15 NOW2

Stops  the  motor  of  the  1st  HD 30 minutes, the motor of the 2nd HD

immediately.  If the 2nd HD was accessed and so spins up, the next spin

down will again happen after 15 mins.

5. HDOff 50 10 UDPATE

If  you  already  started  HDOff,  the  timeout`s  for the HD`s will be

changed to the new values.

6. HDOff STOP QUIET

This  will  just  and  only  stop  the 1st drive, there will not be any

timeout!  Additionally there will be no output.

TECHNICAL

How does it work? The essential things are:

1. Test if ide controller is present (if not: quit)

2.   Patch  scsi.device  (using  SetFunction,  of  course).   The patch

signals HD accesses to the main task.

3.  Count the timer down and reinitialize it if a signal from the patch

occures.

4.   If  timer  reaches 0:  Signal the patch to delay all commands (for

both  drives)  till  the  signal  is  retracted.  Security delay 1 sec.

Forbid(),     Wait_IDE_unbusy,     select     drive     (master/slave),

Wait_IDE_unbusy,  spin  down  HD, Permit().  Retract the signal for the

patch for the HD that was not stopped, for the other one after a 5 secs

security delay (spin up is done by the HD itself).

5. Check, if there are any messages and process them.

6.  Repeat step 3-4 until abort.

For   more   information   about   the   messageport  have  a  look  in

Developers.doc!

DISCLAIMER & COPYRIGHT

This  software  is subject to the "Standard Amiga FD-Software Copyright

Note"  It is Freeware as defined in paragraph 4a.  For more information

please read "AFD-COPYRIGHT" (Version 1 or higher).

AUTHOR

Gideon Zenz

Giersbergstr. 41

53229 Bonn

GERMANY

EMail: gzenz@ixc.net

-Gideon Zenz, 20-Jul-97

BUGS

None known.

FUTURE

I  have  now a SCSI-only system, but this didn`t prevent me not to do a

SCSI  version  of  HDOff.  The reasons are that HDOff wasn`t planned to

support more than 2 drives, and not more then one device, so it will be

quite difficult to implement.  But we`ll see B-)

HISTORY

v1.0: (08-Mai-94)

- Initial release

v1.1: (30-Jan-95)

- Timerloop in minutes and much more precisely

- No need to start with run any more

- At all more secure and more systemconform

- A4000 support

v1.2: (20-Apr-95)

- docs completely rewritten

- Support of 2nd HD`s

- New args OTHER and OTHER2 and NOW and NOW2

v1.33: (7-Jan-96)

- HDOff crashed if started on heavy diskaccess.

- new options SET1200 and SET4000

- Implementation of the onlineport. Have a look in

Developers.doc for usage.

v1.34: (22-Feb-96)

- Fixed a bug in the onlineport: PortVer wasn`t set.

- Added GetStats&Source

- Completely reworked Developer.doc

- Now working 100% on A600

- New arg UPDATE (thx to Ingo Eickelkamp for suggestion)

v1.35: (16-Mar-96)

- New option QUIET

- Bugfix in UPDATE, wrong values were taken

- Now there is again a check if you enter too big values

- MAJOR bug: HD spun only the 1st time down on a1200 [thx to Oli Lohr]

- System of timecounting changed, now 100% accurate

- NOW(2) working again [thx to Oli Lohr]

- New options STOP(2) [thx to A.BOERNER@bbrandes.berlinet.de]

- New, bugfixed and improed version of Getstats [Thx Matthias Andree!]

v1.36: (26-Jun-96)

- Some bugfixes

- Message port expanded/changed (look at Developers.doc)

- Everything is now multi threaded, so msg`s etc are immediately

processed now.

- Included new, advanced GetStats

v1.37 (1-Sep-96)

- Fixed Mungwallhit

- Optimized

- Msgport updated for GUI by M.Andree (look at Developers.doc)

- adapted GetStats

v1.38 (20-Jul-97)

- some bugfixes (thx to Dave Jones, Oli Lohr)

THANKS

Many    thanks    to   heureka@bigben.dssd.sub.org   and   M.ZIELINSKI@

amtrash.comlink.de for their helps on A4000 and 2nd HD support, and all

other`s  who wrote me!  A special thank must go out to Matthias Andree,

who had a look at the docs, and who is now doing a nice GUI!

SECURITY

If  you  want  to be shure you have the original programs, check

with  "md5sum  -c  HDOff.doc".   (md5sum is part of the PGP package)

dfdf15f58f042ffa0f018ba8277124f7  AFD-COPYRIGHT

f7da7a8653f2153aea03f73d664f2a40  AFD-COPYRIGHT.de

5be909723874ce17be7fc3c426025f98  Developers.doc

732fcce06b2f7c5e3e1abbaf8f9ab2d6 *GetStats

eb56a33c57b9992656dcaf45cf3da761  GetStats.c

2b624ffa9e8fdac01ff101ab2a298dde *HDOff

96dc1c9138a28d041c5fd8e8406096dd  HDOff_1.38.readme

- -----BEGIN PGP PUBLIC KEY BLOCK-----

Version: 2.6.3ia

mQCNAi3izr8AAAEEAMi+7o+iKDG26t8EuoX0NJ92iwhkviRC3GdJ1Uvef4+xJA3V

ey20ZnzBg/OokPdo0a3VxhwyjD2auyFmp7DLupQTko7Wx2zLk19EzVBxI6NggUev

ep+eaVvAi8V/YosYh0Xg4/dScOq391irO6k9+BPqkQPH+bRNCUBgnhXGkfElAAUR

tBtHaWRlb24gWmVueiA8Z3plbnpAaXhjLm5ldD6JAJUDBRAzJLB9QGCeFcaR8SUB

AYlyA/9vod8ZjMXVMG9dua5vHOS88m2dfg81A/c8X3u+PCoGmRS5i/V6zFKBu+43

zBtjCw6g1bVwMMjEz+wBcGswfFflwV/5q7jqGi/ABBP1SP23jttBFVKoco8ZWNRQ

Q/mNV8RDLqPzjZAnmEA0dCO+ZkBHR3Bo8nfmeon6HjXgpjuGrbQ6Tk9URTogPGd6

ZW56QEVybmllLk1JLlVuaS1Lb2Vsbi5ERT4gaXMgbm90IHZhbGlkIGFueSBtb3Jl

IYkAlQMFEDMksKpAYJ4VxpHxJQEBnykD/3JcoNyUEjHH8GIp3E1IMGBo0LmHIfII

LaJkrT8XbM+6+6qSEIhAJ3MwUuwksWyVqaGNUMVlUpG2lIDe9uVaK+kzmfrK3iTG

L5syQrctbaqvgIFbDdKuJKFPUGAaG4ttXhhxU0H88h4A4nhnza0vEtDO7Aqxg7E+

QZJL+ZpLE4H4tClHaWRlb24gWmVueiA8Z3plbnpARXJuaWUuTUkuVW5pLUtvZWxu

LkRFPokAlQIFEDKyDmW2+Q3cu22vgQEBUjgEAIb3vZjKei10f0wsNWhlnJ0jo0wR

gX/+9/9tOf9AM30Nxb1V9Mc2FtywFuzhU6u+pf0PDDZ3/3W7f4dhoDdyvPTSY3m5

WOQIT0Uwfe9A3KSsqYEydqMOmpjQT10kDJi7gLw1pgLPKkR4DqgbOuWVNNr772HZ

WgYJNWtwIGDh9r0xiQEVAwUQMfa2uQBgAoLzpE8JAQGBNQf8DwpNz/00qL+ZvOQ8

ZTLLAjeSun8eoYMBBU3z1AXKerk+1NK4rLlDgMCYnBDiCdr0YlDK2Z7VhtpcqM03

EyLZvfdGF31qPkcpUfHptGC4nsumACi3PoW4jo3LvpXVxzBBCdvGujL4Qo5Ha46c

6H8avg1tXPVBNWqSxjKn9PmvAjV6yAmVMrGjDpj0EC63ILgzef1KdzqxWIZDBmwf

1fG2z1z2Z4jstGOV5aNIVyXsu5lXJ/osUW7Oj2NBBO34CTr3AJ43hCrmhPMslBr0

rOBBrGPvSg/0vJM8GR1g9Ydr4lg8LGDIGiNjyKCrs+ZJGIyckqFaRDH65FBuem4M

c+C+BIkAlQMFEDHR1zdAYJ4VxpHxJQEByIMD/ippEW/kpkaUhsB4amGVHff5oGMx

kN/UyULVBwiTbI9j21MwiKzPTohyRnDs6r5FUGYk685m0kkm5UK65/IcpKDf4dGP

pgTiVDv+iMJTvI9yt1+fLigsoSl6FEH8Xz0zNqftyPIsj2ofTsjbCrtr3ALA/gEO

lrSXEwBB31ST5Q3q

=JxHF

- -----END PGP PUBLIC KEY BLOCK-----

-----BEGIN PGP SIGNATURE-----

Version: 2.6.3ia

Charset: latin1

iQCVAwUBM9J6D0BgnhXGkfElAQELRwP/R+h/N2X9jliCdVAqebe38v1kj3Up21QF

FAgFvV83PuEGO8usF8/5EkTLrpVfy23AWfFRCKih9VmEc7nq6h78ytGuN1POzJOS

KP1tnzt4h8H6qSXPyIbYRDtMtrQ/tkX208FhFiR5AwTIjGRUD2qgbi7vHUSY9kKh

+NYvSDubPVo=

=3o72

-----END PGP SIGNATURE-----