+-----------------------------------------------------------------------+

| Vision Technoolgies Viaion VGA Video Utility			06/07/90|

+-----------------------------------------------------------------------+

VIDUTIL is a small (1K) memory resident program which allows the user

to change the video characteristics "on-the-fly."  This utility allows

the user to enable and disable flicker filter, underscan, and video

as well as change the horizontal and vertical centering while running

another program.

Normally a user can set these options using VTMODE.  However, users

may wish to change the video characteristics while running another

application program.  To install VIDUTIL, type:

VIDUTIL [ENTER]

To activate VIDUTIL, the user must simultaneously hold down the

[ALT][CTRL] and [N] keys.  VIDUTIL will now be active, and the

application previously running will be inactive.  Note that VIDUTIL

DOES NOT display any type of popup window.  In the interest of

keeping things simple for users wishing to modify VIDUTIL.ASM,

VIDUTIL does not display a help window of any sort.

While VIDUTIL is active, the following keys can be used to change

the video characteristics:

[UpArrow] - Move screen up approximately 8 scan lines

[DnArrow] - Move screen down approximately 8 scan lines

[LeftArrow] - Move screen left approximately 8 pixels

[RightArrow] - Move screen right approximately 8 pixels

[U] - Underscan enable/disable toggle

[F] - Flicker filter enable/disable toggle

[V] - Video output enable/disable toggle

[0] - Return screen back to default screen centering

[ESC] - Exit VIDUTIL and return to current application

When the user is finished using VIDUTIL, they should press [ESC]

once to resume the application they were using before activating

VIDUTIL.

Users who are knowledgeable in Intel 8086 assembly language may wish

to examine the file VIDUTIL.ASM to see how VIDUTIL works.

-------------------------------------------------------------------------

-------------------------------------------------------------------------