Everex Extended VGA BIOS Functions Quick Reference Sheet

---------------------------------------------------------------

* Extended Set Mode Function *

Entry:	AH = 00h	;Set Mode Function

AL = 70h	;Everex Extended Set Mode

b7 = 1 = Don't clear video buffer during Set Mode

BL = Mode Number

Exit:	None

- Mode Table -

Grfx

Modes	Text	Font	Gender	Type	  Colors   Planes  Res	  Segment

-----	-----	----	------	----	  ------   ------  -----  -------

x02	100x42	8x14	Color	Grfx16	  16	     4	  800x600   A000

x03	80x34	8x14	Color	Text	  16	     1		    B800

x04	80x60	8x 8	Color	Text	  16	     1		    B800

x07	100x43	8x14	Color	Text	  16	     1		    B800

x08	100x75	8x 8	Color	Text	  16	     1		    B800

x0A	132x25	8x14	Color	Text	  16	     1		    B800

x0B	132x44	8x 8	Color	Text	  16	     1		    B800

x0E	132x25	8x14	Mono	Text		     1		    B000

x0F	132x44	8x 8	Mono	Text		     1		    B000

x14	80x25	8x16	Color	Grfx256	  256	     1	  640x400   A000

x15	64x30	8x16	Color	Grfx256   256	     1	  512x480   A000

x16	80x30	8x16	Color	Text	  16	     1		    B800

x18	100x37	8x16	Color	Text	  16	     1		    B800

x20	128x48	8x16	Color	Grfx16	  16	     4   1024x768   A000

x21	160x64	8x16	Color	Grfx16	  16	     4   1280x1024  A000

x23	208x75	8x16	Color	Grfx16	  16	     4	 1664x1200  A000

x30	80x30	8x16	Color	Grfx256	  256	     1	  640x480   A000

x31	100x37	8x16	Color	Grfx256	  256	     1	  800x600   A000

x32	128x48	8x16	Color	Grfx256	  256	     1   1024x768   A000

x40	132x30	8x16	Color	Text	  16	     1		    B800

x50	132x30	8x16	Mono	Text		     1		    B000

(1)x62	40x25	8x8	Color	16Bit	  32K	     1	  320x200   A000

(1)x63	100x37	8x16	Color	16Bit	  32K	     1	  800x600   A000

(1)x70	64x30	8x16	Color	16Bit	  32K	     1	  512x480   A000

(1)x71	80x30	8x16	Color	16Bit	  32K	     1    640x480   A000

(1)x72	90x30	8x16	Color	16Bit	  32K	     1    720x480   A000

(1)x73	96x30	8x16	Color	16Bit	  32K	     1    768x480   A000

(1)x74	90x35	8x16	Color	16Bit	  32K	     1    720x574   A000

(1)x76	64x30   8x16    Color   24Bit     16M        1    512x480   A000

(1)x77  80x30   8x16	Color	24Bit	  16M	     1	  640x480   A000

Note(1)	: These mode are on supported on Everex TC and HC VGA cards

--------------------------------------------------------------------------

* Return Everex VGA State Info *

Entry:	AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0000h	;Return Everex VGA State Info

Exit:	AL = 70h	;Everex VGA is present/function successful

CL = monitor type

00 = monochrome

01 = CGA

02 = EGA

03 = digital (TTL) multi-frequency

04 = IBM PS/2 or compatible (8503, 8512, 8513)

05 = IBM 8514 or compatible

06 = super VGA (NEC 2A or similar)

07 = analog multi-frequency (NEC 3D or similar)

08 = analog super multi-frequency (NEC 4D or similar)

CH = emulation status

b0 = 1 = CGA/HGC emulation enabled

b1 = 1 = emulation automode enabled	(EVGA only)

b2 = 1 = EverNMI (clobber) enabled		(EVGA only)

b3 = 1 = VGA emulation enabled		(EVGA only)

b4 = 1 = VGA register protect enabled

b5 = 1 = optional special oscillator present

b6..b7 = memory size (256K/512K/1024K/2048K)

DX = board model (BCD number)

b15..b4: model number

b3..b0 : revision code

DI = BIOS version (BCD number)

--------------------------------------------------------------------------

* Everex VGA CGA/HGC Emulation *

Entry:	AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0001h	;Everex VGA Emulation

CH = Enable/Disable emulation function

00 = Disable 6845 emulation

01 = Enable  6845 emulation

Exit:	AL = 70h	;Everex VGA is present/function successful

--------------------------------------------------------------------------

* Everex VGA Register Protect *

Entry:	AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0002h	;Everex VGA Register Protect

CH = Enable/Disable protection function

00 = Disable VGA register protection

01 = Enable  VGA register protection

Exit:	AL = 70h	;Everex VGA is present/function successful

--------------------------------------------------------------------------

* Everex VGA Fast Mode *

Entry:	AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0003h	;Everex VGA Fast Mode

CH = Enable/Disable protection function

00 = Disable Fast Mode

01 = Enable  Fast Mode

Exit:	AL = 70h	;Everex VGA is present/function successful

--------------------------------------------------------------------------

* Everex VGA Get Paging Function Pointer *

Entry:	AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0004h	;Everex VGA Get Paging Function Pointer

Exit:	AL = 70h	;Everex VGA is present/function successful

ES:DI = FAR pointer to a function which maps a 64K block of

memory in at A000:0000 for 64K.

ES:[DI-2] = Word that tells the length of the paging function in bytes.

This can be used for copying the function to a local buffer.

ES:DI = Address of function to be used for paging.  RETF (FAR return)

is the last instruction.

DL    = Input parameter for the requested 64K page when calling the

Paging Function (ES:DI)

--------------------------------------------------------------------------

* Everex VGA Get Mode Supported List *

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0005h	;Everex VGA Get Mode Supported List

CL = maximum number of modes to get list of

CH = mode type to get list of

00 = get all mode types

01 = get all mono  text modes

02 = get all color text modes

03 = get all 4-color (CGA) graphics modes

04 = get all 2-color (CGA) graphics modes

05 = get all 16-color (EGA/VGA) graphics modes

06 = get all 256-color (VGA) graphics modes

07 = get all True Color graphics modes

DL = monitor type to get mode list of

(Typically the current monitor)

ES:DI points to a buffer of sufficient size

Exit:   AL = 70h	;Everex VGA is present/function successful

CL = total number of modes found fitting criteria

CH = Size of each mode information structure

ES:DI points to buffer of information returned

The buffer is currently defined to have the following format:

(b) = Mode number (b7 set if it is an extended mode)

(b) = Mode format (same definition as ch above)

(b) = Info bits

b0 = 1 = paged mode

b1..b2 = requires 256K/512K/1024K/2048K

b3 = 1 = requires optional special clock

b4 = 1 = interlaced mode

b5 = 1 = monochrome mode

b6 = 1 = Mode has no BIOS support for writing characters

or dots.

b7 = reserved

(b) = font size

b0..b4 = font height

b5..b6 = reserved

b7 = font width

0 = 8-dot

1 = 9-dot

(b) = Columns on screen (text format)

(b) = Rows on screen (text format)

(w) = Number of scan lines

(b) = Color information

b0..b5 = Bits per pixel

b6..b7 = reserved

-------------------------------------------------------------------------

* Everex VGA Program Mode Parameters *

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0006h	;Everex VGA Program Mode Parameters

ES:DI points to a standard VGA format 64-byte table

DS:DX points to a supplemental register table

(This table is strongly card dependent)

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Return VGA Video Status Info * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 00h	;Return VGA Video Status Info

Exit:   AL = 70h	;Everex VGA is present/function successful

CH = Video Status Info

b0 = 1 = Flicker Filter is enabled

b1 = 1 = Underscan is enabled

b2 = 1 = Video output is enabled

b3..b7 = reserved

CL = reserved

DH = Horizontal Pan value (-128 to 127)

DL = Vertical   Pan value (-128 to 127)

-------------------------------------------------------------------------

* Everex VGA Video Output Control * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 01h	;Everex VGA Video Output Control

CH = Enable/Disable Video Output function

00 = Disable Video Output

01 = Enable  Video Output

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Everex VGA Flicker Filter Control * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 02h	;Everex VGA Flicker Filter Control

CH = Enable/Disable Flicker Filter function

00 = Disable Flicker Filter

01 = Enable  Flicker Filter

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Everex VGA Underscan Control * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 03h	;Everex VGA Underscan Control

CH = Enable/Disable Underscan function

00 = Disable Underscan

01 = Enable  Underscan

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Everex VGA Horizontal Pan Control * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 04h	;Everex VGA Horizontal Pan Control

CH = amount to pan left or right (-128 to 127)

(Typical values range from -4 to 3)

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Everex VGA Vertical Pan Control * (NTSC and PAL boards only)

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0007h	;Everex VGA Video Control Functions

CL = 05h	;Everex VGA Vertical Pan Control

CH = amount to pan up or down (-128 to 127)

(Typical values range from -4 to 3)

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

* Everex VGA Backdoor Set Mode *

Entry:  AX = 7000h	;Everex Extended VGA BIOS Function

BX = 0009h	;Everex VGA Backdoor Set Mode

CH = Set Mode AL parameter

CL = Set Mode BL parameter

Exit:   AL = 70h	;Everex VGA is present/function successful

-------------------------------------------------------------------------

#############################################################################################################################